3GPP TSG-RAN WG4 Meeting #117                                                                    R4-2522003

Dallas, US,  November 17 - 21, 2025

Source:		Ericsson

Title:			A-IoT BS conformance remaining issue

Agenda item:		6.14.2

Document for:		Approval

# Introduction

In this paper, we present our views on remaining issue for Rel-19 A-IoT conformance testing.

# Discussion

In WF[1], FFS on the test of FDM operation on D2R and if to be tested, whether SC or MC to be used.

### Issue 2-5: A-IoT test configuration

- Tentative Agreement:

  - Use the single carrier operation for A-IoT BS R2D conformance testing

  - FFS whether and how to test FDM operation for different devices with different R under same CW frequency operation case if needed

  - FFS whether use the single or multi carrier operation for A-IoT BS D2R conformance testing

To study this, the baseband PSD figure is simulated and illustrated in Figure 1 using the waveform defined in 38.294. It can be observed that the spectrum regrowth outside the wanted signal received bandwidth (2/Tb) is substantial and noise contribution (side lobs from other R) to the wanted signal main lobe quite high (SNR of main lobe signal of R=1 equals to 2.4 dB counting the noise contribution from side lobe of signal of R=2). With bigger separation of different R, the SNR due to side lobe interference can be improved, e.g SNR of main lobe of signal of R=1 is 13.6 dB. It is not considered in the core requirement for the impact of the side lobe interference on the SNR for the FDM operation of D2R, therefore, we think there is no need to consider it in the conformance test.

The side lobe of D2R signal of bigger R interfere with the main lobe of D2R signal of smaller R

SNR due to side lobe interference can be improved with larger separation of FDM D2R signal

No interference analysis done in core requirement for FDM operation in core requirement

No need to consider the FDM operation test in conformance test.

Figure 1: PSD of R=1, 2 with same Tb

Figure 2: PSD of R=1, 4 with same Tb

For the test configuration, it is used to reduce the test load for different permutation of the contiguous/non-contiguous allocation, MC and multiband. As only single carrier test is applied to uplink, there is no need to define the test configuration.

For the test configurations for the MC operation

| BS test case |  |
| --- | --- |
| 6.2 Base station output power | SC |
| 6.3 Transmit ON/OFF power | SC |
| 6.4 Transmitted signal quality | SC |
| 6.6 Unwanted emissions | SC |
| 7.2 Reference sensitivity level | SC |
| 7.3 In-channel selectivity and blocking | SC |
| 7.4 Out-of-band blocking | SC |
| 7.5 Receiver spurious emissions | SC |
| 7.6 Receiver intermodulation | SC |

### Issue 2-4: Test model

- Agreement：

  - Use the A-FR1-B1-1/2/3/4 configuration for the conformance testing as baseline, FFS whether certain requirements (such as modulation quality etc.) can be tested only under A-FR1-B1-1 to optimize testing efficiency.

For the test model, the A-FR1_B1-1/2/3/4 are basically differentiated with number of RB and M.  The simulation shows the synthesized waveform with the less RB and high M can have more prominent “distortion” and may be subject to further impairment by RF chain and worth to be tested.  We suggest that for modulation test, use the low RB number and high M value combination to test, which is A-FR1-B1-1 (M=6 for postamble) or A-FR1-B1-2 (M=12 for postamble) and only the postamble is tested for modulation signal quality. As the channel bandwidth is declared by manufacturer, at least the 200kHz (RB) can be supported, therefore, it will be safe to use A-FR1-B1-1.  Note in clause 6.2.4,the 4 bits [1 1 1 1 ] is defined for postamble and with Manchester coding, it will be always low to high transition so the bit pattern of [0 1 0 1 0 1 0 1].

The higher M and lower RB size introduce more “distortion” on the synthesized signal

Modulation signal test on the postamble only and use A-FR1-B1-1 to test.

Figure 3: Modulation quality comparison with different RB nubmer and M

For other Tx test, the test model should be selected to fit in the declared channel bandwidth.

For other Tx requirement test, the test model should be selected to fit in the declared channel bandwidth

For the data content of the PRDCH, we suggest to reuse the PN random sequence in TS 38.141-1 as below:

“Randomisation of the data content is obtained by utilizing a PN sequence generator and the length-31 Gold sequence scrambling of TS 38.211 [17], clause 5.2.1”

Reuse the PN random sequence generation in TS 38.141-1 for data content of PRDCH.

In the modulation quality requirement, the E-field strength (V/m) is specified as requirement. E-field strength has been used as a requirement in OTA and the measurement method is OTA test measured in a far field anechoic chamber. In 38.141-2, the RMS filter-strength can be converted to EIRP at certain distance r.

Table 7.6.5.1.1-1: OTA out-of-band blocking performance requirement

| Wanted signal mean power (dBm) | Interfering signal RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- |
| EISminSENS + 6 dB (Note 1) | 0.36 V/m | CW carrier |
| NOTE 1: EISminSENS depends on the channel bandwidth as specified in TS 38.104 [2], clause 10.2.1. NOTE 2: The RMS field-strength level in V/m is related to the interferer EIRP level at a distance described as , where EIRP is in W and r is in m; for example, 0.36 V/m is equivalent to 36 dBm at fixed distance of 30 m. |  |  |

The mathematical relation between the E-field and the received power is explained well in [2]. The power flux density received at effective antenna area is used as an intermediate parameter to link the received power and the E-field strength. As the OTA measurement is needed for the modulation signal test, this may not be considered in the beginning as the BS type 1-C only for conducted test.  The beam related test parameter are declared in BS type 1-H not type 1-C. If OTA test is to be added, the additional beam related declaration is needed and maybe it is better to define the A-IoT BS as type 1-H instead of the 1-C from OTA test perspective.

OTA test for Tx modulation signal needs additional beam related test parameter and such parameters declared in BS type 1-H not type 1-C.

As the modulation signal quality requirement refers to the EPC Gen2 standard due to the reason of the reusing the RFID  design. There are some considerations quoted below according the EPC conformance test [3] and quoted below:

Q10: Testing of Reader modulated RF envelope characteristics and symbol durations are specified in 6.3.1.2.3 and 6.3.1.2.5 of the conformance document. These parameters are determined based on A and B measurements as shown in Figure 6.2 of the Gen2 protocol specification. In test, how is A determined?

A: In 6.3.1.2.5 of the Gen2 protocol specification, A is referred to as the maximum amplitude of the RF envelope. In Figure 6.2, A is shown as the midpoint between the maximum and minimum ripple excursions. The ripple represents inter-symbol interference associated with the band-limiting of the transmit symbols. Inter-symbol interference can case the RF envelope to exceed the maximum amplitude of an un-modulated signal with the same power. For consistency with the Gen2 protocol specification, the value of A in 6.3.1.2.3 and 6.3.1.2.5 shall be determined by measuring the unmodulated envelope immediately preceding modulation from the first Reader command issued after the end of the settling interval following a power-up. The test facility shall determine the optimal measurement time to establish an accurate estimate of A

In modulation depth requirement in 38.194, it is defined as average value measured during ½chip duration. . There is no information on what type of detector should be used in test case. As we discuss below, the E-field test may be related to the conducted power so OTA test can be skipped. In this sense, RMS detector should be used to align with the output power test case in BS (e.g output power, dynamic range etc). In identification card test[4], the maximum magnetic field strength is tested with RMS detector over the RBW.

RMS detector should be used in [4] and TS 38.141-2.

The E-field can be derived from the power measurement and as the modulation signal quality requirement are relative so it is possible to using the output power measurement instead of the E-field and mathematically it is illustrated in [2].

Assuming the E-field measurement is measured with a certain distance to DUT (BS). The relation between E field and transmitted conducted power P can be expressed below.

Assume the power density of the incident radio wave is in a distance r to BS and power flex density S is:

S=Pt∙Gt4πr2 						(1)

Where the Pt is conducted power of BS and Gt is the transmitter antenna gain.

The power flux density is the product of electric and magnetic field strength:

S=E∙H=E2Z0

Where Z0 is the characteristic impedance of free space Z0~=377Ω

E=Z0∙S=Z0∙S=Z0∙Pt∙Gt4πr2=α∙Pt				(2)

So E field can be derived from the conducted power and if other parameters (transmitter antenna gain, distance r to BS) are fixed, E is proportional to the square root of the conducted power with a scaling factor of α .

Using this formular the modulation quality requirement can be derived from the conducted power using equation (2) as below:

For ripple requirement:

Ripplehigh%= An-An_avgAn_avg-Bn_avg∙100%=αPAn-αPAnαPAn-αPBn∙100%=PAn-PAnPAn-PBn∙100%

Ripplelow%= Bn-Bn_avgAn_avg-Bn_avg∙100%=PBn-PBnPAn-PBn∙100%

For modulation depth:

Modulation depth= An_avg-Bn_avgAn_avg=PAn-PBnPAn

For the envelop rising/falling time:

X%= CnAn_avg-Bn_avg=PCnPAn-PBn

In above equations PAn is the RMS average power measured over 1/2 duration of the high level of n-th chip, PBn is the RMS average power measured over 1/2 duration of the low level of m-th chip. PCn is the power measured between low and high level of a chip. PAn is the power measured of the high level of n-th chip. PBn is the power measured of the low level of m-th chip.

As the OTA test on modulation test needs further consensus on test method and declared parameters, we suggest RAN4 discuss further:

Issue 1 : test method for modulation signal quality in general

- OTA test: Define the type 1-H instead of type 1-C so the beam related test parameters can be declared for OTA test, additional EIPR test may be needed to complete the modulation quality test

- Conducted test: confirm the equations between E-field to core requirement and OTA test can be avoided

Issue 2: clarification of the An/Bn/Anavg/Bnavg

- RMS detector  is used and align with [4] and 38.141-2 (OTA out of blocking)

Discuss above issues for Tx modulation signal test.

# Conclusions

In this contribution, we present our overview on the A-IoT conformance testing with below observation and proposals:

Observation 1 The side lobe of D2R signal of bigger R interfere with the main lobe of D2R signal of smaller R

Observation 2 SNR due to side lobe interference can be improved with larger separation of FDM D2R signal

Observation 3 No interference analysis done in core requirement for FDM operation in core requirement

Proposal-1: No need to consider the FDM operation test in conformance test.

Observation 4 The higher M and lower RB size introduce more “distortion” on the synthesized signal

Proposal-2: Modulation signal test on the postamble only and use A-FR1-B1-1 to test.

Proposal-3:For other Tx requirement test, the test model should be selected to fit in the declared channel bandwidth

Proposal-4:Reuse the PN random sequence generation in TS 38.141-1 for data content of PRDCH.

Observation 5 OTA test for Tx modulation signal needs additional beam related test parameter and such parameters declared in BS type 1-H not type 1-C.

Observation 6 RMS detector should be used in [4] and TS 38.141-2.

Proposal-5: Discuss above issues for Tx modulation signal test.

# References

- R4-2515058, WF for [116bis][314] A-IoT_BSCW, Huawei

- https://scdn.rohde-schwarz.com/ur/pws/dl_downloads/dl_application/application_notes/1ma85/1MA85_7e_Estimator.pdf

- EPC Compliant Class-1 Generation-2 UHF RFID Devices Conformance Requirements

- ISO/IEC FCD 10373-6; Identification cards — Test methods — Part 6: Proximity cards