3GPP TSG-RAN WG4 Meeting #117 	R4-2522018

Dallas, TX, USA, November 17th – 21st, 2025

Agenda Item:	6.18.2

Source: 	Ericsson

Title:	On RRM performance issues for XR

Document for:	Approval

# Introduction

In RAN4#116bis, the following agreements [4] were made for the performance part of the RAN4 XR work:

Issue 1-1-1: Scenarios for the definition of the test cases

- At least for measurement procedure RAN4 RRM to define test cases for NR-SA for both FR1 and FR2-1

  - For FR1 TCs,

    - all cells are in FR1

  - For FR2 TCs

    - all cells in FR2-1

  - FFS if all TCs also cover FR2-1

- Test cases are specified only for per-UE MGs

Issue 1-1-2: Type 1 and Type 2 gaps

- Test cases are defined only for Type 1 gaps

Issue 1-1-3: DRX proposals

- Each type of test case is specified for either DRX or non-DRX but not both.

Issue 1-1-4: Test target

- Proposals 1, 2, and 3 are addressed in the same test case

  - Proposal 1: measurement delay extension with gap cancellation

  - Proposal 2: Reception/transmission without scheduling restrictions during cancelled gap occasions

  - Proposal 3: The UE processing timeline of receiving the DCI-based MG skipping indication

Issue 1-1-5: List of test cases

- Define test case for:

  - Inter-frequency measurements procedure with gaps

  - Define test case for: Inter-frequency L1-RSRP measurements procedure with MG for FR1 and FR2-1

  - FFS whether to define at least one accuracy test case in FR1 and in FR2-1

# Discussion: Further details for RRM test cases for UE under XR operation

Even if there are the same accuracy requirements with XR as in legacy, there is still a need to verify whether accuracy can be met in the new XR scenarios since the measurement period is defined differently for XR and the time between samples can become very non-uniform.

- Proposal 1: Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1.

- Proposal 2: The following test cases for measurement accuracy for XR can be specified:

Table 1: Candidate test cases for measurement accuracy (NR-SA) under XR operation

|  | TC title | FR (FR1, FR2-1) | DRX or non-DRX |
| --- | --- | --- | --- |
| Inter-frequency with gaps (SS-RSRP) | A.6.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | All cells in FR1 | Non-DRX |
|  | A.7.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell | All cells in FR2-1 | Non-DRX |
| Inter-frequency with gaps (SS-RSRQ) | A.6.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | All cells in FR1 | Non-DRX |
|  | A.7.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell | All cells in FR2-1 | Non-DRX |

On the time between DCI and MG start, there is no need for different configurations in the test cases, depending on the UE capability (UEs supporting 5 ms and 3 ms). Furthermore, the 3 ms capability is optional.

- Proposal 3: In all XR test cases, the DCI indication is configured 5 ms or more before the start of the measurement gap occasion.

# Summary

- Proposal 1: Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1.

- Proposal 2: The following test cases for measurement accuracy for XR can be specified:

Table 1: Candidate test cases for measurement accuracy (NR-SA) under XR operation

|  | TC title | FR (FR1, FR2-1) | DRX or non-DRX |
| --- | --- | --- | --- |
| Inter-frequency with gaps (SS-RSRP) | A.6.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | All cells in FR1 | Non-DRX |
|  | A.7.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell | All cells in FR2-1 | Non-DRX |
| Inter-frequency with gaps (SS-RSRQ) | A.6.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | All cells in FR1 | Non-DRX |
|  | A.7.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell | All cells in FR2-1 | Non-DRX |

- Proposal 3: In all XR test cases, the DCI indication is configured 5 ms or more before the start of the measurement gap occasion.

# References

[1] RP-240791, WID on XR (eXtended Reality) for NR Phase 3, March 2024.

[2] R4-2512144, WF on RRM requirements for NR_XR_Ph3, Nokia, Aug. 2025.

[3] R4-2504911, WF on RRM requirements for XR_Ph3, Nokia, Apr. 2025.

[4] R4-2514869, WF on NR_XR_Ph3_RRM, Nokia, Oct. 2025.