3GPP TSG-RAN WG4 Meeting #117	R4-2522019

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	8.10

Source:	Ericsson

Title:	On radio-based sensing in 6G radio communications networks

Document for:	Approval

# Introduction

On sensing, a.k.a. ISAC (Integrated Sensing and Communication), there is currently an on-going SI for NR [2], focusing primarily on UAVs as sensing targets and mono-static sensing by means of gNBs.

For 6GR, a SI [1] has started, while discussion contributions on sensing specifically are not expected until RAN1#124b (April, 2026).

In RAN4#116bis, sensing was discussed for the first time in RAN4, with the following high-level agreements [4]:

Before April 2026, the agenda for sensing will be kept. RAN4 discussion will primarily focus on the following aspect

- Sensing related regulatory status and requirement survey

- Potential architecture consideration based on the RAN plenary use case study

- views sharing on the potential RAN4 scope based on the identified use cases out of RAN plenary study

- Identify less RAN1 design dependent RAN4 aspects if any

# Discussion

## General

ISAC aims to enable radio-signals based sensing in cellular networks used for radio communications. There are different integration levels for sensing and radio communications, e.g., sharing RF spectrum, sharing infrastructure and sites, sharing radio resources in time and frequency, sharing protocols, using the same type of radio signals or even the same radio signal configuration, etc. Many of these aspects are still to be discussed and agreed upon in other groups, the entire ISAC concept within 3GPP for 6G is still a bit open, and the 6G ISAC scope is expected to be larger than that of the 5G ISAC.

However, one can already note a few aspects which need to be taken into account during the ISAC concept design and requirements discussions, e.g.:

- There are some fundamental differences between the traditional radar technology and ISAC, including the characteristics of transmitters and receivers, spectrum, deployment strategies, etc. For example, the latter is going to share the spectrum with radio communications networks, can potentially involve non-collocated transmitters and receivers, including UEs assisting in sensing by transmitting or receiving radio signals for sensing, etc.;

- There is no unique definition of the sensing transmitting and receiving entities, neither the concept of DL or UL sensing: sensing transmitters and receivers can be the same node or different, any of these can be a BS and/or UE, a BS may need to receive a radio signal for sensing from the same or different BS which is normally transmitted in DL resources, etc.;

- The radio signal level and quality of reflected radio signals can be very low, at least in some deployments;

- The line-of-sight probability may be very low, at least in some deployments;

- As a starting point, sensing targets (objects with respect to which a sensing task is to be performed) are passive, i.e., not equipped with antennas and cannot actively transmit/receive radio signals or perform radio  measurements – which is a big difference, e.g., from positioning targets which are typically UEs; that is, a sensing target may be neither a transmitter nor a receiver of any radio signals;

- The size of a sensing target matters more, e.g., compared to positioning, and impacts directly the sensing result as well as the result representation, e.g., in positioning a location result is associated with an antenna location and at the same time the UE size is relatively small, while in sensing a sensing target can be rather big and it can be important which part/side of the sensing target the sensing result is associated to (e.g., the front or the back part of a vehicle, or the top or the bottom part of it);

- All the different use cases can be decomposed into one or more simpler sensing tasks, e.g., detection, characterization, localization, tracking, etc., regardless of the environment, application, type of sensing target, etc. (NOTE: the list of use cases can be endless, while the list of sensing tasks is rather limited, and the latter are easier to analyse);

- Sensing result is a term which can include many things and different types of information at very different levels of details, depending on a sensing task, e.g., the form of a sensing result for such sensing tasks as object presence/absence detection, movement detection, object characterization, velocity determination, passive object localization, object tracking, etc. can vary from a simple “yes/no” or “detected/not detected” to a detailed trajectory description of a moving sensing target for high-accuracy collision avoidance;

- A variety of sensing targets is big, e.g., they can be ground-level objects or UAVs (NOTE: during SID for Rel-19 ISAC channel modelling SI five types of sensing targets were prioritized: UAV, vehicle, human, AGV, and road hazards).

All the above bullets indicate that the potential set of the ISAC scenarios can be huge, therefore it is important that the ISAC scope is more clearly defined by RAN/RAN1 before RAN4 starts looking into more specific issues.

## RRM

ISAC will be relying primarily on reflected signals, which will most likely to be received at low SINRs, hence regardless of whether ISAC will use new or the existing 6G measurements, RAN4 may need to ensure good measurements performance at lower SINR levels compared to those observed for communication.

- Observation 1 (RRM): The reflected signals in ISAC may have lower SINRs than usually considered for other RRM measurements.

Some new radio measurements can be expected for ISAC. Depending on the agreed ISAC topologies, the measurements will be performed by radio network nodes and/or UEs. For the measurements reported to another node, RAN4 may need to specify, as usual, measurement and accuracy requirements.

- Observation 2 (RRM): Some ISAC measurement and accuracy requirements may need to be specified by RAN4, since new sensing-specific measurements may be introduced.

A range of possible ISAC deployment scenarios is being discussed for 6G, including those where a UE can be involved in ISAC in different ways, e.g., a UE could assist in ISAC by transmitting radio signals for sensing only, could receive radio signals for sensing and perform ISAC measurements, could do both, or could even transmit and receive reflected radio signals transmitted by itself. Not all ISAC deployment scenarios will have the same RAN4 specification impact.

- Observation 3 (RRM): Not all ISAC deployment scenarios will have the same RAN4 specification impact.

Given that sensing target may be neither a transmitter nor a receiver of any radio signals in an ISAC deployment, the legacy simulation setups and legacy (6G) test cases can be more difficult to reuse as such for ISAC.

- Observation 4 (RRM): Legacy (6G) simulation setups and test cases can be more difficult to reuse as such for ISAC, e.g., because:

  - a sensing target can be neither a transmitter nor a receiver of a radio signal(s) for sensing,

  - more entities are to be modelled in ISAC simulations and test cases (e.g., at least one transmitter, one receiver, and one passive object can be envisioned).

- Observation 5 (RRM): Since the legacy simulation setups cannot be just directly reused as such and a lot of simulator development work may potentially be needed, it is important to identify the scenarios and the simulations needed earlier on.

ISAC implies that the sensing feature is integrated into the radio communications network, hence the ISAC studies and requirements must be based on the assumptions relevant for general radio communications network deployments and an acceptable trade-off level between resources needed for radio communications services and resources needed to support sensing.

- Observation 6 (RRM): RAN4 ISAC studies and requirements must be based on the assumptions, which are relevant for general radio communications network deployment scenarios and an acceptable trade-off level between resources needed for radio communications services and resources needed to support sensing.

3GPP SA2 is considering to include a task for 6G sensing on sensing continuity in consideration of mobility of UE sensing entities and/or target objects. The sensing continuity consideration may potentially also impact the RAN4 assumptions and the requirements for sensing.

- Observation 7 (RRM): The sensing continuity consideration may potentially also impact the RAN4 assumptions, the requirements, and the measurement procedure for sensing.

And as usual, the UE resources will be shared for different measurement types, including sensing measurements, hence RAN4 shall discuss not only the requirements for RRM sensing measurements but also the requirements ensuring the general RRM performance for the UE performing sensing measurements and/or transmitting radio signals for sensing.

- Observation 8 (RRM): RAN4 needs to discuss not only the requirements for RRM sensing measurements but also the requirements ensuring the general RRM performance for the UE performing sensing measurements and/or transmitting radio signals for sensing.

## Co-existence & RF

ISAC introduces new co-existence challenges because sensing requires detection of weak reflected signals while operating in environments with strong neighboring high power transmissions. From a RAN4 perspective, a BS receiver is impacted both by emissions from nearby operators’ sites and by its own limited selectivity, especially in tower-sharing deployments where coupling loss is low.

- When other operators’ BSs are in transmit mode (e.g., downlink), ISAC receivers are vulnerable to interference from neighboring BSs transmitting at high power, as the reflected signal power is orders of magnitude weaker than communication signals. Similarly, when ISAC transmits sensing signals in downlink, it is important to assess the impact of these emissions on neighboring BS receivers.

- When other operators’ BSs are in receive mode (e.g., uplink) and an ISAC transmitter transmits UL sensing signals, it becomes essential to study the allowable ISAC emission levels and transmit power to ensure that neighbouring uplink reception is not degraded. Similarly when ISAC is in listen mode for echoes during UL, interference is from UEs and there is a need to study UE emission limits.

Under the above conditions, there may be a need to perform co-existence studies in RAN4 using an updated co-existence framework to ensure conventional communication services are protected while also safeguarding ISAC reception against strong communication signals.

- Observation 9 (Co-existence & RF): RAN4 needs to establish a co-existence framework to study and define upon the need appropriate emission and reception requirements for ISAC operation.

The legacy RAN4 co-existence protected communication links against BS/ UE interference using throughput-based performance metric. However, it is important to define ISAC performance metrics and their relation to SNR under interference as a pre-requisite to make ISAC work in a general deployment.

- Observation 10 (Co-existence & RF): The co-existence framework should specify what counts as “performance loss” for 6G sensing and how to define it.

In the latest WF [4], it was proposed to identify topics which are less RAN1 dependent. However, in our view and based on all the above observations suggest that the potential ISAC system space is very broad, encompassing multiple deployment models, frequency arrangements, and operational modes across both uplink and downlink. Therefore, it is important that the overall ISAC scope and key operational assumptions are first clarified and agreed in RAN/ RAN1 (e.g., definition of sensing modes, time/frequency alignment with communications, use-cases and so on) before detailed discussions progress in RAN4.

- Observation 11 (Co-existence & RF): The ISAC scope and key operational assumptions in context to any discussion on Co-existence and RF requirements are in general RAN1 dependent.

When it comes to regulatory aspects, these are limited to the current primary allocations and whether they can be used for the different types of ISAC targets. For example, one frequency band may be more suitable for sensing flying objects than other frequency bands and/or for other sensing targets. Another aspect to consider is the co-existence with other services, if the current network deployment assumptions would change based on the sensing target and hence would possibly introduce more restrictive requirements on the current communication services.

- Observation 12 (Regulatory): Regulatory aspects are limited to the current primary allocations and usage for the different ISAC targeted bands based on sensing targets.

- Proposal 1: RAN4 should await detailed discussions on 6G ISAC until the fundamental concepts are first clarified and agreed in RAN and RAN1.

# .Summary

- Observation 1 (RRM): The reflected signals in ISAC may have lower SINRs than usually considered for other RRM measurements.

- Observation 2 (RRM): Some ISAC measurement and accuracy requirements may need to be specified by RAN4, since new sensing-specific measurements may be introduced.

- Observation 3 (RRM): Not all ISAC deployment scenarios will have the same RAN4 specification impact.

- Observation 4 (RRM): Legacy (6G) simulation setups and test cases can be more difficult to reuse as such for ISAC, e.g., because:

  - a sensing target can be neither a transmitter nor a receiver of a radio signal(s) for sensing,

  - more entities are to be modelled in ISAC simulations and test cases (e.g., at least one transmitter, one receiver, and one passive object can be envisioned).

- Observation 5 (RRM): Since the legacy simulation setups cannot be just directly reused as such and a lot of simulator development work may potentially be needed, it is important to identify the scenarios and the simulations needed earlier on.

- Observation 6 (RRM): RAN4 ISAC studies and requirements must be based on the assumptions, which are relevant for general radio communications network deployment scenarios and an acceptable trade-off level between resources needed for radio communications services and resources needed to support sensing.

- Observation 7 (RRM): The sensing continuity consideration may potentially also impact the RAN4 assumptions, the requirements, and the measurement procedure for sensing.

- Observation 8 (RRM): RAN4 needs to discuss not only the requirements for RRM sensing measurements but also the requirements ensuring the general RRM performance for the UE performing sensing measurements and/or transmitting radio signals for sensing.

- Observation 9 (Co-existence & RF): RAN4 needs to establish a co-existence framework to study and define upon the need appropriate emission and reception requirements for ISAC operation.

- Observation 10 (Co-existence & RF): The co-existence framework should specify what counts as “performance loss” for 6G sensing and how to define it.

- Observation 11 (Co-existence & RF): The ISAC scope and key operational assumptions in context to any discussion on Co-existence and RF requirements are in general RAN1 dependent.

- Observation 12 (Regulatory): Regulatory aspects are limited to the current primary allocations and usage for the different ISAC targeted bands based on sensing targets.

- Proposal 1: RAN4 should await detailed discussions on 6G ISAC until the fundamental concepts are first clarified and agreed in RAN and RAN1.

# References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator), June 2025.

[2] RP-251861, New SID: Study on Integrated Sensing and Communication (ISAC) for NR, AT&T, June 2025.

[3] R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 Chair, August 2025.

[4] R4-2514592, WF on 6G sensing, ZTE Corp., Oct. 2025.