3GPP TSG-RAN WG4 Meeting #117                                                           R4-2522022

Dallas, USA, 17 – 21 November, 2025

Source:	Samsung

Title:	Discussion on RRM for 6GR

Agenda item:			8.6

Document for:	Discussion

- Introduction

In RAN#108, RAN plenary approved a new RAN working group level SID on 6G Radio [1], which has been further revised and approved in [2] in RAN#109. This study item is to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements. Thus, it involves all 3GPP working groups and multiple aspects and areas to for a foundational of 6G system in future decades. Obviously, the importance of RAN4 including RRM cannot be overstated. It is the cornerstone that it ensures the 6GR a robust, efficient, and interoperable reality. In last RAN4#116bis meeting, RAN4 started the discussion on RRM for 6GR, the latest agreements are captured in [3].

In this contribution, we would like to further provide our viewpoints on RAN4 RRM for 6GR.

- Discussion on RRM for 6GR

In the RRM for 6GR study, based on latest WF [3] and previous Chair’s guidance in [4], it can be categorized as follows:

- Measurement gap(MG) and interruption

  - Including MG and interruption related proposals

- RRM framework: Measurement capability/delay/overhead/accuracy/quantity/unified-measurement

- Mobility related RRM

- RRM related energy efficiency

- Spectrum aggregation related RRM

- MIMO and mTRP operation related RRM

- NTN related RRM

- Initial access related RRM

- Other PHY signal/channel/procedure related RRM

In the subsequent sections, we will analyze each category in detail.

  - Measurement gap(MG) and interruption

The key agreement is last meeting is RAN4 agreed the measurement gap will be part of RAN4 RRM 6G study:

Issue 4-1: MG related scope Agreement in main session (Tue): Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion.

While the study of 6G measurement gaps admits so many subtopics, our discussion proceeds with an elaboration of two major categories:

- MG pattern reduction from 5G

- Gap-less measurement and its side conditions

    - MG pattern reduction from 5G

Measurement gap was used from 4G LTE system. In LTE, the measurement gap design is much simpler than that in 5G system. Before discussion measurement gap, it is crucial to redefine the concepts of intra-frequency and inter-frequency measurement.

In LTE framework, the primary synchronization signal and secondary synchronization signa are always at the center of bandwidth. Thus, the definition of Intra-frequency measurement and Inter-frequency measurement in LTE is shown as:

- Intra-frequency neighbour (cell) measurements: Neighbour cell measurements performed by the UE are intra-frequency measurements when the current and target cell operates on the same carrier frequency. - Inter-frequency neighbour (cell) measurements: Neighbour cell measurements performed by the UE are inter-frequency measurements when the neighbour cell operates on a different carrier frequency, compared to the current cell.

On the other hand, PSS/SSS in LTE are with fixed 5ms periodicity. This also led to few periodic gap patterns such as 6ms gap every 40ms is the most typical used in real deployments.

Therefore, in LTE, number of gap patterns are fewer than that in 5GNR. And for Intra-frequency measurement, no gap is needed. For Inter-frequency measurement, gap is needed.

However, in 5GNR framework, the flexibility of SSB design complicates the design of Measurement Gap both in time domain and frequency domain.

In 5GNR, the SSB are quite different from LTE. In frequency domain, it is more flexible. It led to the different Intra-frequency and Inter-frequency definition. Take SSB as an example:

Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows: - SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the SSB frequency configured in the measurement object associated with the serving cell and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same. - SSB based inter-frequency measurement: a measurement is defined as an SSB based inter-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are different, or the subcarrier spacing of the two SSBs is different.

It caused different cases for intra-frequency and inter-frequency measurement:

- Intra-frequency without MG

- Intra-frequency with MG

- Inter-frequency with MG

- Inter-frequency without MG

In 6GR, the fundamental assumption in RAN1 has evolved.

According to latest RAN1 agreement, the SCS shall be the same in a band:

Agreement 6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band. - FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1. - Note: ISAC is separately discussed in ISAC session.

The assumption of SCS has been changed. The measurement occasion in frequency domain and relationship with BWP may be changed as well. We think it’s the time to re-consider the scenarios and definition of Intra-frequency and Inter-frequency measurement.

Proposal 1: In 6GR, redefine Intra-frequency and Inter-frequency measurement as:

Intra-frequency measurement: The SSB is completely contained in the active [BWP] of the UE (If there is still BWP definition. If no BWP definition in 6G, change to CBW).

Inter-frequency measurement: otherwise.

The motivation for this redefinition is to maintain simplicity principle between intra-frequency measurement, inter-frequency measurement and the measurement gaps as:

- Intra-frequency measurement without MG

- Inter-frequency measurement with MG.

This figure below represents the underlying design principles for MG in 5G:

Observation 1: For RF retuning time, the assumption in 5G is 500us for FR1, and 250us for FR2.

Table 9.1.2-1: Gap Pattern Configurations

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |
| 24 | 10 | 80 |
| 25 | 20 | 160 |

Table 9.1.2-3: Applicability for Gap Pattern Configurations supported by the UE with NR standalone operation (with single carrier, NR CA and NR-DC configuration)

| Measurement gap pattern configuration | Serving cell | Measurement Purpose NOTE 2 | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
|  | FR1 NOTE5, or FR1 + FR2 | non-NR RAT NOTE3,6 | 0,1,2,3 |
|  |  | FR1 and/or FR2 NOTE 8,9 | 0-11, 24, 25 |
|  |  | non-NR RAT and FR1 and/or FR2 NOTE3,6,8,9 | 0, 1, 2, 3, 4, 6, 7, 8,10, 24 |
| Per-UE measurement | FR2 NOTE5 | non-NR RAT only NOTE3,6 | 0,1,2,3 |
| gap |  | FR1 only NOTE 8.9 | 0-11, 24, 25 |
|  |  | FR1 and FR2 NOTE 8,9 | 0-11, 24, 25 |
|  |  | non-NR RAT and FR1 and/or FR2 NOTE3,6,8,9 | 0, 1, 2, 3, 4, 6, 7, 8,10, 24 |
|  |  | FR2 only NOTE 8,9 | 12-25 |
|  | FR1 if configured | non-NR RAT only | 0,1,2,3 |
|  | FR2 if configured | NOTE3,6 | No gap |
|  | FR1 if configured | FR1 only NOTE 8.9 | 0-11,24,25 |
|  | FR2 if configured |  | No gap |
|  | FR1 if configured | FR2 only NOTE 8.9 | No gap |
| Per-FR | FR2 if configured |  | 12-25 |
| measurement | FR1 if configured | non-NR RAT and | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
| gap | FR2 if configured | FR1 NOTE3,6,8,9 | No gap |
|  | FR1 if configured | FR1 and FR2 NOTE 8.9 | 0-11, 24,25 |
|  | FR2 if configured |  | 12-25 |
|  | FR1 if configured | non-NR RAT and | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
|  | FR2 if configured | FR2 NOTE3,6,8,9 | 12-25 |
|  | FR1 if configured | non-NR RAT and FR1 and FR2 NOTE3,6,8,9 | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
|  | FR2 if configured |  | 12-25 |

RAN4 support more than 25 gap patterns for different time level. For different dimensions, these gap patterns can be per-UE gap or per-FR gap. Take an example, the periodicity of 40ms with different MGL are marked in red.

Observation 2: Due to the flexibility of SSB and different cases/definition of intra-frequency and inter-frequency measurement, complicated measurement gaps IDs were specified in 5GNR: (not all measurement gaps are really and well deployed in real deployment)

- More than 25 gap patterns, for different MGRP and different MGL.

- Per-UE gap or per-FR gap

Observation 3: In 5G, Per-FR (Frequency Range) gap is designed to minimize interruption, depending on the UE capability to support Per-FR (Frequency Range) measurement. If not, per-UE gap is used.

Observation 4: For Gap pattern ID applicability: #0~#11, FR1 and per-UE basis; #12~#23, FR2; #24 #25 are introduced for positioning.

For per-UE gap, this is simple than per-FR gap. (LTE is also using per-UE gap. FR1 and FR2 have the same MG. RF chains are sharing (only IF can be dependently.))

For per-FR gap, this is more complex. (FR1 and FR2 have independent MGs, RF chains are independent.)

According to observation #1, the RF retuning time is assumed as 500us and 250us in Rel-15. After that, we use RF retuning/switching in multiple RRM requirements but with different RF values assumption.

Such as:

In later releases such as uplink Tx switching (Rel-16/R17 inter-band CA) uplinkTxSwitchingPeriod-r16/ uplinkTxSwitchingPeriod2T2T-r17): 35us 140us 210us

Rel-19 Low band CA switching (FR1 only): 35us 70us 140us

Even in other feature based WIs, there are so many other UE capabilities, take RedCap as example: rf-RxRetuneTimeFR1-r18 rf-TxRetuneTimeFR1-r18 etc…

Observation 5: In later releases, RAN4 assumption the RF switching time in FR1 is shortened to ~210us which is closed to 250us in FR2.

Proposal 2: In 6GR in FR1, if the worst of the RF retuning time assumption can be shortened to the same level as that in FR2 in RF session. The MG pattern ID reduced due to no need for distinct RF retuning time difference in FR1 and FR2.

In addition, according to latest agreements in RAN-P, there is no need to support DC for 5G+6G.

Proposal 3:

MG pattern reduction principle:

- The RF retuning time can be re-considered to update.

- To consider the typical SSB periodicities which will be used in real deployment such as 160ms.

- No DC for 5G+6G. No need to consider the applicability for Measurement gap pattern configuration for 5G+6G

- Support per-UE MG as the basis.

In addition, in Rel-17 and following releases, multiple types of measurement gap were introduced including: Concurrent measurement gaps; Network controlled small gap; Pre-configured measurement gap; MUSIM gaps etc.

For Concurrent measurement gap, it supports two different gaps for parallel measurement. But in 6GR, whether it is a  typical scenario for different measurement patterns are not so clear. We still need to consider it with RAN1 new SSB design.

For Network controlled small gap, it can provide benefits for shorter/flexible interruption by NW. It can be considered as one type of 6G MG design.

For pre-configured measurement gap, it is designed for BWP related. In 6GR RAN1 discussion, even BWP may be disappeared in 6GR. We think pre-configured measurement gap is deprioritized in 6G MG design.

For MUSIM Gap:

Table 9.1.10-1: MUSIM Gap Pattern Configurations

| MUSIM Gap Pattern Id | MUSIM Gap Length (MGL, ms) | MUSIM Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 10 | 80 |
| 13 | 20 | 160 |
| 14 | 6 | 320 |
| 15 | 6 | 640 |
| 16 | 6 | 1280 |
| 17 | 6 | 2560 |
| 18 | 10 | 320 |
| 19 | 10 | 640 |
| 20 | 10 | 1280 |
| 21 | 10 | 2560 |
| 22 | 20 | 320 |
| 23 | 20 | 640 |
| 24 | 20 | 1280 |
| 25 | 20 | 2560 |
| 26 | 20 | 5120 |
| 27 | 10 | NA |
| 28 | 20 | NA |
| NOTE 1: Measurement gap pattern #27, #28 are the aperiodic gap pattern without MGRP. |  |  |  |

Compare to legacy MG, there are some different MG pattern for MUSIM in response to varied periodicity and measurement length. In 6G, RAN4 to discuss and consider whether it can be designed as general measurement gap but not a dedicated measurement gap type.

Proposal 4: In 6GR, RAN4 to design the 6G measurement gap as:

- Concurrent measurement gap is need to be considered together with new SSB to check the necessity.

- Deprioritize pre-configured measurement gap.

- Take NCSG in 6GR measurement gap design.

- Further discuss whether MUSIM gap can be re-designed as general measurement gap.

    - Gap-less measurement and its side conditions

In RAN4#116bis meeting, many companies would like to further discuss Gap-less measurement:

- Gap-less measurement and its side conditions (10 companies: MTK, Samsung, CMCC, ZTE, NTT DCM, HW, Ericsson, Nokia, QC, Xiaomi) - In scenarios with and without an available RF chain.

In LTE and 5GNR, the inter-frequency measurement is assumed to be based on MG. This is fundamentally rooted in the assumption of UE hardware architecture, specifically the assumption of a single RF chain and single local oscillator. When the UE is with the communication with serving cell, to measure a different carrier frequency, the UE has to retune its RF to the measured frequency. That’s why measurement gap is needed. In 6GR, if the UE can be improved by the hardware architecture with capabilities, we can suppose it can be improved for Gap-less measurement or less interruption. It can be further discussed by two Dimensions:

Proposal 5: For Gap-less measurement and side condition, RAN4 to further discuss on:

- Available RF chain in frequency domain (carriers)

  - Depends on RF concept. (The condition with current frequency and to be measured freqncy)

    - If the RF chain belongs to different Bands or far enough, the RF chain which is not used can be used for inter-frequency without MG (Gap-less).

    - Typical UE implementation: 2~3 RF chains.

    - From RF specs: up to 6 independent RF chains for inter-band CA.

    - FFS on the measurement to consider with searcher together.

- Available RF branches in diversity Reception.

  - Depends on UE capabilities, UE usually supports more than 2RX branches. The unused branches can be used for measurement without MG. (Gap-less in some time)

    - FFS on the measurement to consider with searcher together.

  - RRM framework

Another key agreement is last meeting is RAN4 agreed the RRM framework will be part of RAN4 RRM 6G study:

Issue 5: RRM framework: Measurement capability/delay/overhead/accuracy Agreement: RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. Note: the title of the main topic can be revised after selection of the sub-topics.

    - Measurement capability/delay/overhead/accuracy/quantities

In RAN1#122 meeting, RAN1 has already discussed the SSB design in 6GR. SSB is the most fundamental referent signal in RRM. In RRM design, there can be multiple reference signals to perform measurement in DL including SSB, CSI-RS, other measurement resources. But the SSB is the most important one. It can be used in different RRM procedures such as: initial access, serving cells measurement, neighbor cells measurement, beam management, etc. Therefore, the RRM mechanisms must accurately measure and analyze SSB quality including RSRP, SINR and T/F tracking etc.

In current RAN1 discussion, we observed there is potential change for SSB in 6GR which means RRM assumption for SSB quality cannot be assumed the same as that in 5GNR.

Take an example: serving cell/neighbor cells cell identification and measurement.

In 5GNR, e.g. for intra-frequency measurement without MGs

DRX cycle; TPSS/SSS_sync_intra. No DRX; max( 600 ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra. DRX cycle≤ 320 ms; max( 600 ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320 ms; ceil(5 x Kp) x DRX cycle x CSSFintra. DRX cycle; TSSB_measurement_period_intra. No DRX; max(200 ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320 ms; max(200 ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320 ms; ceil( 5 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

In L3 measurement, 5+5 samples for SSB are the minimum requirements for each target. If there are other factor, the time is even worse, such as in FR2, it should be 8 times of 10 samples. Take an example, if everything is the same as that in 5GNR with only change of SSB periodicity (20ms->160ms), the latency will be more than 12s in FR2. There is big challenge of latency issue.

In 5GNR, although RAN4 also support the measurement of configurable SSB periodicity, values can be chosen from {5, 10, 20, 40, 80, and 160 ms}. But for the initial access, the typical case is 20ms SSB periodicity. If RAN1 changed the SSB design to consider:

- Extend the SSB periodicity

- Change SSB sequence

- Multiple-types of SSBs

It will have huge impact on all RRM requirements to lead to inefficient RRM, poor mobility management, inability to meet the goal of “reliability and latency, seamless” in 6GNR.

Observation 6: RAN1 potential new SSB design will impact RRM framework a lot such as for measurement delay. The default SSB periodicity may extended to 160ms and number of RB in frequency may be changed to less than 5G, e.g. the shortest one is 1RB.

In typical RRM requirements, the following aspects are key in RRM:

- Number of samples of SSB

- The side condition of the measurement

- Measurement accuracy

RAN4 can start to discuss and perform the link level simulation in RAN4 for measurement delay and side condition.

Proposal 6: In 6GR, RAN4 shall evaluate SSB for measurement delay and side condition including:

- Extend the SSB periodicity

- Change SSB sequence and/or length in frequency domain.

    - Unified measurement

In 5GNR, RAN4 RRM specified different RRM requirements for L1 and L3 measurement respectively. In 5GNR, layer 1 measurement and L3 measurement are fundamentally different because they serve distinct purposes, which request vastly different time units and side conditions, and they are consumed by different part of the gNB for different decisions.

L1 and L3 are different in following aspects:

- Purpose. Part of L3 measurement is aimed to cell coverage and mobility among cells.

- Measurement resources. In L3 measurement, as the example listed in 2.10, the measurement is based on the measObjects and SMTC/Measurement Gaps. In contrast, in L1 measurement, the measurement is based on physical layer CSI frameworks, including CSI for tracking, CSI for beam management.

- Timing. In L3, for the new detectable cells, UE has no information of the timing of the cells. In other word, UE has to perform coarse synchronization of the detectable cells.  In contrast, for L1 measurement, UE has already synchronized to the serving cell. In other word, UE has to perform fine time/frequency tracking other than coarse synchronization.

- Time unit and latency. In L3 measurement, as mentioned above, the measurement is based on the measObjects and SMTC/Measurement Gaps. Thus, the measurement periods always depend on the periodicity of SMTC MG and CSSF. The latency is much longer than L1, such as hundreds of ms or seconds. In L1 measurement, the measurement is based on periodicity of SSB or CSI-RS in physical layer. The latency is much less than L3 measurement such as dozen or hundreds of ms.

- Different interferences and side condition. In L3 measurement, the main interferences are coming from neighboring cells. In L1 measurement, the main interferences are coming from beams. In 5GNR, L1 has higher condition than L3 and typically L1 is using L3 as precondition.

Observation 7: RAN4 specified different L1 and L3 RRM requirements due to following aspects:

- Purpose: L3 – cell coverage and mobility; L1 – beam and physical layer quality

- Measurement resources: L3 – MO; L1 – RS in physical layer

- Timing: L3 – coarse frequency and synchronization; L1 – fine time/frequency tracking

- Time unit and latency: L3 mainly based on SMTC/MG/CSSF longer; L1 – periodicity of reference signals, shorter

- Different interferences and side condition: L3 – cell, lower side condition and no precondition; L1 – beam, higher side condition and precondition by L3

However, there are common aspects for L1 measurement and L3 measurement. From the UE measurement perspective, all measurement is based on some types of reference signals to be measured such as SSB. In other word, L1 measurement can be the raw data for L3 measurement. UE physical layer can continuously measure the RSRP/[RSRQ]/SINR on the reference signals to be measured. Although L1 measurement cannot replace L3 measurement at all, RAN4 can find some cases to integrate L1 and L3 measurement.

Proposal 7: In 6GR, although L3 measurement cannot be dropped at all due to the reason above, L1 and L3 are both based on raw measurement based on the quality of reference signals to be measured in physical layer. RAN4 can find some cases to integrate L1 and L3 measurement from UE measurement perspective.  RAN4 to discuss whether/how to integrate L1 and L3 measurement from above aspects in Observation 7.

    - Virtual RRM UE group

From the concept of the virtual RRM UE group, the motivation is to check whether multiple UEs can be logically grouped for coordinated for RRM tasks. In 5GNR, each UE perform RRM measurement and communicate with NW independently. From ecosystem perspective, if the company is with a diverse portfolio of devices (such as smartphones, watches, XR glasses, FWA device), it may achieve benefit by collaborative RRM tasks. We are open to discuss whether there will be benefit especially for power consumption gain by coordination grouped RRM measurement. By our understanding, the key factors of grouped UE RRM measurement shall be:

- How to group?

- How can multiple UEs perform RRM measurement for coordination?

- Evaluation of the level of gain which can be achieved.

Proposal 8: RAN4 to discuss the feasibility of virtual RRM UE group for the benefit especially for power consumption gain by coordination grouped RRM measurement.

  - Mobility related RRM

In the 6G SID, it explicitly mentioned, RAN4 shall discuss and evaluate for: Mobility for 6GR (for all RRC states), including related RRM.

In RAN4#116bis meeting, many sub-topics are captured in latest WF:

Issue 6: Mobility related RRM [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to start from mobility related RRM solutions with less RAN1/2-dependency. - RAN4 to identify which of the following topics can be starts directly in RAN4: - Latency and/or interruption reduction for mobility through RAN4-defined components (Apple, MTK(interruption only), QC, LGE, ZTE, HW, vivo, Ericsson, Nokia, CMCC, Samsung) - discuss RRM part reduction during mobility, e.g., L1/L3 measurement, beam sweeping and etc. - discuss the scenarios/conditions for such reduction (known, unknown, or other status) - discuss NW controlled and UE initiated L1/L3 measurement report - Solutions for longer SSB periodicity in mobility (MTK, Samsung, Ericsson) - Early RRC decoding, and/or, DL/UL sync, and/or, early T/F tracking for mobility (MTK, Nokia) - Virtual RRM UE group (Apple, ZTE) - Unified measurement and mobility framework (QC) - E.g., based on 5G LTM - End-to-end handover latency target (QC) - RAN4 to study the practically achievable end-to-end handover latency target, taking into account user-plane data forwarding latency, to better align handover requirements with practical effectiveness. - Unified UE measurement requirements across cell changes (Nokia) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other mobility related RRM topics if there are sufficient conclusions from other WGs - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other mobility related RRM topics once there are sufficient conclusions from other WGs.

The mobility related RRM may be related to RAN2, the latest agreements for mobility in RAN2 are shown:

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

It is high level design principle in RAN2 which are aligned with RAN4 purpose as well.

From RAN4 perspective, we think RAN4 shall study on:

- Latency and/or interruption reduction for mobility through RAN4-defined components

- Solutions for longer SSB periodicity in mobility

For the necessity of second bullet, as mentioned in RRM framework, this change is a mandatory task which RAN4 shall investigate in 6GR.  Because it is observed from RAN1, the trend is that the SSB periodicity may be extended to 160ms for typical and default configuration. The SSB measurement affects RRM measurement especially for delay related requirements. When UE moves in the deployed multiple cells, RAN4 shall evaluate whether and how to ensure the mobility performance especially for different scenarios such as high speed UEs.

For the first bullet, we agree this is quite important and RAN4 shall investigate in 6GR. However, the basic definition in other group such as RAN2 hasn’t been agreed. Take an example, we are not sure the similar handover or conditional handover will be adopted in 6GR as well. It’s a little early to discuss the interruption for RAN4 defined components for consecrate RRM requirements.

Proposal 9: RAN4 shall study on:

- Latency and/or interruption reduction for mobility through RAN4-defined components

- Solutions for longer SSB periodicity in mobility such as 160ms

  - RRM related energy efficiency

According to RAN4 Chair’s guidance, it also required to discuss the RRM related to energy efficiency.

According to the 6G plan, RAN1 started in last RAN1#122 meeting on the discussion on energy efficiency. In RAN1 discussion, it mainly focusses on the power model of energy efficiency. There is no detailed solution yet.

In 5GNR, diverse energy-saving techniques have been introduced in multiple 3GPP release to enhance energy efficiency including: DRX, eDRX, WUS, MIMO layer adaptation, relaxed measurement in RRC Idle mode and RRC connected mode, NES, LP-WUS, etc. RAN4 specified different RRM requirements for those.

Observation 8: RAN4 specified different RRM requirements for diverse energy efficiency techniques in 5GNR including: DRX, eDRX, WUS, MIMO layer adaptation, relaxed measurement in RRC Idle mode and RRC connected mode, NES, LP-WUS, etc.

Proposal 10: In 6GR, according to the progress in RAN1, RAN4 to discuss whether/how to define a simple/unified RRM requirements for energy efficiency. The RAN4 discussion can be deferred until further RAN1 progress. RAN4 can set check point to check if there are sufficient conclusion from RAN1 in Q1, 2026.

  - Spectrum aggregation related RRM

According to RAN4 Chair’s guidance, it also required to discuss the RRM related to spectrum aggregation.

According to the 6G plan, RAN1 hasn’t started the discussion on spectrum aggregation:

6GR spectrum utilization and aggregation Placeholder only. No contributions before RAN1#124.

However, we can discuss from high level RAN4 points.

Observation 9: In LTE and 5GNR, RAN4 specified many RRM requirements for spectrum aggregation including: CA related technique; DC related technique. For detailed RRM requirements, e.g. Carrier switching; SCell related procedures (including SCell activation and deactivation; SCell L3 measurements) etc.

Although there is no RAN1 discussion yet, we believe in 6GR, some concepts will be remained in 6GR in framework.

From RRM perspective, there are below aspects may be related to RRM:

- CA and/or DC

In 5GNR, RAN4 has already specified separate RRM requirements for CA and DC respectively. For DC, even more complicated EN-DC NE-DC NR-DC are discussed and specified respectively. Some DC types like EN-DC, NE-DC, they were developed for 4G and 5G migration. More solutions will cause more parallel workload. In 6GR, we prefer to simpler framework as only keep CA and 6GR-6GR as DC. No other DC solution to support between 5GNR and 6GR.

- SCS for spectrum

In 5GNR, RAN4 encountered many issues and has already specified so many RRM requirements related to SCS. Take an example, BWP switching requirements. RRM discussed different scenarios including whether the SCS is changed or not before and after the switching in a CC or in multiple CCs because the assumption of RRM procedures is quite different. Take another example, RAN4 specified many different RRM requirements to for scheduling restriction/measurement restriction to treat the case with mixed numerology. However, this flexibility introduced at the very beginning has rare deployments in real but take more effort. In 6GR, the target it to design a simpler system. Single SCS per band will ensure the network and UE sides for more efficient and well deployed in real deployments. In last RAN1#122 and RAN#109 meetings, there were discussion on it with no decision yet. From RAN4 perspective, we also support single SCS per band to avoid unnecessary and unrealistic RRM request.

- DL and UL decoupling

In RAN#109 meeting, there was such discussion on DL and UL decoupling. We observed this aspect will impact RRM a lot. We need to keep the progress and consider whether/how to impact RRM.

- Single Cell Multi-Carriers

In 5G network, there are multiple multi-carriers solutions including:

CA (Intra-band contiguous/non-contiguous CA; Inter-band CA)

DC

LB-CA switching in Rel-19

LB-CA switching in Rel-20

From the commercial demands, there are some pieces of spectrum especially in FR1 bands. The single Cell multi-Carriers is aimed to avoid the complexity for fragmented pieces of spectrum, leading to a unified architecture for one cell to support multi-carriers but not CA solution. For details of the definition, we still need RAN1 agreements. The RAN4 discussion can be deferred until further RAN1 progress. RAN4 can set check point to check if there are sufficient conclusion from RAN1 in Q2, 2026.

Proposal 11: In 6GR, for spectrum aggregation, RAN4 RRM to discussion on following aspects:

- CA and/or DC. In 6GR, we prefer to simpler framework as only keep CA and 6GR-6GR DC. No other DC solution to support between 5GNR and 6GR.

- SCS for spectrum. We prefer to only single SCS per band to avoid unnecessary and unrealistic RRM request.

- DL and UL decoupling. RAN4 to track other working groups to consider whether/how to impact RRM.

- Single Cell Multi-Carriers: RAN4 RRM should discuss how to support the RRM for Single Cell Multi-Carriers

RAN4 can set check point to check if there are sufficient conclusion from RAN1 in Q2, 2026.

  - MIMO and mTRP operation related RRM

According to RAN4 Chair’s guidance, it also required to discuss the RRM related to MIMO operation and multi-TRP.

According to the 6G plan, RAN1 hasn’t started the discussion on MIMO operation and multi-TRP:

MIMO operation No contributions before RAN1#124.

However, we can discuss from high level RAN4 points.

Observation 10: In LTE and 5GNR, RAN4 specified many RRM requirements for MIMO and multi-TRP including: TCI-States related; Beamforming measurement related including L1-RSRP; L1-SINR; BFD/CBD; ICBM; Single-DCI; Multi-DCI; asymmetric TRPs; CJT; etc.

Although there is no RAN1 discussion yet, we believe in 6GR, some concepts will be remained in 6GR in framework.

From RRM perspective, there are below procedures may be related to RRM:

- Multiple Tx and Multiple Rx with/without simultaneously in transmission/reception

As detailed analysis from technical points in UE device types, this is one key factor which should be discussed together with MIMO operation and multi-TRPs. In multi-TRP scenarios, there can be single-DCI, multi-DCI, CJT, NCJT, asymmetric DL/UL TRPs, etc. The assumption of antenna architectures are key aspect to design RRM requirements for different dimensional including: time domain, frequency domain, spatial domain.

- TCI states

As the most foundation for MIMO, TCI states is most likely hesitated from 5GNR. However, unlike 5G, we prefer to keep only unified TCI states. Unlike 5G, RAN4 specified different RRM requirements for different TCI states framework. It makes the discussion and spec more complicated.

- Further harmonized “TR point” and “cell” for mobility and measurement

In multi-TRP operation, it is defined to improve the performance of cell-edge. However, from RRM or measurement perspective, there are some common concepts between “TRP” and “cell” including: whether they are using the same measurement reference signals; whether they are under the same condition assumption. In cell concept, it may lead to handover procedures. The measurements are related for serving cells or neighbor cells. For “TRP” concept, it may lead to beam management procedures.  The measurements are related for all beams. RAN4 can discuss and identify the common part to design with a harmonized RRM requirements.

Proposal 12: In 6GR, for MIMO operation and multi-TRP, RAN4 RRM to discussion on following aspects:

- Multiple Tx and Multiple Rx with/without simultaneously in transmission/reception

- TCI states: RRM only support unified TCI states framework

- Further harmonized “TR point” and “cell” for mobility and measurement

RAN4 can set check point to check if there are sufficient conclusion from RAN1 in Q2, 2026.

  - NTN related RRM

In 6GR SID, the system is aim at a harmonized 6GR for TN and NTN:

Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

In 5GNR, RAN4 has already specified separate RRM requirements for TN and NTN respectively. Within NTN clauses, several RRM procedures and requirements are reused from the TN. However, there are quite a lot RRM procedures and requirements are specifically defined to address the unique NTN characteristics including: Location-based/time-based cell re-selection; Location-based/time-based Conditional handover, satellite re-sync; different SMTC mechanisms from TN etc. In addition, 5GNR has defined preliminary scenarios for TN+NTN coexistence and mobility from TN to NTN or NTN to TN.

In 6GR, the objective is to design a fundamentally integrated TN-NTN architecture with unified RRM framework. RAN4 can co-work with other working groups bases on below aspects:

- Re-consider which procedures can be common for TN and NTN.

If procedures are using satellite ephemeris, prediction of satellite, it is more specific for NTN system. But some mechanisms like SMTC adjustment, it can also benefit for TN system.

- Integrated TN-NTN mobility support

In 6GR, RAN4 should support more seamless TN+NTN systems. NTN is not the backup or supplementary solution of TN. UE can move smoothly from NTN cells/beams to TN cells/beams or from TN cells/beams to NTN cells/beams. There is not a simple solution from one type to another type. In these complicated networks, the synchronization and timing mechanism for RRM are quite challenged.

- NTN UE types

In 5GNR, Rel-17, 3GPP specified mechanisms and requirements for UEs in FR1-NTN. In 5GNR Rel-18, RAN4 discussed the NTN support for Ka band especially in RAN4. Later, RAN4 have different WIs in later release to further support in NTN. There are special UE types called VSAT. Unlike UE in TN, especially for mechanical steering UE type, it is very special for NTN. In 5GNR, it has been defined preliminary mechanisms and support due to the limited resources. Take an example, for the most import procedure in RRC connected mode Handover, the handover procedure is under a limited assumption for mechanical steering UE type due to the physics of mechanical. In 6GR, RAN4 should collaborate with other working groups to re-consider the framework.

Proposal 13: RAN4 to discuss for harmonized 6G Radio design for TN and NTN, can start from these aspects:

- Re-consider which procedures can be common for TN and NTN.

- Integrated TN-NTN mobility support.

- Re-consider the framework and mechanisms for the different NTN UE types.

  - Initial access related RRM

According to RAN4 Chair’s guidance, it also required to discuss the RRM related to initial access.

According to the 6G plan, RAN1 hasn’t started the discussion on initial access:

Initial Access No contributions before RAN1#124.

However, we can discuss from high level RAN4 points.

From RRM perspective, there are below procedures may be related to RRM:

- Cell Search & Selection

Cell search and selection is key procedures in initial access.

Observation 11: In LTE and 5GNR, there is no RRM requirements for initial cell search.

In previous LTE and 5GNR, the performance of initial cell search is partially left to UE implementation. The procedure of initial cell search is a procedure performed autonomously by the UE before the communication with gNB. Typically, when RAN4 specifies a delay or “certain time” for RRM requirements, RAN4 need to specify the start point and end point. For the end point, it may use the time which UE transmit RACH like MSG1. However, it is a little difficult to clearly specify the start point like “power on”. Our understanding is that is the reason of why there is no exact RRM requirement to ensure the “exact initial cell search” requirements by now. But RAN4 to use other RRM requirements for other procedures to make sure part of the UE performance, such as cell identification time in L3 measurement. or sync raster specified in RF spec.  Meanwhile, the initial cell search typically happened when the UE is powered on which is rare in mobility of the cases.

- RACH

In previous LTE and 5GNR, RAN4 RRM specified RRM requirement for RACH. Take an example: in 5GNR 38.133,

### 6.2.2 Random access #### 6.2.2.1 Introduction #### 6.2.2.2 Requirements for 4-step RA type ##### 6.2.2.2.1 Contention based random access ##### 6.2.2.2.2 Non-Contention based random access ##### 6.2.2.2.3 UE behaviour when configured with supplementary UL #### 6.2.2.3 Requirements for 2-step RA type ##### 6.2.2.3.1 Contention based random access ##### 6.2.2.3.2 Non-Contention based random access ##### 6.2.2.3.3 UE behaviour when configured with supplementary UL

In each clause, RAN4 specified the correct behavior for each case. It is like a functionality description and part of them are repeated as RAN2 spec.

Proposal 14: In 6GR, for initial access, RAN4 RRM to discussion on following aspects:

- Whether to specify the RRM requirements for initial cell search. To consider on following aspects:

  - Whether can find the start point to define such RRM requirement like “power on”

  - Necessity to specify such RRM requirements if “UE is powered on” happened infrequently.

  - Part of UE performance in initial cell search can be ensured by other procedures like cell identification; sync raster

  - RRM requirements are needed for cell selection

If the first 3 bullets are the same as 5GR, which means initial cell search requirements cannot be specified in 6GR.

- Whether to specify the RACH RRM requirements as functionality as correct UE behavior and tests in RRM.

  - Other PHY signal/channel/procedure related RRM

    - RRM for different UE device types

In 6GR SID, the system will be designed as native support for more diverse UE device types: taking account to:

- target scalable and forward compatible design for diverse UE device types.

- considering different device types and implementations

In last RAN#109 plenary meeting, RAN and RAN1 has already started the discussion on UE device types. RAN4 RF will discuss on how to consider the RF requirements by considering different device types and implementations. Although there is no final decision, we can provide from RRM perspective as well.

In 5GNR, 3GPP also support different UE types including: normal UE like smartphones; handheld UE; RedCap UE, high-speed train CPE; FWA; Vehicle; NTN UE types etc.

From RRM perspective, we care several characteristics which will impact RRM requirements:

- Number of Rx: 1Rx, 2Rx, even more number of Rx. with more than 1 of number of Rx, it can use as receive diversity to combine the signal from multiple RX. In typical RRM requirements, UE measure the reference signals on different RXs or single RX within a certain period and report the typical RRM metrics RSRP/RSRQ/SINR… to the NW. The “certain period” or the performance of the metrics are relied on the number of RXs. It is a key factor of RRM requirements.

- Bandwidth support: Whether to have different minimum BW for different UE devices? In different bandwidth assumption, there can be different RRM procedures. Take an example, for IoT devices, it is supposed to use narrow bandwidth. However, for high-capable UEs, it can support a wideband bandwidth. It will lead to different assumption of RRM design including: BWP switching, whether to need switching to inactive BWP to perform measurement etc. It is also a key factor of RRM design for different procedures.

- Multiple Rx chains including simultaneous Rx reception. It is similar as a). However, it highlighted the capability of different Rx chains with simultaneous reception. For different UE device types, it may have different capabilities. Take an example, for more high capability FWA, it may have more than 4 panels to receive the RX from 4 directions at the same time. But for IoT devices, it may only support 1 direction at the same time.

- Multiple panels for uplink transmission with/without simultaneous transmission. It is similar as the capability in Rx but this is for transmission. For different UE device types, it may have different capabilities. Take an example, for more high capability FWA, it may have more than 4 panels to transmit different UL beams to different multi-TRPs for 4 directions at the same time. But for IoT devices, it may only support 1 direction at the same time.

For c) and d), it can impact the “certain period” and UE behavior of reception and transmission and RRM performance. It will also be discussed together with MIMO operation since the multiple Rx and multiple Tx is part of MIMO techniques.

- Power consumption: different UE types have different power consumption. For IoT devices, the data transmission/reception for services is not needed always but the battery life is more critical issue. For larger sizes UE devices types, the battery life and power consumption are not the critical issue. This will effect RRM requirements and this one is also can be discussed together with energy efficiency.

- Mobility status. From different UE devices types, the necessity of mobility is quite different. For example, for IoT sensors, fixed FWAs, their locations are quite stable. In most scenarios, they are stationary. For normal smartphones or smart watches, XRs, it may need mobility from different networks. For vehicle CPEs or high-speed train CPEs, the mobility from different cells/beams are more challenged. From RRM perspective, due to RRM consider multiple mobility procedures. This is the key factor to define such RRM requirements. Take an example, for stationary UEs, the UEs do not need to measure neighboring cells so frequently. UE can perform infrequent measurement. In contrast, high speed UE types need measure frequently. Otherwise, it cannot support smoothly movement to another cell/beam which caused the re-connection to new cell/beam again and again.

Proposal 15: In 6GR, RAN4 RRM should support diverse UE device types. From RRM perspective, RRM for different procedures and different requirements/performance are highly relied on below aspects, it should be discussed from below aspects:

- Number of Rx

- Bandwidth

- Multiple Rx chains including simultaneous Rx reception

- Multiple panels for uplink transmission with/without simultaneous transmission

- Power consumption

- Mobility status

    - RRM for duplexing

duplexing [RAN1, RAN4]

According to the SID, RAN1 and RAN4 will study on duplexing as well.

In RAN1#122 meeting, RAN1 achieved the agreements as below:

Agreement Study and identify the lessons learned from NR duplex modes. On 6GR duplexing study, RAN1 considers at least following duplex types: - FD-FDD - Semi-static TDD - gNB semi-static SBFD - HD-FDD on UE side - Dynamic TDD Study whether to consider following duplexing types - gNB dynamic SBFD - UE SBFD - gNB FD - Note: Other duplex modes are not precluded. Duplexing Placeholder only. No contributions before RAN1#124. None.

In 5GNR, RAN4 specified RRM requirements to support different duplexing modes including: FDD; TDD, HD-FDD on UE side. In Rel-19 SBFD WI, RRM requirements for UE to support gNB semi-static SBFD with limited assumption: gNB SBFD is operated in a signal carrier. In 6GR, duplexing can be further studied and in order to achieve better performance in other scenarios including SBFD with muti-carriers, mTRP, or energy saving. Even in Rel-19, RAN4 RRM specified RRM requirements with some assumption due to limited time. From RAN4 RRM perspective, RAN4 shall collaborate with RAN1 to discuss on how to support multiple types of duplexing including SBFD.

Proposal 16: In 6GR, RAN4 RRM shall collaborate with RAN1 to discuss on how to support multiple types of duplexing including SBFD. RAN4 can set check point to check if there are sufficient conclusion from RAN1 in Q2, 2026.

- Conclusion

In this contribution, we have provided our viewpoints on RAN4 RRM for 6GR and our proposals are:

Proposal 1: In 6GR, redefine Intra-frequency and Inter-frequency measurement as:

Intra-frequency measurement: The SSB is completely contained in the active [BWP] of the UE (If there is still BWP definition. If no BWP definition in 6G, change to CBW).

Inter-frequency measurement: otherwise.

The motivation for this redefinition is to maintain simplicity principle between intra-frequency measurement, inter-frequency measurement and the measurement gaps as:

- Intra-frequency measurement without MG

- Inter-frequency measurement with MG.

Observation 1: For RF retuning time, the assumption in 5G is 500us for FR1, and 250us for FR2.

Observation 2: Due to the flexibility of SSB and different cases/definition of intra-frequency and inter-frequency measurement, complicated measurement gaps IDs were specified in 5GNR: (not all measurement gaps are really and well deployed in real deployment)

- More than 25 gap patterns, for different MGRP and different MGL.

- Per-UE gap or per-FR gap

Observation 3: In 5G, Per-FR (Frequency Range) gap is designed to minimize interruption, depending on the UE capability to support Per-FR (Frequency Range) measurement. If not, per-UE gap is used.

Observation 4: For Gap pattern ID applicability: #0~#11, FR1 and per-UE basis; #12~#23, FR2; #24 #25 are introduced for positioning.

Observation 5: In later releases, RAN4 assumption the RF switching time in FR1 is shortened to ~210us which is closed to 250us in FR2.

Proposal 2: In 6GR in FR1, if the worst of the RF retuning time assumption can be shortened to the same level as that in FR2 in RF session. The MG pattern ID reduced due to no need for distinct RF retuning time difference in FR1 and FR2.

Proposal 3:

MG pattern reduction principle:

- The RF retuning time can be re-considered to update.

- To consider the typical SSB periodicities which will be used in real deployment such as 160ms.

- No DC for 5G+6G. No need to consider the applicability for Measurement gap pattern configuration for 5G+6G

- Support per-UE MG as the basis.

Proposal 4: In 6GR, RAN4 to design the 6G measurement gap as:

- Concurrent measurement gap is need to be considered together with new SSB to check the necessity.

- Deprioritize pre-configured measurement gap.

- Take NCSG in 6GR measurement gap design.

- Further discuss whether MUSIM gap can be re-designed as general measurement gap.

Proposal 5: For Gap-less measurement and side condition, RAN4 to further discuss on:

- Available RF chain in frequency domain (carriers)

  - Depends on RF concept. (The condition with current frequency and to be measured freqncy)

    - If the RF chain belongs to different Bands or far enough, the RF chain which is not used can be used for inter-frequency without MG (Gap-less).

    - Typical UE implementation: 2~3 RF chains.

    - From RF specs: up to 6 independent RF chains for inter-band CA.

    - FFS on the measurement to consider with searcher together.

- Available RF branches in diversity Reception.

  - Depends on UE capabilities, UE usually supports more than 2RX branches. The unused branches can be used for measurement without MG. (Gap-less in some time)

    - FFS on the measurement to consider with searcher together.

Observation 6: RAN1 potential new SSB design will impact RRM framework a lot such as for measurement delay. The default SSB periodicity may extended to 160ms and number of RB in frequency may be changed to less than 5G, e.g. the shortest one is 1RB.

Proposal 6: In 6GR, RAN4 shall evaluate SSB for measurement delay and side condition including:

- Extend the SSB periodicity

- Change SSB sequence and/or length in frequency domain.

Observation 7: RAN4 specified different L1 and L3 RRM requirements due to following aspects:

- Purpose: L3 – cell coverage and mobility; L1 – beam and physical layer quality

- Measurement resources: L3 – MO; L1 – RS in physical layer

- Timing: L3 – coarse frequency and synchronization; L1 – fine time/frequency tracking

- Time unit and latency: L3 mainly based on SMTC/MG/CSSF longer; L1 – periodicity of reference signals, shorter

- Different interferences and side condition: L3 – cell, lower side condition and no precondition; L1 – beam, higher side condition and precondition by L3

Proposal 7: In 6GR, although L3 measurement cannot be dropped at all due to the reason above, L1 and L3 are both based on raw measurement based on the quality of reference signals to be measured in physical layer. RAN4 can find some cases to integrate L1 and L3 measurement from UE measurement perspective.  RAN4 to discuss whether/how to integrate L1 and L3 measurement from above aspects in Observation 7.

Proposal 8: RAN4 to discuss the feasibility of virtual RRM UE group for the benefit especially for power consumption gain by coordination grouped RRM measurement.

Proposal 9: RAN4 shall study on:

- Latency and/or interruption reduction for mobility through RAN4-defined components

- Solutions for longer SSB periodicity in mobility such as 160ms

Observation 8: RAN4 specified different RRM requirements for diverse energy efficiency techniques in 5GNR including: DRX, eDRX, WUS, MIMO layer adaptation, relaxed measurement in RRC Idle mode and RRC connected mode, NES, LP-WUS, etc.

Proposal 10: In 6GR, according to the progress in RAN1, RAN4 to discuss whether/how to define a simple/unified RRM requirements for energy efficiency. The RAN4 discussion can be deferred until further RAN1 progress. RAN4 can set check point to check if there are sufficient conclusion from RAN1 in Q1, 2026.

Observation 9: In LTE and 5GNR, RAN4 specified many RRM requirements for spectrum aggregation including: CA related technique; DC related technique. For detailed RRM requirements, e.g. Carrier switching; SCell related procedures (including SCell activation and deactivation; SCell L3 measurements) etc.

Proposal 11: In 6GR, for spectrum aggregation, RAN4 RRM to discussion on following aspects:

- CA and/or DC. In 6GR, we prefer to simpler framework as only keep CA and 6GR-6GR DC. No other DC solution to support between 5GNR and 6GR.

- SCS for spectrum. We prefer to only single SCS per band to avoid unnecessary and unrealistic RRM request.

- DL and UL decoupling. RAN4 to track other working groups to consider whether/how to impact RRM.

- Single Cell Multi-Carriers: RAN4 RRM should discuss how to support the RRM for Single Cell Multi-Carriers

RAN4 can set check point to check if there are sufficient conclusion from RAN1 in Q2, 2026.

Observation 10: In LTE and 5GNR, RAN4 specified many RRM requirements for MIMO and multi-TRP including: TCI-States related; Beamforming measurement related including L1-RSRP; L1-SINR; BFD/CBD; ICBM; Single-DCI; Multi-DCI; asymmetric TRPs; CJT; etc.

Proposal 12: In 6GR, for MIMO operation and multi-TRP, RAN4 RRM to discussion on following aspects:

- Multiple Tx and Multiple Rx with/without simultaneously in transmission/reception

- TCI states: RRM only support unified TCI states framework

- Further harmonized “TR point” and “cell” for mobility and measurement

RAN4 can set check point to check if there are sufficient conclusion from RAN1 in Q2, 2026.

Proposal 13: RAN4 to discuss for harmonized 6G Radio design for TN and NTN, can start from these aspects:

- Re-consider which procedures can be common for TN and NTN.

- Integrated TN-NTN mobility support.

- Re-consider the framework and mechanisms for the different NTN UE types.

Observation 11: In LTE and 5GNR, there is no RRM requirements for initial cell search.

Proposal 14: In 6GR, for initial access, RAN4 RRM to discussion on following aspects:

- Whether to specify the RRM requirements for initial cell search. To consider on following aspects:

  - Whether can find the start point to define such RRM requirement like “power on”

  - Necessity to specify such RRM requirements if “UE is powered on” happened infrequently.

  - Part of UE performance in initial cell search can be ensured by other procedures like cell identification; sync raster

  - RRM requirements are needed for cell selection

If the first 3 bullets are the same as 5GR, which means initial cell search requirements cannot be specified in 6GR.

- Whether to specify the RACH RRM requirements as functionality as correct UE behavior and tests in RRM.

Proposal 15: In 6GR, RAN4 RRM should support diverse UE device types. From RRM perspective, RRM for different procedures and different requirements/performance are highly relied on below aspects, it should be discussed from below aspects:

- Number of Rx

- Bandwidth

- Multiple Rx chains including simultaneous Rx reception

- Multiple panels for uplink transmission with/without simultaneous transmission

- Power consumption

- Mobility status

Proposal 16: In 6GR, RAN4 RRM shall collaborate with RAN1 to discuss on how to support multiple types of duplexing including SBFD. RAN4 can set check point to check if there are sufficient conclusion from RAN1 in Q2, 2026.

- Reference

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

[2] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

[3] R4-2514644, WF on [116bis][105] 6G RRM, Apple

[3] R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 Chair