3GPP TSG-RAN WG4 Meeting #117		R4-2522024

Dallas, USA, 17 – 21 November, 2025

Agenda item:		6.12.2.2

Source:		Samsung

Title:	Discussion on RRM performance requirements of Rel-19 MIMO Part2

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of performance part of this WI are described in [1] as:

- Specify necessary performance requirements for the enhancements listed above.

In last RAN4#116bis meeting, RAN4 achieved agreements for the test cases of part2 in the latest WF [2]. There is still an open issue for accuracy requirements for CJT reporting. In this contribution, we provide our view of accuracy requirements for CJT reporting.

- Discussion

In last RAN4#116bis meeting, the way forward of accuracy requirements for CJT reporting are agreed in the WF [2]:

Issue 1-1: Accuracy requirements for delay and frequency offset reporting for CJT calibration Way forward: Alternatives for further discussion: Alt 1: Define one set of [tighten] accuracy requirements based on SNR >= 0 or 3dB Encourage further input on the exact performance requirement Note: “tighten” means it is tighten compared to -3dB side condition Alt 2: Define two sets of requirements with UE capability Encourage input on details of the UE capability

In previous meetings, RAN4 achieved the core requirements for CJT reporting for delay offset and frequency offset. Due to different simulation results, there were different approaches to define the accuracy requirements. By our understanding, the use cases of CJT reporting can be divided into two categories:

- TRP selection or MIMO scheme chosen.

- Pre-compensation for Multi-TRPs at NW side.

The first usage for CJT reporting is for NW to perform TRP selection or MIMO scheme chosen for UEs. If the delay/frequency offset are too large, which means the coherent performance in multiple TRPs turns worse. The TRP selection means the NW can change the multi-TRP schemes to single TRP who serve to this UE. Another scheme is: there are many MIMO schemes in 5GNR, such as: m-DCI, s-DCI, CJT schemes; NCJT schemes. NW can perform proper MIMO scheme for UE depends on the reporting of each UE. By this usage, this can be per-UE level.

The second usage for CJT reporting is the most useful of the scenario. NW have multiple TRPs to co-work to the UE. If there are delay offset or frequency offset, it leads to the un-sync condition at UE side. Therefore, RAN1 introduced this feature to let UE performance the CJTC-Dd for delay measurement and CJTC-F for frequency measurement and report to NW to help NW to perform the pre-compensation at NW side. After the pre-compensation, the signals from all TRPs arrive at the UE coherently with better performance. By this motivation, the pre-compensation at NW side cannot be UE specific but based on cell level. Introduce two sets of requirements with different UE capabilities doesn’t help in real implementation of this feature. From implementation view at NW side, if there are two sets of different UE capabilities, NW will always pick up the better performance of the UE capability. In addition, in real deployments, there will be more than hundreds of UEs in the network. The truly reporting accuracy depends on the UE implementation algorithm, UE condition like UE is in cell center or cell edge. There is no any other information known by NW but only the all the reporting from all UEs. It doesn’t make sense to add further weight factor for each UE. The straightforward NW implantation is to average all the reporting results received. That is why we said the pre-compensation is per-cell level but not per-UE level.

Observation 1: For pre-compensation of delay/frequency offset is per-cell level but not per-UE level. No such benefit can be observed to introduce two sets of UE capability.

On the other hand, it is also required more UE vendors to support this feature to make the whole procedure works together. We understand the -3dB is typical L1-RSRP side condition and from NW side, by using lower side condition means for cell coverage. However, considering there are many UEs in the network, increasing SNR condition can improve the reliability of the pre-compensation.

Proposal 1: RAN4 to specify Accuracy requirements for delay and frequency offset reporting based on one set of accuracy requirements based on SNR >= 3dB.

- Conclusion

In this contribution, we provide our proposals of accuracy requirements for CJT reporting:

Observation 1: For pre-compensation of delay/frequency offset is per-cell level but not per-UE level. No such benefit can be observed to introduce two sets of UE capability.

Proposal 1: RAN4 to specify Accuracy requirements for delay and frequency offset reporting based on one set of accuracy requirements based on SNR >= 3dB.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2514804, WF on NR_MIMO_Ph5_RRM_Part2, MediaTek