3GPP TSG-RAN WG4 Meeting #117		R4-2522026

Dallas, USA, 17 – 21 November, 2025

Agenda item:		6.13.3

Source:		Samsung

Title:	Discussion on RRM performance requirements of Rel-19 SBFD

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#108 meeting. The objectives of performance part of this WI are described in [1] as:

The objectives are as follows [RAN4]: - Specify BS RF conformance requirements to support SBFD operation at gNB - Specify necessary RRM performance requirements to support SBFD operation - Specify necessary BS and/or UE demodulation performance requirements to support SBFD operation

In last RAN4#116bis meeting, RAN4 achieved agreements for accuracy requirements and most of the test cases in the latest WF [2]. There are still some open issues in the WF. In this contribution, we provide our view of the remained open issues.

- Discussion

In last RAN4#116bis meeting, RAN4 achieved the agreements as below:

- Issue 1-1-1: Time offset for L1-SRS-RSRP measurements - Agreement - Specify the time offset conditions for L1-SRS-RSRP measurement accuracy requirements with cell phase error for [15kHz SCS and] 30kHz SCS and without cell phase error for other SCS

In the last minute, we proposed to remove the square bracket of [15kHz SCS and]. However, there was different understanding from company which want to remove [15kHz SCS and] totally as shown: “Specify the time offset conditions for L1-SRS-RSRP measurement accuracy requirements with cell phase error for 30kHz SCS and without cell phase error for other SCS”. In our understanding, the reason is 15kHz is not a typical commercial NW deployment in FR1 TDD. In real 5G NR FR1 deployment, TDD with 30kHz SCS is the typical scenario. This is also the comment from our side during online discussion. That’s why we support and agreed the “FR1: NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex” in the test cases for test configurations.

However, the agreement above is not only for test cases but for the general accuracy requirements. By the description as “Specify the time offset conditions for L1-SRS-RSRP measurement accuracy requirements with cell phase error for 30kHz SCS and without cell phase error for other SCS” is to move 15kHz SCS in to “other SCS” without cell phase error. It went against the original intention to use SCS lower than 60kHz and higher SCS for without cell phase error.

In addition, according to previous agreement the BSRF session and latest spec 38.104 v19.2.0, 15KHz SCS is not precluded from spec perspective.

38.104 v19.2.0 5.3.2 Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1 SCS (kHz); 3 MHz; 5 MHz; 7 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 35 MHz; 40 MHz; 45 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB. 15; 15; 25; 35; 52; 79; 106; 133; 160; 188; 216; 242; 270; N/A; N/A; N/A; N/A; N/A. 30; N/A; 11; N/A; 24; 38; 51; 65; 78; 92; 106; 119; 133; 162; 189; 217; 245; 273. 60; N/A; N/A; N/A; 11; 18; 24; 31; 38; 44; 51; 58; 65; 79; 93; 107; 121; 135. Note: In FR1 bands applicable to SBFD operation, the BS channel bandwidth for SBFD operation shall be no less than 40MHz and no less than 20MHz for band n39..

There is another big table in 5.3.5 of 38.104 which only the Note is captured as below:

omitted Table here NOTE 9: In FR1 bands applicable to SBFD operation, the BS channel bandwidth for SBFD operation shall be no less than 40MHz and no less than 20MHz for band n39.

Proposal 1: Remove the square bracket in RAN4#116bis agreement, RAN4 to agree: Specify the time offset conditions for L1-SRS-RSRP measurement accuracy requirements with cell phase error for 15kHz SCS and 30kHz SCS and without cell phase error for other SCS.

In last RAN4#116bis meeting, the Measurement accuracy for CSI-RS based L1 measurements are still open as captured in WF [2]:

### Issue 2-1-1: Measurement accuracy for CSI-RS based L1 measurements - Background - Different cases for DUD - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). - Agreement from RAN4#114bis - Note: The following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE. - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - Way forward - Define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding [TBD] relaxation on top of existing accuracy (based on 48 RB) - For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB), - for a UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB apply. - Option 2: accuracy requirements based on 24 RB apply. - for UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB apply

In last meeting, companies have different understanding for case 3. When we agreed for case 3, the assumption of the UE processing can be different for DUD case. If the UE has the capability of CSI processing time scaled by 2, the UE can treat D1 and D2 together by our understanding. It means UE can apply the legacy accuracy requirements based on 48RBs. There is no need to further relax the accuracy requirements for L1-RSRP and L1-SINR. Otherwise, the relaxed accuracy requirements based on 24 PRBs shall be applied for UE not support such capability.

Proposal 2: For measurement accuracy for CSI-RS based L1 measurements of case 3, RAN4 to agree:

- for a UE indicating CSI processing time scaled by 2, existing accuracy requirements based on 48 RB apply.

- for UE not indicating CSI processing time scaled by 2, accuracy requirements based on 24 RB apply.

In last RAN4#116bis meeting, there are still two FFS bullets in the Test configurations, captured in WF [2]:

### Issue 3-1-2: Test configurations - Agreements - FR1: NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex - FR2: - Option 1: NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode - Option 2: NR 120 kHz SCS, 400 MHz bandwidth, TDD duplex mode - TDD UL/DL configuration : DDDSU - Time domain configuration of SBFD slots/symbols: DXXXU - Frequency domain configuration of SBFD subbands for FR1 - DU: 80 MHz – 20 MHz - DUD: 40 MHz – 20 MHz – 40 MHz - FFS: Frequency domain configuration of SBFD subbands for FR2

According to the previous agreement in BS-RF session and the latest 38.104 v19.2, according to the declaration, the BS channel bandwidth for SBFD shall be no less than 100MHz.

Table 5.3.5-2: BS channel bandwidths and SCS per operating band in FR2-1

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n257 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n258 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n259 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n260 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n261 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n262 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| NOTE 1: In FR2 bands applicable to SBFD operation, the BS channel bandwidth for SBFD operation shall be no less than 100MHz. |  |  |  |  |  |

For SBFD operation, the transmission bandwidth configuration for SBFD UL subband NRB,SBFD,UL shall be configured according to the values provided for BS transmission bandwidth configuration NRB corresponding to bandwidth no less than 10MHz in Table 5.3.2-1 for FR1, and NRB in Table 5.3.2-2 for FR2-1. Transmission bandwidth configuration for SBFD DL subband in SBFD (i.e., NRB,SBFD,DL for SBFD DU or UD operation, and NRB,SBFD,DL,1 and NRB,SBFD,DL,2 for SBFD DUD operation) and guardband between SBFD UL subband and DL subband shall be configured by SBFD-capable BS. For SBFD DUD operation, NRB,SBFD,DL,1 and NRB,SBFD,DL,2 shall have the same size or different sizes by only 1RB. Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1 SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A. 120; 32; 66; 132; 264. Note: The BS channel bandwidth for SBFD operation shall be no less than 100MHz..

In addition, the minimum UL subband size is 50MHz in FR2. In existing RRM tests for FR2, the typical used configuration is 100MHz. Therefore, we propose the test configuration of SBFD for FR2:

Proposal 3: For test configuration of SBFD in FR2, RAN4 to agree:

- NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode

- DU: 50MHz – 50MHz

In last RAN4#116bis meeting, there is still FFS for CSI-RS L1-SINR measurement highlighted in yellow:

### Issue 4-1-1: Test case scenarios - Agreements - Agreed test-cases (details in the work-split table) - L1-SRS-RSRP measurement delay and scheduling restrictions - L1-CLI-RSSI measurement delay and scheduling restrictions - L1-SRS-RSRP measurement accuracy - L1-CLI-RSSI measurement accuracy - FFS: Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM in SBFD - Radio Link Monitoring In-sync Test for PCell configured with CSI-RS-based RLM in SBFD - Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in SBFD - CSI-RS based L1-RSRP measurement in SBFD - CSI-RS based L1-RSRP measurement accuracy in SBFD - Further discuss whether to define the following test-cases: - CSI-RS L1-SINR measurement (delay and accuracy)

There was a proposal from one company to remove CSI-RS L1-SINR measurement since UE has already been tested by L1-RSRP measurement. However, it cannot be inferred that if UE can pass CSI-RS based L1-RSRP measurement, it will pass CSI-RS L1-SINR measurement as well. L1-SINR reporting with CSI-RS based CMR and with/no dedicated IMR is an important UE behavior to verify which cannot be replaced and inferred by L1-RSRP. In addition, in SBFD core requirements, RAN4 further specified the RRM requirements for SBFD feature compared to legacy behavior in Rel-19 which shall be verified.

Proposal 4: RAN4 to add the test cases for CSI-RS L1-SINR measurement in SBFD.

- Conclusion

In this contribution, we further analyze the impacts on RRM performance part of Rel-19 SBFD and provide our proposals:

Proposal 1: Remove the square bracket in RAN4#116bis agreement, RAN4 to agree: Specify the time offset conditions for L1-SRS-RSRP measurement accuracy requirements with cell phase error for 15kHz SCS and 30kHz SCS and without cell phase error for other SCS.

Proposal 2: For measurement accuracy for CSI-RS based L1 measurements of case 3, RAN4 to agree:

- for a UE indicating CSI processing time scaled by 2, existing accuracy requirements based on 48 RB apply.

- for UE not indicating CSI processing time scaled by 2, accuracy requirements based on 24 RB apply.

Proposal 3: For test configuration of SBFD in FR2, RAN4 to agree:

- NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode

- DU: 50MHz – 50MHz

Proposal 4: RAN4 to add the test cases for CSI-RS L1-SINR measurement in SBFD.

- Reference

[1] RP-251874, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2515166, Way Forward for [116bis][309] Rel-19 RRM performance_Part1, Qualcomm