3GPP RAN4 Meeting #117		R4-2522037

Dallas, USA, 17th – 21th November, 2025

Agenda item:	8.11

Source:	Ericsson

Title:	Discussion on testability and OTA in 6G

WI/SI:	FS_6G_Radio

Release:	Rel-20

Document for:	Approval

# Introduction

During RAN4#116bis a WF [1] was agreed on testability and OTA in 6G with the following agreements:

This contribution presents our views on the above issues.

# Discussion

## Improved testability of conducted requirements

Some conducted requirements from the previous generations, ex. Pout, sensitivity, spurious emissions, EVM, Maximum Sensitivity Degradation (MSD), and MIMO verification, are not sufficient for guaranteeing device good performance in network operation. For example, MSD for CA is currently tested using conducted setups, where the UE is connected via RF cables to a tester that provides controlled downlink signals while forcing uplink transmission with antennas disabled. This method is highly repeatable and isolates the effect of the internal RF chain, but it ignores the real antenna environment. A key drawback is that conducted tests do not capture antenna mutual coupling, radiation leakage, housing effects, or user interaction, all of which contribute to real-world desensitization. They also assume ideal impedance matching and duplexer isolation, which are rarely true in practice. By shifting MSD to OTA testing, these missing effects would be included, leading to results that better reflect end-to-end device performance. OTA testing would reveal degradations caused by antenna placement, device form factor, and near-field coupling, giving operators a more realistic view of UE behaviour in the field. It would also allow assessment of self-interference paths through radiation, not just circuit leakage. It would also indicate whether simultaneous Rx-Tx can be supported for a band combination.

Observation 1. The conducted MSD requirements do not capture real antenna mutual coupling, radiation leakage, housing effects, or user interaction, all of which contribute to real-world desensitization. Shifting MSD to OTA testing, these missing effects would be included, leading to results that better reflect end-to-end device performance.

Proposal 1. RAN 4 to start working on method /methods for OTA testing of MSD as part of 6G SI.

## New test methodologies for new 6GR frequencies

The newly added frequency range to 6G extending from 7 to 15 GHz has a shorter wavelength and higher propagation losses compared to FR1 frequency range. This means that antennas with smaller physical dimensions can be designed, and more antennas can be integrated to mobile phones to accommodate for higher propagation losses. So, the discussion about the preferred test method (conducted or OTA) should not be limited to the availability or not of connectors at a certain frequency but also to the projected number of antennas that can yield some impracticalities of adding to many connectors.

Observation 2. The discussion about the preferred test method (conducted or OTA) should not be limited to the availability or not of connectors at a certain frequency but also to the projected number of antennas that can yield some impracticalities of adding to many connectors.

Proposal 2. Expand the evaluation of preferred test methods (conducted vs. OTA) to include not only connector availability at specific frequencies but also the anticipated number of antennas.

## Testability for different device types

Currently, many types of devices, e.g. mobile phones, wearables, FWA, XR, IoT and so on, are used with cellular networks. With 6G some other type of devices can appear. While all these devices are important, mobile phones are the most spread devices. Hence, mobile phones should be the highest priority in the context of this study item. Indeed, while in the past releases there has been a good progress on TRP/TRS OTA measurement of mobile devices, there are still very important issues, e.g. OTA measurement of MSD, dynamic MIMO OTA, OTA testing of AI/ML and so on, to handle before targeting new types of devices.

Observation 3. Many types of devices are used with cellular networks, 6G will add more devices. Among all these device types, mobile phones are the most spread devices.

Observation 4. In the past releases there has been a good progress on TRP/TRS OTA measurement of mobile devices. However, there are still very important issues, OTA measurement of MSD, dynamic MIMO OTA, OTA testing of AI/ML and so on, to handle before targeting new types of devices.

Proposal 3. RAN4 to focus on addressing key challenges such as MSD OTA measurement, dynamic MIMO OTA, and OTA testing of AI/ML features before expanding to new device categories.

## AI/ML OTA testability

6G will introduce AI/ML functionalities such as beam prediction, mobility, positioning, and CSI compression/prediction. Robust testing will be required to ensure reliable operation and interoperability of AI/ML based features across vendors, which may necessitate new test setups. Moreover, testing time and reproducibility will be important considerations, and standardized approaches will be critical, as inadequate validation could limit adoption of certain AI-native features.

Proposal 4. Study OTA testing for the added AI/ML functionalities with the appropriate channel models as Spatial Channel Model (SCM).

Overfitted AI/ML functionalities can pass a specific testing scenario, but they will not function in any other real use scenario. This can be avoided by testing the AI/ML functionalities under divers scenarios/ use cases (different channel models and deployment scenarios (RMa, UMa, UMi, …) different doppler shifts and so on.

Observation 5: AI/ML models can be over-trained/ overfitted, performing well in a specific test scenario but failing to generalize to real-world conditions.

Proposal 5: Implement diverse testing across multiple channel models, deployment scenarios, and Doppler shifts to ensure robustness and generalization of the AI/ML functionalities.

# Conclusions

This contribution provides our views related to OTA and testability. The contribution makes the following observations and proposals.

Observation 1. The conducted MSD requirements do not capture real antenna mutual coupling, radiation leakage, housing effects, or user interaction, all of which contribute to real-world desensitization. Shifting MSD to OTA testing, these missing effects would be included, leading to results that better reflect end-to-end device performance.

Proposal 1. RAN 4 to start working on method /methods for OTA testing of MSD as part of 6G SI.

Observation 2. The discussion about the preferred test method (conducted or OTA) should not be limited to the availability or not of connectors at a certain frequency but also to the projected number of antennas that can yield some impracticalities of adding to many connectors.

Proposal 2. Expand the evaluation of preferred test methods (conducted vs. OTA) to include not only connector availability at specific frequencies but also the anticipated number of antennas.

Observation 3. Many types of devices are used with cellular networks, 6G will add more devices. Among all these device types, mobile phones are the most spread devices.

Observation 4. In the past releases there has been a good progress on TRP/TRS OTA measurement of mobile devices. However, there are still very important issues, OTA measurement of MSD, dynamic MIMO OTA, OTA testing of AI/ML and so on, to handle before targeting new types of devices.

Proposal 3. RAN4 to focus on addressing key challenges such as MSD OTA measurement, dynamic MIMO OTA, and OTA testing of AI/ML features before expanding to new device categories.

Proposal 4. Study OTA testing for the added AI/ML functionalities with the appropriate channel models as Spatial Channel Model (SCM).

Observation 5: AI/ML models can be over-trained/ overfitted, performing well in a specific test scenario but failing to generalize to real-world conditions.

Proposal 5: Implement diverse testing across multiple channel models, deployment scenarios, and Doppler shifts to ensure robustness and generalization of the AI/ML functionalities.

# References

[1]	R4-25114640, “WF for [116bis][110] 6G testability and OTA”, Vivo, 3GPP RAN4#116bis.