3GPP TSG-RAN WG4 Meeting #117	R4-2522056

Dallas (TX), USA, November 17-21, 2025

Source: 	Qualcomm Incorporated

Title: 	Draft LS for RAN5 regarding Intra-band non-collocated Type 4b UE requirements

Agenda item:	 6.2.3

Document for:	Endorsement

- Introduction

In the previous meeting it was agreed to send an LS to RAN5 informing them to extend the application of the agreements for Type 2 UE regarding minimum separation across CCs for demodulation requirements testing to Type 4b UE considered in this WI.

In this contribution we submit a draft LS for RAN4 endorsement.

# Draft LS Content

3GPP TSG-RAN WG4 Meeting #117	R4-25xxxxx

Dallas (TX), USA, November 17-21, 2025

Title:	LS on frequency separation for Type 4b UE NR-CA PDSCH demodulation requirements

Response to:	-

Release:	Release 19

Work Item:	NonCol_intraB_ENDC_NR_CA_Ph2

Source:	TSG RAN WG4

To:	TSG RAN WG5

Cc:	-

Contact person:

Name:	Pierpaolo Vallese

E-mail Address:	pvallese@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

## Overall description

RAN4 has agreed to define NR-CA PDSCH demodulation requirements for Type4b UEs in the context of the Rel-19 WI on supporting intra-band non-colocated EN-DC/NR-CA deployment. PDSCH demodulation requirements are specified in TS 38.101-4 for UE indicating ‘intraBandNR-CA-non-collocated-r19’ and providing ‘nonCollocatedTypeNR-CA-v1900’ with value ‘type4’.

It was previously agreed [2] that Demodulation tests for Type 2 UEs supporting intra-band non-colocated NR-CA should introduce the following test condition in RAN5:

- Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC in R18.

The same agreement should be applied to NR-CA PDSCH demodulation requirement for Type 4b UEs.

## Actions

To RAN5:

ACTION: RAN4 respectfully asks RAN5 to consider the agreed test condition in their future specification work.

## References

[1] R4-2316951, “LS on frequency separation for Type 2 UE NR-CA PDSCH demodulation requirements”, RAN4.

Proposal 1: RAN4 to approve the Draft LS include above and send it to RAN5.

- Conclusions

Proposal 1: RAN4 to approve the Draft LS include above and send it to RAN5.