3GPP TSG-RAN WG4 Meeting #117 	R4-2522065

Dallas, US, Nov 17th – 21st, 2025

Agenda Item: 6.13.3

Source: MediaTek inc.

Title: Discussion on SBFD simulation performance

Document for: Discussion

# Introduction

In the previous meeting RAN4 agreed to run simulation [1] to provide performance results based on the agreed simulation assumptions [2]. In the last meeting, we agreed to have the following parameters used to define the accuracy requirements:

# Topic #1: UE-to-UE CLI handling ### Issue 1-1-1: Time offset for L1-SRS-RSRP measurements - Agreement - Specify the time offset conditions for L1-SRS-RSRP measurement accuracy requirements with cell phase error for [15kHz SCS and] 30kHz SCS and without cell phase error for other SCS ### Issue 1-1-2: SRS BW for L1-SRS-RSRP measurements - Agreement - SRS BW of 24 RBs shall be used for performance requirements of L1-SRS-RSRP measurements ### Issue 1-1-3: SRS Es/Iot for L1-SRS-RSRP measurements - Agreement - SRS Es/Iot side-condition of 1db shall be used for performance requirements of L1-SRS-RSRP measurements ### Issue 1-1-4: L1-SRS-RSRP measurement accuracy requirements - Agreement - L1-SRS-RSRP measurements accuracy requirements are defined based on simulation results for the agreed side conditions

# Simulation performance

Following the agreed simulation assumptions:

- Channel: AWGN Channel

- SCS: 15kHz, 30kHz, 60kHz, and 120kHz SCS

- SNR: 1dB SNR

- num. of measurement sample: 1/2/3 L1 sample

- SRS BW: 24 PRBs

- SRS symbol: 1 OFDM symbol

- performance metric: 5%/50%/95%-tile delta RSRP

- Cell phase error for FR1, without cell phase error for FR2

Table 1: simulation performance for SR-RSRP measurement accuracy (delta RSRP) for Case 1 (4.67us FR1)

| FR | SCS | SRS BW | Sample | MTK |  |
| --- | --- | --- | --- | --- | --- |
|  | (kHz) | (PRBs) |  | SNR (dB) |  |
|  |  |  |  | 1 dB |  |
|  |  |  |  | 5% | 95% |
| FR1 | 15 | 24 | 1 | -1 | 0.42 |
|  |  |  | 2 | -0.85 | 0.15 |
|  |  |  | 3 | -0.81 | 0.014 |
|  | 30 | 24 | 1 | -1.59 | -0.06 |
|  |  |  | 2 | -1.43 | -0.37 |
|  |  |  | 3 | -1.39 | -0.5 |
|  | 60 | 24 | 1 | -3.06 | -1.23 |
|  |  |  | 2 | -2.87 | -1.57 |
|  |  |  | 3 | -2.82 | -1.74 |

Table 2: simulation performance for SR-RSRP measurement accuracy (delta RSRP) for Case 2 (0.67us FR2)

| FR | SCS | SRS BW | Sample | MTK |  |
| --- | --- | --- | --- | --- | --- |
|  | (kHz) | (PRBs) |  | SNR (dB) |  |
|  |  |  |  | 1 dB |  |
|  |  |  |  | 5% | 95% |
| FR2 | 60 | 24 | 1 | -0.76 | 0.63 |
|  |  |  | 2 | -0.61 | 0.36 |
|  |  |  | 3 | -0.57 | 0.23 |
|  | 120 | 24 | 1 | -1.08 | 0.34 |
|  |  |  | 2 | -0.94 | 0.06 |
|  |  |  | 3 | -0.89 | -0.07 |

# Summary

Simulation performance is provided in Table 1 and Table 2, based on the agreed simulation assumptions and latest agreements.

# References

- R4-2515166, “WF for [116bis][309] Rel-19 RRM performance_Part1”, Qualcomm, Prague, CZ, Oct 2025.

- R4-2416910, “Simulation assumptions for L1 SRS-RSRP measurement”.

- R4-2502083, “Discussion on SBFD simulation performance”, MediaTek, Athens, Greece, Feb, 2025.

- R4-2512190, “updated summary sim results for L1-SRS-RSRP”, Huawei, Bengaluru, India, Aug 2025.