3GPP TSG-RAN WG4 Meeting #117	R4-2522102

Dallas, USA, November 17th – November 21st, 2025

Source:	Nokia

Title:	Discussion on XR ph3 RRM performance requirements

Agenda item:	6.18.2

Document for:	Discussion

Introduction

This contribution provides Nokia’s views on open issues after RAN4#116bis on RRM performance requirements for XR enhancements [1].

Discussion

Test Case List

In the RAN4#116bis meeting, significant progress was achieved, and the following agreements were made for defining the XR test cases.

### Issue 1-1-1: Scenarios for the definition of the test cases <Agreement> - At least for measurement procedure RAN4 RRM to define test cases for NR-SA for both FR1 and FR2-1 - For FR1 TCs, - all cells are in FR1 - For FR2 TCs - all cells in FR2-1 - FFS if all TCs also cover FR2-1 - Test cases are specified only for per-UE MGs ### Issue 1-1-2: Type 1 and Type 2 gaps <Agreement> - Test cases are defined only for Type 1 gaps ### Issue 1-1-3: DRX proposals <Agreement> - Each type of test case is specified for either DRX or non-DRX but not both.

But there remain some open issues related to test target and whether to define accuracy test cases for XR which need to be addressed before the agreed test cases can be developed.

### Issue 1-1-4: Test target <Agreement> Proposals 1, 2, and 3 are addressed in the same test case - Proposal 1: measurement delay extension with gap cancellation - Proposal 2: Reception/transmission without scheduling restrictions during cancelled gap occasions - Proposal 3: The UE processing timeline of receiving the DCI-based MG skipping indication <Way forward> Further discuss on the following proposals - Proposal 1: Further discuss on the UE processing timeline of receiving the DCI-based MG skipping indication - Proposal 2: Accuracy requirements - Proposal 3: Cancellation of scheduling restrictions due to L3 and L1 measurements without gaps during cancelled gaps

In the following, we provide our further views on the 3 proposals (Proposals 1-3) mentioned in the above way forward. Regarding Proposal 1, it is worth mentioning that DCI timeline is the most important requirement for XR. Hence, RAN4 must verify whether UE processing timeline of receiving the DCI-based MG skipping indication respects the corresponding capability (3/5 ms) or not.

RAN4 must verify the UE processing timeline of receiving the DCI-based MG skipping indication.

Regarding Proposal 2, we recommend that RAN4 should verify the accuracy requirements in XR test case(s). This is because it is possible that some UE implementation might not adapt to the measurement filter after gap skipping. For such scenarios, UE can easily satisfy the measurement delay requirement, but accuracy will be significantly degraded. In one example a UE implementation could not re-schedule the measurements in response to a measurement, and simply reduce the number of filtered samples of the measurement to keep the same measurement timeline. In that case, the UE would easily comply with all the measurement delay requirements, but due to the filtering reduction the accuracy would be degraded.

RAN4 should verify the accuracy requirements in XR test case(s).

Regarding Proposal 3, we recommend that RAN4 should verify that the UE can successfully transmit or receive data without scheduling restrictions during cancelled gap occasions. For this, we need to have non-fully overlapping L1 and L3 intra-freq without gaps on the cancelled gap.

RAN4 should verify cancellation of scheduling restrictions due to L3 and L1 measurements without gaps during cancelled gaps.

### Issue 1-1-5: List of test cases <Agreement> - Define test case for: - Inter-frequency measurements procedure with gaps - Define test case for: Inter-frequency L1-RSRP measurements procedure with MG for FR1 and FR2-1 - FFS whether to define at least one accuracy test case in FR1 and in FR2-1

In the paragraph before Proposal 2, we have justified the need to verify the accuracy requirements in XR test case(s). However, the number of such test cases should be as small as possible to minimize the RAN4 workload. Therefore, we recommend to RAN4 that the accuracy requirements should be tested in accuracy specific tests for which a single test case for each FR1 and FR2 should be sufficient.

Define at least one accuracy test case in FR1 and in FR2-1.

The accuracy requirements are tested in limited number of accuracy specific test, including

  - Inter-frequency measurements with Type 1 gaps, without DRX in FR1,

  - Inter-frequency measurements with Type 1 gaps, without DRX in FR2-1.

Test Case Configuration

### Issue 1-2-1: SSB time index detection <Way forward> Proposals - Proposal 1: For measurement procedure, define test cases without SSB time index detection. - Proposal 2: Use SSB index detection for intra-/inter-frequency measurement TCs, use no SSB index detection with lower priority for intra-/inter-frequency measurement TCs.

For the configuration of test cases, we think that all test cases may be defined without SSB index detection.

For measurement procedure, define test cases without SSB time index detection.

MG Cancellation Model

### Issue 1-3-1: Cancelation model for XR test cases: conditions for Type 1 gaps and Type 2 gaps <Way forward> Proposals for discussion - Option 1: Measurement gap cancelation is modelled as a fixed pattern for the test cases. - Option 2: Measurement gap cancelation is determined randomly by the test equipment.

As part of the Rel-16 NR-U performance work, a model for CCA failure was developed for the test cases. This model was developed because NR-U core requirements included limitation on the number of samples during a measurement cycle over which the requirements don’t apply.

The discussion on the conditions for measurement gap cancellation is still open for the RRM core requirements. In case the limitation on the maximum number of skipped occasions is not specified by RAN4, then the measurement cancellation model can be very simplified in comparison to the NR-U model.

NR-U CCA model for RRM test cases includes limitation on the maximum number of samples with non-successful CCA.

Measurement gap cancellation is determined randomly by the test equipment.

Our preference is for Option 2, but since some companies showed concerns on completely random gap cancellation, we propose a pseudo random pattern as a compromise. This pattern can be captured as a table, as shown in our companion CR. The test equipment can then start with a random position of the table for every new test iteration.

Measurement gap cancelation is modelled as a fixed pattern for the test cases, where

  - The pseudo random pattern is generated for at least 40 gap occasions.

  - For every test iteration, the TE starts at a random position of the pseudo random pattern.

### Issue 2-3-3: Cancelation ratio <Way forward> Proposals for discussion - Proposal 1: Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests. - Proposal 2: For Type 1 MGs test cases: - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset

We have provided simulation results of ratio of satisfied UEs Vs gap cancellation ratio in [8] and it is shown through analysis that the cancellation ratio should be in the order of 40% to 50% to achieve a good XR capacity. Additionally, a larger XR cancellation ratio is needed for UEs that supports Toffset = 5 ms in comparison to UE supporting Toffset = 3 ms. Therefore, considering our previous simulation results, we propose to adopt a cancellation ratio of 50% for UEs supporting Toffset =5 ms and 40% for UEs supporting Toffset = 3 ms as a baseline.

For Type 1 MGs test cases, RAN4 should agree on Proposal 2, i.e.,

  - Measurement gap cancellation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset.

  - Measurement gap cancellation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset.

Figure 1: Static scenario: Ratio of satisfied UEs vs skipping ratio for Alt 1-1 when TSO={1, 3, 5} ms. Ratio of satisfied UEs has been measured when cell load is equal to 7 UEs/cell (i.e., the XR capacity of ‘w/o MG’ when PDB=10ms).

DCI Aspects

### Issue 1-4-1: Toffset between DCI and start of MG <Way forward> Proposals for discussion - Option 1: The processing time 3ms/5ms shall be verified in the test cases. - Option 1a: Define a single test case that includes time offset of 5ms and 3ms with UE capability used as a condition - Option 2: Define test case for 5ms offset only.

Before RAN4 decides the Toffset between DCI and start of MG during the test, we would like to reiterate that DCI timeline is the most important requirement for XR. Hence, RAN4 must verify whether UE processing timeline of receiving the DCI-based MG skipping indication respects the corresponding capability (3/5 ms) or not.

The workload to define test cases for both 3ms offset and 5ms offset is the same as that for defining test cases for only 5ms offset as it is just a matter of configuration of the test case. Furthermore, to ensure that UEs declaring 3ms Toffset capability are respecting their declared processing timeline of receiving the DCI-based MG skipping indication, such UEs need to be tested with 3ms offset only. Otherwise, there will be no guaranteed minimum performance corresponding to 3ms capability. Therefore, we support Option 1.

The processing time 3ms/5ms shall be verified in the test cases.

Define a single test case that includes time offset of 5ms and 3ms with UE capability used as a condition.

### Issue 1-4-2: Types of DCI with cancellation indication <Way forward> Proposals for discussion - Option 1: In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancellation. - Option 2: Define RAN4 test cases using DCI 1-1 for the canceling indication.

Considering the types of DCI for test case definition, it is not clear why we need to cover more than 1 DCI type. From RRM functionality perspective we don’t define different UE behaviour depending on the DCI format. The most important aspect to be considered in this case is regarding UE capabilities. As shown below, the XR gap canceling feature includes a field for the UE to declare support of additional DCI, where formats 0_1 and 1_1 are mandatory.

enableTx-RxDuringMeasGap-r19 Indicates whether the UE supports reception/transmission during measurement gaps/scheduling restrictions is enabled based on explicit indication by DCI format 0_1 and DCI format 1_1. The capability signalling of this feature includes the following: - additionalDCI-r19 indicates additional supported DCI formats for enabling reception/transmission in gap/restriction. Value ‘dci0-1and1-2’ indicates DCI format 0_2 and DCI format 1_2, value ‘dci0-3’ indicates DCI format 0_3, value ‘dci1-3’ indicates DCI format 1_3; - indicationField-r19 indicates UE behaviour for indication field value of “0”. Value ‘option1’ means that, for explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. Value ‘option2’ means that, for explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 behaviour; - minimumTimeOffset-r19 indicates the minimum time offset from the end of PDCCH carrying the gap/restriction cancellation to the start of cancelled gap/restriction. Value ‘ms5’ indicates 5ms, value ‘ms3’ indicates 3ms. If a UE indicates support for the reception of the indication with DCI format 0_3 and/or 1_3, the UE supports reception of the indication on a band with that DCI format if the UE also indicates support for that DCI format on that band.

XR gap canceling feature includes a field for the UE to declare support of additional DCI, where formats 0_1 and 1_1 are mandatory.

Considering the RAN2 UE capability, it is proposed that only 1 mandatory DCI format is tested, and for simplification we propose to UE DCI 1-1.

Define RAN4 test cases using DCI 1-1 for the cancelling indication.

### Issue 1-4-3: Behaviour of Option 1 and Option 2 for indication of value ‘0’ <Way forward> Proposals for discussion - Proposal 1: RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. - Proposal 2: RAN4 to agree that UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Further details are as follows: - Test Option 2 in every test case. - For testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability.

Regarding the open issue of testing the behaviour of Option 1 and Option 2 for indication of value ‘0’, we think that the testing of Option 2 is expected in every test case. However, for testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability. This will ensure that test coverage is broad while keeping the similar workload.

Our companion CR includes how to test option2 as part of the gap cancellation model.

RAN4 to agree that UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Further details are as follows:

  - Test Option 2 in every test case.

  - For testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability.

Conclusion

The following Observations and Proposals were made:

Proposal 1: RAN4 must verify the UE processing timeline of receiving the DCI-based MG skipping indication.

Proposal 2: RAN4 should verify the accuracy requirements in XR test case(s).

Proposal 3: RAN4 should verify cancellation of scheduling restrictions due to L3 and L1 measurements without gaps during cancelled gaps.

Proposal 4: Define at least one accuracy test case in FR1 and in FR2-1.

Proposal 5: The accuracy requirements are tested in limited number of accuracy specific test, including

- Inter-frequency measurements with Type 1 gaps, without DRX in FR1,

- Inter-frequency measurements with Type 1 gaps, without DRX in FR2-1.

Proposal 6: For measurement procedure, define test cases without SSB time index detection.

Observation 1: NR-U CCA model for RRM test cases includes limitation on the maximum number of samples with non-successful CCA.

Proposal 7: Measurement gap cancellation is determined randomly by the test equipment.

Proposal 8: Measurement gap cancelation is modelled as a fixed pattern for the test cases, where

- The pseudo random pattern is generated for at least 40 gap occasions.

- For every test iteration, the TE starts at a random position of the pseudo random pattern.

Proposal 9: For Type 1 MGs test cases, RAN4 should agree on Proposal 2, i.e.,

- Measurement gap cancellation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset.

- Measurement gap cancellation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset.

Proposal 10: The processing time 3ms/5ms shall be verified in the test cases.

Proposal 11: Define a single test case that includes time offset of 5ms and 3ms with UE capability used as a condition.

Observation 2: XR gap canceling feature includes a field for the UE to declare support of additional DCI, where formats 0_1 and 1_1 are mandatory.

Proposal 12: Define RAN4 test cases using DCI 1-1 for the cancelling indication.

Proposal 13: RAN4 to agree that UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Further details are as follows:

- Test Option 2 in every test case.

- For testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability.

- References

- RP-250107, Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia, Qualcomm

- R4-2514869, WF on RRM requirements for NR_XR_Ph3, RAN4#116bis, Nokia

- 3GPP TS 38.133, Requirements for support of radio resource management (Release 19)

- R4-2416864, WF on RRM requirements for NR_XR_Ph3, RAN4#112-bis, Nokia

- R4-2420103, WF on RRM requirements for NR_XR_Ph3, RAN4#113, Nokia

- R4-2504911, WF on RRM requirements for NR_XR_Ph3, RAN4#114-bis, Nokia

- R4-2508285, WF on RRM requirements for NR_XR_Ph3, RAN4#115, Nokia

- R4-2512144, WF on RRM requirements for NR_XR_Ph3, RAN4#116, Nokia

- R4-2504505, Analysis of skip ratio on XR capacity and mobility performance, RAN4#114bis, Nokia