3GPP TSG-RAN4 Meeting #117 	R4-2522103

# Dallas, US, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR XR RRM performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of RRM test cases for measurement gap cancellation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1 – model for gap cancellation Change 2 – test case for LTM measuremets with gap cancellation in FR2 - TC on A.7.6.21.2 is used as a baseline Change 3 – Accuracy test case with gap cancellation in FR1 - TC on A.6.7.1.2.1-1 is used as baseline Whenever possible, repetition of tables of existing test cases is avoided, and references are given to the tables from the reference test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurement gap cancellation cannot be verified in conformance tests |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.x (new), A.7.6.y (new), A.6.7.1.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change #1>

## A.3.38	DL-PRS Measurement Time Window configurations

Table A.3.38-1: Reference configuration for DL-PRS Measurement Time Window

| DL-PRS Measurement Time Window Parameters | Unit | Values |  |
| --- | --- | --- | --- |
| Reference DL-PRS Measurement Time Window |  | MTW.1 | MTW.2 |
| Number of configuration elements |  | 1 | 1 |
| Start SFN |  | 0 | 0 |
| Periodicity | slot | Note 1 | Note 4 |
| Slot and Symbol Offset | slot | Note 2 | Note 2 |
| Duration | slot | 4 | 4 |
| Associated PFL index |  | 0 | 0 |
| Associated PRS resource set index |  | Note 3 | Note 3 |
| NOTE 1: Same as 2*(PRS resource set periodicity as used in the test case). NOTE 2: Same as PRS resource with smallest slot and symbol offset as used in the test case. NOTE 3: Include all PRS resource sets of all TRPs used in the test case. NOTE 4: Same as 2*(DRX cycle as used in the test case). |  |  |  |

## A.3.x	Measurement gap cancellation model

The measurement gap cancellation model is used in the RRM test cases verifying UE behaviour with gap cancellation.

The system simulator performs the following procedure for modelling cancellation of measurement gaps:

1>	Start a random position of Table A.3.x.1-1, n in the range [0, 39];

1>	For each measurement gap occasion

2>	If the nth entry in Table A.3.x.1-1 contains gap cancellation equal to ‘1’

3>	The next gap occasion is to be cancelled

3>	Send a DCI containing measurement gap cancellation indication on the last available PDCCH occasion X ms before the measurement gap, where X is described in clause 9.1.14.4.

2>	If the UE indicates support ‘option2’ in indicationField-r19 of enableTx-RxDuringMeasGap-r19, and the nth entry in Table A.3.x.1-1 contains gap cancellation equal to ‘0’

3>	The next gap occasion is not to be cancelled

3>	If the nth entry in Table A.3.x.1-1 contains change of DCI indication equal to ‘1’

4>	The system simulator starts sending DCI containing measurement gap cancellation indication 10 ms before the next measurement gap occasion

4>	The system simulator sends DCI without measurement gap cancellation indication on the last available PDCCH occasion X ms before the measurement gap, where X is described in clause 9.1.14.4

2>	otherwise

3>	The next gap occasion is not to be cancelled

3>	All DCI transmissions before the next gap occasion do not include indication of measurement gap cancellation

2>	the value of n is incremented, as mod( n+1, 40 )

Table A.3.x-1: Gap cancellation pattern

| Gap occasion # | Gap cancellation Note 1 |  | Change of DCI indication Note 2 |  |
| --- | --- | --- | --- | --- |
|  | TimeOffset = ms5 | TimeOffset = ms3 | TimeOffset = ms5 | TimeOffset = ms3 |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| 1 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| 2 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 3 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 4 |  |  | 0 |  | 0 |  | 1 |  | 1 |
| 5 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 6 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 7 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| 8 |  |  | 1 |  | 0 |  | N/A |  | 1 |
| 9 |  |  | 0 |  | 0 |  | 1 |  | 1 |
| 10 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 11 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| 12 |  |  | 0 |  | 0 |  | 1 |  | 1 |
| 13 |  |  | 0 |  | 0 |  | 1 |  | 1 |
| 14 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| 15 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 16 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| 17 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 18 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 19 |  |  | 1 |  | 0 |  | N/A |  | 1 |
| 20 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| 21 |  |  | 0 |  | 0 |  | 1 |  | 1 |
| 22 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 23 |  |  | 1 |  | 0 |  | N/A |  | 1 |
| 24 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| 25 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| 26 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 27 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 28 |  |  | 0 |  | 0 |  | 1 |  | 1 |
| 29 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 30 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 31 |  |  | 0 |  | 0 |  | 1 |  | 1 |
| 32 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| 33 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 34 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| 35 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| 36 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| 37 |  |  | 0 |  | 0 |  | 1 |  | 1 |
| 38 |  |  | 1 |  | 0 |  | N/A |  | 1 |
| 39 |  |  | 1 |  | 1 |  | N/A |  | N/A |
| NOTE 1: Table entries equal to ‘1’ indicate gap occasion that is cancelled, and table entries equal to ‘0’ indicate gap occasion that is not cancelled NOTE 2: Only used for UEs supporting indicationField-r19 = option2 |  |  |  |  |

# <End of Change #1>

# <Start of Change #2>

##### A.7.6.25.1.2	Test requirements

The DL RSCP with UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.8 with Nsample=4 for UE Rx-Tx time difference.

The UE shall perform and report the DL RSCP and UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified DL RSCP with UE Rx-Tx time difference measurement time specified in clause 9.9.8 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%. The reported DL RSCP measurement shall be within the DL RSCP reporting range specified in clause 10.1.44 and the reported UE Rx-Tx measurement shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.

### A.7.6.y	LTM Inter-frequency L1-RSRP measurement with measurement gap cancellation

#### A.7.6.y.1	Inter-frequency SSB-based L1-RSRP measurement with measurement gap cancellation for LTM in FR2

##### A.7.6.y.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of inter-frequency L1-RSRP measurement with MG cancellation for LTM. This test will partly verify the L1-RSRP measurement requirements in clause 9.15.5, with the testing configurations in tables A.7.6.21.1.1-1, A.7.6.21.1.2-1, A.7.6.21.1.2-2 and A.7.6.21.1.2-3.

The AoA setup of FR2 cell for this test is Setup 1 as defined in clause A.3.15.

Note: 	The parameters and configurations of this test follow the tables defined in A.7.6.21.2 as a baseline.

##### A.7.6.y.1.2	Test parameters

There are two cells in the test, Cell 1 is the PCell on NR RF channel number 1 and Cell 2 is a neighbor cell NR RF channel number 2. The test parameters for Cell 1 and Cell 2 are given in table A.7.6.21.1.2-1, table A.7.6.21.1.2-2 and table A.7.6.21.1.2-3.

The test consists of two successive time periods, with time duration of T1 and T2 respectively. SSB_RP of Cell 2 in T1 and T2 are different. Measurement gap is configured in the test.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on RF channel 1 (PCC).

-	A measurement object is configured for the RF channel 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used. Before the start of the T1, event is triggered, and UE has sent a measurement report for the Cell 2 with SSB Index.

-	UE is provided with LTM-Candidate-r18 for Cell 2.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

At the beginning of T2, SSB_RP of Cell 2 changes to a different value from T1.

During the entire duration of the test, the system simulator performs measurement gap cancellation in accordance with the measurement gap cancellation model described in A.3.x.

The TE schedules continuous DL data on PCell throughout the test.

##### A.7.6.y.1.3	Test Requirements

The UE shall send L1-RSRP report every 320 slots in T2. The UE shall start to report a larger L1-RSRP value of Cell 2 in no later than [TBD] ms plus 320 slots from the beginning of time period T2. UE shall send L1-RSRP report including the valid results for Cell 2 while meeting the accuracy requirements defined in clause 10.1.20B.

The reported L1-RSRP value shall include the Rx antenna gain in the range of -10 to +20 dB.

The rate of correct events observed during repeated tests shall be at least 90 %.

UE shall report corresponding HARQ-ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the non-scheduled measurement gap. UE is not allowed to cause any interruption outside the configured non cancelled  measurement gap occasions.

## A.7.7	Measurement Performance requirements

Unless explicitly stated otherwise:

-	Reported measurements shall be within defined range of accuracy limits defined in clause 10 for at least 90 % of the reported cases. If multiple measurement performance requirements are verified in the same test, the reported measurements for each requirement shall be within defined range of accuracy limits of the corresponding requirement defined in clause 10 for at least 90 % of the reported cases.

-	Measurements are performed in RRC_CONNECTED state.

-	The reference channels assume transmission of PDSCH with a maximum number of 5 HARQ transmissions unless otherwise specified.

# <End of Change #2>

# <Start of Change #3>

##### A.6.7.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil the absolute requirement in clause 10.1.4.1.1 and relative requirement in clause 10.1.4.1.2.

A.6.7.1.3	Void

#### A.6.7.1.z	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell with measurement gap cancellation

##### A.6.7.1.z.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits when measurement gap cancellation is applied.

Note: 	The parameters and configurations of this test follow the tables defined in A.6.7.1.2 as a baseline.

This test will verify the requirements in clauses 10.1.4.1.1 and 10.1.4.1.2 for inter-frequency measurements with the testing configurations for NR cells in table A.6.7.1.2.1-1.

##### A.6.7.1.z.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on a different frequency than the PCell. The test parameters for the Cell 1 and Cell 2 are given in table A.6.7.1.2.2-1. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in table A.6.7.1.2.2-1. The inter-frequency measurements are supported by a measurement gap.

During the entire duration of the test, the system simulator performs measurement gap cancellation in accordance with the measurement gap cancellation model described in A.3.x.

The system simulator schedules continuous DL data on PCell throughout the test.

##### A.6.7.1.z.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil the absolute requirement in clause 10.1.4.1.1 and relative requirement in clause 10.1.4.1.2.

UE shall report corresponding HARQ-ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the non-scheduled measurement gap. UE is not allowed to cause any interruption outside the configured non cancelled  measurement gap occasions.

### A.6.7.2	SS-RSRQ

#### A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.7.1.1.

# <End of Change #3>