3GPP TSG-RAN WG4 Meeting #117                                           R4-2522131

# Dallas, TX, USA, November 17-21, 2025

Title:	Qualcomm views on 6G Channel bandwidth

Source:	Qualcomm Incorporated

Agenda item:	8.2.3

Release:	Rel-20

SI: 	FS_6G_Radio

Document for:	Approval

# 1	Introduction

In RAN4#116bis a WF was agreed for system parameters [7]. In the WF, discussion on the maximum channel BW feasibility was encouraged. In this paper we discuss feasibility to support 400 MHz channel BW. WF also encouraged to submit simulation assumptions for the Spectral utilization. In this paper we propose those assumptions and discuss some principles how to define the guardbands.

# 2	Discussion

## 2.1	FFT Size and Channel Bandwidth

In RAN4#116bis there was an initial discussions on the maximum FFT size and channel bandwidth. Some high level agreements were captured in [7]. The main points for discussion were the feasibility of 8k and/or 16k FFT size and corresponding channel bandwidths of 200/400MHz.

The FFT operation is generally performed with a radix-2 butterfly algorithm which iteratively splits an FFT into two half size FFTs and then merges them through multiply and add to calculate the final output. The algorithm complexity is in the order of N*log(N). In the case of 16k FFT, the computational complexity is ~7% higher compared to performing two 8k FFT operations. Considering that the FFT operation is only a small part of the overall baseband complexity, this increase is marginal or even negligible.

Handling a wider bandwidth with a CA like approach (2x8k FFT instead of 16k FFT) brings additional complexity in filtering, processing multiple control channels, PDSCH transport blocks, etc. This added complexity could be higher compared to a larger FFT operation.

Observation 1: The added complexity for 16k FFT is marginal, hence, supporting 16k FFT in the 6G timeframe is feasible.

400MHz channel bandwidth can be supported with 16k FFT and 30kHz SCS. Such bandwidth is already supported in some NR bands, hence, it should be feasible to support in the 6G timeframe.

Proposal 1: Supporting 16k FFT and/or 400MHz channel bandwidth in the 6G timeframe is feasible.

## 2.2	Spectral utilization

In previous meeting an WF [7] for Spectrum Utilization was agreed two main points, agree simulation assumptions and evaluate RF performance.

For simulation assumptions, we propose to keep the lengths of time domain spectral confinement techniques to 1 / 4  or 1 / 3 of the CP length and that the used lengths are declared with the simulation results. This should not be hard limit and if longer lengths are used, then evaluation should consider used SCS and the delay spread of the frequency. We also propose that this may not be constant but may change depending on the RB allocation and position.

For LO leakage and IQ image we propose to use the value 28 dBc as is specified in clause 6.4 of TS 38.101-1.

Simulation should consider all requirements, ACLR, SEM, EVM, IBE and state clearly the guardband size with the channel bandwidth. Simulation should consider all possible allocation sizes for the chosen channel BW and not just look at the narrow allocation at the edge. Starting point should be clauses 5.3.2 and 5.3.3 of TS 38.101-1.

In summary:

Proposal 2:

- Use mainly 0.25 – 0.3 of the CP length for timedomain confinement techniquest and discuss delay spread handling if the length is longer

- LO leakage and IQ image use 28 dBc as in clause 6.4 of TS 38.101-1

- Consider all requirements ACLR, SEM, EVM and IBE

- Report guardband size with the channel bandwidth

- Analyse all allocation sizes

- Starting point are in clauses 5.3.2 and 5.3.3 of TS 38.101-1 and aim to improve these

Further, RAN4 should discuss what would be the approach to define the guard band. There are two principle that can be derived from physics:

- Define guard band so that the relative % that the guard band occupies from the channel BW is the same for all channel bandiwidths

- Define guard band to be the same absolute value for all channel bandwidths

The option 1 is physical since it assumes the spectral confinement is equally challenging e.g. in case a frequency domain filter, the filter order remains the same or in case of Wola, the length is the same for all allocation and bandwidths.

The option 2 is physical since the same assumptions would be used for the edge RBs. In this case, it is assumed that the wola length or the filter width is changed based on the allocation and it is always the same for the edge RBS regardless of the channel BW or NRB.

Observation 2: RAN4 to discuss principles how to define the guard band considering the implementation aspects.

# 3 	Conclusions

For channel BW and related aspects we concluded that:

Observation 1: The added complexity for 16k FFT is marginal, hence, supporting 16k FFT in the 6G timeframe is feasible.

Proposal 1: Supporting 16k FFT and/or 400MHz channel bandwidth in the 6G timeframe is feasible.

For spectral utilization we concluded that:

Proposal 2:

- Use mainly 0.25 – 0.3 of the CP length for timedomain confinement techniquest and discuss delay spread handling if the length is longer

- LO leakage and IQ image use 28 dBc as in clause 6.4 of TS 38.101-1

- Consider all requirements ACLR, SEM, EVM and IBE

- Report guardband size with the channel bandwidth

- Analyse all allocation sizes

- Starting point are in clauses 5.3.2 and 5.3.3 of TS 38.101-1 and aim to improve these

Observation 2: RAN4 to discuss principles how to define the guard band considering the implementation aspects.

# 4.	References

[1]	RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2]	TR 38.914, “Study on 6G Scenarios and Requirements”, v0.1.0, June 2025

[3]	R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple), 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

[4]	R1-2506216, “Overview of 6GR air interface”, Qualcomm, 3GPP TSG RAN WG1 #122, Bengaluru, India, Aug 25th – 29th, 2025

[5] 	R1-2506218, Waveforms for 6GR, RAN1#122, 25th-29th August 2025

[7]	R4-2514650, “WF on 6G system parameter”, Feature lead (Huawei, HiSilicon), 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, Oct 13 – 17, 2025