3GPP TSG-RAN WG4 Meeting #117                                         R4-2522135

Dallas, USA, Nov 17 – 21, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on HPUE RF requirements for NR single carrier operation

Agenda Item:	7.1.4

Document for:	Approval

# Introduction

For the HPUE objectives in the phase 5 UE RF enhancements WID [1], a WF [2] was agreed in the last meeting. There’re a number of open issues to be discussed, including: power class definition, antenna isolation, ACLR, MPR and etc.

# Discussion

## 4x26dBm for CPE/FWA UEs on TDD bands

In the last meeting, there were concerns that 0dB MPR relative to 32dBm MOP might not be achieved for 4x26dBm architecture. We think this should not prevent the MOP from being defined as 32dBm. As seen from the current spec, PC1.5 with 4Tx has minimum MPR=2dB and 0.5dB for the inner RB region for antenna isolation of 10dB and 20dB, respectively.

Moreover, MPR is an allowance. It’s defined by calibrating the PA to operate at a very specific point and barely meet the emission requirements. Practical UEs usually work at more linear operating point and can often use less MPR.

Table 6.2D.2-4: Maximum power reduction (MPR) for power class 1.5 with 4 Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 8.0 | ≤ 3.0 | ≤ 2.0 |
|  | QPSK | ≤ 8.5 | ≤ 3.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 8.5 | ≤ 4.0 | ≤ 2.5 |
|  | 64 QAM | ≤ 8.5 | ≤ 4.7 | ≤ 4.5 |
|  | 256 QAM | ≤ 9.5 | ≤ 7.0 | ≤ 7.0 |
| CP-OFDM | QPSK | ≤ 9.5 | ≤ 5.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 9.5 | ≤ 5.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 7.0 | ≤ 7.0 |
|  | 256 QAM | ≤ 9.5 | ≤ 9.5 | ≤ 9.5 |
| NOTE 1: This table is targeted to vehicular UE or other industrial device form factor with 10dB antenna isolation. |  |  |  |  |

Table 6.2D.2-5: Maximum power reduction (MPR) for power class 1.5 with 4 Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 7.5 | ≤ 2.0 | ≤ 0.5 |
|  | QPSK | ≤ 8.0 | ≤ 2.5 | ≤ 0.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 3.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 4.0 | ≤ 3.5 |
|  | 256 QAM | ≤ 8.0 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 8.0 | ≤ 4.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 8.0 | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 5.0 | ≤ 5.0 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ 8.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Observation 1: Existing MPR requirements for PC1.5 with 4Tx are >0dB. It does not prevent a practical UE from reaching the MOP (29dBm).

As per the WID [1], 1x29dBm and 2x29dBm will be studied from 2026Q4. With the reduction of the number of Tx, the MPR could be decreased, as seen from PC1.5 with 2Tx.

Observation 2: MPR for 2Tx could be smaller than that for 4Tx as seen from PC1.5. It’s possible that 2x29dBm architecture (from 2026Q4) could have 0dB MPR relative to 32dBm MOP.

Proposal 1: The MOP for 4x26dBm is defined as 32dBm, and there’s no need to wait for the exact ACLR number.

Compared with the existing PC1 power class, it’s expected that the new architecture of 4x26dBm will demand different MOP (32dBm), ACLR (<37dB) as well as MPR. For clarity, it’s better to define a new power class such as PC1bis to indicate distinct RF requirements as well as different hardware assumptions.

Proposal 2: It’s better to define a new power class for 4x26dBm with new set of RF requirements including MOP, ACLR, MPR and etc.

The CPE/FWA devices can have different physical sizes to meet different consumers’ demand. The existing PC1.5 with 4Tx indeed supports both 10dB and 20dB antenna isolation.

Proposal 3: Follow PC1.5 with 4Tx, study and define MPR requirements for both 10dB and 20dB antenna isolation for 4x26dBm architecture.

Proposal 4: Follow Rel-18 agreement that supporting 4Tx is release independent from Rel-18.

## 2Tx PC1.5 for both handheld and FWA UEs on FDD bands

It’s more challenging to achieve good antenna isolation for sub-1GHz bands, especially for handheld UEs. On the other hand, antenna isolation is not the only factor that affects MPR. Overall, we think it’s feasible to reuse the existing PC1.5 MPR requirements for FDD bands.

Proposal 6: It’s feasible to reuse the existing PC1.5 MPR requirements for 10dB and 20dB antenna isolation.

In the course of adding PC2 support for FDD bands in Rel-17, little interest was expressed by operators on the protection of UTRA services. As a result, the UTRA ACLR for PC2 is defined the same as for PC3. The same can be used for PC1.5 if no concerns from operators.

Proposal 7: For UTRA ACLR, the same requirement for PC2 can be reused if no concerns from operators.

Table 6.5.2.4.2-1: UTRA ACLR requirement

|  | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| UTRAACLR1 | 33 dB | 33 dB | 33 dB |
| UTRAACLR2 | 36 dB | 36 dB | 36 dB |

The 2Tx PC2 RSD for n25 has been specified as follows:

Table 7.3.2-1d Reference Sensitivity Degradation from PC3 to PC2 for
FDD bands for UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 7 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n25 |  | 1.5 |  | 1.5 | 1.6 | 1.6 | 1.7 | 6.0 | 6.2 | 6.7 | 7.1 |  |

For 2Tx PC1.5, an extra 3dB can be added on top of the above PC2 RSD.

Proposal 8: The 2Tx PC1.5 RSD for n25 can be defined as 2Tx PC2 RSD + 3dB.

# Conclusion

In this paper, we have shared further analysis on the HPUE requirements for 4x26dBm and 2Tx PC1.5. The following observations and proposals are made:

4Tx FWA with 32dBm MOP

Observation 1: Existing MPR requirements for PC1.5 with 4Tx are >0dB. It does not prevent a practical UE from reaching the MOP (29dBm).

Observation 2: MPR for 2Tx could be smaller than that for 4Tx as seen from PC1.5. It’s possible that 2x29dBm architecture (from 2026Q4) could have 0dB MPR relative to 32dBm MOP.

Proposal 1: The MOP for 4x26dBm is defined as 32dBm, and there’s no need to wait for the exact ACLR number.

Proposal 2: It’s better to define a new power class for 4x26dBm with new set of RF requirements including MOP, ACLR, MPR and etc.

Proposal 3: Follow PC1.5 with 4Tx, study and define MPR requirements for both 10dB and 20dB antenna isolation for 4x26dBm architecture.

Proposal 4: Follow Rel-18 agreement that supporting 4Tx is release independent from Rel-18.

2Tx PC 1.5 for FDD

Proposal 6: It’s feasible to reuse the existing PC1.5 MPR requirements for 10dB and 20dB antenna isolation.

Proposal 7: For UTRA ACLR, the same requirement for PC2 can be reused if no concerns from operators.

Table 6.5.2.4.2-1: UTRA ACLR requirement

|  | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| UTRAACLR1 | 33 dB | 33 dB | 33 dB |
| UTRAACLR2 | 36 dB | 36 dB | 36 dB |

Proposal 8: The 2Tx PC1.5 RSD for n25 can be defined as 2Tx PC2 RSD + 3dB.

# References

[1] RP-252952, “New WID: UE RF enhancements for NR FR1, Phase 5”, Moderator (vivo), TSG RAN#109

[2] R4-2515087 Way Forward for [116bis][316] NR_UE_RF_Ph5, Nokia, Murata, RAN4#116bis