3GPP TSG-RAN WG4 Meeting #117	R4- 2522138

Dallas, USA, Nov 17 – 21, 2025

Agenda Item:	4.20.1.1

Source: 	Huawei, HiSilicon

Title:	Discussion on test configuration and RMC for AIoT devices

Document for:	Approval

# 1	Introduction

With the completion of core RF/RRM requirements, the TS 38.191 for Ambient IoT device was approved at RAN#109 [1]. There’re some open issues related to test configuration and RMC yet to be concluded. Some of them are captured in the WF [2].

# 2	Discussion

### 2.1 Test configuration for backscatter power

As agreed previously, for the measurement of the D2R signal power, only the 1st lower/upper sideband around Fc+/-fSFS are included. How to capture this requirement into the test configuration need to be decided.

Assuming the D2R signal is received and down-converted to baseband. The sub-carrier frequency corresponding to the Small Frequency Shift (SFS), the bandwidth of the sideband, the lower frequency edge of the sideband and the upper frequency edge of the sideband are denoted as fSFS, 1SB, fl and fh, respectively. And the nominal bit duration and the SFS factor are denoted as Tb and RSFS, respectively.

The lower/upper edge frequency of the upper sideband can be obtained as follows:

fl=fSFS-1SB2,  fh=fSFS+1SB2

Assume that the Sample Frequency Offset (SFO) is -0.1<= SFO <=0.1. The frequency characteristics of the upper sideband can be analysed as follows for three cases: no frequency error, sample frequency higher/lower than the nominal frequency.

Table 1: Frequency Characteristics for D2R upper sideband

|  | Sub-carrier fSFS | 1SB | Lower edge fl | Upper edge fh |
| --- | --- | --- | --- | --- |
| No Frequency Error (SFO=0) | RSFSTb | 2Tb | RSFS-1Tb | RSFS+1Tb |
| Higher Sample Frequency (SFO>0) | RSFSTb(1+SFO) | 2Tb(1+SFO) | RSFS-1Tb(1+SFO) | RSFS+1Tb(1+SFO) |
| Lower Sample Frequency (SFO<0) | RSFSTb(1-¦SFO¦) | 2Tb(1-¦SFO¦) | RSFS-1Tb(1-¦SFO¦) | RSFS+1Tb(1-¦SFO¦) |

The three cases can be illustrated by Figure 1 below.

DC

fSFS

- fSFS

fSFS  -fl

fSFS  +fh

-fSFS  +fh

-fSFS  -fl

- SFO=0

b) SFO>0

c) SFO<0

Figure 1: Illustration of D2R measurement filters for different SFOs

The spectrum of a simulated D2R signal is shown in Figure 2 below.

Figure 2: PSD of D2R signal with SFO

In order to cover the worst case for both the lower edge and the upper edge of the sideband (as highlighted in Table 1), the parameters for the measurement filter are recommended as follows.

- Use the test configuration in Table 2 for D2R measurement.

Table 2: Test configuration for D2R measurement

| Measurement parameter | Value |
| --- | --- |
| CW frequency (FCW) | According to TS 38.192 |
| Filter passband range for D2R upper sideband | FCW+RSFS-1Tb*0.9 to FCW+RSFS+1Tb*1.1 |
| Filter passband range for D2R lower sideband | FCW-RSFS+1Tb*1.1 to FCW-RSFS-1Tb*0.9 |
| Note 1: The power at the carrier frequency is excluded from the D2R power. |  |

### 2.1 RMC for D2R

It has been agreed that the following D2R RMC is used for SEM and spurious emission tests. The remaining issue is to confirm whether it’s also used for backscatter loss verification.

Table 3: FRC for [backscatter power, SEM and] spurious emissions

| Parameter | Unit | Value |
| --- | --- | --- |
| Transmission BW | kHz | 15 |
| TBS | Bits | Depending on the length of AIoT device ID |
| CRC | Bits | 16 |
| FEC code rate |  | 1/3 |
| Block repetition number |  | 1 |
| Preamble length | Bits | 31 |
| Midamble length | Bits | 31 |
| Interval for midamble insersion | Bits | 48 |
| Additional midamble insertion |  | No |
| Small frequency shift | kHz | 480 |
| Modulation |  | BPSK/OOK (NOTE1) |
| NOTE 1: The modulation scheme used is up to device implementation. |  |  |

It has been clarified that the maximum allowable backscatter loss requirements as defined in Clause 6.2.1 are applicable for D2R channel bandwidth. The question for conformance tests is whether all D2R CBWs or more than one CBW should be verified. Due to very limited feedback from device/TE vendors at this stage, the answer to the question is unclear.

- The same D2R RMC used for testing SEM and spurious emission requirements can also be used to test backscatter power requirements. Additional RMCs can be added in the future if the need is identified.

# 3	Conclusion

In this paper, we have shared the analysis on the test configuration and RMC channels needed for conformance tests. The following proposals are made:

Proposal 1: Use the test configuration in Table 2 for D2R measurement.

Table 2: Test configuration for D2R measurement

| Measurement parameter | Value |
| --- | --- |
| CW frequency (FCW) | According to TS 38.192 |
| Filter passband range for D2R upper sideband | FCW+RSFS-1Tb*0.9 to FCW+RSFS+1Tb*1.1 |
| Filter passband range for D2R lower sideband | FCW-RSFS+1Tb*1.1 to FCW-RSFS-1Tb*0.9 |
| Note 1: The power at the carrier frequency is excluded from the D2R power. |  |

Proposal 2: The same D2R RMC used for testing SEM and spurious emission requirements can also be used to test backscatter power requirements. Additional RMCs can be added in the future if the need is identified.

# References

- RP-252811, “Presentation of Specification: TS 38.191, Version 1.1.0”, RAN4, TSG RAN#109

- R4-2515156 WF on [116bis][326] A-IoT_Maintenance, CMCC, RAN4#116bis