3GPP TSG-RAN4 Meeting #117	R4-2522146

# Dallas, US, November 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 6129 | rev |  | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of reference measurement channels for time-frequency interleaving demod test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_Ph2-Perf |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It has been agreed to introduce new reference measurement channels for time-frequency interleaving demod test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing new reference measurement channels for time-frequency interleaving |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreed demod tests can not be defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.1, A.3.8.1 FDD |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==== Start of Change ====

### A.3.1.1	Overview of DL reference measurement channels

Table A.3.1.1-1O: Overview of DL reference measurement channels (PMCH Performance)

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.8.1-1 | R.40 FDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-1 | R.37 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-2 | R.38 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-3 | R.39-1 FDD | 5 | 64QAM | 2/3 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-3 | R.39 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-4 | R.81-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-4 | R.81-2 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-5 | R.82-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-6 | R.83-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-6 | R.83-2 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-7 | R.84-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-8 | R.85-1 FDD | 3 | QPSK | 1/3 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-8 | R.85-2 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-8 | R.85-3 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-1 FDD | 10 | 64QAM | 0.48 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-2 FDD | 10 | 64QAM | 0.52 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-3 FDD | 6 | 64QAM | 0.49 | 30 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-4 FDD | 7 | 64QAM | 0.48 | 35 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-5 FDD | 8 | 64QAM | 0.48 | 40 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-6 FDD | 6 | 64QAM | 0.54 | 30 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-7 FDD | 7 | 64QAM | 0.53 | 35 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-8 FDD | 8 | 64QAM | 0.52 | 40 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-10 | R.107 FDD | 10 | 16QAM | 0.46 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-10 | R.107-1 FDD | 6 | 16QAM | 0.46 | 30 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-10 | R.107-2 FDD | 7 | 16QAM | 0.46 | 35 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-10 | R.107-3 FDD | 8 | 16QAM | 0.46 | 40 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-11 | R.108-1 FDD | 8 | 16QAM | 0.41 | 40 |  |  |  |
| FDD | Table A.3.8.1-11 | R.108-2 FDD | 8 | QPSK | 0.3 | 40 |  |  |  |
| TDD |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.8.2-1 | R.40 TDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-1 | R.37 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-2 | R.38 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-3 | R.39-1 TDD | 5 | 64QAM | 2/3 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-3 | R.39 TDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |

==== End of Change ====

==== Start of Change ====

Table A.3.8.1-11: Fixed Reference Channel for subcarrier spacing 1.25 kHz with LTE based 5G terrestrial broadcast MBMS dedicated cell with time-frequency interleaving

| Parameter | PMCH |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.108-1 FDD | R.108-2 FDD |
| Channel bandwidth | MHZ | 8 | 8 |
| Subcarrier spacing | kHz | 1.25 | 1.25 |
| Allocated resource blocks |  | 39 | 39 |
| Allocated slots per 40 ms (Note 1) |  | 32 | 39 |
| Time domain spreading and scaling (M, N) |  | (8, 4) | (32, 16) |
| Frequency domain interleaving |  | Enabled Codebook cycling disabled | Enabled Codebook cycling disabled |
| Modulation |  | 16QAM | QPSK |
| Target coding rate |  | 0.41 | 0.3 |
| Information bit payload (Note 2) |  |  |  |
| For each slot | Bits | 31704 | 45352 |
| Number of code blocks per subframe |  | 6 | 6 |
| Binary channel bits per subframe |  |  |  |
| For each slot | Bits | 19200 | 9600 |
| Transport block size before scaling with N | Bits | 7992 | 2825 |
| Transport block size after scaling with N | Bits | 31704 | 45353 |
| MBMS UE category |  |  |  |
| Note 1: First subframe every 40 ms is allocated for non-MBMS transmission Note 2: Zero OFDM symbols are reserved for PDCCH and no CRS allocated as per TS 36.211 [4] Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 bits is allocated to each Code Block (otherwise L = 0 bits) |  |  |  |

==== End of Change ====