3GPP TSG-RAN4 Meeting #117		R4-2522150

# Dallas, TX, USA 17th – 21st November, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM Core Requirements for AI/ML Beam Management

Agenda item:	6.11.4

Document for:	Discussion

# Introduction

This paper focuses on RRM core requirements for AI/ML based beam management.

# Discussion

## 2.2. Definition of Relative Accuracy

During one of the previous meetings, RAN4 decided to define requirements for relative RSRP accuracy through the following agreement.

Issue 2-9: Metrics/KPIs for beam ID prediction Agreement: - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided.

RAN4 defined relative RSRP accuracy in the following way in a previous meeting:

Previous Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Above agreement contains two FFS points.

- First, it is open if reported L1-RSRP of beam index n can be measured L1-RSRP, too.

  - Note: Beam index n owns the largest reported L1-RSRP value.

- Second, it is open if the above definition applies for spatio-temporal beam prediction.

First, in this work item, UE measures properties of setB beams and reports properties of top beams of setA beams. SetB can be a subset of setA, e.g., in narrow-to-narrow beam prediction scenario, and the beam index n owning the largest reported L1-RSRP value can be an element of setB. In this scenario, the reported L1-RSRP of beam index n can come from measured L1-RSRP. However, UE still reports it as predicted L1-RSRP of beam index n. It does not matter whether UE calculates this predicted L1-RSRP of beam index n based on measured L1-RSRP of the same beam. Hence, the definition of relative L1-RSRP accuracy can simply focus on “predicted L1-RSRP” and remove the “measured RSRP” related phrase.

The above definition of relative L1-RSRP accuracy can apply to spatio-temporal beam prediction, in general. However, the key question is whether the reported L1-RSRP across two different time instances will be considered in the definition of spatio-temporal beam prediction.

In spatio-temporal beam prediction, UE predicts and reports properties (L1-RSRP and beam ID) of predicted beams for up to eight prediction instances. Channel may vary significantly across these eight prediction instances. Hence, it would not make sense to compare reported L1-RSRP across two different prediction instances to define requirement. So, the previous definition of relative L1-RSRP accuracy can be applied to case 2 where predicted and ground truth L1-RSRP of both beam index i and n are obtained at the same time instance.

Observation 1: Beam index n, owning the largest reported value, can be an element of setB. However, UE reports it as predicted L1-RSRP while reporting it as part of a top beam of setA. It does not matter whether the reported and predicted L1-RSRP of this beam is exactly equal to the measured L1-RSRP of this beam.

Observation 2: Channel may vary significantly across different prediction time instances. It is not meaningful to compare relative accuracy of reported L1-RSRP across two different prediction time instances.

Proposal 1: RAN4 uses the agreed baseline for relative L1-RSRP of case 1 with the following changes:

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i – reported predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

    - Reported L1-RSRP can be predicted,

    - FFS whether reported L1-RSRP can be measured RSRP

Proposal 2: Define relative L1-RSRP accuracy in the following way for case 2:

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i at time instance t –  predicted L1-RSRP of beam index n at time instance t) -  (ground truth of L1-RSRP of beam index i  at time instance t- ground truth of L1-RSRP of beam index n at time instance t), where the beam index n owns the largest reported value at time instance t.

## 2.2 Evaluation Period of Spatio-Temporal Beam Prediction

RAN4 made the following agreement during the last meeting [21].

Issue 2-1: Measurement period for inference Agreement: observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report) For case 1 reuse M,N,P from legacy measurement requirements For case 2, use T*M, N P (M,N,P same as legacy measurement requirement) FFS on T value, T can also be 1. T can also be based on capability This observation period is to be used for the prediction delay requirement Issue 2-1: Measurement period for inference – case 2 Agreement: Measurement period for inference – case 2 K(number of samples)*[M]*N*P K as defined by RAN1

During the last meeting, RAN4 agreed most of the aspects of measurement period for case 2. The only remaining aspect is whether M will be considered in case 2 measurement period or not. Note that, the value of M is 1 when TimeRestrictionForChannelMeasurement IE is configured and it is 3 otherwise.

According to last meeting’s agreement, UE will get K samples per RX beam to evaluate for case 2. UE does not need 3 * K samples per RX beam to generate inference for case 2.

K * N * P can make the measurement period of case 2 quite long. An additional factor of 3 does not need to be considered in case 2 measurement period.

Observation 3: the value of M is 1 when TimeRestrictionForChannelMeasurement IE is configured and it is 3 otherwise.

Observation 4: K * N * P can make the measurement period of case 2 long. An additional scaling factor of 3 will make case 2 measurement period even longer.

Proposal 3: RAN4 does not consider M in case 2 measurement period, i.e., measurement period for inference in case 2 is equal to K * N * P.

# Conclusions

Observation 1: Beam index n, owning the largest reported value, can be an element of setB. However, UE reports it as predicted L1-RSRP while reporting it as part of a top beam of setA. It does not matter whether the reported and predicted L1-RSRP of this beam is exactly equal to the measured L1-RSRP of this beam.

Observation 2: Channel may vary significantly across different prediction time instances. It is not meaningful to compare relative accuracy of reported L1-RSRP across two different prediction time instances.

Observation 3: the value of M is 1 when TimeRestrictionForChannelMeasurement IE is configured and it is 3 otherwise.

Observation 4: K * N * P can make the measurement period of case 2 long. An additional scaling factor of 3 will make case 2 measurement period even longer.

Proposal 1: RAN4 uses the agreed baseline for relative L1-RSRP of case 1 with the following changes:

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i – reported predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

    - Reported L1-RSRP can be predicted,

    - FFS whether reported L1-RSRP can be measured RSRP

Proposal 2: Apply proposal 1 to case 2 where predicted and ground truth L1-RSRP of both beam index i and n correspond to the same time instance. In other words, define relative L1-RSRP accuracy in the following way for case 2:

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i at time instance t –  predicted L1-RSRP of beam index n at time instance t) -  (ground truth of L1-RSRP of beam index i  at time instance t- ground truth of L1-RSRP of beam index n at time instance t), where the beam index n owns the largest reported value at time instance t.

Proposal 3: RAN4 does not consider M in case 2 measurement period, i.e., measurement period for inference in case 2 is equal to K * N * P.

# References

- R4-2414446, ‘AI/ML ad-hoc meeting minutes’, Moderator (Qualcomm)

- R4-2414323, ‘WF on AI/ML’, Qualcomm

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #117.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #118.

- 38.133, Requirements for support of radio resource management, V 17.11.0.

- R4-2417212, WF on Requirements for AI/ML air interface, 3GPP TSG-RAN WG4 #112bis.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 #118bis.

- R4-2420513, WF on AI/ML air interface part 1, RAN4 #113.

[9] R4-2502856, “WF on the requirements for AI/ML air interface”, 3GPP TSG-RAN WG4 Meeting#114

[10] R4-2505105, “WF on requirements for AI/ML air interface normative work”, 3GPP TSG-RAN WG4 Meeting#114bis

[11] 3GPP TSG WG RAN1 Chairman’s notes, RAN1 #120bis

[12] R4-2502856, WF on the requirements for AI/ML air interface, 3GPP RAN WG4 Meeting#114

[13] R1-2504992, “Introduction of AI/ML BM Prediction”, 3GPP TSG-RAN WG#121

[14] R4-2511890, “WF on AI-ML air interface”,  3GPP TSG RAN WG4 #116.

[22] 3GPP TSG WG RAN1 122 Chairman’s notes.