3GPP TSG-RAN4 Meeting #117	R4-2522153

# Dallas, Texas, USA 17th – 21st November, 2025

Source: 	Qualcomm Incorporated

Title: 	AI-ML based CSI Prediction: Simulation Results

Agenda item:	6.11.8

Document for:	Discussion

# Introduction

In this paper, we present simulation results for AI-ML CSI prediction.

# Simulation Framework

During the last few meetings, RAN4 agreed to a diverse set of options to simulate the performance of AI-ML based CSI prediction. Companies were requested to submit results in terms of SGCS and throughput during RAN4 116 [1]-[3]. This contribution focuses on those options and contains SGCS centric simulation results. The simulation results have been re-submitted from one of our previous meeting’s contribution [5].

Table 1 shows the set of simulation assumptions that were used to generate result for this contribution.

Table 1: Updated simulation assumptions for AIML based CSI prediction [6]

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Subcarrier spacing | 15kHz for FDD, 30kHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol for step-1 and step-2 simulation |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| gNB TX antennas | 16(N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 with XP medium correlation as baseline |
| Doppler spread | 20Hz, 50Hz |
| Channel estimation for CSI prediction | Step-1: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Step-2: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., to check SGCS after CSI feedback). Step-3: - Practical DL channel estimation, use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| CSI-RS configuration | For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5). - For 32Tx Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0, l1); (5, 12). |
| Rank per UE | Step-1: Rank 1 and 2 to do quick check Step-2 and Step-3: Rank 2 as baseline |
| CSI feedback assumption | Step-1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account |
|  | Step-2 and Step-3: take CSI feedback into account - Use Rel-18 eTypeII-Doppler, N4 = 1， N4>1 can be further discussed in the future if needed |
| Test setup(timeline) | Only for step-3 For FDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 4 slots Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4) - Test setup for FDD test setup with Periodic CSI-RS configuration is captured in Table2 For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note : If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3 FFS test setup for aperiodic CSI-RS configuration |
| Beam steer approach | For step-3, randomization of the dual-cluster beam directions should be used as specified in AnnexB.2.3.2.3A in 38.101-4. The value of relative power ratio (p) shall be fixed as 1 during the test |
| SNR assumption | Only for step-3 Claimed by companies for step-3 first round simulation (TP as KPI) FFS: other suggestions |
| CSI feedback overhead | PC7 as baseline |
| Baseline | FFS - Random PMI with Rel-15 Type I single panel codebook if needed (e.g., for Step-3 or performance requirements) |
| KPI | Step-1: - check SGCS1 before CSI feedback - SGCS1 is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), Step-2: - check SGCS2 and SGCS3 - SGCS2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD SGCS3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2) Step-3: - check SGCS1 and SGCS3, under practical channel estimation, corresponding to 90% of maximum TP with follow PMI using AI/ML-based CSI prediction - SGCS1: Compares the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - SGCS3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2) - check TPs after CSI feedback for both AI based CSI prediction and Random PMI with Rel-15 Type I single panel codebook, as well as the SNR value corresponding to 90% of maximum TP of the AI based CSI prediction γ1_AI/ML: AI/ML based CSI prediction is compared with Rel-15 Type I random PMI - check TPs after CSI feedback for both AI based CSI prediction and Rel-16 eType II codebook(sample and hold), as well as the SNR value corresponding to 90% of maximum TP of the AI based CSI prediction γ2_AI/ML: AI/ML based CSI prediction is compared with Rel-16 eType II CB(sample and hold) - Companies are also encouraged to provide the TP curve with corresponding SNRs. - The test parameters for TPs simulation can be referred as 38.101-4 section of 6.3 - FDD with 16Tx4Rx: Table 6.3.3.1.8-1 - FDD with 32Tx4Rx: Table 6.3.3.1.4-1: - TDD with 16Tx4Rx: Table 6.3.3.2.8-1 - TDD with 32Tx4Rx: Table 6.3.3.2.4-1: |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training data and test data | Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz, 100Hz); 20Hz Doppler spreads. 50Hz Doppler spreads. |
| Companies can provide the model complexity information (e.g., flops, backbone) of their AI/ML CSI prediction models Companies are encouraged to clarify whether the SNR dataset for model training is per SNR or with an SNR range when providing the results of step-3 simulation |  |
| Clarifications on the SGCS calculation: SGCS1: Compares the SVD of model output (before feedback) with the SVD of the ground truth Raw channel SGCS2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD. SGCS3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth. |  |

Table2: Test setup for FDD test setup with Periodic  CSI-RS configuration (based on R.PDSCH.1-24.1 FDD)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | MCS13 | MCS17 | MCS19 |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames | Slots | 16 | 16 | 16 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 17 | 19 |  |  |
| Modulation |  | 16QAM | 64QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.43 | 0.50 |  |  |
| Number of MIMO layer |  | 2 | 2 | 2 |  |  |
| Number of DMRS REs (Note 3) |  | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 5) = {0}, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot | Bits | 24072 | 32264 | 37896 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 5) = {0}, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 5) = {0}, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot | CBs | 3 | 4 | 5 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data An example for FDD test setup with Periodic CSI-RS configuration |  |  |  |  |  |  |

Table3: Test setup for TDD test setup with Periodic  CSI-RS configuration (based on R.PDSCH.2-38.1 TDD)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | MCS13 | MCS17 | MCS19 |  |  |
| Channel bandwidth | MHz | 40 | 40 | 40 |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |  |
| Number of allocated resource blocks | PRBs | 106 | 106 | 106 |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames | Slots | 24 | 24 | 24 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 17 | 19 |  |  |
| Modulation |  | 16QAM | 64QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.43 | 0.50 |  |  |
| Number of MIMO layer |  | 2 | 2 | 2 |  |  |
| Number of DMRS REs (Note 3) |  | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {0} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {1,2,3,4,5,6} for i from {1,…,39} |  | 49176 | 65576 | 77896 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {0} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot i | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {0} for i from {0,…,39} | CBs | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot | CBs | 6 | 8 | 10 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data An example for TDD test setup with Periodic CSI-RS configuration |  |  |  |  |  |  |

# Simulation Results

## Results

Table 2: SGCS results at step 1, 2 and 3 with rank 2

| Prediction algorithm | Doppler during training (Hz) | Doppler during test (Hz) | Layer | Rank | Step 1 SGCS1 | Step 2 SGCS2 | Step 2 SGCS3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sample and hold | -- | 20 | 0 | 2 | 0.7805 | 0.7102 | 0.7157 |
| Sample and hold | -- | 20 | 1 | 2 | 0.6245 | 0.5363 | 0.5427 |
| AI-ML20 | 20 | 20 | 0 | 2 | 0.9938 | 0.8957 | 0.9435 |
| AI-ML20 | 20 | 20 | 1 | 2 | 0.9892 | 0.8351 | 0.9071 |
| AI-ML Mix | 20+50+100 | 20 | 0 | 2 | 0.9926 | 0.8947 | 0.9410 |
| AI-ML Mix | 20+50+100 | 20 | 1 | 2 | 0.9866 | 0.8332 | 0.9029 |
| Sample and hold | -- | 50 | 0 | 2 | 0.3127 | 0.2970 | 0.2959 |
| Sample and hold | -- | 50 | 1 | 2 | 0.1734 | 0.1662 | 0.1694 |
| AI-ML50 | 50 | 50 | 0 | 2 | 0.9572 | 0.8653 | 0.8931 |
| AI-ML50 | 50 | 50 | 1 | 2 | 0.9173 | 0.7788 | 0.8167 |
| AI-ML Mix | 20+50+100 | 50 | 0 | 2 | 0.9590 | 0.8668 | 0.8945 |
| AI-ML Mix | 20+50+100 | 50 | 1 | 2 | 0.9197 | 0.7806 | 0.8187 |

Table 3: SGCS results at step 1, 2 and 3 with rank 1

| Prediction Algorithm | Doppler during training (Hz) | Doppler during test (Hz) | Layer | Rank | Step 1 SGCS1 | Step 2 SGCS2 | Step 2 SGCS3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sample and hold | -- | 20 | 0 | 1 | 0.7805 | 0.7102 | 0.7157 |
| AI-ML20 | 20 | 20 | 0 | 1 | 0.9938 | 0.8957 | 0.9435 |
| AI-ML Mix | 20+50+100 | 20 | 0 | 1 | 0.9926 | 0.8947 | 0.9410 |
| Sample and hold | -- | 50 | 0 | 1 | 0.3127 | 0.2970 | 0.2959 |
| AI-ML50 | 50 | 50 | 0 | 1 | 0.9572 | 0.8653 | 0.8931 |
| AI-ML Mix | 20+50+100 | 50 | 0 | 1 | 0.9590 | 0.8668 | 0.8945 |

Table 2 and table 3 show the following things:

- The SGCS of AI-ML based CSI prediction in step 1 and step 2 are significantly higher than those of “sample and hold” methods in 20 and 50Hz.

- The SGCS performances of AI-ML model, trained based on the mixed dataset of 20, 50 and 100Hz, are almost identical to those of AI-ML model, trained based on individual datasets.

Observation 1: For 25Hz and 50Hz Doppler frequencies, the SGCS obtained with AI-ML based CSI prediction in both step 1 and step 2 are significantly higher than those obtained with “sample and hold” approach.

Observation 2: The SGCS performances of AI-ML model, trained based on the mixed dataset of 20, 50 and 100Hz, are almost identical to those of AI-ML model, trained based on individual datasets.

# Conclusions

Observation 1: For 25Hz and 50Hz Doppler frequencies, the SGCS obtained with AI-ML based CSI prediction in both step 1 and step 2 are significantly higher than those obtained with “sample and hold” approach.

Observation 2: The SGCS performances of AI-ML model, trained based on the mixed dataset of 20, 50 and 100Hz, are almost identical to those of AI-ML model, trained based on individual datasets.

# References

- R4-2505105, WF on requirements for AI/ML air interface normative work, 3GPP TSG RAN WG4 #114bis..

- R4-2505229, “Updated simulation assumptions for AIML CSI prediction”, RAN4 #114bis.

- R4-2508102, “WF on updated simulation assumptions for AI based CSI prediction”, RAN4 #115

- R4-2505513, “CSI Prediction: Simulation Results”, Qualcomm Incorporated, RAN4 #115.

- R4-2511573, “CSI Prediction: Simulation Results”, Qualcomm Incorporated, RAN4 #116.

- R4-2511796, “WF on updated simulation assumptions for AI based CSI prediction final”, RAN4 #116bis