3GPP TSG-RAN4 Meeting #117	R4-2522156

# Dallas, TX, USA 17th – 21st November, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM requirements for Rel-20 low NR band carrier aggregation via switching

Agenda item:	7.8.4

Document for:	Discussion

# Introduction

3GPP introduced a new Rel-20 work item on enhancement of low NR band carrier aggregation via switching in RAN #109[2]. This work item is intended to enhance the scope of the previous Rel-19 work item that focused on low band carrier aggregation via switching [1]. We focus on some relevant RRM requirements for this topic.

We discuss some relevant RRM requirements for this topic.

# Discussion

## General Focus

Figure 1: Supported band combinations, so far, in Rel-19

The key objectives of Rel-19 work item are shown below:

- Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - - Specify the switching delay and time mask for carrier switching [RAN4] - - Specify necessary RRM requirements [RAN4] - Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach

Figure 1 shows the band combination that was supported in Rel-19 work item. Other band combinations may be added later.

Figure 2: Supported band combinations, so far, in Rel-20 NR LB-CA

The objectives of Rel-20 work item are shown below

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28-n67, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

Figure 2 shows the band combinations that are supported in Rel-20 work items. Other band combinations may be added later.

RAN plenary guides RAN4 to use Rel-19 requirements as much as possible in Rel-20. Rel-20 WID also suggests reusing the semi-static switching mechanism based on RRC configuration defined in Rel-19 for LB-CA via switching.

RAN4 made the following agreements during the last meeting:

Agreement: - Switching pattern applicability for RRM requirement in Rel-20 LB-CA via switching: - For the SCell which has been activated (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is applied. - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. Agreement: - For L3 measurement and L1 measurement on activated SDL SCell, reuse the Rel-19 requirement as starting point. - For TCI state switch delay requirements, reuse the Rel-19 requirement as starting point. - For SDL SCell activation/deactivation delay requirements, reuse the Rel-19 requirement as starting point.

The major difference between Rel-19 and Rel-20 lies in the dual DL nature of Rel-20. That means, PCell DL remains ON in both cases of the Rel-20 switching pattern. Figure 3 shows an example switching pattern of Rel-20. Figure 4 shows the impact of this switching pattern in ON/OFF nature of various links of PCell and SCell in Rel-20.

Figure 4: ON and OFF patterns of PCell DL, PCell UL and SDL SCell DL in an example Rel-20 switching pattern

## Potential PCell DL Interruption during Transition between Two Cases of Switching Pattern

One of the most important questions of Rel-20 work item is whether UE interrupts PCell DL while it is transitioning between case 1 (PCell DL + PCell UL ON) and case 3 (PCell DL + SCell DL ON) of the switching pattern.

In one solution, UE may keep its PCell DL turned on while it is transitioning case 1 and case 3. In another solution, UE’s DL may get impacted while UE is transitioning between case 1 and case 3. RAN4 RF is the suitable session to discussion. RAN4 RRM session should wait for RAN4 RF session to discuss the architecture of Rel-20 NR LBCA work item and conclude whether PCell DL gets interrupted during transition between the two cases of Rel-20 switching pattern.

Observation 1: RAN4 RF session is the suitable session to discuss the UE architecture of Rel-20 NR LB-CA work item and conclude whether PCell DL gets interrupted during transition between the two cases of Rel-20 switching pattern.

## Interruption During Deactivated SCell Measurement Requirement

RAN4 made the following agreement regarding interruption during deactivated SCell measurement in Rel-19 NR LB-CA [3].

Issue 1-3-2: Interruption requirement design of deactivated SDL SCell measurement [Agreement]: - For R19 LB CA via switching, interruption length on PCC due to measurements on deactivated SDL is (tuning time in slot level + TSMTC_duation + retuning time in slot level), - Tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell LBCA-SwitchingGap-Duration-PCelltoSCell-r19 - Returning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap- SCelltoPCell-r19 - Missed ACK/NACK rate For R19 LB CA via switching, missed ACK/NACK rate due to measurements on deactivated SDL are allowed with up to X= YmeasCycleSCell, where Y=2* interruption length.

Observation 2: During Rel-19 LB CA switching, interruption length on PCC due to measurement of deactivated SCell was determined to be the summation of SMTC, tuning time and returning time.

Observation 3: During Rel-19 LB CA switching, missed ACK/NACK rate due to measurements on deactivated SDL were allowed to be the ratio of (2 * interruption length) and measCycleSCell.

It is clear that UE would interrupt PCell UL while it is measuring deactivated SCell in Rel-20 NR LB-CA. However, RRM session should wait for RF session to decide if transitioning between two cases of switching pattern impacts PCell DL to decide overall interruption during deactivated SCell measurement requirement.

Proposal 1: RRM session waits for RF session to decide if transitioning between two cases of switching pattern impacts PCell DL before deciding overall interruption during deactivated SCell measurement.

## Measurement of PCells

RAN4 mad the following agreement regarding measurement of PCell in Rel-19 NR LB-CA

Issue 1-4-2: others RRM requirement of FDD PCell in LB CA via switching [Agreement]: - The principle of agreed activated SDL SCell requirement in LB-CA can be reused for the corresponding PCell requirement in LB-CA.

Above agreement simply mentions that the measurement delay of PCell will be extended according to the fraction of available measurement occasions. In Rel-20, PCell DL will be ON during both cases of the switching pattern. RAN4 RF session should decide whether UE interrupts PCell DL while it is transitioning between the two cases of the switching pattern. RRM should wait until this decision before deciding the principle of PCell measurement period in Rel-20 NR LB-CA.

Observation 4: In Rel-19, RAN4 has agreed to extend the measurement delay of PCell according to the fraction of available measurement occasions.

Proposal 2: RRM session waits for RF session to decide if transitioning between two cases of switching pattern impacts PCell DL before deciding PCell measurement delay in Rel-20 NR LB-CA.

# Conclusions

Observation 1: RAN4 RF session is the suitable session to discuss the UE architecture of Rel-20 NR LB-CA work item and conclude whether PCell DL gets interrupted during transition between the two cases of Rel-20 switching pattern.

Observation 2: During Rel-19 LB CA switching, interruption length on PCC due to measurement of deactivated SCell was determined to be the summation of SMTC, tuning time and returning time.

Observation 3: During Rel-19 LB CA switching, missed ACK/NACK rate due to measurements on deactivated SDL were allowed to be the ratio of (2 * interruption length) and measCycleSCell.

Observation 4: In Rel-19, RAN4 has agreed to extend the measurement delay of PCell according to the fraction of available measurement occasions.

Proposal 1: RRM session waits for RF session to decide if transitioning between two cases of switching pattern impacts PCell DL before deciding overall interruption during deactivated SCell measurement.

Proposal 2: RRM session waits for RF session to decide if transitioning between two cases of switching pattern impacts PCell DL before deciding PCell measurement delay in Rel-20 NR LB-CA

# References

[1] RP-243317, New WID on low band carrier aggregation via switching, 3GPP TSG RAN #106.

[2] RP-252907, New WID on ehancement of low NR band carrier aggregation via switching, 3GPP TSG RAN #109.

[3] R4-2512119, “WF on RRM requirements for NR_LBCA_Sw”, 3GPP TSG-RAN WG4 #116.

[4] R4-2514824, “WF for [116bis][224] NR_LBCA_Sw_enh_RRM”, 3GPP TSG-RAN WG4 #116bis