3GPP TSG-RAN4 Meeting #117	R4-2522157

# Dallas, Texas, USA, 17th – 21th November, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM Core Requirements for AI-ML based Positioning

Agenda item:	6.11.5

Document for:	Discussion

# Introduction

In this paper, we present our views regarding RRM core requirements for case 1. Figure 1 shows different cases of 5G AI-ML based positioning. RAN4 made the following agreement regarding case 1 during the last two meetings [1][2]

Issue 3-1: Reporting Delay Requirements for case 1 Take the framework of the existing reporting delay requirements - Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report - Check until next meeting on the number of samples needed for the measurements Issue 3-1: Requirements for case 1 To be clarified in the requirements: - Request location information message should refer to “case 1” positiniong - Reuse the legacy RSTD measurement delay for all the cases (with gap, w/o gap, PRS aggregation) - Beam sweeping factor for FR2 including sweeping reduction factor - Take the legacy values - Rx TEG: check on signaling, take the legacy(used in RSTD measurements) approach if it is signaled - Cover all RRC states

We provide our views regarding inference delay and different aspects of measurement delay in this contribution.

# Discussion

## Inference Time of Case 1

RAN4 already agreed to reuse the legacy RSTD measurement delay as the measurement delay for case 1. Appendix A shows that the legacy RSTD measurement delay scales with the number of positioning frequency layers (PFL). UE may see different TRPs in different PFLs. UE has two options to infer its co-ordinates based on the channel impulse response obtained from TRPs across different PFLs.

- Option 1:

  - Run inference based on aggregated channel response of TRPs across all PFLs and derive one estimate of UE’s co-ordinate

- Option 2:

  - Run inference separately based on channel response of TRPs in each PFL and estimate UE’s co-ordinate separately for each PFL.

  - Post-process the estimated co-ordinates for different PFLs to derive one estimate of UE’s location.

The input size of option 1 is greater than that of option 2. Hence, option 1 requires UE to use an AI-ML model that has larger number of parameters and that is computationally more complex. Besides, option 1 would require UE to

Figure: Different cases of AI-ML based positioning in AI-ML air interface

decide inference time based on the worst-case assumption that the maximum possible number of PFLs has been configured. Hence, total delay will be unnecessarily extended when the actual number of configured PFLs is smaller than the maximum possible value. Option 2 does not suffer from this issue. Hence, RAN4 should select option 2 and introduce inference time per PFL.

Observation 1: UE has two options to infer its co-ordinates based on the channel impulse response obtained from TRPs across different PFLs.

  - Option 1: UE can run inference based on aggregated channel response of TRPs across all PFLs and derive one estimate of its co-ordinate.

  - Option 2: UE runs inference separately based on channel response of TRPs in each PFL and estimates its co-ordinate separately for each PFL. UE post-processes the estimated co-ordinates of different PFLs to derive one estimate of its location.

Observation 2: Running inference based on aggregated channel response of TRPs across all PFLs requires UE to use a large and more complex AI-ML model.

Observation 3: Running inference based on aggregated channel response of TRPs across all PFLs requires UE to decide inference based on the worst-case assumption that the maximum number of PFLs has been configured. This will increase total delay unnecessarily when the actual number of configured PFLs is smaller than the maximum possible value.

Proposal 1: RAN4 introduces inference time per configured positioning frequency layer in the reporting delay framework of AI-ML positioning case 1.

UEs may experience different number of TRPs in different deployment scenarios. For example, the number of observed TRPs in indoor factory could be smaller than that in outdoor locations. Required inference time to process the channel response of the TRPs and estimate UE’s location will also vary across deployment scenarios. Hence, RAN4 should introduce a UE capability for inference time per PFL in case 1.

In AI-ML positioning case 1, UE reports its co-ordinates to the network. Network can use this reported co-ordinate for different services. As Appendix C shows, the required response time can vary significantly among these services, ranging from milliseconds to hundreds of seconds [5]. UE needs to run inference very quickly for services whose response time is in the order of milliseconds. On the other hands, some UEs may only support services whose required response time is in the order of tens or hundreds of seconds. Those UEs should be allowed to infer via a very low complexity model and consume longer inference time. Hence, the newly introduced UE capability should allow UE to report required inference time ranging from few ms to tens of ms.

The activation of most of the Rel-19 AI-ML features, except positioning, is controlled by gNB. The activation of AI-ML based UE co-ordinate reporting is co-ordinated by LMF. It is possible that UE may be configured to run AI-ML based beam or CSI prediction concurrently with AI-ML based positioning case 1. That’s why, the CSI processing criteria of RAN1 spec only covers Rel-19 AI-ML based beam and CSI prediction; and not AI-ML based positioning. If UE cannot fit in the models of positioning case 1 and beam/CSI prediction in its AI engine, it will have to run the AI-ML models serially. Hence, the inference timeline of AI-ML positioning case 1 should be large enough to accommodate the field deployment scenario where UE may get configured to infer for beam and/or CSI prediction while it is inferring its co-ordinates for positioning case 1. RAN4 AI-ML based positioning tests should avoid concurrent configuration of AI-ML based positioning and other spec defined AI-ML features (e.g., beam prediction, CSI prediction, etc.)

Observation 4: UEs may experience different number of TRPs in different deployment scenarios. Required inference time to process the channel response of the TRPs will also vary across deployment scenarios.

Observation 5: UE’s reported co-ordinates can be used for different quality of services. The required response time can vary significantly among these services, ranging from milliseconds to hundreds of seconds.

Observation 6: The activation of AI-ML based positioning case 1 and other Rel-19 AI-ML features (AI-ML based beam/CSI prediction) are controlled by LMF and gNB respectively. It is possible that UE may be configured to run AI-ML based beam or CSI prediction concurrently with AI-ML based positioning case 1.

Observation 7: The inference timeline of AI-ML positioning case 1 should be large enough to accommodate the field deployment scenario where UE may get configured to infer for beam and/or CSI prediction while it is inferring its co-ordinates for positioning case 1

Proposal 2: RAN4 introduces following UE capability for inference time per positioning frequency layer in case 1.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos_AIML | 58-x-x | Inference time per positioning frequency layer for AI-ML positioning case 1 | The maximum required inference time per configured positioning frequency layer for AI-ML positioning case 1 | 58-x-x | n/a | n/a | Candidate values: {3, 4, 5, 6, 7, 8, 9, 10, 15, 20, 25, 30, 35, 40, 45, 50} ms | Optional with capability signaling |

Proposal 3: RAN4 AI-ML based positioning tests should avoid concurrent configuration of AI-ML based positioning and other spec defined AI-ML features (e.g., beam prediction, CSI prediction, etc.)

## Different Factors in Measurement Delay of Case 1

RAN4 agreed to reuse the RSTD measurement delay of case 1 as the measurement delay of AI-ML positioning case 1. Legacy RSTD measurement delay framework contained parameters that are associated with three legacy NR UE capabilities: 1) Support of reduced measurement samples, 2) Support of lower Rx beam sweeping factor in FR2, 3) RX timing error group. Appendix B shows these legacy NR UE capabilities.

A similar set of capabilities has not yet been introduced in RAN1 and RAN4 for AI-ML positioning case 1. Ideally, RAN4 should introduce similar capabilities in AI-ML positioning case 1. The other option would be to assume the maximum possible value for these parameters in the measurement delay requirement of case 1. That means, RAN4 would have to assume the required number of measurement samples, FR2 RX beam sweeping factor and RX timing error group to be 4, 8 and 8 respectively. Introduction of UE capability would allow network to assume smaller reporting delay for the UEs that support these capabilities.

Observation 8: Legacy RSTD measurement delay framework contained parameters that are associated with three legacy NR UE capabilities: 1) Support of reduced measurement samples, 2) Support of lower Rx beam sweeping factor in FR2, 3) RX timing error group. These capabilities have not yet been introduced for AI-ML positioning case 1.

Observation 9: In the absence of UE capabilities, RAN4 would have to assume the maximum possible values for the required number of measurement samples, FR2 RX beam sweeping factor and Rx-TEG in the measurement delay framework of AI-ML case 1.

- Note: The required number of measurement samples, FR2 RX beam sweeping factor and RX timing error group would have to be assumed as 4, 8 and 8 respectively

Proposal 4: RAN4 introduces UE capabilities for measurement sample, RX beam sweeping factor and Rx-TEG for AI-ML case 1 positioning.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos_AIML | 58-x-x | PRS measurement for reduced sample for AI-ML positioning case 1 | The capability to support reporting UE co-ordinate based on measuring on 1 or 2 samples of a DL PRS resource set in AI-ML positioning case 1 | 58-x-x | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_AIML | 58-x-x | Support of lower RX beam sweeping factor for AI-ML positioning case 1 | 1. Support of the lower RX beam sweeping factor than 8 for FR2 in AI-ML positioning case 1 2. Number of RX beam sweeping factor in AI-ML positioning case 1 | 58-x-x | n/a | n/a | Component 2: candidate values {1, 2, 4, 6} Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_AIML | 58-x-x | UE-RxTEGs for AI-ML positioning case 1 | 1. Support of UE-RxTEGs for UE-based AI-ML positioning case 1 2. The maximum number of UE-RxTEG, which is supported and reported by UE for UE-based AI-ML positioning case 1 | 58-x-x | n/a | n/a | Component 2 candidate values: {1, 2, 3, 4, 6, 8} Need for location server to know if the feature is supported | Optional with capability signaling |

# Conclusions

Observation 1: UE has two options to infer its co-ordinates based on the channel impulse response obtained from TRPs across different PFLs.

  - Option 1: UE can run inference based on aggregated channel response of TRPs across all PFLs and derive one estimate of its co-ordinate.

  - Option 2: UE runs inference separately based on channel response of TRPs in each PFL and estimates its co-ordinate separately for each PFL. UE post-processes the estimated co-ordinates of different PFLs to derive one estimate of its location.

Observation 2: Running inference based on aggregated channel response of TRPs across all PFLs requires UE to use a large and more complex AI-ML model.

Observation 3: Running inference based on aggregated channel response of TRPs across all PFLs requires UE to decide inference based on the worst-case assumption that the maximum number of PFLs has been configured. This will increase total delay unnecessarily when the actual number of configured PFLs is smaller than the maximum possible value.

Observation 4: UEs may experience different number of TRPs in different deployment scenarios. Required inference time to process the channel response of the TRPs will also vary across deployment scenarios.

Observation 5: UE’s reported co-ordinates can be used for different quality of services, the required response time can vary significantly among these services, ranging from milliseconds to hundreds of seconds.

Observation 6: The activation of AI-ML based positioning case 1 and other Rel-19 AI-ML features (AI-ML based beam/CSI prediction) are controlled by LMF and gNB respectively. It is possible that UE may be configured to run AI-ML based beam or CSI prediction concurrently with AI-ML based positioning case 1.

Observation 7: The inference timeline of AI-ML positioning case 1 should be large enough to accommodate the field deployment scenario where UE may get configured to infer for beam and/or CSI prediction while it is inferring its co-ordinates for positioning case 1

Observation 8: Legacy RSTD measurement delay framework contained parameters that are associated with three legacy NR UE capabilities: 1) Support of reduced measurement samples, 2) Support of lower Rx beam sweeping factor in FR2, 3) RX timing error group. These capabilities have not yet been introduced for AI-ML positioning case 1.

Observation 9: In the absence of UE capabilities, RAN4 would have to assume the maximum possible values for the required number of measurement samples, FR2 RX beam sweeping factor and Rx-TEG in the measurement delay framework of AI-ML case 1.

- Note: The required number of measurement samples, FR2 RX beam sweeping factor and RX timing error group would have to be assumed as 4, 8 and 8 respectively

Proposal 1: RAN4 introduces inference time per configured positioning frequency layer in the reporting delay framework of AI-ML positioning case 1.

Proposal 2: RAN4 introduces following UE capability for inference time per positioning frequency layer in case 1.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos_AIML | 58-x-x | Inference time per positioning frequency layer for AI-ML positioning case 1 | The maximum required inference time per configured positioning frequency layer for AI-ML positioning case 1 | 58-x-x | n/a | n/a | Candidate values: {3, 4, 5, 6, 7, 8, 9, 10, 15, 20, 25, 30, 35, 40, 45, 50} ms | Optional with capability signaling |

Proposal 3: RAN4 AI-ML based positioning tests should avoid concurrent configuration of AI-ML based positioning and other spec defined AI-ML features (e.g., beam prediction, CSI prediction, etc.)

Proposal 4: RAN4 introduces UE capabilities for measurement sample, RX beam sweeping factor and Rx-TEG for AI-ML case 1 positioning.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos_AIML | 58-x-x | PRS measurement for reduced sample for AI-ML positioning case 1 | The capability to support reporting UE co-ordinate based on measuring on 1 or 2 samples of a DL PRS resource set in AI-ML positioning case 1 | 58-x-x | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_AIML | 58-x-x | Support of lower RX beam sweeping factor for AI-ML positioning case 1 | 1. Support of the lower RX beam sweeping factor than 8 for FR2 in AI-ML positioning case 1 2. Number of RX beam sweeping factor in AI-ML positioning case 1 | 58-x-x | n/a | n/a | Component 2: candidate values {1, 2, 4, 6} Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_AIML | 58-x-x | UE-RxTEGs for AI-ML positioning case 1 | 1. Support of UE-RxTEGs for UE-based AI-ML positioning case 1 2. The maximum number of UE-RxTEG, which is supported and reported by UE for UE-based AI-ML positioning case 1 | 58-x-x | n/a | n/a | Component 2 candidate values: {1, 2, 3, 4, 6, 8} Need for location server to know if the feature is supported | Optional with capability signaling |

# References

[1] R4-2514631, WF on Air Interface, 3GPP TSG WG RAN4 116bis.

RP-2423999, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN 105, Sep 2024.

[2] RP-250792, “Revised WID for NR_AIML_air”, 3GPP TSG RAN meeting #107.

[3] R4-2511890, “WF on AI-ML air interface”, 3GPP TSG-RAN WG4 Meeting #116.

[4] 3GPP TS 38.133, v 17.11.0

[5] 3GPP TS 37.355, v 19.0.0

# Appendix A: Rel-17 Measurement Period Requirement of RSTD [4]

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

Where ,

i is the index of positioning frequency layer,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below:

TRSTD,i=kmultiTEG,i*CSSFPRS,i*ceil( Kp,PRS,i)*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i ,

where:

NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1;

and in FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

CSSFPRS,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in clause 9.1.5.2.

kmultiTEG,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i=1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation;

otherwise,

kmultiTEG=NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure a PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

-	For a window W of duration max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioningfrequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable =0.

NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the

sampling time and processing time. If all of the PRS resources to be measured are available in the same MG occasion during Tavailabe, Tlast,i = Ti +MGL. Otherwise, Tlast,i = Ti + Tavailable_PRS,i ,

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Where,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi.

MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the PRS frequency layer i.	TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource,

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where

TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Note: For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34].

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot specified in TS 37.355 [34].

# Appendix B: Legacy NR Capabilities for Measurement Samples, FR2 Rx Beam Sweeping Factor and Rx Timing Error Groups

| Features | Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-1 | UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning | 1. Support of UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning 2. The maximum number of UE-RxTEG, which is supported and reported by UE for UE assisted DL TDOA and/or Multi-RTT positioning | 13-1, one or more of {13-3, 13-4} | Component 1 candidate values: {UE-assisted DL TDOA, Multi-RTT positioning, UE-assisted DL TDOA and Multi-RTT positioning} Component 2 candidate values: {1, 2, 3, 4, 6, 8} Note: a single value is reported when both multi-RTT and DL-TDOA are supported Need for location server to know if the feature is supported If the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the UE Rx timing errors for this measurement Note: The "per band" reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |

| 27. NR_pos_enh | 27-9 | Support of lower Rx beam sweeping factor | 1. Support of the lower Rx beam sweeping factor than 8 for FR2 2. Number of Rx beam sweeping factors |  | Component 2 candidate values: {1,2,4,6} Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- |

| 27. NR_pos_enh | 27-3-1 | M-sample measurements in RRC_CONNECTED | The capability to support reporting a measurement based on measuring M=1 or 2 samples (instances) of a DL PRS resource set | 13-1 | Need for location server to know if the feature is supported Note: this feature is supported for both UE-assisted and UE based positioning | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- |

# Appendix C: Response Time of Different QoS for UE Co-ordinate Reporting

37.355 provides following range of response time for different quality of services,

QoS ::= SEQUENCE {

horizontalAccuracy			HorizontalAccuracy		OPTIONAL,	-- Need ON

verticalCoordinateRequest	BOOLEAN,

verticalAccuracy			VerticalAccuracy		OPTIONAL,	-- Need ON

responseTime				ResponseTime			OPTIONAL,	-- Need ON

velocityRequest				BOOLEAN,

...,

[[	responseTimeNB-r14		ResponseTimeNB-r14		OPTIONAL	-- Need ON

]],

[[	horizontalAccuracyExt-r15	HorizontalAccuracyExt-r15		OPTIONAL,	-- Need ON

verticalAccuracyExt-r15		VerticalAccuracyExt-r15			OPTIONAL	-- Need ON

]]

}

ResponseTime ::= SEQUENCE {

time								INTEGER (1..128),

...,

[[	responseTimeEarlyFix-r12		INTEGER (1..128)		OPTIONAL		-- Need ON

]],

[[	unit-r15				ENUMERATED { ten-seconds, ... , ten-milli-seconds-v1700 }

OPTIONAL		-- Need ON

]]

}

This IE indicates the quality of service and comprises a number of sub-fields. In the case of measurements, some of the sub-fields apply to the location estimate that could be obtained by the server from the measurements provided by the target device assuming that the measurements are the only sources of error. Fields are as follows:

-	horizontalAccuracy indicates the maximum horizontal error in the location estimate at an indicated confidence level. The 'accuracy' corresponds to the encoded uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15].

-	verticalCoordinateRequest indicates whether a vertical coordinate is required (TRUE) or not (FALSE)

-	verticalAccuracy indicates the maximum vertical error in the location estimate at an indicated confidence level and is only applicable when a vertical coordinate is requested. The 'accuracy' corresponds to the encoded uncertainty altitude as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15].

-	responseTime

-	time indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation. If the unit field is absent, this is given as an integer number of seconds between 1 and 128. If the unit field is present with enumerated value 'ten-seconds', the maximum response time is given in units of 10-seconds, between 10 and 1280 seconds. If the unit field is present with enumerated value 'ten-milli-seconds', the maximum response time is given in units of 10-milli-seconds, between 0.01 and 1.28 seconds. If the periodicalReporting IE is included in CommonIEsRequestLocationInformation, this field should not be included by the location server and shall be ignored by the target device (if included).

-	responseTimeEarlyFix indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation containing early location measurements or an early location estimate. If the unit field is absent, this is given as an integer number of seconds between 1 and 128. If the unit field is present with enumerated value 'ten-seconds', the maximum response time is given in units of 10-seconds, between 10 and 1280 seconds. If the unit field is present with enumerated value 'ten-milli-seconds', the maximum response time is given in units of 10-milli-seconds, between 0.01 and 1.28 seconds. When this IE is included, a target should send a ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing early location information according to the responseTimeEarlyFix IE and a subsequent ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing final location information according to the time IE. A target shall omit sending a ProvideLocationInformation if the early location information is not available at the expiration of the time value in the responseTimeEarlyFix IE. A server should set the responseTimeEarlyFix IE to a value less than that for the time IE. A target shall ignore the responseTimeEarlyFix IE if its value is not less than that for the time IE.

-	unit indicates the unit of the time and responseTimeEarlyFix fields. Enumerated value 'ten-seconds' corresponds to a resolution of 10 seconds. Enumerated value 'ten-milli-seconds' corresponds to a resolution of 0.01 seconds. If this field is absent, the unit/resolution is 1 second. Enumerated value 'ten-milli-seconds' is only applicable for NR E-CID Positioning, NR DL-TDOA Positioning, NR DL-AoD Positioning, and NR Multi-RTT Positioning. If the enumerated value 'ten-milli-seconds' is included for methods others than NR E-CID Positioning, NR DL-TDOA Positioning, NR DL-AoD Positioning, and NR Multi-RTT Positioning the target device shall ignore the unit field