3GPP TSG-RAN4 Meeting #117	R4-2522169

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

# Title:	3MHz CHBW change from Optional to Mandatory for selected bands in Rel-19

# Source:	Rakuten Mobile Inc , T-Mobile, Anterix, Southern Linc

# Agenda Item:	8.2.4

# Document for:	Approval

# Introduction

At RAN4#115, a Way Forward on handling optional channel bandwidth across releases [1] was approved. This contribution provides Rakuten Mobile, T-Mobile, Anterix, Southern Linc input on WF1 from [1].

# Discussion

[1] included the following agreement:

Agreement 1:

- 3MHz CBW:

  - For MSD analysis, do not consider 3MHz CBW except when the DL affected band is band n106.

  - Companies are invited to share their views at next meeting whether 3MHz CBW should become mandatory for bands n26, n28, n31, n72, n85 and n100.

The supporting companies propose “Support of 3MHz CBW from Optional to mandatory in Rel-19 for band n26,n28,n85 and n100” based on the following rationale:

- The proposal requires no new RF configurations, introduces no testing burden, and aligns with an already validated approach used for Band n106.

- This change will ensure all NR UEs supporting Band n28 in Rel-19 are capable of operating at 3 MHz, aligning support expectations across the ecosystem.

- It is agreed that no MSD analysis is considered for 3MHz CBW for these bands.

Also, a corrosponding CR [3] is submitted for agreement in this meeting to remove optionality for 3MHz support from the mentioned bands.

# Conclusion

Proposal 1:   Change the 3 MHz CBW for Band n28,n26,n85,n100 from optional to mandatory in Release 19. This CBW to be excluded from MSD interference impact analysis.

# Reference

[1] R4-2508050 – “WF on optional CHBW across releases”
[2] R4-2506184 – “Input on Optional vs. Mandatory Channel BWs”

[3] R4-2522167 - CR to 38.101-1 for 3MHz Bandwidth change from Optional to Mandatory for n28,n26,n85,n100