3GPP TSG-RAN WG4 Meeting #117								                          R4-2522170

Dallas, TX, USA, November 17 – 21, 2025

Agenda item:			7.1.4

Source:	Ericsson

Title:	On HPUE RF requirements for 4Tx with up to 32 dBm and 2Tx PC1.5 FDD

Document for:	Approval

# Introduction

In RAN#109, a new WID has been approved [1] for “UE RF enhancements for NR FR1, Phase 5” WI. According to RAN#109 discussion, for “High power UE (HPUE) for NR single carrier operation” part, only the work on the following two objectives started in RAN4#116-bis meeting:

The objectives of core part for UE RF enhancements for NR FR1 Phase 5 include: High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25

In this contribution, we will focus on the HPUE RF requirements for both 4 Tx with up to 32 dBm for TDD bands

and PC1.5 with 2Tx for FDD bands parts.

# Discussion

## 4Tx for CPE/FWA UE with 4x26dBm PA configurations

In RAN4#116-bis meeting, the following was agreed in the approved WF [2]:

1.1 MPR/A-MPR assumptions Agreement: - Evaluate assuming following antenna isolation assumptions - For CPE, assuming both 10dB and 20dB, - For FWA, assuming 20dB. Come back once MPR results are available, to evaluate if it is possible to select a single CPE antenna isolation assumption based on the MPR results. 1.2 NS to be evaluated Agreement: Evaluate A-MPR for NS_04 and NS_47 for n41.

### Power class definition and the ACLR requirement

In RAN4#116-bis meeting, there was still no agreement on whether the nominal maximum output power should be 31 dBm or 32 dBm and whether PC1 should be reused or a new power class should be introduced. 32 dBm MOP is feasible in theory by using 4 x 26 dBm PAs configuration but the main concern of some companies was whether it would require too stringent ACLR which would consequently make the MPR allowance excessive and remove the virtue of the higher power class for the front-end configuration assumed.

For this meeting, we have provided the co-existence simulation results in [3], where based on our evaluations the ACLR could be specified as 33 dBc for 32 dBm nominal MOP. Such ACLR could represent a good compromise between the UE implementation challenges and the current PC1 ACLR requirement of 37 dBc.

Proposal 1: Specify ACLR of 33 dBc for 32 dBm nominal maximum output power.

Since the nominal MOP would be 32 dBm with a new ACLR requirement of [33 dBc] and associated MPR, a new power class would normally be introduced. However, the difference in the nominal maximum output power to PC1 is only 1 dB, which may not merit introduction of additional UE capability indication of a new higher power class. A power boosting of 1 dB could be used to achieve 32 dBm nominal maximum output power. Moreover, specification of a MPR = 0 dB reference may be possible for inner allocations using PC1 nominal power for 4Tx. Therefore, we propose to keep PC1:

Proposal 2: a new power class should not be introduced - PC1 should be used with power boosting capability of 1 dB.

The relaxed ACLR requirement and associated MPR for these devices could be distinguished from that of existing devices by using the modified MPR-behaviour bitmap for the example bands of the 4 x 26 dBm CPE devices. The modified MPR behaviour is specified per band.

Observation 1: the (band-specific) modified MPR behaviour can be used to distinguish PC1 support for the example bands of the 4 x 26 dBm CPE devices with their relaxed ACLR requirement and associated MPR from legacy PC1 devices.

This would make the modified requirements for PC1 band specific and no new capabilities would be needed.

### ULFPTx mode support

One of the most controversial discussion points in Rel-18 when PC1.5 was specified for 4 Tx was on PUSCH configurations for uplink full power transmission (ULFPTx) (Table 6.2D.1-3 in TS38.101-1). For 4 Tx, ULFPTx Mode-1 and ULFPTx Mode-2 were specified for both single (DFT-s-OFDM, CP-OFDM) and two layers (CP-OFDM) with the corresponding TPMI indices. ‘Mode-full power’ (Mode-0) wasn’t specified since it was assumed that 29 dBm PAs were not feasible in Rel-18.

Similar holds for 32 dBm power class achieved by 4 x 26 dBm PAs. ‘Mode-full power’ is not feasible since it is assumed that in Rel-20 FWA or CPE devices are not equipped with 4 x 32 dBm PAs (even though 1 PA of 31 dBm is considered feasible for public safety type of UEs).

Observation 2: ULFPTx ‘Mode-0’ is not feasible for 32 dBm power class.

Assuming 4 x 26 dBm PA configuration and that the assumption on coherent ports is maintained, ULFPTx ‘Mode-1’ is feasible for 32 dBm power class for both single and two layers with TPMI indices 13 () and 6 (), respectively.

Observation 3: ULFPTx ‘Mode-1’ is feasible for 32 dBm power class for both single- and two-layers cases.

On the other hand, under the same assumptions, the ULFPTx ‘Mode-2’ is not feasible (neither for single nor two layers cases), since that mode would require that the UE is equipped with at least two PAs of 29 dBm. For example, for single layer, TPMI index 4 points to precoding matrix  while for two-layers TPMI index 1 points to .

Observation 4: ULFPTx ‘Mode-2’ is not feasible for 32 dBm power class.

Proposal 3: Specify only ULFPTx ‘Mode-1’ for 32 dBm power class.

It should be discussed how to capture the ULFPTx ‘Mode-1’ only support in the specifications since this would be the first case that for a certain combination of power class and the number of Tx ports one of the already specified PUSCH configurations for an ULFPTx mode is not specified.

Observation 5: It should be discussed how to capture the ULFPTx ‘Mode-1’ only support in the specifications.

On the other hand, by considering 4 x 26 dBm PA configuration feasible in Rel-20, ‘Mode-full power’ becomes feasible in theory for PC2. It should be discussed whether it can be introduced in this WI. However, in that case it should be clarified in the specifications that ‘mode-full power’ for 4 Tx for single layer transmission is only available for PC2 and not for PC1.5 and PC1.

Observation 6: ‘Mode-full power’ can be introduced for PC2 under 4 x 26 dBm PA configuration assumption. It should be discussed whether it can be introduced in this WI.

## PC1.5 with 2Tx for FDD bands for handheld and FWA UE

As stated in the WID justification part [1], PC2 for FDD bands wasn’t introduced before Rel-17 due to larger implementation challenges compared with TDD bands case (e.g. UE heat dissipation, duplexer feasibility etc.). In Rel-20, PC1.5 with 1 Tx will only be studied for TDD bands where for FDD bands it is still considered not feasible.

PC1.5 with 2 Tx is already specified for TDD bands since Rel-16, but only now (in Rel-20) it is considered feasible for FDD bands as well.

In RAN4#116-bis meeting, there was a slightly better progress on this topic [2]. The nominal MOP was agreed to be 29 dBm, while the ACLR requirement will be reused from TDD bands, which is 31 dBc. Given that fact, and since in the current specification there has never been a case where the MPR requirements were different between TDD and FDD bands for a given power class and number of Tx ports (and antenna isolation assumptions), in our view the already existing MPR requirements for PC1.5 with 2 Tx ports for TDD bands could be reused.

The current MPR requirements for PC1.5 with 2 Tx ports are given in two tables, a ”default one” and a separate one ”targeted to large FWA form factor with 20 dB or above antenna isolation.” Since in this part of the WI both handheld and FWA UE types are in the scope, both tables could reused for the MPR requirements for PC1.5 with 2Tx for FDD bands (the ”default” one targeting handheld UEs which are assumed to have ~10 dB antenna isolation).

Proposal 4: For the MPR requirements, reuse the current MPR requirements for PC1.5 with 2 Tx derived for TDD bands. Both tables should be included: one targeting handheld UEs and the other targeting UEs with large FWA form factor with 20 dB or above antenna isolation.

Regarding the RSD requirements, due to a relatively large duplexing distance for band n1, 0 dB could specified for PC1.5 RSD.

Proposal 5: Adopt 0 dB as PC1.5 RSD requirement for band n1 due to a large duplexing distance.

# Conclusion

Based on the discussion in the previous sections we have made the following observations:

Observation 1: the (band-specific) modified MPR behaviour can be used to distinguish PC1 support for the example bands of the 4 x 26 dBm CPE devices with their relaxed ACLR requirement and associated MPR from legacy PC1 devices.

Observation 2: ULFPTx ‘Mode-0’ is not feasible for 32 dBm power class.

Observation 3: ULFPTx ‘Mode-1’ is feasible for 32 dBm power class for both single- and two-layers cases.

Observation 4: ULFPTx ‘Mode-2’ is not feasible for 32 dBm power class.

Observation 5: It should be discussed how to capture the ULFPTx ‘Mode-1’ only support in the specifications.

Observation 6: ‘Mode-full power’ can be introduced for PC2 under 4 x 26 dBm PA configuration assumption. It should be discussed whether it can be introduced in this WI.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Specify ACLR of 33 dBc for 32 dBm nominal maximum output power.

Proposal 2: a new power class should not be introduced - PC1 should be used with power boosting capability of 1 dB.

Proposal 3: Specify only ULFPTx ‘Mode-1’ for 32 dBm power class.

Proposal 4: For the MPR requirements, reuse the current MPR requirements for PC1.5 with 2 Tx derived for TDD bands. Both tables should be included: one targeting handheld UEs and the other targeting UEs with large FWA form factor with 20 dB or above antenna isolation.

Proposal 5: Adopt 0 dB as PC1.5 RSD requirement for band n1 due to a large duplexing distance.

# References

- RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, RAN#109.

- R4-2515087, “Way Forward for [116bis][316] NR_UE_RF_Ph5”, Nokia, Murata, RAN4#116-bis.

- R4-2522171, “On HPUE 4Tx PC1 initial coexistence study results”, Ericsson, RAN4#117.