3GPP TSG-RAN WG4 Meeting #117								                        R4-2522171

Dallas, TX, USA, November 17 – 21, 2025

Agenda item:			7.1.3

Source:	Ericsson

Title:	On HPUE 4Tx PC1 initial coexistence study results

Document for:	Approval

# Introduction

In RAN#109, a new WID has been approved [1] for “UE RF enhancements for NR FR1, Phase 5” WI. For “High power UE (HPUE) for NR single carrier operation” part, only the work on the following two objectives started in RAN4#116-bis meeting:

The objectives of core part for UE RF enhancements for NR FR1 Phase 5 include: High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25

In the following, we will focus on the co-existence study for the ACLR for 4Tx with 4x26dBm PA configurations part, where we will present our initial simulation results.

# Discussion

In RAN4#116-bis meeting, the simulation assumptions have been agreed (most of them only as a starting point), which was captured in the approved WF [2].

In the following, our simulation results are being presented and discussed, considering the preliminarily agreed simulation assumptions. The considered scenario is the Urban Macro deployment. Two frequency ranges are considered: 3.5 GHz and 7 GHz. For both, reference simulation parameters are based on TR 38.922. As for the Inter-Site Distance (ISD), 500 m and 450 m are considered, respectively, for 3.5 GHz and 7 GHz.

To evaluate the ACLR of aggressor UEs, two values for the maximum UE transmission power are considered, 23 dBm and 32 dBm, while the victim network only considers UEs transmitting at maximum output power of 23 dBm.

Figure 1: 5th Percentile Throughput Loss for Victim Network for 3.5 GHz

Figure 2: 5th Percentile Throughput Loss for Victim Network for 7 GHz

When the aggressor maximum transmission power is 23 dBm, it can be observed from Figure 1 and Figure 2 that the UL 5%tile throughput loss is below the threshold of 5% for the considered ACIR values above 30 dB, for both deployments at 3.5 GHz and 7 GHz. This suggests that coexistence is achieved considering available requirements.

It can be observed that the propagation characteristics at 7 GHz and the bigger antenna arrays facilitate the co-existence, further reducing the 5%tile throughput loss, compared to the deployment at 3.5 GHz, where propagation characteristics are better and the antenna array offers less directionality compared to the one at 7 GHz.

Observation 1: When the aggressor UE maximum transmission power is 23 dBm the current ACLR requirements already guarantee the coexistence, at both 3.5 GHz and 7 GHz.

Observation 2: A deployment at 7 GHz is characterized by reduced interference from an adjacent channel, due to the propagation characteristics and the bigger antenna array, which increases the directionality of transmissions and receptions.

When the aggressor maximum transmission power is 32 dBm, and for a deployment at 3.5 GHz, it can be observed from Figure 1that the UL 5%tile throughput loss exceeds the threshold of 5%, and it is necessary to increase the ACLR compared to 30 dB, by a factor X=3 dB, in order to reduce the 5%tile throughout loss at 5%.

On the other hand, for a deployment at 7 GHz and the maximum aggressor transmission power of 32 dBm, the UL 5%tile throughput loss is below the threshold of 5% for the considered ACIR values, so there is no need to increase the ACLR. Figures 3 and 4 show the CDF of the UL power of both aggressor and victim networks, for both considered frequencies.

Figure 3: CDF of UE Power for Aggressor and Victim Networks for 3.5 GHz

Figure 4: CDF of UE power for Aggressor and Victim Networks for 7 GHz

Proposal 1: When the aggressor UEs transmit at a maximum power of 32 dBm, the ACLR at the UE needs to be improved by X = 3 dB, when the operation frequency is 3.5 GHz, and X= 0 dB, when the operation frequency is 7 GHz.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: When the aggressor UEs transmit at a maximum power of 32 dBm, the ACLR at the UE needs to be improved by X = 3 dB, when the operation frequency is 3.5 GHz, and X= 0 dB, when the operation frequency is 7 GHz.

# References

- RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, RAN#109.

- R4-2515085, “Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 - HPUE coexistence”, Huawei, HiSilicon, RAN4#116-bis.