3GPP TSG-RAN WG4 Meeting #117								                        R4-2522172

Dallas, TX, USA, November 17 – 21, 2025

Agenda item:			7.6.3

Source:	Ericsson

Title:	On HPUE for NTN UE RF requirements

Document for:	Approval

# Introduction

In RAN#109, a new WID has been approved [1] for “Enhanced requirements for NR NTN and IoT NTN Phase 2” WI. The part on HPUE for NTN (PC1.5) has the following objectives:

HPUE for NTN Core Part - Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types - Example bands are Band 256 and 255 - Necessary Tx requirements, at least including - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives - The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

In RAN4#116-bis meeting, a very good progress has been made [2]. In the following, we will provide our view on one of the remaining issues.

# Discussion

Other than the MPR and the A-MPR requirements, the only remaining issue in this part of the WI is the following:

Issue 2-2-3: UE RF front end assumptions and related accommodations WayFoward: - FFS on power handling challenges when NB-IoT NTN UE is scheduled for long duration e.g. 256ms in UL with high power e.g. +29dBm. (e.g. potential accommodations to UE maximum output power requirements) - The timeline of the PC1.5 IoT NTN work shall not be impacted by this issue, even if no solution is agreed, no discussion after RAN4#118.

In current LTE specifications, the fallback to the default power class (PC3) is mandated for power class 2 UEs but only for TDD bands (“the UE shall meet…”). It occurs if TDD frame configuration is 0 or 6, as well as if IE P-max is not provided or set to the maximum output power of the default power class or lower (band 41 is a small exception). For FDD band 14, which is the only FDD band in LTE which supports PC2, fallback to the default power class is not defined for PC2, i.e. UEs are never mandated to meet all requirements for PC3. Unlike NR, reporting mechanisms based on the maximum uplink duty cycle are not defined in LTE.

The above issue from the WF originated from the following proposal in RAN4#116-bis meeting: “Proposal 1: Allow but do not mandate PC1.5 NB-IoT NTN UE to reduce transmit power to PC3 power level when 25% Tx duty cycle in a radio frame is exceeded” [3]. Allowing UEs to transmit with higher powers for longer duration can indeed improve the uplink performance, but one of the concerns was how to be sure that all UEs will meet SAR requirements if they were allowed not to fall back to the default power class when 25% Tx duty cycle in a radio frame was exceeded. However, ultimately, compliance with the SAR requirements is UE’s responsibility, while  RAN4 only defines provisions to facilitate compliance with the SAR requirements, such as P-MPR.

Hence, not mandating UE to reduce its transmit power to PC3 power level when 25% uplink duty cycle in a radio frame is exceeded could be specified, meaning that likely no changes would be needed for the core specifications. On the other hand, these requirements should be properly tested, where an RMC with up to 25% uplink duty cycle should be defined and where no P-MPR mechanism is being used.

Proposal: Allow, but do not mandate the UEs to reduce their transmit power to PC3 power level when 25% uplink duty cycle in a radio frame is exceeded. At the same time, introduce an RMC with up to 25% uplink duty cycle where P-MPR should be disabled to properly test these devices.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal: Allow, but do not mandate the UEs to reduce their transmit power to PC3 power level when 25% uplink duty cycle in a radio frame is exceeded. At the same time, introduce an RMC with up to 25% uplink duty cycle where P-MPR should be disabled to properly test these devices.

# References

- RP-252962, “New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2”, RAN#109.

- R4-2515157, “WF for [116bis][318] NR_IoT_NTN_Ph2”, CHTTL, RAN4#116-bis.

- R4-2514383, “NB-IoT NTN PC1.5 requirements”, Qualcomm Inc., RAN4#116-bis.