# 3GPP TSG-RAN WG4 Meeting #117	R4-2522196

Dallas, USA, 17th-21st of November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.108 | CR | - | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on demodulation performance for TS 36.108 - IoT_NTN_TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES, Iridium Satellite LLC, CCL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_TDD-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SAN RF demodulation performance requirements for frequency band 249 as per IoT_NTN_TDD WID, RP-242415 (with revised WID approved in RP-243293 and then RP-252935). SAN TS 36.108 specification currently supports NB-IoT NTN TDD L-band (MSS band 1616 MHz-1626.5 MHz for UL and DL, the first TDD band introduced for NTN). Moreover, SAN core RF requirements in TS 36.108 were finished at RAN4#116 (see also corresponding R4-2511149 with TS 36.108 Big CR for IoT NTN TDD SAN RF). Now, the SAN RF performance requirements have to be properly specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Satellite Access Node (SAN) performance requirements with all impacted clauses under TS 36.108, as a result of finished SAN RF core requirements under TS 36.108. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 249 may not be properly introduced under Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------<<< Start Change 1 >>>----------------------------------

## 8.5	Performance requirements for Narrowband IoT

### 8.5.1	Requirements for NPUSCH format 1

#### 8.5.1.1	Requirements

The performance requirement of NPUSCH format 1 is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A.7. The performance requirements assume HARQ retransmissions.

An NB-IoT Base Station supports 15 kHz subcarrier spacing requirements, or 3.75 kHz subcarrier spacing requirements, or both.

For 15kHz subcarrier spacing multi-subcarrier, the demodulation requirements apply for the supported number of subcarriers.

Table 8.5.1.1-1: Test parameters

| Parameter | Value |
| --- | --- |
| Maximum number of HARQ transmissions | 4 |
| RV sequence | RV0, RV2 |

##### 8.5.1.1.1	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput stated in table 8.5.1.1.1-1 for the FDD single-subcarrier of 3.75KHz subcarrier spacing, in table 8.5.1.1.1-2 for FDD multi-subcarrier of 15KHz subcarrier spacing at the given SNR for 1Tx, and in table 8.5.1.1.1-3 for TDD multi-subcarrier of 15KHz subcarrier spacing at the given SNR for 1Tx.

Table 8.5.1.1.1-1: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 3.75KHz subcarrier spacing, 1Tx, FDD

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN-TDLA100-1 | A7-1 | 128 Hz | 256 | 4 | 70% | -2.0 |
| 1 | 1 | 3.75KHz | 1 | NTN-TDLC5-1 | A7-1 | 128 Hz | 256 | 4 | 70% | -3.6 |
| 1 | 2 | 3.75KHz | 1 | NTN-TDLA100-1 | A7-1 | 128 Hz | 256 | 4 | 70% | -5.4 |
| 1 | 2 | 3.75KHz | 1 | NTN-TDLC5-1 | A7-1 | 128 Hz | 256 | 4 | 70% | -6.5 |

Table 8.5.1.1.1-2: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx, FDD

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN-TDLA100-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -3.2 |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -4.0 |
| 1 | 2 | 15KHz | 12 | NTN-TDLA100-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -7.1 |
| 1 | 2 | 15KHz | 12 | NTN-TDLC5-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -7.8 |

Table 8.5.1.1.1-3: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx, TDD

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN-TDLA100-1 | A7-2 | 8 Hz | 4 | 4 | 70% | 11.7 |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A7-2 | 8 Hz | 4 | 4 | 70% | 8.5 |

### 8.5.2	Performance requirements for NPUSCH format 2

#### 8.5.2.1	DTX to ACK performance

The DTX to ACK probability for NPUSCH format 2 case denotes the probability that ACK is detected when nothing is sent on the wanted signal and only the noise is present per NPUSCH format 2 transmission.

An NB-IoT Base Station supports 15 KHz sub-carrier spacing requirements, or 3.75 KHz sub-carrier spacing requirements, or both.

##### 8.5.2.1.1	Minimum requirement

The DTX to ACK probability, i.e. the probability that ACK is detected when nothing was sent, shall not exceed 1% per NPUSCH format 2 transmission. Where the performance measure definition is as follows:

where:

-	#(false ACK bits) denotes the number of detected ACK bits.

-	#(ACK/NACK bits) denotes the number of HARQ-ACK information bit per NPUSCH format 2 transmission.

-	#( NPUSCH format 2 DTX) denotes the number of DTX occasions.

#### 8.5.2.2	ACK missed detection requirements

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent per NPUSCH format 2 transmission.

##### 8.5.2.2.1	Minimum requirements

The ACK missed detection probability shall not exceed 1% at the SNR given in table 8.5.2.2.1-1, table 8.5.2.2.1-2 and table 8.5.2.2.1-3 for 1Tx case.

Table 8.5.2.2.1-1: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 3.75KHz subcarrier spacing, 1Tx for FDD frequency bands

| Number of TX antennas | Number of RX antennas | Propagation conditions (Annex D) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 3.75KHz | 64 Hz | 256 | 4 | 1.8 |
| 1 | 2 | NTN-TDLA100-1 | 1 | 3.75KHz | 64 Hz | 256 | 4 | -3.9 |

Table 8.5.2.2.1-2: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, 1Tx for FDD frequency bands

| Number of TX antennas | Number of RX antennas | Propagation conditions (Annex D) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 15KHz | 16 Hz | 32 | 16 | 5.1 |
| 1 | 2 | NTN-TDLA100-1 | 1 | 15KHz | 16 Hz | 32 | 16 | -3.3 |

Table 8.5.2.2.1-3: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, 1Tx, for TDD frequency bands

| Number of TX antennas | Number of RX antennas | Propagation conditions (Annex D) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 15KHz | 16 Hz | 16 | 8 | 0.4 |

### 8.5.3	Performance requirements for NPRACH

#### 8.5.3.1	NPRACH False alarm probability

The false alarm requirement is valid for any number of receive antennas, for all repetition numbers and for any number of subcarriers.

The false alarm probability is the conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

##### 8.5.3.1.1	Minimum requirement

The false alarm probability shall be less than or equal to 0.1%.

#### 8.5.3.2	NPRACH detection requirements

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. For NTN-TDLA100 and NTN-TDLC5, a timing estimation error occurs if the estimation error of the timing of the largest delay path is larger than the time error tolerance given in Table 8.5.3.2-2.

Table 8.5.3.2-1: Test preambles for NPRACH

| Parameter | Value |
| --- | --- |
| Narrowband physical layer cell identity | 0 |
| Initial subcarrier index | 0 |

Table 8.5.3.2-2: Time error tolerance for AWGN, NTN-TDLA100-1 and NTN-TDLC5-1

| PRACH | PRACH SCS | Time error tolerance |
| --- | --- | --- |
| preamble | (kHz) | NTN-TDLA100-1, NTN-TDLC5-1 |
| 0 | 3.75KHz | 3.646us |
| 1 | 3.75KHz | 3.646us |

##### 8.5.3.2.1	Minimum requirements

The probability of detection shall be equal to or exceed 99% for the SNR levels listed in table 8.5.3.2.1-1 and table 8.5.3.2.1-2.

Table 8.5.3.2.1-1: NPRACH missed detection requirements for FDD frequency bands

| Number of TX antennas | Number of RX antennas | TxDuration | Repetition number | Propagation conditions (Annex D) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | 8 | 8 | NTN-TDLA100-1 | 200 Hz | 13.3 | 13.7 |
| 1 | 1 | 16 | 16 | NTN-TDLA100-1 | 200 Hz | 9.8 | 9.6 |
| 1 | 2 | 8 | 8 | NTN-TDLA100-1 | 200 Hz | 6.0 | 6.1 |
| 1 | 2 | 16 | 16 | NTN-TDLA100-1 | 200 Hz | 1.6 | 1.7 |

Table 8.5.3.2.1-2: NPRACH missed detection requirements for TDD frequency bands

| Number of TX antennas | Number of RX antennas | TxDuration | Repetition number | Propagation conditions (Annex D) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | 8 | 8 | NTN-TDLA100-1 | 200 Hz | 9.9 | 10.1 |
| 1 | 1 | 16 | 16 | NTN-TDLA100-1 | 200 Hz | 5.6 | [5.9] |
| 1 | 1 | 8 | 8 | NTN-TDLC5-1 | 200 Hz | 6.3 | 6.5 |
| 1 | 1 | 16 | 16 | NTN-TDLC5-1 | 200 Hz | 3.4 | [3.4] |

------------------------------<<< End Change 1 >>>-------------------------