3GPP TSG-RAN WG4 Meeting #117	R4-2522197

Dallas, US, 17th – 21th November 2025

Source:	BT

Title:	Discussion on NR SU-MIMO performance requirements using SCM

Agenda item:	7.5.3

Document for:	Discussion

Introduction

A WF for SCM was agreed in RAN4#116-bis meeting [1]. The Way Forward document captured the agreements, open issues and candidate options for Rel-20 Demod performance requirements WI and Rel-19 Demod performance requirements maintenance. In this contribution, we provide our views on the WF.

Discussion

Frequency generalization for rCDL channel modelling

In the last meeting, RAN4 agreed to define the requirements based on the same channel model for all different FR1 frequencies.

- The maximum Doppler fD should be fixed as X Hz

    - As baseline for Rel-20 demod requirement, X =10

    - FFS if other fD can be considered

Fix the maximum doppler to X Hz, as the frequency has no impact on the array gain at half-wavelength spacing, whereas the array gain is equal to the number of elements.

Angular Scaling for rCDL

In the last meeting, RAN4 agreed to update the rCDL channel model based on the latest angular scaling procedure as in TR 38.901 v19.1.0 and further discuss the details. Further agreement are required on the details.

- Agreement on the details:

  - Use the ‘s’ values for tabulated desired AS values rather than the desired AS values we used in 38.753 to save us the effort of obtaining ‘s’ and aligning among companies.

  - Obtain ‘s’ from the tabulated example AS desired values and use the following desired AS: ASD – 25; ASA – 60; ZSD – 5, ZSA – 15 as baseline to configure rCDL channel model

  - Additional rCDL Channel Models based on different AS are not precluded

For alignment among companies preference should be given to the use of the tabulated desired AS values.

eType II PMI reporting requirements based on SCM

In the last meeting, RAN4 presented the following candidate options.

- Candidate options:

  - Option 1: Specify requirements for PMI performance with eType II precoding

- Option 1a: Select the lowest γ value among the ones companies provided.

- Option 1b: Outlier removal to be applied to form requirements, if outliers are not aligned in second simulation results delivery round.

  - Option 2: Further simulation result alignment is required

- Option 2a: Collect both TP gain and SNR @ 90% Max TP to evaluate alignment

- Option 2b: RAN4 can take 8Tx 4 layers Type-I PMI case for initial alignment if rCDL model is updated.

Specify the requirements for PMI performance with eType II and simulation implementations should be reviewed to improve alignment.

eType II PMI reporting

- Candidate options:

  - Option 1: Only consider rank 2

  - Option 2: Consider rank 4

To include Rank 4 in the study along with the study of Rank 2, as Rank 4 is highly dependent on the spatial channel.

Conclusion

We propose the following

- Fix the maximum doppler to X Hz, as the frequency has no impact on the array gain at half-wavelength spacing, whereas the array gain is equal to the number of elements.

- For alignment among companies preference should be given to the use of the tabulated desired AS values.

- Specify the requirements for PMI performance with eType II and simulation implementations should be reviewed to improve alignment.

- To include Rank 4 in the study along with the study of Rank 2, as Rank 4 is highly dependent on the spatial channel.

References

- RP-2514823, “WF on NR_demod_Ph6_Ph5”, October 2025