3GPP TSG-RAN WG4 Meeting #116bis	R4-2514627

# Prague, Czech Republic,  13th - 17th October 2025				(revision of R4-2513845)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | <draft> | rev | <#> | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the NTN NR band n250 UL configuration for REFSENS testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_combinedLband-Core |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 3GPP introduced two new L-bands – n250 and n251 – where band n250 UL is mapped to the ETSI sub-range 2, size of which is 7MHz. Thus, band n250 UL cannot accommodate NR channels larger than 5MHz, but REFSENS should be tested for all applicable DL channel bandwidths, i.e. 5, 10, 15 and 20MHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 7.3.2-2, a clarificatiion is added which UL configuration should be used to test REFSENS for DL channel bandwidths larger than 5MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN band n250 will have incorrect UL configuration for the REFSENS testing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.3.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A3.2.2 of 3GPP TS 38.101-1 [5], with parameters specified in Table 7.3.2-1.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -101.7 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -101.7 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -101.7 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
|  | 15 |  | -100.0 |  |  |  |  |  |  |  |  |  |
| n253 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n252 | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n251 | 30 |  |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n250 | 30 |  |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
| NOTE: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |  |

For power class 2 and power class 1 UEs, certain degradation of the reference sensitivity in Table 7.3.2-1 is allowed. For a power class 2 UE the maximum amount of degradation is specified in Table 7.3.2-1a, and in Table 7.3.2-1b for a UE that indicates txDiversity-r16 or txDiversity2Tx-r18. For a power class 1 UE the maximum amount of degradation is specified in Table 7.3.2-1c, and in Table 7.3.2-1d for a UE that indicates txDiversity-4Tx-r18.

Table 7.3.2-1a Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE with 1Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| n255 | 0.8 | 0.8 | 0.9 | 0.9 |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-1b Reference Sensitivity Degradation from PC3 to PC2 for
FDD bands for UE with 2Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| n255 | 1.6 | 1.6 | 1.7 | 1.7 |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-1c Reference Sensitivity Degradation from PC3 to PC1 for
FDD bands for UE with 1Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 1.5 | 1.5 | 1.5 | 1.5 |  |  |  |  |  |  |
| n255 | 2 | 2 | 2 | 2 |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-1d Reference Sensitivity Degradation from PC3 to PC1 for
FDD bands for UE with 4Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 2.1 | 2.1 | 2.1 | 2.1 |  |  |  |  |  |  |
| n255 | 3.3 | 3.3 | 3.3 | 3.3 |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |

The reference receiver sensitivity (REFSENS) requirement specified in Table 7.3.2-1 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-2 and with default Tx-Rx carrier center frequency separation except for cases specified in Table 7.3.2-3.

Table 7.3.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  |  | 24 | 36 | 50 | FDD |
|  | 60 |  |  | 10 | 18 | 24 |  |
|  | 15 | 15 | 25 | 50 | 75 | 752 503 |  |
| n255 | 30 |  |  | 24 | 36 | 362 243 | FDD |
|  | 60 |  |  | 10 | 18 | 182 103 |  |
|  | 15 | 15 | 25 | 50 | 75 |  |  |
| n254 | 30 |  |  | 24 | 36 |  | FDD |
|  | 60 |  |  | 10 | 18 |  |  |
|  | 15 |  | 25 |  |  |  |  |
| n253 | 30 |  |  |  |  |  | FDD |
|  | 60 |  |  |  |  |  |  |
|  | 15 |  | 25 | 50 | 75 | 100 |  |
| n252 | 30 |  |  | 24 | 36 | 50 | FDD |
|  | 60 |  |  | 10 | 18 | 24 |  |
|  | 15 |  | 25 | 50 | 75 | 752 503 |  |
| n251 | 30 |  |  | 24 | 36 | 362 243 | FDD |
|  | 60 |  |  | 10 | 18 | 182 103 |  |
| n250 | 15 |  | 25 | 254 | 254 | 254 | FDD |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. NOTE 2: Applicable for Tx-Rx frequency separation of -101.5 MHz and -115.5 MHz. NOTE 3: Applicable for Tx-Rx frequency separation of -87.5 MHz. NOTE 4: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used |  |  |  |  |  |  |  |

Table 7.3.2-3: TX – RX carrier centre frequency separation for REFSENS verification

| Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| n256 | 5 MHz | 165 MHz, 215 MHz |
|  | 20 MHz | 180 MHz, 200 MHz |
| n255 | 5 MHz | -72.5 MHz, -130.5 MHz |
|  | 20 MHz | -87.5 MHz, -115.5 MHz |
| n253 | 5 MHz | -148MHz, -152MHz |
| n252 | 5 MHz | 165 MHz, 195 MHz |
|  | 15 MHz | 175 MHz, 185 MHz |
| n251 | 5 MHz | -72.5 MHz, -137.5 MHz |
|  | 20 MHz | -87.5 MHz, -122.5MHz |
| n250 | 5 MHz | -114 MHz, -152 MHz |
|  | 20 MHz1 | -129 MHz, -137 MHz |
| NOTE 1: For this DL channel bandwidth, the highest UL channel bandwidth is specified in Table 5.3.6-1 |  |  |

The minimum requirements specified in Table 7.3.2-1 shall be verified with the network signalling value NS_01 configured in Table 6.2.3.1-1.