3GPP TSG-WG4 Meeting #116bis	draft_R4-2514596

# Prague, Czech Republic, 13-17 Oct 2025

# Title:	WF on R19 spectrum enhancement

# Agenda Item:	4.2.8

# Source:	Skyworks Solutions, Inc., Murata Manufacturing Corp, Qualcomm France.

# Document for:	Approval

# 1		Introduction

This WF captures the agreements on the mandatory 25MHz CBW for band n71 and on MSD re-evaluation to account for Band n41 10MHz CBW based on proposals [1…6] as a follow up of the agreed WF [7].

# 2		WF#1 on MSD re-evaluation due to Band n41 10MHz CBW

Agreement 1 on MSD for band n41 within an inter-band CA combination where the band n41 DL CBW is changed to 10MHz.

Correct the band n41 5MHz CBW PC3 MSD requirement using the following correction factors:

- Band n41 5MHz CBW MSD>=10dB: 			2.0dB correction,

- 7dB <= Band n41 5MHz CBW MSD < 10dB:	1.5dB correction,

- Band n41 5MHz CBW MSD < 7dB:				1.0dB correction.

The band n41 HPUE MSD requirements can be revisited using the MSD simplification Look-Up Tables.

Agreement 2 on MSD for other DL band within an inter-band CA combination where the band n41 UL CBW is changed to 10MHz.

Correct the other band PC3 MSD requirement using the following correction factors:

- Other DL band MSD>=10dB: 					1.5dB correction,

- 1dB for 7dB<= Other DL band MSD < 10dB:	1.0dB correction,

- Other DL band MSD < 7dB:						0.5dB correction.

The other DL band MSD HPUE MSD requirements can be revisited using the MSD simplification Look-Up Tables.

# 3		WF#2 on Band n71 mandatory support of 25MHz CBW

Agreements:

25MHz UL for n71 is mandatory from Rel-19 onwards.

The following modifications into 38.101 are to be made:

Modify n71 Channel Bandwidths, REFSENS, and UL RB allocation for REFSENS as follows for Rel-19

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n71 | 15 |  | 5 | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |
| NOTE 12: This UE channel Bandwidth is optional for uplink in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n71 | 15 |  | -97.2 | -94.0 | -91.6 | -86.0 | -84.19 -74.810 | -82.59 -67.110 | -80.79 -64.010 |  |  |  |
|  | 30 |  |  | -94.3 | -91.9 | -87.4 | -84.29 -74.910 | -82.69 -67.210 | -80.89 -64.110 |  |  |  |
| NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 9: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 10: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n71 | 15 |  | 25 | 251 | 201 | 201 | 201,6 | 201,6 | 201,6 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 121 | 101 | 101 | 101,6 | 101,6 | 101,6 |  |  |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. For UEs not supporting this uplink channel bandwidth, the UL configuration of the 20MHz UL channel bandwidth and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 |  | 0.5 | 0.9 | 0.9 | 2.2 | 2.42 2.5 | 2.52 2.43 | 2.92 3.13 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |

Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 |  | 1.1 | 1.1 | 1.7 | 5.5 | 5.92 6.9 | 6.22 7.23 | 6.52 7.33 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |

Modify CA_n5A-n71A MSD with n71 UL as follows for Rel-19

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 688685.5 | 2025 | 15 | 20 (RBstart=113) | 871.5 | 5 | 2.06 | >ACLR2 |

Modify CA_n71A-n85A PC3 MSD with n71 UL as follows

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 8.210.26 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 237 | ACLR1 |

Modify CA_n71A-n85A PC2 MSD with n71 UL as follows

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 13.24,6 16.24,7 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 265,6 32.35,7 | ACLR1 |
| NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |  |

# 4			References

- R4-2514293 Symmetrical UL 25MHz  DL 25MHz CBW for band n71, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Huawei.

- R4-2513295 MSD analysis for 10MHz CBW of n41/n90, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Murata Manufacturing Co Ltd.

- R4-2513322 MSD analysis for UL CBW 25MHz of n71, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Murata Manufacturing Co Ltd.

- R4-2514315 25MHz UL BW for n71, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Qualcomm.

- R4-2514494 On Band n71 mandatory support for UL/DL 25MHz CBW, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Skyworks Solutions, Inc.

- R4-2514499 Impact of replacing Band n41 5MHz with 10MHz CBW, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Skyworks Solutions, Inc.

- R4-2511779 WF on optional channel bandwidth, 3GPP TSG-RAN WG4 Meeting # 116, Bengaluru, India, Skyworks Solutions, Inc., Huawei, Murata.