3GPP TSG-RAN WG4 Meeting #116bis					        R4-2513606

Prague, Czech Republic, October 13 – 17, 2025

Title:	LS of RAN4 RRM agreement update on switching pattern application

Response to:

Release:	Rel-19

Work Item:	NR_LBCA_Sw-Core

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Jie Cui,  jie_cui@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN4 has further discussed the applicability of switching pattern for SDL SCell for RRM requirements and would like to share the following update to RAN1.

- Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options. - For MAC CE based SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length. - Other options are not precluded. - For RRC based direct SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length. - Other options are not precluded. Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline.

# 2	Actions

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to take the above into account in their future work.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #117	17th – 21st Nov 2025			Dallas, USA

TSG-RAN WG4 Meeting #118	09th – 13th Feb 2026			Gothenburg, Sweden