# 3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514821

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:			7.4.1

Source:	Ericsson

Title:	WF on NR_RRM_Ph6

Document for:	Approval

# Topic #1: (e)RedCap UE enhancement requirements (AI 7.4.3.2)

Issue 2-1-1: Scope for LT5 (e)RedCap UE

Agreement:

- For (e)RedCap UE supporting LT5:

  - CSI-RS based measurement is out of scope.

  - RAN4 only defines the requirements for PCell.

  - RAN4 only defines the requirements for NR FR1.

  - Introduce support for only 15 PRB transmission bandwidth configuration and 12 PRB SSB

Issue 2-1-2: LT5 for 2Rx (e)RedCap UE

Agreement:

- The existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes.

  - Handover

  - RLM/BFD

  - Intra-frequency/Inter-frequency measurement

- Further check for HD-FDD

Issue 2-2-1: Scope of relaxed RLM/BFD for (e)RedCap UE

Agreement:

RAN4 only considers the relaxed RLM/BFD requirements on PCell when the DRX cycle is no longer than 80 ms.

Issue 2-2-2: Relaxed RLM/BFD for 2Rx (e)RedCap UE

Agreement:

- Reuse the existing R17 RLM/BFD relaxation requirements for both FR1 and FR2, and for full-duplex FDD and TDD.

Sub-topic 2-3: Simulation assumption for LT5 1Rx (e)RedCap

Agreement:

Simulation Assumptions for SSB index reading and MIB reading Evaluation

| Parameter | Unit | Value |
| --- | --- | --- |
| - Number of PRBs for PBCH |  | 12 PRBs |
| - Carrier frequency | - GHz | - 900MHz |
| - Subcarrier spacing | - kHz | - 15 kHz |
| - Number of Tx antennas | - - | - 1 - 2 |
| - Number of Rx antennas | - - | - 1 |
| - DMRS | - - | - PBCH DMRS |
| - Other assumptions |  | - Tx BW and SSB puncturing are known at the Rx side |
| - CP Length | - - | - Normal |
| - Number of transmitted SS block within a SS burst set period (K) | - - | - 1 |
| - SS burst set periodicity | - ms | - 20 |
| - Frequency Offset relative to UE frequency reference | - Hz | - 0 |
| - PBCH symbols within the SS block |  | - PSS-PBCH-SSS-PBCH |
| - Data and Control Power offset with respect to PSS and SSS | - dB | - Baseline 0 |
| - PBCH power offset with respect to PBCH-DMRS | - dB | - 0 |
| - PBCH-DMRS power offset with respect to PSS and SSS | - dB | - 0 |
| - PSS and SSS sequences | - - | - No changes expected |
| - PBCH-DMRS sequences | - - | - No changes expected |
| - PBCH-DMRS RE positions within the PBCH resource | - - | - No changes expected except for puncturing impact |
| - PBCH Channel coding |  | - No changes expected to actual Channel coding - (Polar code with 512 length and 24bit CRC) |
| - PBCH Modulation | - - | - QPSK |
| - PBCH Payload (including the CRC) | - bits | - 56bit (CRC 24bit) |
| - PBCH SNR | - dB | - -10 : 0 dB, with 1 dB spacing |
| - Propagation Condition / Channel models | - - | - For 3 km/h UE speed - TDL-C 300ns - Additional scenarios can be considered |
| - Detection Method |  | - Baseline: One shot detection (i.e. no combination for different PBCHs) - FFS whether to remove Soft Combining or keep Soft Combining as Optional |
| - Metrics |  | - TBA |
| - NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned. |  |  |

Issue 2-1-4: RLM/BFD impact of LT5 for 1Rx (e)RedCap UE

Agreement:

For discussing the PDCCH hypothetical parameters, RAN4 discuss whether it is feasible to target on coverage equivalence between 1Rx and 2Rx RedCap UEs.

Issue 2-2-4: Entry and exit criteria, singling indication, applicability rules for 2Rx

Agreement:

Reuse R17 RLM/BFD Entry and exit criteria from legacy UE as a baseline