3GPP TSG-RAN WG4 Meeting #117                                                	R4-2522971

# Dallas, USA, 17th – 21st of November 2025

Agenda item:			4.1.3

Source:	Moderator (Iridium)

Title:	Ad-Hoc minutes: [117][325] NR_IoT_NTN_Maintenance_RRM

Document for:	Information

# Introduction

Proposals discussed came from the contributions submitted under AIs 4.11.2.3, 4.25.3, 4.26.2, 4.27.4 for the RAN4#117 meeting.

The following documents submitted into [117][325] NR_IoT_NTN_req_test_enh-Core under AI 4.11.2.3 are:

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2521629 | CR | CR on requirements for less than 5MHz for NTN | Huawei, HiSilicon | Agreement | 4.11.2.3 |

- Recommendation for treatment of the above: to be discussed during the meeting; to be agreed if no objections.

The following documents submitted into [117][325] NR_NTN_Ph3-Core under AI 4.25.3 are:

(Split by Tdoc type):

- Type #1: Discussion papers

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2521540 | Other (discussion) | Discussion on RRM requirements on NTN phase 3 | ZTE Corporation, Sanechips | Approval? | 4.25.3 |
| R4-2521730 | Other (discussion) | Impact of configuring 2 SMTC periodicities over multiple SMTC occasions in NTN | Nokia | Approval? | 4.25.3 |
| R4-2521217 | discussion | Discussion on RRM requirements for NTN for NR Phase 3 | Ericsson | discussion | 4.25.3 |
| R4-2520977 | discussion | Discussion on the RRM requirement maintenance for NTN phase3 | CMCC | discussion | 4.25.3 |
| R4-2520664 | discussion | On R19 NTN RRM core part | Apple | discussion | 4.25.3 |
| R4-2520107 | discussion | Discussion on RRM core maintenance for Rel-19 NTN phase3 | CATT | discussion | 4.25.3 |
| R4-2520497 | discussion | (NR_NTN_Ph3-Core) Discussion on remaining issues on RRM core requirements of NTN for NR Phase 3 | Xiaomi | discussion | 4.25.3 |

- Recommendation for treatment of the above: to be discussed during the meeting.

- Type #2: (Draft) CRs

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2521577 | CR | (NR_NTN_Ph3-Core) CR on Handover for SAN with 6 SMTCs and different periodicities | Ericsson | Agreement | 4.25.3 |
| R4-2521554 | CR | (NR_NTN_Ph3-Core)_Modification on radio link monitoring for HD-FDD UE with SAN | ZTE Corporation, Sanechips | Agreement | 4.25.3 |
| R4-2521555 | CR | (NR_NTN_Ph3-Core)_Modification on applicability rule for SDT | ZTE Corporation, Sanechips | Agreement | 4.25.3 |
| R4-2521556 | CR | (NR_NTN_Ph3-Core)_Modification on RedCap UE in NTN scenario requirements in RRC IDLE state | ZTE Corporation, Sanechips | Agreement | 4.25.3 |
| R4-2521557 | CR | (NR_NTN_Ph3-Core) Supplement random access for HD-FDD UE with SAN | ZTE Corporation, Sanechips | Agreement | 4.25.3 |
| R4-2521643 | CR | CR on RRM requirements for RedCap UE in NTN | Huawei, HiSilicon | Agreement | 4.25.3 |
| R4-2521644 | CR | CR on DL coverage enhancement in NTN | Huawei, HiSilicon | Agreement | 4.25.3 |
| R4-2521680 | CR | CR on Handover for SAN with different periodicities | Ericsson | Agreement | 4.25.3 |
| R4-2521247 | CR | CR 38.133 Correction to applicability requirements for RedCap in NTN | Nokia | Agreement | 4.25.3 |
| R4-2521218 | CR | (NR_NTN_Ph3-Core) CR on Handover for SAN with 6 SMTCs and different periodicities | Ericsson | Agreement | 4.25.3 |
| R4-2520978 | CR | CR on correction of SMTC enhancement requirement for NTN phase 3 | CMCC | Agreement | 4.25.3 |
| R4-2520106 | CR | (NR_NTN_Ph3-Core) CR on RRM core maintenance for NTN Phase 3 | CATT | Agreement | 4.25.3 |
| R4-2520514 | CR | (NR_NTN_Ph3-Core) CR on RRM core maintenance for NTN Phase 3 part 2 | CATT | Agreement | 4.25.3 |
| R4-2520498 | Draft CR | (NR_NTN_Ph3-Core) draftCR on Cell Re-selection requirememts for SMTC enhancement | Xiaomi | Agreement | 4.25.3 |
| R4-2520499 | Draft CR | (NR_NTN_Ph3-Core) draftCR on L3 measurement requirememts for SMTC enhancement | Xiaomi | Agreement | 4.25.3 |
| R4-2520105 | (Big) CR (reserved) | Big CR on RRM core maintenance for NR_NTN_Ph3-Core | CATT | Agreement | 4.25.3 |

- Recommendation for treatment of the above: to be discussed during the meeting, to be agreed/endorsed if no objections.

- Type #3: LS

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2521642 | LS out | Discussion on RRM core requirements for Rel-19 NR NTN | Huawei, HiSilicon | Approval | 4.25.3 |

- Recommendation for treatment of the above: to be discussed during the meeting, to be approved if no objections.

The following documents submitted into [117][325] IoT_NTN_Ph3-Core under AI 4.26.2 are:

(Split by Tdoc type):

- Type #1: Discussion papers

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2520979 | Discussion | Discussion on the RRM requirement maintenance for IoT NTN phase3 | CMCC | Discussion | 4.26.2 |
| R4-2521215 | Discussion | On TA command adjust timing for NB-IoT NTN | Ericsson | Discussion | 4.26.2 |

- Recommendation for treatment of the above: to be discussed during the meeting.

- Type #2: CRs

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2520036 | CR | CR on channel quality report in Msg-3-EDT for anchor carrier | MediaTek | Agreement | 4.26.2 |
| R4-2521731 | CR | CR on conditional HARQ-ACK feedback for CB-Msg3 Requirements | Nokia | Agreement | 4.26.2 |

- Recommendation for treatment of the above: to be discussed during the meeting, to be agreed/endorsed if no objections.

- Type #3: LS out (Reply)

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2520980 | LS out | Reply LS on CQI reporting for CB-Msg3 EDT for eMTC UE | CMCC | Approval | 4.26.2 |

- Recommendation for treatment of the above: to be discussed during the meeting, to be approved if no objections.

The following documents submitted into [117][325] IoT_NTN_TDD-Core under AI 4.27.4 are:

(Split by Tdoc type):

- Type #1: Discussion papers

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2521734 | Discussion (Submitted as: Other) | On removing support of segmented pre-compensation for IoT NTN TDD | Nokia | Discussion (Submitted for: Approval) | 4.27.4 |
| R4-2521651 | Discussion | Discussion on time pre-compensation for IoT NTN TDD | Huawei, HiSilicon | Discussion | 4.27.4 |

- Recommendation for treatment of the above: to be discussed during the meeting.

- Type #2: CRs

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2520890 | CR | CR on pre-compensation for timing error requirement for IoT NTN TDD (Clause 7.20B.2) | LG Electronics Inc. | Agreement | 4.27.4 |
| R4-2521733 | CR | CR with Editorial Correction for IoT NTN TDD Requirements | Nokia | Agreement | 4.27.4 |
| R4-2521735 | CR | (IoT_NTN_TDD-Perf) CR on removing support for segmented precompensation | Nokia | Agreement | 4.27.4 |
| R4-2521652 | CR | CR on core requirements for IoT NTN TDD | Huawei, HiSilicon | Agreement | 4.27.4 |

- Recommendation for treatment of the above: to be discussed during the meeting; to be agreed if no objections.

# List of topics/sub-topics/issues prior to the meeting:

| TDoc Number | Company | General Purpose | Agenda Item |
| --- | --- | --- | --- |
| R4-2521629 | Huawei, HiSilicon | Agreement | 4.11.2.3 |

# Topic #1 AI 4.11.2.3 RRM Core Req [NR_IoT_NTN_req_test_enh-Core]

For ONline and OFFline sessions:

- Topic #1: [NR_IoT_NTN_req_test_enh-Core] under AI 4.11.2.3 (NWM)

  - R4-2521629 CR for agreement

Recommended WF: Not flaggedAgree to the CR. (NWM)

# Topic #2 AI 4.25.3 RRM Core Req [NR_NTN_Ph3-Core]

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2521540 | ZTE Corporation, Sanechips | Observation 1: There is a controversial between RAN2 understanding and RAN4 specification description on SMTC selection if more than 4SMTCs are configured by network. Proposal 1: RAN4 shall clarify the principle on SMTC selection if more than 4SMTCs are configured with RAN2 understanding. Observation 2: If UE is capable of twoSMTC-Periodicities-r19, UE supports NTN SSB based RRM measurements on target cells using two SMTC periodicities on a single frequency carrier. If UE does not support this capability, UE will have same periodicity with smtc1. Proposal 2: RAN4 shall consider RAN4 specification description with RAN2 outcome on two different SMTC periodicities. |
| R4-2521730 | Nokia | Observation 1: In the example of Figure 1, the requirements related to SMTC1 are expanded by 2, even though only 1/8 of the SMTC occasions are colliding with other SMTCs. Proposal 1: When SMTCs with different periodicities collide, for measurements purposes, the SMTC with longer periodicity shall always take precedence over the SMTC with shortest periodicity. Observation 2: In the example above, the total time for the UE to have 5 measurement opportunities in SMTC2 will be enhanced from 1285 ms to 645 ms. Proposal 2: The value of K_multi_SMTC is to be calculated only considering SMTCs with same periodicity. Proposal 3: For SMTCs with shorter periodicity that overlap with SMTCs with larger periodicity the formula for the measurement requirements shall consider a factor L_multi_SMTC, which corresponds to the number of measurement opportunities lost due to collision with SMTCs with larger periodicity within 5 SMTC occasions. Proposal 4: Adopt proposals 3, 4 and 5 for all cases in which the longest SMTC periodicity equals 160 ms, and in the other cases, adopt the proposal from RAN4 #116: Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i. |
| R4-2521217 | Ericsson | Proposal 1: We are open to the solutions of optimizing value of Kmulti_SMTC,i to mitigate the degradation of long SMTC periodicity. Proposal 2: RAN4 to take below scaling factor into account of Kmulti_SMTC,i optimization. - Koccupy,i : = 1-SMTCi / MAX(SMTC), if SMTCi < MAX(SMTC) = 1, if SMTCi = MAX(SMTC) - Kmulti_SMTC,i only calculates the collsion between SMTCs with same periodicity Table 9.2C.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1-NTN) DRX cycle; TPSS/SSS_sync_intra. No DRX; max( 600 ms, ceil( 5 x Kp x Klayer1_measurement x Koccupy_i) x Kmulti_SMTC,i x SMTC period )Note 1 x CSSFintra. DRX cycle≤ 320 ms; max( 600 ms, ceil(1.5 x 5 x Kp x Klayer1_measurement x Koccupy_i) x Kmulti_SMTC,i x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320 ms; ceil(5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Proposal 3: K_satellite in inter-frequency measurement requirements shall be updated in the same way as Kmulti_SMTC. Proposal 4: For supporting configuration of more than 4 SMTCs per frequency (i.e., 6), the SMTC selection is left to UE implementation, and no update is needed for current RRC Idle/Inactive mode RRM requirements. |
| R4-2520977 | CMCC | Proposal 1: In RRC-IDLE/Inactive, update the SMTC selection rule, following wording for intra-frequency cell re-selection measurement is proposed: The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise - For UE which support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE shall select the SMTC to be used of all the configured SMTCs up to the parallelSMTC-r17, the selection is up to UE implementation, and the requirements in this clause apply. - For UE which does not support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, if UE does not support twoSMTC-Periodicities-r19 and the number of SMTCs with a periodicity equal to the serving cell SMTC periodicity doesn’t exceed the parallelSMTC-r17, the requirements in this clause apply. Otherwise, UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected Proposal 2: In RRC-IDLE/Inactive, update the SMTC selection rule, following wording for inter-frequency cell re-selection measurement is proposed: The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the values indicated by parallelSMTC-r17, otherwise - For UE which support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE shall select the SMTC to be used of all the configured SMTCs up to the parallelSMTC-r17, the selection is up to UE implementation, and the requirements in this clause apply. - For UE which does not support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE may select one or subset of all the configured SMTCs sequentially until all of the SMTCs can be measured, the selection of SMTCs to be used is up to UE implementation, and longer measurement delay than the corresponding measurement period specified in table 4.2C.2.4-1 and table 4.2C.2.4-2 is expected. |
| R4-2520664 | Apple | Proposal 1: For RRC Idle/Inactive mode RRM requirements, if the NW configures more than 4 SMTCs, the UE in idle/inactive selects up to 4 SMTCs (or less, depending on the UE capability) based on the closest reference locations for measurement. Proposal 2: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE. |
| R4-2520107 | CATT | Proposal 1: If the NW configures more than 4 STMCs, RAN4 to use the same description as TS 38.306 to support SMTC selection based on UE position and associated location for each SMTC in NTN (as specified in TS 38.331) in RRC_ILDE/RRC_INACTIVE mode. Proposal 2: RAN4 need to consider the impact of two different SMTC periodicities (with different offsets) in one frequency layer for idle/inactive/connected mode. Proposal 3: RAN4 to clarify that the definition of overlapping when configuring two different SMTC periodicities (with different offsets) in one frequency layer, such as: two SMTCs are considered as overlapping if they overlap in one or more occasions. Proposal 4: For each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i. |
| R4-2520497 | Xiaomi | Proposal 1: From RAN4 perspective, the SMTC enhancement capability can apply to FR2-NTN, based on the legacy assumption on the measurement: - no inter-satellite measurement is configured; - single SAN Tx beam per radio cell in DL; - same UE Rx beam is used for both serving and neighbouring cells which belong to the same satellite. Proposal 2: If the UE supports Location based SMTC selection and related NW configuration is present, requirements in section 4.2C.2.3 and 4.2C.2.4 apply and the SMTCs referenced in the requirements refer to the SMTCs selected by the UE. Proposal 3: RAN4 to clarify which SMTC periodicity used during PSS/SSS detection for intra-frequency measurement and inter-frequency measurement requirements in RRC_IDLE/INACTIVE mode. |

For ONline and OFFline sessions:

Topic #2 [NR_NTN_Ph3-Core] AI 4.25.3

- Sub-topic 2-1: SMTC related

Issue 2-1-1: Decide on the value of Kmulti_SMTC,i

- Proposal 1: For each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i. (CATT, NOK)

- Proposal 2: RAN4 to take below scaling factor into account of Kmulti_SMTC,i optimization.
   (Ericsson)

- Koccupy,i :

=1/ (1-SMTCi / MAX(SMTC)), if SMTCi  < MAX(SMTC)

= 1, if SMTCi  = MAX(SMTC)

- Kmulti_SMTC,i  only calculates the collision between SMTCs with same periodicity

- Recommended WF: Proposal 1

Issue 2-1-2: If the NW configures more than 4 STMCs

(For RRC Idle/Inactive mode RRM requirements)

- Proposal 1: if the NW configures more than 4 SMTCs, the UE in idle/inactive selects up to 4 SMTCs (or less, depending on the UE capability) based on the closest reference locations for measurement.

- Proposal 2: for supporting configuration of more than 4 SMTCs per frequency (i.e., 6), the SMTC selection is left to UE implementation, and no update is needed for current RRC Idle/Inactive mode RRM requirements.

When UE supports location based, then P3:

- Proposal 3: If the NW configures more than 4 STMCs, RAN4 to use the same description as TS 38.306 to support SMTC selection based on UE position and associated location for each SMTC in NTN (as specified in TS 38.331) in RRC_ILDE/RRC_INACTIVE mode.
(reflected in the CR R4-2520978)

Proposal 3A (intra): In RRC-IDLE/Inactive, update the SMTC selection rule, following wording for intra-frequency cell re-selection measurement is proposed:

The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise

-	For UE which support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE shall select the SMTC to be used of all the configured SMTCs up to the parallelSMTC-r17, the selection is up to UE implementation, and the requirements in this clause apply.

-	For UE which does not support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, if UE does not support twoSMTC-Periodicities-r19 and the number of SMTCs with a periodicity equal to the serving cell SMTC periodicity doesn’t exceed the parallelSMTC-r17, the requirements in this clause apply. Otherwise, UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected

#### 4.2C.2.3	Measurements of intra-frequency NR cells

Table 4.2C.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
|  | FR1 and FR2-NTN |  |  |  |
| 0.32 | 1 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| NOTE 1: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 4 upon more than 1 SMTC configured at the UE; M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; otherwise M2=1. Where, NSMTC is the number of SMTCs configured by SAN If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. NOTE 2: The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment. |  |  |  |  |

Proposal 3B (inter): In RRC-IDLE/Inactive, update the SMTC selection rule, following wording for inter-frequency cell re-selection measurement is proposed:

The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the values indicated by parallelSMTC-r17, otherwise

-	For UE which support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE shall select the SMTC to be used of all the configured SMTCs up to the parallelSMTC-r17, the selection is up to UE implementation, and the requirements in this clause apply.

-	For UE which does not support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE may select one or subset of all the configured SMTCs sequentially until all of the SMTCs can be measured, the selection of SMTCs to be used is up to UE implementation, and longer measurement delay than the corresponding measurement period specified in table 4.2C.2.4-1 and table 4.2C.2.4-2 is expected.

Apple: Prop1. To double check, Apple might be fine with P2 or P3.

(R2 LS R2-2506298 referred to by Apple)

MTK: Prop2.

CATT: Use Prop3 as base wording to update the core req

IF UE supports 2 SMTC periodicities (discuss separately)

Proposal4: RAN4 to clarify which SMTC periodicity used during PSS/SSS detection for intra-frequency measurement and inter-frequency measurement requirements in RRC_IDLE/INACTIVE mode.

- Recommended WF for Proposal4: Address online (Thu PM)

Issue 2-1-3: STMC in FR2 (one company contributed to FR2 aspect)

Proposal 1: From RAN4 perspective, the SMTC enhancement capability can apply to FR2-NTN, based on the legacy assumption on the measurement:

- no inter-satellite measurement is configured;

- single SAN Tx beam per radio cell in DL;

- same UE Rx beam is used for both serving and neighbouring cells which belong to the same satellite.

- Recommended WF:

ZTE: Ka band is targeted here.

The wording 1,2,3 already implemented. This is only for the measurement part.

Xiaomi: not essential, but there were other’s supporting it last time.

Moderator: If agreed, then LS to R2 to inform them that capability can be applied to FR2 also.

Sub-topic 2-2: CRs - NWM

Sub-topic 2-3: LS – NWM

Proposal 1: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE (Apple)

Proposal 2: (in the LS R4-2521642, Huawei, HiSilicon), includes the topic raised in Proposal 1. (LS Flagged)

WF: Companies agreed to work together on unifying views in the (new, revised) LS out.

# Topic #3 AI 4.26.2 RRM Core Req  [IoT_NTN_Ph3-Core]

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520979 | CMCC | Proposal 1 – Do not introduce CQI reporting for CB-Msg3 EDT for eMTC UE, due to feasibility concerns (no preceding Msg1/Msg2 for measurement). Proposal 2 – Send a Reply LS to RAN2 to clarify and align on CQI reporting for eMTC UE with CB-Msg3-EDT. |
| R4-2521215 | Ericsson | (In order to align timing advance application between TS 36.213 (RAN1) and TS 36.133 (RAN4)): Proposal 1 – Send LS to RAN1 requesting update of TS 36.213 §16.1.2 to align with TS 36.133 §7.22A.2.1, ensuring consistent use of Koffset. |
| R4-2520036 (CR) | MediaTek | CQI for Anchor only. |
| R4-2521731 (CR) | Nokia | Reflects support for optional HARQ-ACK resource; UE ACK transmission depends on network configuration for CB-Msg3-EDT |
| R4-2520980 (LS out) | CMCC | Reply LS to RAN2 to clarify and align on CQI reporting for eMTC UE with CB-Msg3-EDT |

For ONline and OFFline sessions:

- Topic #3: [IoT_NTN_Ph3-Core] under AI 4.26.2

  - Sub-topic 3-1: CQI, CB‑Msg3‑EDT & eMTC

    - Issue 3-1-1: Applicability of channel quality reporting in CB‑Msg3‑EDT to eMTC

Proposal 1 – Do not introduce CQI reporting for CB-Msg3 EDT for eMTC UE (LS: R4-2520980)

If agreed, Send a Reply LS to RAN2 to clarify and align on CQI reporting for eMTC UE with CB-Msg3-EDT (LS: R4-2520980, not flagged)

- Recommended WF:

Proposal 1 (CQI reporting for CB-Msg3 EDT only applicable to NB IoT UE)

tentative, NOK to check.

- Issue 3-1-2: CQI for anchor vs non-anchor (NB IoT)

  - Proposal 1: CQI reporting is defined for the anchor carrier only; no non-anchor CQI scope addressed. (MTK, CMCC)
(Implemented in CR: R4-2520036)

  - Recommended WF: Proposal 1

  - Sub-topic 3-2: Align specs to ensure consistent use of Koffset

    - Proposal 1: Support aligning TA timing units by updating RAN1(TS 36.213 §16.1.2) on Koffset, aligning with RAN4 and other NTN RATs (1 company)

    - Recommended WF: Proposal 1 LS to R1 during this meeting:
Proposal 1: Send an LS to RAN1 to request RAN1 to update Koffset in TS 36.213 clause 16.1.2 to align with that in TS 36.133 clause 7.22A.2.1 of the RAN4 spec.
(Moderator: An offline held with R1 & R4 on this topic on Wed.

    - Moderator: LS drafted, shared via Drafts, need TDoc number.

  - Sub-topic 3-3: HARQ-ACK for CB-Msg3-EDT

    - Proposal 1: Support optional HARQ-ACK resource; UE ACK transmission depends on network configuration for CB-Msg3-EDT. (CR R4-2521731) (1 company)

NOK: re-submission of endorsed CR (last time.)

Recommended WF: Not flagged Agreed.

- Sub-topic 3-4: CRs for approval - two submitted for 36.133 (NWM, based on decisions made here)

# Topic #4 (AI 4.27.4) RRM Core Req [IoT_NTN_TDD-Core]

| TDoc # | Company | Proposals/Observations |
| --- | --- | --- |
| R4-2520890 (CR) | LG Electronics Inc. | CR on pre-compensation for timing error requirement for IoT NTN TDD (36.133 Clause 7.20B.2) 7.20B.2 Requirements The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.20B.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The UE may adjust its time at the beginning of the uplink burst of 8 consecutive transmitted subframes, and no pre-compensation gap is allowed before the beginning of each uplink burst of 8 consecutive transmitted subframes. When segmentation is applied, UE pre-compensation gap may or may not be used for timing adjustment between two consecutive segments. Pre-compensation gap is not allowed for the first segment. |
| R4-2521651 Discussion | Huawei, HiSilicon | Proposal 1: RAN4 to confirm that there is no concern on the updated RAN1 agreement from RRM perspective, and RAN4 to make the following update to clause 7.20B. 7.20B.2 Requirements … when a repetition is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary, as defined above. |
| R4-2521652 (CR) | Huawei, HiSilicon | A re-submission of 36.133 draftCR R4-2515128 endorsed in RAN4#116-bis. - The delay of 1400ms is still in RRC re-estalishement requirements. Also, the definition of Ksatellite needs to be updated based on agreement. - The RRC re-drection requirements need to be updated for the TDD pattern. (Moderator : in NWM, propose to Agree to this CR) |
| R4-2521733 (CR) | Nokia | 36.133 CR with editorial correction for IoT NTN TDD requirements by removing the duplicate sentence in clause 8.14B.6.3 (Moderator : in NWM, propose to Agree to this CR) |
| R4-2521734 Discussion | Nokia | Observation 1: From RAN1 LS it is possible to confirm there is no need for a pre-compensation gap in IoT NTN TDD, but that UEs may change the pre-compensation prior to the set of 8 consecutive uplink subframes. - Proposal 1: Confirm to RAN1 that there is no issues with their agreements. Proposal 2: RAN4 to update the specification and send the agreed text to RAN1 as information. |
| R4-2521735 (CR) | Nokia | CR on removing support for segmented pre-compensation (36.133 clause 7.20B.2) 7.20B.2 Requirements The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.20B.2-1. This requirement applies when it is the first transmission in each set of consecutive uplink subframes; or the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, , or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus |

For ONline and OFFline sessions:

## Sub-topic 4-1. Segmentation and pre-compensation for IoT NTN TDD?

Issue 4-1-1: Is Segmented pre-compensation supported for IoT NTN TDD?

  - Proposal 1: Segmented pre-compensation is not supported for IoT NTN TDD (Nok, Huawei)

    - Remove support for it for IOT NTN TDD case from the 36.133 (CR R4-2521735) – Flagged.

  - Proposal 2: Segmented pre-compensation is/can be supported for IoT NTN TDD (LG)

- Recommended WF: Proposal 1:

  - It is aligned w/ RAN1 understanding

  - Due to built-in gaps (DL to UL; UL to DL), no need to include extra segmentation/gaps.

Huawei: NOK CR is sufficient. R4 does not need to say “not needed” or “not supported”. 
RF thread not affected.

LG: We need to explicitly state (in the CR) that the UE performs pre-compensation without any pre-compensation gap.

QC: wants to follow “ue shall do the precomp at the beginning….”

Thales: Network will not configure; UE is allowed to….

LS to R1: the feature not supported for IOT NTN TDD.

The next two issues could go away, IF 4-1-1 decides for Proposal 1.

Issue 4-1-3: Timing and frequency pre-compensation within the transmission of U=8 subframes

  - Proposal 1: No timing and frequency pre-compensation is allowed within the transmission of U=8 subframes (in line with RAN1 agreements)
(Nok, Huawei)

  - Proposal 2: The UE may adjust its time within uplink burst of 8 consecutive transmitted subframes 
(LG)

- Recommended WF: Proposal 1

Issue 4-1-4: Segmentation gap & uplink transmission burst

- Proposals:

- Proposal 1: No segmentation gap is needed at the beginning of U=8 uplink transmission burst

- Recommended WF: Proposal 1 (everyone agrees) implement it in CR 36.133

## Sub-topic 4-2 (to be handled in NWM)

- CRs for agreement – Two for 36.133: editorial or already endorsed:

Issue 4-2-1: This CR (R4-2521652) is re-submission of 36.133 draftCR (was R4-2515128) endorsed in RAN4#116-bis

Recommended WF: Not flaggedAgree to this CR.

Issue 4-2-2: 36.133 CR (R4-2521733) with editorial correction for IoT NTN TDD requirements by removing the duplicate sentence in clause 8.14B.6.3

Recommended WF: Not flaggedAgree to this CR.