3GPP TSG-RAN4 Meeting #117	R4-2522983

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on introduction of BFD and LR test for RedCap UE with Satellite Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In RAN4#116bis meeting, RAN4 agreed to introduce the BFD and LR test for RedCap UE with Satellite Access including: - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode for 1Rx/2Rx RedCap UE with NTN - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS based BFD and LR in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS based BFD and LR in DRX mode for 1Rx/2Rx RedCap UE with NTN |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the BFD and LR test for RedCap UE with Satellite Access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The R19 NTN spec will not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.X.4.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2520994 |  |  |  |  |  |  |  |  |

# ==============First change==============

A.X.4.2	Beam Failure Detection and Link recovery procedures for RedCap UE with satellite access

A.X.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode for 1Rx RedCap UE

A.X.4.2.1.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.2.1.1 shall apply for 1RX RedCap UE except that:

clause 8.5 is replaced with clause 8.5E

Table A.14.4.2.1.1-1 is replaced with A.X.4.2.1.1-1

Table A.14.4.2.1.1-2, Table A.14.4.2.1.1-3 shall apply to configuration 1,2,3,4, except those described in the tables A.X.4.2.1.1-2.

Table A.X.4.2.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.X.4.2.1.1-2: General test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| NTN reference Serving satellite configuration | Config 1, 3 |  | SSC.1 |  |
|  | Config 2, 4 |  | SSC.2 |  |
| Duplex mode | Config 1, 2 |  | FDD |  |
|  | Config 3, 4 |  | HD-FDD |  |
| Dedicated CORESET Reference Channel | Config 1, 2, 3, 4 |  | CCR.3.2 FDD |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x1 Low |  |
| Beam failure detection transmission parameters | Aggregation level | CCE | 16 |  |
| T3 |  | s | 0.44 |  |

A.X.4.2.1.2	Test Requirements

The test requirements defined in A.14.4.2.1.2 are reused.

A.X.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode for 2Rx RedCap UE

A.X.4.2.2.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.2.1.1 shall apply for 2RX RedCap UE except that:

clause 8.5 is replaced with clause 8.5E

Table A.14.4.2.1.1-1 is replaced with A.X.4.2.1.1-1

Table A.14.4.2.1.1-2, Table A.14.4.2.1.1-3 shall apply to configuration 1,2,3,4, except those described in the tables A.X.4.2.2.1-1.

Table A.X.4.2.2.1-1: General test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| NTN reference Serving satellite configuration | Config 1, 3 |  | SSC.1 |  |
|  | Config 2, 4 |  | SSC.2 |  |
| Duplex mode | Config 1, 2 |  | FDD |  |
|  | Config 3, 4 |  | HD-FDD |  |
| Dedicated CORESET Reference Channel | Config 1, 2, 3, 4 |  | CCR.1.3 FDD |  |

A.X.4.2.2.2	Test Requirements

The test requirements defined in A.14.4.2.1.2 are reused.

A.X.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode for 1Rx RedCap UE

A.X.4.2.3.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.2.2.1 shall apply for 1RX RedCap UE except that:

clause 8.5 is replaced with clause 8.5E

Table A.14.4.2.2.1-1 is replaced with A.X.4.2.1.1-1

Table A.14.4.2.2.1-2, Table A.14.4.2.2.1-3 shall apply to configuration 1,2,3,4, except those described in the tables A.X.4.2.3.1-1.

Table A.X.4.2.3.1-1: General test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| NTN reference Serving satellite configuration | Config 1, 3 |  | SSC.1 |  |
|  | Config 2, 4 |  | SSC.2 |  |
| Duplex mode | Config 1, 2 |  | FDD |  |
|  | Config 3, 4 |  | HD-FDD |  |
| Dedicated CORESET Reference Channel | Config 1, 2, 3, 4 |  | CCR.3.2 FDD |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x1 Low |  |
| Beam failure detection transmission parameters | Aggregation level | CCE | 16 |  |
| T3 |  | s | 6.44 |  |

A.X.4.2.3.2	Test Requirements

The test requirements defined in A.14.4.2.2.2 are reused

A.X.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode for 2Rx RedCap UE

A.X.4.2.4.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.2.2.1 shall apply for 2RX RedCap UE except that:

clause 8.5 is replaced with clause 8.5E

Table A.14.4.2.2.1-1 is replaced with A.X.4.2.1.1-1

Table A.14.4.2.2.1-2, Table A.14.4.2.2.1-3 shall apply to configuration 1,2,3,4, except those described in the tables A.X.4.2.4.1-1.

Table A.X.4.2.4.1-1: General test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| NTN reference Serving satellite configuration | Config 1, 3 |  | SSC.1 |  |
|  | Config 2, 4 |  | SSC.2 |  |
| Duplex mode | Config 1, 2 |  | FDD |  |
|  | Config 3, 4 |  | HD-FDD |  |
| Dedicated CORESET Reference Channel | Config 1, 2, 3, 4 |  | CCR.1.3 FDD |  |

A.X.4.2.4.2	Test Requirements

The test requirements defined in A.14.4.2.2.2 are reused.

A.X.4.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode for 1Rx RedCap UE

A.X.4.2.5.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.2.3.1 shall apply for 1RX RedCap UE except that:

clause 8.5 is replaced with clause 8.5E

Table A.14.4.2.3.1-1 is replaced with A.X.4.2.1.1-1

Table A.14.4.2.3.1-2, Table A.14.4.2.3.1-3 shall apply to configuration 1,2,3,4, except those described in the tables A.X.4.2.5.1-1.

Table A.X.4.2.5.1-1: General test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| NTN reference Serving satellite configuration | Config 1, 3 |  | SSC.1 |  |
|  | Config 2, 4 |  | SSC.2 |  |
| Duplex mode | Config 1, 2 |  | FDD |  |
|  | Config 3, 4 |  | HD-FDD |  |
| Dedicated CORESET Reference Channel | Config 1, 2, 3, 4 |  | CCR.3.2 FDD |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x1 Low |  |
| Beam failure detection transmission parameters | Aggregation level | CCE | 16 |  |
| T3 |  | s | 0.27 |  |

A.X.4.2.5.2	Test Requirements

The test requirements defined in A.14.4.2.3.2 are reused.

A.X.4.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode for 2Rx RedCap UE

A.X.4.2.6.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.2.3.1 shall apply for 2RX RedCap UE except that:

clause 8.5 is replaced with clause 8.5E

Table A.14.4.2.3.1-1 is replaced with A.X.4.2.1.1-1

Table A.14.4.2.3.1-2, Table A.14.4.2.3.1-3 shall apply to configuration 1,2,3,4, except those described in the tables A.X.4.2.6.1-1.

Table A.X.4.2.6.1-1: General test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| NTN reference Serving satellite configuration | Config 1, 3 |  | SSC.1 |  |
|  | Config 2, 4 |  | SSC.2 |  |
| Duplex mode | Config 1, 2 |  | FDD |  |
|  | Config 3, 4 |  | HD-FDD |  |
| Dedicated CORESET Reference Channel | Config 1, 2, 3, 4 |  | CCR.1.3 FDD |  |

A.X.4.2.6.2	Test Requirements

The test requirements defined in A.14.4.2.3.2 are reused.

A.X.4.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode for 1Rx RedCap UE

A.X.4.2.7.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.2.4.1 shall apply for 1RX RedCap UE except that:

clause 8.5 is replaced with clause 8.5E

Table A.14.4.2.4.1-1 is replaced with A.X.4.2.1.1-1

Table A.14.4.2.4.1-2, Table A.14.4.2.4.1-3 shall apply to configuration 1,2,3,4, except those described in the tables A.X.4.2.7.1-1.

Table A.X.4.2.7.1-1: General test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| NTN reference Serving satellite configuration | Config 1, 3 |  | SSC.1 |  |
|  | Config 2, 4 |  | SSC.2 |  |
| Duplex mode | Config 1, 2 |  | FDD |  |
|  | Config 3, 4 |  | HD-FDD |  |
| Dedicated CORESET Reference Channel | Config 1, 2, 3, 4 |  | CCR.3.2 FDD |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x1 Low |  |
| Beam failure detection transmission parameters | Aggregation level | CCE | 16 |  |
| T3 |  | s | 12.24 |  |

A.X.4.2.7.2	Test Requirements

The test requirements defined in A.14.4.2.4.2 are reused.

A.X.4.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode for 2Rx RedCap UE

A.X.4.2.8.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.2.4.1 shall apply for 2RX RedCap UE except that:

clause 8.5 is replaced with clause 8.5E

Table A.14.4.2.4.1-1 is replaced with A.X.4.2.1.1-1

Table A.14.4.2.4.1-2, Table A.14.4.2.4.1-3 shall apply to configuration 1,2,3,4, except those described in the tables A.X.4.2.8.1-1.

Table A.X.4.2.8.1-1: General test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| NTN reference Serving satellite configuration | Config 1, 3 |  | SSC.1 |  |
|  | Config 2, 4 |  | SSC.2 |  |
| Duplex mode | Config 1, 2 |  | FDD |  |
|  | Config 3, 4 |  | HD-FDD |  |
| Dedicated CORESET Reference Channel | Config 1, 2, 3, 4 |  | CCR.1.3 FDD |  |

A.X.4.2.8.2	Test Requirements

The test requirements defined in A.14.4.2.4.2 are reused.

# ==============End of change==============