# 3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514807

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:			6.15.1

Source:	Moderator (Ericsson)

Title:	Ad-hoc minutes for Netw_Energy_NR_enh_RRM

Document for:	Approval

# Introduction

This document is the Ad-hoc minutes for Netw_Energy_NR_enh with the following topics covered.

Note 1: The following terminologies are used.

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

  - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification) when the center frequency locations of always-on SSB and on-demand SSB are same.

  - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported when the center frequency locations of always-on SSB and on-demand SSB are same.

  - Alt Scenario 3: When the center frequency locations of always-on SSB and on-demand SSB are different and AO-SSB and OD-SSB are located in the same BWP.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #2A refers to “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

- Scenario #3A refers to “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

- Scenario #3B refers to “When SCell activation is completed and SCell is activated” or “After SCell activation is completed and SCell is activated”

Note 2: The following abbreviations are used.

- AO-SSB: Always-on SSB

- OD-SSB: On-demand SSB

- FMW: Fast Measurement Window

- EMP: Effective Measurement Periodicity

# Topic #1: On-demand SSB(OD-SSB) performance part (AI 6.15.2.1)

Issue 1-1: Test case list

RAN4 to discuss the following OD-SSB test cases.

Background

Issue 1-1-1: General – Case 1 and Case 2 Agreement - The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell. - Case #1: scenario #2, scenario #2A. - Case #2: scenario #2, scenario #2A and scenario #3B. Issue 1-1-3: General – Different OD-SSB and AO-SSB relation in Case 2 Agreement: RAN4 to define TCs for Alt Time-C1. Meanwhile, - Option 1: Not define TC for Alt Time-C2 - Option 2: Select one test for Alt Time-C2. For UE supporting Alt Time-C2, the corresponding test case for Alt Time-C1 can be skipped. - Further discuss in one test case or separate test cases. Issue 1-1-4: Scenario – Scell activation scope Agreement: RAN4 to only define the test cases for single Scell activation, Direct SCell activation. Issue 1-1-6: T2 for deactivated SCell measurement Agreement: - Define one test case in case 1 to verify the UE measurement behaviour when the OD-SSB is reactivated relevant to T2. Issue 1-3-1: General principle Agreement: RAN4 to apply the following principles for OD-SSB test - Both FR1 and FR2 - Test non-DRX mode, and select one test case with DRX configuration for deactivated SCell measurement. - The test without SSB time index detection - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting - Only NCD-SSB for OD-SSB test case

Test Case List

Agreement

- SCell activation

- TC1: SCell activation in Case 1

| No. | Test category | Test scenario | Test configuration | Test purpose | Company |
| --- | --- | --- | --- | --- | --- |
| TC1-a FFS | SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation FR1 | OPPO, vivo: Not needed. Scenario 2 is verified from TC5. CMCC, Nokia, ZTE: Keep both of them. Nokia: test one DRX Vivo: whether we can remove TC1-a? CMCC, Nokia: The difference is OD-SSB is NCD-SSB. |
| TC1-b (Nokia) | SCell Activation and deactivation of unknown SCell in DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a Unknown SCell activation FR1 and FR2 |  |

- TC2: SCell activation in Case 2 Alt Time C-1

| No. | Test category | Test scenario | Test configuration | Test purpose | Company |
| --- | --- | --- | --- | --- | --- |
| TC2-a | SCell Activation and deactivation of unknown SCell in DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - OD-SSB(5ms) is overlapping with AO-SSB(20ms) OD-SSB scenario 2a Unknown SCell activation FR1 | OPPO: Not needed Nokia, CMCC, ZTE: We want to keep both. OPPO: We need to consider EMP and it already have AO-SSB. CMCC: In legacy, we also have known and unknown case. HW: Do we need to consider to skip the cell identification? Vivo: Can we pick up three of the group 1 and 2? |
| TC2-b | SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - The union of OD-SSB and AO-SSB meets the legacy SSB periodicity OD-SSB scenario 2a known SCell activation FR1 and FR2 |  |

- TC3: OD-SSB based direct SCell Activation at SCell addition in Case 1

- TC4: OD-SSB based direct SCell Activation at SCell addition in Case 2 Alt Time C-1

| No. | Test category | Test scenario | Test configuration | Test purpose | Company |
| --- | --- | --- | --- | --- | --- |
| TC3 FFS | Direct SCell activation at SCell addition based on OD-SSB | Direct SCell activation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a FR1 |  |
| TC4 | Direct SCell activation at SCell addition based on OD-SSB | Direct SCell activation | OD-SSB periodicity 20ms AO-SSB periodicity 80ms | OD-SSB Case 2 Alt Time-C1 OD-SSB scenario 2a FR1 | OPPO: Not needed |

- Deactivated SCell L3 measurement

- TC5: Intra-frequency measurement(deactivated SCell) in Case 1

- TC6: Intra-frequency measurement(deactivated SCell) in Case 2

| No. | Test category | Test scenario | Test configuration | Test purpose | Company |
| --- | --- | --- | --- | --- | --- |
| TC5 | Intra-frequency measurement(deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 10ms | OD-SSB Case 1 FMW; UE behaviour relevant to T2 FR1 |  |
| TC6 | Intra-frequency measurement(deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW FR2 FFS FR1 | Moderator: What about FR2 |

- Active SCell L3 measurement

- TC7: Intra-frequency measurement in Case 1

- TC8: Intra-frequency measurement in Case 2

- TC9: L1-RSRP measurement with OD-SSB parameter update(Scenario 3B) in Case 2

| No. | Test category | Test scenario | Test configuration | Test purpose | Company |
| --- | --- | --- | --- | --- | --- |
| TC7 | Intra-frequency measurement- SA event triggered reporting tests without gap for SCell under non DRX based on OD-SSB | L3-RSRP measurement | OD-SSB periodicity 10ms | OD-SSB scenario 3B; OD-SMTC and SMTC FR1 |  |
| TC8 | Intra-frequency measurement- SA event triggered reporting tests without gap under non DRX based on OD-SSB | L3-RSRP measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB scenario 3B; OD-SMTC and SMTC FR2 FFS FR1 | OPPO: Not needed |
| TC9 | SSB based L1-RSRP measurement when DRX is not used based on OD-SSB | L1-RSRP measurement | OD-SSB periodicity 20ms -> 10ms | OD-SSB scenario 3B; Scheduling restriction to AO-SSB occasions; L1-RSRP based on EMP FR1 |  |

- Interruption

The interruption test for OD-SSB activation/deactivation and during measurement on deactivated NR SCC are merged to SCell activation and deactivated SCell measurement test cases.

# Topic #2: Adaptation of common signal/channel transmission (AI 6.15.2.2)

Issue 2-1: Test case list

Background

Issue 1-1: TC(s) to be specified besides L3 intra-frequency measurement Agreement RAN4 to at least define the following SSB adaptation test case. - L3 intra-frequency measurement Agreement: - Besides L3 intra-frequency measurement, specify the following test case for SSB adaptation - L1-RSRP measurement - L1-SINR measurement Issue 1-2: General principle Agreement: - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only - Both FR1 and FR2 - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement - Only test non-DRX mode - without SSB time index detection - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. Issue 1-3: SMTC and SSB configuration Tentative Agreement: - For L1-RSRP measurement test case: - SMTC periodicity from 80ms to 20ms, and vice versa. - SSB periodicity from 80ms to 20ms, and vice versa. - For other test cases: - SMTC periodicity from 80ms to 20ms. - SSB periodicity from 80ms to 20ms.

Tentative Agreement

  - TC1: L3 intra-frequency measurement (Company: )

    - FR1

    - SMTC periodicity from 80ms to 20ms.

    - SSB periodicity from 80ms to 20ms.

  - TC2-1: L1-RSRP measurement (Company: )

    - FR2

    - SMTC periodicity from 20ms to 80ms, and from 80ms to 20ms.

    - SSB periodicity from 20ms to 80ms, and from 80ms to 20ms.

  - TC2-2: L1-RSRP measurement (Company: )

    - FR1

    - SMTC periodicity from 20ms to 80ms.

    - SSB periodicity from 20ms to 80ms.

  - TC3: L1-SINR measurement (Company: )

    - FR1

    - SMTC periodicity from 80ms to 20ms.

    - SSB periodicity from 80ms to 20ms.

Agreement

  - TC1: L3 intra-frequency measurement

    - FR1

  - TC2: L1-RSRP measurement

    - PCell FR1 and SCell FR2

  - TC3: L1-SINR measurement

    - FR1

Issue 1-3: SMTC and SSB configuration

Tentative Agreement:

- For L1-RSRP measurement test case:

  - SMTC periodicity from 20ms to 80ms, and from 80ms to 40ms.

  - SSB periodicity from 20ms to 80ms, and from 80ms to 40ms.

- For other test cases:

  - SMTC periodicity from 80ms to 20ms.

  - SSB periodicity from 80ms to 20ms.

Samsung: We need to consider 3 values.

# Topic #3: OD-SIB1 (AI 6.15.2.3)

Issue 3-1: Test case list

Background

Issue 2-1: Whether to define TC for OD-SIB1 Agreement: - RAN4 to define cell reselection TC with longer TSI-NR for OD-SIB1. Issue 2-3: TC scope Agreement: - Cell reselection FR1 to FR1 target Cell - Cell reselection FR2 to FR2 target Cell

Tentative Agreement

- TC1: OD-SIB1 based Cell reselection FR1 to FR1 target Cell (Company: )

- TC2: OD-SIB1 based Cell reselection FR2 to FR2 target Cell (Company: )

# Topic #4: Maintenance (AI 4.21.1)

Issue 4-1: OD-SSB interruption

- Background

##### 8.2.3.2.x Interruptions at OD-SSB activation/deactivation The requirements in this clause shall apply for the UE configured with E-UTRA PSCell and one SCell. When OD-SSB on one deactivated SCell in MCG is activated or deactivated the UE is allowed an interruption on any serving cell in MCG, provided that the OD-SSB activation/deactivation command and SCell activation/deactivation command are not received at the same time: - of up to X2 slots, if the active serving cell is not in the same band as any of the SCells on which OD-SSB is being activated or deactivated, or - of up to Y2 slots + TSMTC_duration if the active serving cells are in the same band as any of the SCells on which OD-SSB is being activated or deactivated, provided the cell specific reference signals from the active serving cells and the SCells on which OD-SSB is being activated or deactivated are available in the same slot. 8.3.x1 OD-SSB based SCell Activation Delay Requirement for Deactivated SCell .. The starting point of an interruption window on SpCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is: - 0, if Tactivation_time is 3 ms; - TFirst, OD-SSB, for any scenario where Tactivation_time includes TFirst, OD-SSB; - Tuncertainty_MAC + TFineTiming, OD-SSB, for any scenario where Tactivation_time includes only TFineTiming, OD-SSB and no TFirst, OD-SSB.

- Proposals

  - Option 1: Ericsson

    - RAN4 to align the interruption length of OD-SSB based deactivated SCell measurement with SCell activation as the RF retuning time without the SMTC duration.

- Recommended WF

  - Further discussion.

Issue 4-2: The number of od-ssb-nrofBurst

- Background

  - In RAN1 #122 meeting, RAN1 made the following agreement.

Agreement - Confirm the value range of od-ssb-nrofBurst for both FR1 and FR2 - For FR1, the value range of od-ssb-nrofBurst is {5, 10, 15, 20, 25, 30, 40, 50}. - For FR2, the value range of od-ssb-nrofBurst is {25, 30, 40, 50, 75, 100, 150, 200}. - Note: It is up to RAN4 whether to include smaller value than 25 for FR2.

- Proposals

  - Option 1: vivo

    - RAN4 confirm that there is no need to include smaller value than 25 for FR2 for od-ssb-nrofBurst and provide the feedback to RAN1/2.

- Recommended WF

  - Moderator suggests the group to check whether option 1 can be agreed.

- Discussion

Vivo: It’s better to confirm in the meeting to avoid RAN2 impact.

Nokia: We don’t need to have any action for this RAN1 agreement.

OPPO:  We don’t need any agreement.

Agreement

There is no action from RAN4 for this issue.

Issue 4-3: Terminology

- Proposals

  - Option 1: Nokia

    - RAN4 to use “OD-SSB”/”AO-SSB” instead of “first/second SSB”, and provide clear definitions for the OD-SSB/AO-SSB referring to the first/second SSB specified in RAN1.

- Recommended WF

  - Further discussion.

Agreement

RAN4 to clarify the definition of first SSB and OD-SSB in TS38.133 Terminology based on RAN2 signalling.