3GPP TSG-RAN WG4 Meeting #116bis                                                                                                      R4-2513156

Prague, CZ, 13rd –17th October, 2025

Source: 	CATT

Title: 	Discussion on angle and coordinate system for Expected EIRP mask for upper 6GHz

Agenda item:	6.8.4.1

Document for:	Approval

- Introduction

In this contribution, we provide our views on angle and coordinate system for Expected EIRP mask for upper 6GHz.

- Discussion

The  Θmin, Θmax, Φmax, Φmin for test beam directions set

There are two types of coordinate systems in TS 38.141-2. The first coordinate system is defined in sub-clause 4.14 of TS 38.141-2 for OTA peak directions set maximum steering direction(s) (D.10).

Observation 1: The reference coordinate system defined in sub-clause 4.14 of TS 38.141-2 is for OTA peak directions set maximum steering direction(s) (D.10).

Figure 4.14-1 in sub-clause 4.14 for OTA peak directions set maximum steering direction(s) (D.10) is copied as below:

Figure 4.14-1: Reference coordinate system

The text in sub-clause 4.14 “The reference coordinate system is should be associated to an identifiable physical feature on the base station enclosure.” indicates that reference coordinate system is local coordinate system associated to an identifiable physical feature on the base station enclosure.

Observation 2: The reference coordinate system in sub-clause 4.14 of TS 38.141-2 is local coordinate system associated to an identifiable physical feature on the base station enclosure.

As example (EXAMPLE: AAS BS with variable downtilt) in TR 37.842 copied as below, the down tilt (only electrical down tilt) is included in OTA peak directions set. Early EIRP accuracy directions set is currently OTA peak directions set.

EXAMPLE: AAS BS with variable downtilt: In this example, the AAS BS is a single column that provides variable downtilt from 0 to 10 degrees. The BS operates in two bands, band X and band Y with two different beam characteristics. In each band, 2 identical beams (one beam per polarization) are declared. The reference direction is taken to be (0,0) and there is only one possible beamwidth in the reference direction. The declarations for this BS could look as follows: Table F-1 Declaration identifier; Declaration; Used in requirements; Notes. D9.10; EIRP accuracy directions set; 9; Provided on directions diagram below. Continuous range along the Φ axis from 0 to 10 degrees. Same for all beams. (Noted by CATT, the Φ should be Θ).

As above, the Θmin, Θmax, Φmax, Φmin in OTA peak directions set maximum steering direction(s) include effect of electrical down tilt. This is reasonable because electrical tilts are related to an identifiable physical feature on the base station enclosure(or antenna).

Observation 3: The Θmin, Θmax, Φmax, Φmin in OTA peak directions set maximum steering direction(s) include effect of electrical down tilt.

But the mechanical tilt is related to antenna installation, the antenna installation is related to global coordinate system. So mechanical tilt is related to global coordinate system instead of local coordinate system associated to an identifiable physical feature on the base station enclosure.

Observation 4: Mechanical tilt is related to global coordinate system instead of local coordinate system associated to an identifiable physical feature on the base station enclosure.

Since the Θmin, Θmax, Φmax, Φmin in OTA peak directions set maximum steering direction(s) (D.10) is declared based on local coordinate system associated to an identifiable physical feature on the base station enclosure and is dependent on BS enclosure, so the OTA peak directions set maximum steering direction(s) (D.10) is not varied with any mechanical tilt. As following figure, the Θmin, Θmax  is defined based on BS enclosure, which is not varied with mechanical down tilt.

Observation 5: The Θmin, Θmax, Φmax, Φmin in OTA peak directions set maximum steering direction(s) (D.10) is not varied with any mechanical tilt, since they are declared based on local coordinate system associated to an identifiable physical feature on the base station enclosure.

The second coordinate system is defined in sub-clause 6.9.5.1 of TS 38.141-2 for Expected EIRP (EEIRP) requirement. The Figure 6.9.5.1-1 in sub-clause 6.9.5.1 for Expected EIRP (EEIRP) requirement is copied as below:

Figure 6.9.5.1-1: Definitions of θHLi and θHHi angles.

From our understanding, the above coordinate system in sub-clause 6.9.5.1 of TS 38.141-2 is global coordinate system, since it is defined based on horizon. So Expected EIRP (EEIRP) requirement is defined based on global coordinate system, although, coordinate system in sub-clause 6.9.5.1 of TS 38.141-2 is not aligned with general spherical coordinate system.

Observation 6: Expected EIRP (EEIRP) requirement is defined based on global coordinate system, which is coordinate system in sub-clause 6.9.5.1 of TS 38.141-2.

As above observation 4, mechanical tilt is related to global coordinate system. The coordinate system for mechanical tilts declaration (D.64) for EEIRP is unclear.

Observation 7: The coordinate system for mechanical tilts declaration (D.64) for EEIRP is unclear.

Based on our common understanding, mechanical tilt is defined with respect to the horizon with elevation angles above the horizon negative and below the horizon positive. But above global coordinate system in sub-clause 6.9.5.1 of TS 38.141-2 is defined with respect to the horizon with elevation angles above the horizon positive and below the horizon negative. So, if using above global coordinate system in sub-clause 6.9.5.1 of TS 38.141-2 for mechanical tilt, common 10 degree mechanical tilt is inverted and corresponding -10 degree. So the global coordinate system in sub-clause 6.9.5.1 of TS 38.141-2 is not applicable for mechanical tilts declaration (D.64). It is reasonable to define that Mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive, and mechanical down tilt is defined between -90° and +90° (positive towards the ground). Add it to Table 6.9.5.1-1 for EEIRP limits.

Table 6.9.5.1-1: EEIRP limits as function of elevation above horizon

| Bin number i | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<<30 | 16 |
| 6 | 30≤<<60 | 15 |
| 7 | 60≤<<90 | 15 |
| Note 1: The requirement shall apply to all supported mechanical tilts. Mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive. |  |  |

Proposal 1: Add the coordinate system for mechanical tilts to Table 6.9.5.1-1 for EEIRP limits as below:

Table 6.9.5.1-1: EEIRP limits as function of elevation above horizon

| Bin number i | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<<30 | 16 |
| 6 | 30≤<<60 | 15 |
| 7 | 60≤<<90 | 15 |
| Note 1: The requirement shall apply to all supported mechanical tilts. Mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive. |  |  |

The Table 4.9.3.2-1 and Figure 4.9.3.2-1 of TS 48.141-2 for test beam directions set for EEIRP are copied as below:

Table 4.9.3.2-1: Test beam direction set for protection of FSS UL Beam index; Elevation angle; Azimuth angle; Weighting factor. 1; Θmin+2.5(Θmax-Θmin)/25; Φmin+7.5(Φmax-Φmin)/25; 1/21. 2; Θmin+2.5(Θmax-Θmin)/25; Φmin+12.5(Φmax-Φmin)/25; 1/21. 3; Θmin+2.5(Θmax-Θmin)/25; Φmin+17.5(Φmax-Φmin)/25; 1/21. 4; Θmin+7.5(Θmax-Θmin)/25; Φmin+2.5(Φmax-Φmin)/25; 1/21. 5; Θmin+7.5(Θmax-Θmin)/25; Φmin+7.5(Φmax-Φmin)/25; 1/21. 6; Θmin+7.5(Θmax-Θmin)/25; Φmin+12.5(Φmax-Φmin)/25; 1/21. 7; Θmin+7.5(Θmax-Θmin)/25; Φmin+17.5(Φmax-Φmin)/25; 1/21. 8; Θmin+7.5(Θmax-Θmin)/25; Φmin+22.5(Φmax-Φmin)/25; 1/21. 9; Θmin+12.5(Θmax-Θmin)/25; Φmin+2.5(Φmax-Φmin)/25; 1/21. 10; Θmin+12.5(Θmax-Θmin)/25; Φmin+7.5(Φmax-Φmin)/25; 1/21. 11; Θmin+12.5(Θmax-Θmin)/25; Φmin+12.5(Φmax-Φmin)/25; 1/21. 12; Θmin+12.5(Θmax-Θmin)/25; Φmin+17.5(Φmax-Φmin)/25; 1/21. 13; Θmin+12.5(Θmax-Θmin)/25; Φmin+22.5(Φmax-Φmin)/25; 1/21. 14; Θmin+17.5(Θmax-Θmin)/25; Φmin+2.5(Φmax-Φmin)/25; 1/21. 15; Θmin+17.5(Θmax-Θmin)/25; Φmin+7.5(Φmax-Φmin)/25; 1/21. 16; Θmin+17.5(Θmax-Θmin)/25; Φmin+12.5(Φmax-Φmin)/25; 1/21. 17; Θmin+17.5(Θmax-Θmin)/25; Φmin+17.5(Φmax-Φmin)/25; 1/21. 18; Θmin+17.5(Θmax-Θmin)/25; Φmin+22.5(Φmax-Φmin)/25; 1/21. 19; Θmin+22.5(Θmax-Θmin)/25; Φmin+7.5(Φmax-Φmin)/25; 1/21. 20; Θmin+22.5(Θmax-Θmin)/25; Φmin+12.5(Φmax-Φmin)/25; 1/21. 21; Θmin+22.5(Θmax-Θmin)/25; Φmin+17.5(Φmax-Φmin)/25; 1/21. NOTE 1: Θmin, Θmax, Φmax, Φmin is defined in the declaration D.10 with the associated declaration D.64.. Figure 4.9.3.2-1 Test beam set for conformance testing

The declaration identifier related to OTA peak directions set (D.9), OTA peak directions set maximum steering direction(s) (D.10), The mechanical down-tilt (D.64), and NOTE 21 for The mechanical down-tilt (D.64) in Table 4.6-1 of TS 38.141-2 are copied as below:

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.9 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3). | x | x | x |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) Φmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) θmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) θmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). For the EEIRP requirement in subclause 6.9, the OTA peak direction set maximum steering direction(s) describes the BS steering range(s) for a declared mechanical tilt range (D.64). | x | x | x |
| D.64 | The mechanical down-tilt | For EEIRP the declaration of mechanical tilt together with D.10.(NOTE 21) | x | x | n/a |
| NOTE 21: The minimum mechanical down tilt and the maximum electrical steering range shall be declared. If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances: One instance is minimum mechanical down tilt and the reduced electrical steering range, and the other instance is maximum electrical steering range and increased mechanical down tilt. |  |  |  |  |  |

As above NOTE 21, since the coordinate system for mechanical down tilt is not defined, so the minimum mechanical down tilt is unclear. And the maximum electrical steering range is Θmin, Θmax, Φmax, Φmin, but the “reduced electrical steering range” in NOTE 21 is unclear.

At the same time, “the OTA peak direction set maximum steering direction(s) describes the BS steering range(s) for a declared mechanical tilt range” indicates Θmin, Θmax, Φmax, Φmin is for per mechanical tilt. But NOTE 21 indicates both “maximum electrical steering range” and “reduced electrical steering range” can be for minimum mechanical down tilt. They are confusion.

Observation 8: The declarations for Θmin, Θmax, Φmax, Φmin for test beam directions are unclear.

The mechanical down-tilt is related to antenna installation and network application, and the EEIRP is also defined for per mechanical down-tilt. EEIRP is dependent on network coverage which decided by vertical coverage and horizontal coverage in global coordinate system for EEIRP. The vertical coverage is dependent on mechanical down-tilt and electrical down-tilt. So the Θmin, Θmax, Φmax, Φmin in OTA peak direction set is only related to electrical down-tilt. And combination of mechanical down-tilt and electrical down-tilt determine network coverage.

So, based on above discussion, the following declaration for EEIRP is more reasonable.

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) Φmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) θmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) θmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). When θmax, θmin, Φmax, Φmin are used for the EEIRP requirement in subclause 6.9, manufacturer shall additional declare θmax, θmin, Φmax, Φmin for an electrical down tilt, where an electrical down tilt is decided by combination of mechanical down tilt and electrical down tilt. (D.64). | x | x | x |
| D.64 | The mechanical down-tilt | For EEIRP the declaration of combination of mechanical down tilt and electrical down tilt (NOTE 21), And mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive. | x | x | n/a |
| NOTE 21: The combination of maximum mechanical down tilt and the maximum electrical down tilt shall be declared. If the maximum electrical down tilt and the maximum mechanical down tilt are not simultaneously supported in application, the manufacturer shall declare and test the following two instances: One instance is combination of maximum mechanical down tilt and the reduced electrical down tilt, and the other instance is combination of maximum electrical down tilt and reduced mechanical down tilt. |  |  |  |  |  |

So, we propose,

Proposal 2: Update the manufacturer declaration for EEIRP as following:

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) Φmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) θmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) θmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). When θmax, θmin, Φmax, Φmin are used for the EEIRP requirement in subclause 6.9, manufacturer shall additional declare θmax, θmin, Φmax, Φmin for an electrical down tilt, where an electrical down tilt is decided by combination of mechanical down tilt and electrical down tilt. (D.64). | x | x | x |
| D.64 | The mechanical down-tilt | For EEIRP the declaration of combination of mechanical down tilt and electrical down tilt (NOTE 21), And mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive. | x | x | n/a |
| NOTE 21: The combination of maximum mechanical down tilt and the maximum electrical down tilt shall be declared. If the maximum electrical down tilt and the maximum mechanical down tilt are not simultaneously supported in application, the manufacturer shall declare and test the following two instances: One instance is combination of maximum mechanical down tilt and the reduced electrical down tilt, and the other instance is combination of maximum electrical down tilt and reduced mechanical down tilt. |  |  |  |  |  |

Measurement and calculation of EEIRP

From above, there are four types of angle directions used for EEIRP testing as following:

- The angle θn,φm  for Expected EIRP defined based on global coordinate system with respect to the horizon in Figure 6.9.5.1-1 in sub-clause 6.9.5.1. The angle θn,φm is defined with respect to the horizon with angles above the horizon positive and below the horizon negative.

- The test beam direction (s) in test beam directions set, which is OTA peak directions set maximum steering direction(s) (D.10) declared based on local coordinate system associated to an identifiable physical feature on the base station enclosure in sub-clause 4.14 of TS 38.141-2.

- Mechanical tilt angle (D.64), as above proposal 1, Mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive.

- The direction angle in coordinate system used by the test system.

From our understanding, there are two methods to do EEIRP measurement: one method is measuring EIRP in upper half of spherical space, another method is measuring EIRP in full spherical space. For first method, the setting mechanical tilt in test procedure is needed, we just need to measure the EIRP in upper half of spherical space, and then use EIRP to calculate EEIRP. And for second method, we can measure EIRP0 with zero mechanical tilt in full spherical space, then we should calculate EIRP in upper half of spherical space using measured EIRP0 and transformation, at last, we can use calculated EIRP to calculate EEIRP. For second method, we need to ensure the measured EIRP0 values corresponding to the angles after coordinate transformation exist.

So for second method, the basic step is as following:

1. Place the BS at the positioner with mechanical down-tilt set to zero, such that blocking effect in the vertical domain is minimized.

2. Align the manufacturer declared coordinate system orientation (D.2) of the BS for test beam direction(s) in test beam direction set in sub-clause 4.9.3.2 with the coordinate system used by the test system.

3. Measure EIRP0kΘl,Φq for test beam direction k, at positioner angle Θl,Φq , where  Θl,Φq is full spherical space, and   Θl,Φq is defined based on manufacturer declared coordinate system orientation (D.2) in sub-clause 4.9.3.2, this is because the above step 2 align it with coordinate system of test system.

4. Averaging EIRP0kΘl,Φq over K beamforming directions as EIRP0Θl,Φq=k=1KwkEIRP0k(Θl,Φq)  .

5.  Calculate  EIRPθn,φm for each mechanical tilt θMT from EIRP0Θl,Φq with zero mechanical tilt using transformation from global angle θn,φm in global coordinate system defined with respect to the horizon in Figure 6.9.5.1-1 to local angle Θl,Φq in local (zero mechanical tilt) reference coordinate system defined in sub-clause 4.14.

6.  Use   EIRPθn,φm to calculate  EEIRPθHLi , θHHi=(θHHi-θHLi)×π180°MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn .

According to above base step for calculating EEIRP, the angle and coordinate system in current test procedure and averaging processes is unclear. So we propose,

Proposal 3: To update the angle and coordinate system used in current test procedure and averaging processes with the following steps:

1. Place the BS at the positioner with mechanical down-tilt set to zero, such that blocking effect in the vertical domain is minimized.

2. Align the manufacturer declared coordinate system orientation (D.2) of the BS for test beam direction(s) in test beam direction set in sub-clause 4.9.3.2 with the coordinate system used by the test system.

3. Measure EIRP0kΘl,Φq for test beam direction k, at positioner angle Θl,Φq , where  Θl,Φq is full spherical space, and   Θl,Φq is defined based on manufacturer declared coordinate system orientation (D.2) in sub-clause 4.9.3.2, this is because the above step 2 align it with coordinate system of test system.

4. Averaging EIRP0kΘl,Φq over K beamforming directions as EIRP0Θl,Φq=k=1KwkEIRP0k(Θl,Φq)  .

5.  Calculate  EIRPθn,φm for each mechanical tilt θMT from EIRP0Θl,Φq with zero mechanical tilt using transformation from global angle θn,φm in global coordinate system defined with respect to the horizon in Figure 6.9.5.1-1 to local angle Θl,Φq in local (zero mechanical tilt) reference coordinate system defined in sub-clause 4.14.

6.  Use   EIRPθn,φm to calculate  EEIRPθHLi , θHHi=(θHHi-θHLi)×π180°MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn .

Based on above proposal, A CR for TS 38.141-2 on conformance testing requirement for EEIRP for upper 6GHz is provided for approval.

- Conclusion

This contribution provides analysis on angle and coordinate system for Excepted EIRP mask for upper 6GHz. The following observation and proposal is concluded as follows:

Observation 1: The reference coordinate system defined in sub-clause 4.14 of TS 38.141-2 is for OTA peak directions set maximum steering direction(s) (D.10).

Observation 2: The reference coordinate system in sub-clause 4.14 of TS 38.141-2 is local coordinate system associated to an identifiable physical feature on the base station enclosure.

Observation 3: The Θmin, Θmax, Φmax, Φmin in OTA peak directions set maximum steering direction(s) include effect of electrical down tilt.

Observation 4: Mechanical tilt is related to global coordinate system instead of local coordinate system associated to an identifiable physical feature on the base station enclosure.

Observation 5: The Θmin, Θmax, Φmax, Φmin in OTA peak directions set maximum steering direction(s) (D.10) is not varied with any mechanical tilt, since they are declared based on local coordinate system associated to an identifiable physical feature on the base station enclosure.

Observation 6: Expected EIRP (EEIRP) requirement is defined based on global coordinate system, which is coordinate system in sub-clause 6.9.5.1 of TS 38.141-2.

Observation 7: The coordinate system for mechanical tilts declaration (D.64) for EEIRP is unclear.

Proposal 1: Add the coordinate system for mechanical tilts to Table 6.9.5.1-1 for EEIRP limits as below:

Table 6.9.5.1-1: EEIRP limits as function of elevation above horizon

| Bin number i | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<<30 | 16 |
| 6 | 30≤<<60 | 15 |
| 7 | 60≤<<90 | 15 |
| Note 1: The requirement shall apply to all supported mechanical tilts. Mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive. |  |  |

Observation 8: The declarations for Θmin, Θmax, Φmax, Φmin for test beam directions are unclear.

Proposal 2: Update the manufacturer declaration for EEIRP as following:

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) Φmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) θmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) θmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). When θmax, θmin, Φmax, Φmin are used for the EEIRP requirement in subclause 6.9, manufacturer shall additional declare θmax, θmin, Φmax, Φmin for an electrical down tilt, where an electrical down tilt is decided by combination of mechanical down tilt and electrical down tilt. (D.64). | x | x | x |
| D.64 | The mechanical down-tilt | For EEIRP the declaration of combination of mechanical down tilt and electrical down tilt (NOTE 21), And mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive. | x | x | n/a |
| NOTE 21: The combination of maximum mechanical down tilt and the maximum electrical down tilt shall be declared. If the maximum electrical down tilt and the maximum mechanical down tilt are not simultaneously supported in application, the manufacturer shall declare and test the following two instances: One instance is combination of maximum mechanical down tilt and the reduced electrical down tilt, and the other instance is combination of maximum electrical down tilt and reduced mechanical down tilt. |  |  |  |  |  |

Proposal 3: To update the angle and coordinate system used in current test procedure and averaging processes with the following steps:

1. Place the BS at the positioner with mechanical down-tilt set to zero, such that blocking effect in the vertical domain is minimized.

2. Align the manufacturer declared coordinate system orientation (D.2) of the BS for test beam direction(s) in test beam direction set in sub-clause 4.9.3.2 with the coordinate system used by the test system.

3. Measure EIRP0kΘl,Φq for test beam direction k, at positioner angle Θl,Φq , where  Θl,Φq is full spherical space, and   Θl,Φq is defined based on manufacturer declared coordinate system orientation (D.2) in sub-clause 4.9.3.2, this is because the above step 2 align it with coordinate system of test system.

4. Averaging EIRP0kΘl,Φq over K beamforming directions as EIRP0Θl,Φq=k=1KwkEIRP0k(Θl,Φq)  .

5.  Calculate  EIRPθn,φm for each mechanical tilt θMT from EIRP0Θl,Φq with zero mechanical tilt using transformation from global angle θn,φm in global coordinate system defined with respect to the horizon in Figure 6.9.5.1-1 to local angle Θl,Φq in local (zero mechanical tilt) reference coordinate system defined in sub-clause 4.14.

6.  Use   EIRPθn,φm to calculate  EEIRPθHLi , θHHi=(θHHi-θHLi)×π180°MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn .

Based on above proposal, A CR[2] for TS 38.141-2 on conformance testing requirement for EEIRP for upper 6GHz is provided for approval.

- References

- R4-209356, Discussion on angle and coordinate system for Expected EIRP mask for upper 6GHz, CATT, RAN4#116

- R4-2513157, (NR_BS_RF_req_evo-Core)CR for TS 38.141-2, On conformance testing requirement for EEIRP for upper 6GHz, CATT, RAN4#116bis