# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2514473

# Prague, Czech Republic, Oct. 13-17, 2025

Source: 	Huawei, HiSilicon

Title: 		Discussion on the proposed conclusions for the preferred OTA test methods

Agenda Item:	6.8.4.2.2

Document for:	Discussion

# Introduction

During RAN4#114bis meeting, discussion paper in [3] was presented, on the European Commission’s feedback on alternative test methods, which was received during ETSI meeting with the European Commission (EC), and reported via ETSI OCG (Operational Coordination Group) RED (Radio Equipment Directive) meeting #89 [1]. Related contribution was presented during ETSI TFES#83 (March 2025) in [2].

This topic was disused during RAN4#116 meeting, WF in [7] was approved:

Issue 2-2: Reference method and preferred method Agreement: - The anechoic chamber to be clarified as the preferred test method for TRP measurements (as the reverberation chamber is already considered as the alternative test method); - Further discuss whether the “preferred” terminology is the right one. - Further work on CR to TR 37.941 to clarify definition of the anechoic chamber. - Enhancements to the Annex I (TRP sampling grids) to be further detailed to address Spherical Equal Angle Grid; related modifications to Annex I to be merged with the above chamber modifications in sub-bullet a. - Further discuss whether the “reference” terminology is the right one.

In this contribution we provide discussion towards the proposed conclusions for the preferred OTA test methods topic.

# Discussion

The following information was captured in [2]:

We draw attention to the EC feedback related to the alternative test methods, which was part of the discussion on the TRP simplification topic during previous meetings. The above referred feedback says that for ETSI Harmonized Standard, a preferred test method now always needs to be identified – otherwise, the Harmonized Standard may not be listed in the Official Journal (OJ). Therefore, this issue is of high importance to all companies who care about efficient use of ETSI Harmonized Standards for products conformance testing and their smooth introduction to the market.

RAN4 conformance testing specifications are the baseline for the ETSI MSG TFES work on Harmonized Standards. Therefore, it is advised that RAN4 seriously considers the above EC feedback (otherwise, decisions on the preferred test methods for BS Harmonized Standards will have to be taken in ETSI MSG TFES, which is not seen as effective an approach).

The following can be observed:

Observation 1: In the above referred text, OTA test chambers are listed as examples (i.e., the information on the test method is not as granular as to provide e.g., TRP sampling grids).

Observation 2: While EC feedback is generic and not related to TRP measurements only, it seems that the focus is on the OTA measurements due to much more complex nature of radiated test methodology compared to well established conducted testing.

## Consideration of TR 37.941

Referring to TR 37.941 clause 6.3.2 (TRP measurement procedures), there is a table listing TRP measurement methods and chambers, mapping them to the type of emissions to be measured (extracted below).

Table 6.3.2.1-1: Applicability of TRP measurement methods and chambers to the type of emissions to be measured BS output power; Unwanted emissions: ACLR (Note 1); Unwanted emissions: SEM, OBUE; Spurious emissions (Note 7). Full sphere using reference steps (accurate); X; X; X; X (Note 2). Methods applicable to; Full sphere using sparse sampling (overestimate); X (Note 2, 3). anechoic chambers; Two cuts + Pattern multiplication (accurate) (Note 4); X; X; X. Two/three cuts (overestimate); X; X; X (Note 2, 3). Beam-based directions; X; X (Note 5); X (Note 5). Peak method; X; X (Note 2). Equal sector with peak average; X; X (Note 2). Pre-scan (Note 2); X; X; X. Reverberation chamber; X; X; X; X (Note 2). Note 1: Two TRP measurements are needed, i.e. one for wanted channel, and the other one for adjacent channel. Note 2: Pre-scan is needed to identify the frequencies of interest. Note 3: At harmonic frequencies the use of this method is not applicable. Note 4: Pattern multiplication is conditional, as detailed in clause 6.3.4.5. Note 5: Applicable if the directivity of corresponding requirement at the reference direction is equivalent to the directivity at the reference direction when BS emits Prated,c,TRP and Prated,c,EIRP. Note 6: If box is blank the method is not excluded but the methodology has not been described in clause 6.3.3 on angular alignment in TRP measurements; if a suitable analysis is shown the method may be applied. Note 7: Applicability of any chamber for the spurious emissions measurements is subject to the supported frequency range of the chamber, and specifically its upper frequency limit..

One could consider adding information on the preferred test method to this table, but such approach would cause problems:

- If information on the preferred test method is included in TR 37.941 table 6.3.2.1-1, then granularity of the information would have to be as fine as TRP sampling grid (e.g., 2cut, 3cut, pre-scan, etc.) and not just e.g., chamber type). It is expected that reaching RAN4 agreements on the “preferred” TRP sampling grid per-requirement would be far from easy.

- Baseline for ETSI HS drafting is RAN4 conformance testing specifications (and not TR 37.941). It would be more straight-forwards to embed information on the preferred test method directly in related RAN4 conformance testing specifications.

Observation 3: It is not preferred to include information on the preferred TRP test method in TR 37.941.

Referring to the measurement procedures and chambers described in TR 37.941, we have extracted information on the chambers used for the derivations of the MU budget:

|  |  | Chamber type |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Indoor Anechoic Chamber | Compact Antenna Test Range | Near Field Test Range | One Dimensional Compact Range | Plane Wave Synthesiser | Reverberation Chamber | General Chamber |
| Tx | EIRP - Normal conditions | X | X | X | X | X |  |  |
|  | EIRP - Extreme conditions | X | X |  |  |  |  |  |
|  | DL RS | X | X | X |  | X |  |  |
|  | Power dynamics | X | X | X |  | X |  |  |
|  | EVM (%) | X | X | - |  | X |  |  |
|  | In-band TRP | X | X | X |  | X | X |  |
|  | ACLR - absolute | X | X | X |  | X | X |  |
|  | ACLR - relative | - | X | - |  | X | X |  |
|  | OBUE | X | X | X |  | X | X |  |
|  | SEM | X | X | X |  | X | X |  |
|  | Tx spurious emissions |  |  |  |  |  | X | X |
|  | Co-ex EM |  | X |  |  |  |  |  |
| Rx | EIS | X | X | X | X | X |  |  |
|  | Rx spurious emissions |  |  |  |  |  |  | X |

The following can be observed based on the table above:

Observation 4: There is no single chamber type that can be used for all requirement measurements.

Observation 5: Conclusion on the preferred chamber type would be requirement-specific.

It shall be clarified here, that the discussion on the preferred test method selection is not going to remove, nor de-prioritize any of the existing OTA test methods in TR 37.941 and related OTA BS conformance testing specifications.

## Consideration of TS 38.141-2

Referring to TRP test procedure descriptions in TR 38.141-2, one can find the following wording, e.g., for OTA BS output power (TRP):

#### 6.3.4.2 Procedure The following procedure for measuring TRP is based on the directional power measurements as described in annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so, follow steps 1, 3, 5, and 7. When calibrated and operated within the guidance of 3GPP TR 37.941 [29] the measurement methods are applicable and selected depending on availability at the test facility.

While the first extracted sentence above refers to TRP measurement procedures described in Annex I (applicable to anechoic chambers), there is also possibility to use alternative method to measure TRP in a reverberation chamber.

The same structure of text is also used for other TRP requirements: ACLR, OBUE, Tx /Rx spur.

As the reverberation chamber is already referred in OTA BS test specification as an “alternative method” for TRP measurements, one potential solution could be to clarify in OTA BS conformance testing specifications that the preferred test method for TRP measurements is the “anechoic chamber” in general.

Observation 6: While OTA BS test specs refer to TRP measurement procedures described in Annex I, it is not clarified that the Annex I is related to anechoic chamber only.

## Discussion on general chamber

The term “general chamber” has been used in TR 37.941 in two different ways during the analysis of the MU budgets the meaning of the term general chamber differs for the following requirement types:

-	Co-location requirements,

-	Out-of-band TRP requirements.

For co-location requirements, the MU calculation focuses on the uncertainties between the test equipment, the CLTA and the DUT as such the precise chamber type is not relevant as such when calculation the MU it was referred to as a General Chamber meaning any of the available chamber types which are capable of performing the appropriate OTA measurement associated with the co-location requirements.

For out-of-band spurious emissions and out-of-band blocking requirements the frequency range specified is very great (1 MHz to 12.75 GHz) as such many of the constraints on the band-specific chambers cannot be met, it is possible that multiple chambers may be required to cover the whole frequency range for example, so in this case the term General Chamber was used to imply a non-specific chamber that was capable of meeting the MU values specified.

Short description of the General chamber for the purposes of co-location and OOB TRP measurements is provided below (reuse from TR 37.941).

- Measurement system description (co-location requirements)

The General Chamber represents any suitable OTA chamber which shields the BS and CLTA form external interference and prevents reflections within the chamber from altering the coupling between the BS and the CLTA, i.e., any of the OTA measurement systems described in TR 37.941 clause 7.2 to 7.6 may be considered as example of the General Chamber.

The General Chamber represents any chamber capable of performing the OTA transmitter spurious emissions requirements, the OTA operating band unwanted emissions requirements and the OTA ACLR requirements to the specified accuracy.

- Measurement system description (out-of-band TRP requirements)

The General Chamber represents any suitable OTA chamber capable of performing the out-of-band TRP requirements. Any OTA measurement systems may be considered as example of the General Chamber as long as the MU is within the specified value (or the test requirement is offset appropriately) and they are suitable for the frequencies being tested.

Observation 7: General chamber term is used in TR 37.941 for co-location requirements and out-of-band TRP requirements.

## WF follow-up

WF in [7] has captured the following open issues related to the handling of anechoic chamber for BS technical specifications:

1. Further discuss whether the “preferred” terminology is the right one.

2. Further work on CR to TR 37.941 to clarify definition of the anechoic chamber.

For bullet #1 above we are flexible on the wording used. If there is no other agreeable alternative proposed, we suggest to stick to the “preferred” wording, when referring to the TRP measurements in the anechoic chamber (as opposed to measurements in reverberation chamber). One argument to keep the “preferred” wording is that it directly addressed concerns from CEPT ECC.

Proposal 1: Proceed with the “preferred” wording, when referring to the TRP measurements in the anechoic chamber, unless there is another agreeable alternative.

For bullet #2 above, it was confirmed that there is no definition of the “anechoic chamber” in TR 37.941. Therefore:

Proposal 2: Introduce generic definition of the “anechoic chamber” in TR 37.941.

# Conclusions

Based on the discussion above, the following proposals are now formulated, considering offline discussion on Draft CRs last meeting:

Proposal 1: Proceed with the “preferred” wording, when referring to the TRP measurements in the anechoic chamber, unless there is another agreeable alternative.

Proposal 2: Introduce generic definition of the “anechoic chamber” in TR 37.941.

Proposal 3: Endorse Draft CR to TS TS 38.141-2 and TS 37.145-2, aiming to proceed with formal CRs during RAN4#117 meeting.

Draft CRs proposing related implementation of the proposed solution were submitted in [8, 9, 10].

# References

[1] 	OCGREDEMCD(25)089010r1, Dialogue EC-ETSI on RED standardisation #3 7 February 2025 - conclusions, ETSI, OCG RED-EMCD, https://docbox.etsi.org/OCG/OCG_Red-Emcd/05-CONTRIBUTIONS/2025/OCGREDEMCD(25)089010r1_Dialogue_EC-ETSI_on_RED_standardisation__3_7_February_202.zip

[2]		TFES(25)83027, Open issues discussed with the European Commission – updates,  https://docbox.etsi.org/msg/tfes/05-CONTRIBUTIONS/2025/TFES(25)83027_Open_issues_discussed_with_the_European_Commission_-_updates.docx

[3]		R4-2504597, Information on European Commission feedback wrt. alternative test methods, RAN4#114bis

[4]		R4-2508658, Way Forward for [115][304] NR_BS_RF_Part3_OTA_TRP, RAN4#115

[5]		R4-2511561, Draft CR to TS 38.141-2: implementation of the preferred test method solution

[6]		R4-2511562, Draft CR to TS 37.145-2: implementation of the preferred test method solution

[7]		R4-2512580, Way Forward for [116][304] NR_BS_RF_Part3_OTA_TRP, RAN4#116

[8]		R4-2514474, Draft CR to TS 38.141-2: implementation of the preferred test method for TRP requirements

[9]		R4-2514475, Draft CR to TS 37.145-2: implementation of the preferred test method for TRP requirements

[10]	R4-2514476, Draft CR to TR 37.941: Definition of anechoic chamber