# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2514475

# Prague, Czech Republic, Oct. 13-17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR |  | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 37.145-2: implementation of the preferred test method solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on related discussion, in this Draft CR we provide proposal to address the preferred/reference test method issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implementation of the preferred test method for TRP measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The issue of the missing preferred test method would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.2, 6.7.3.4.2.1, 6.7.4.4.2, 6.7.5.4.2, 6.7.6.2.4.2, 6.7.6.4.4.2, 7.7.4.2, Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.141-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 6.3.2	OTA Maximum output power

#### 6.3.2.1	Definition and applicability

The rated carrier TRP shall be as specified for UTRA in table 6.3.2.1-1, and for E-UTRA and NR in table 6.3.2.1-2

Table 6.3.2.1-1: UTRA OTA AAS Base Station rated output power limits for BS classes

| OTA AAS BS class | Prated,c,TRP |
| --- | --- |
| Wide Area BS | (NOTE) |
| Medium Range BS | ≤ 44 dBm |
| Local Area BS | ≤ 30 dBm |
| NOTE: There is no upper limit for the Prated,c,TRP of the Wide Area Base Station. |  |

Table 6.3.2.1-2: E-UTRA and NR OTA AAS Base Station rated output power limits for BS classes

| OTA AAS BS class | Prated,c,TRP |
| --- | --- |
| Wide Area BS | (NOTE) |
| Medium Range BS | ≤ 47 dBm |
| Local Area BS | ≤ 33 dBm |
| NOTE: There is no upper limit for the Prated,c,TRP of the Wide Area Base Station. |  |

#### 6.3.2.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], clause 9.3.2.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], clause 9.3.2.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], clause 9.3.2.4.

#### 6.3.2.3	Test purpose

The test purpose is to verify the accuracy of the maximum carrier TRP (Pmax,c,TRP) across the frequency range for all RIBs in the AAS BS.

#### 6.3.2.4	Method of test

##### 6.3.2.4.1	Initial conditions

Test environment: Normal; see annex G.2.

RF bandwidth positions to be tested: BRFBW, MRFBW and TRFBW in single-band operation, see clause 4.12.1.

BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.12.1.

As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex F).

##### 6.3.2.4.2	Procedure

The following is the preferred procedure for measuring TRP, based on the directional power measurements within anechoic chamber (as defined in TR 37.941 [38]), as described in Annex F. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so, follow steps 1, 4, 5, and 7. When calibrated and operated within the guidance of 3GPP TR 37.941 [38] the measurement methods are applicable and selected depending on availability at the test facility.

------------------------------ Next modified section ------------------------------

### 6.7.3	OTA Adjacent Channel Leakage power Ratio

#### 6.7.3.1	Definition and applicability

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

#### 6.7.3.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], clause 9.7.3.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], clause 9.7.3.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], clause 9.7.3.4.

#### 6.7.3.3	Test purpose

To verify that the adjacent channel leakage power ratio requirement shall be met as specified by the minimum requirement.

#### 6.7.3.4	Method of test

##### 6.7.3.4.1	Initial conditions

6.7.3.4.1.1	General test conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested for single carrier:

-	B and T; see clause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW and TRFBW in single-band operation; see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.12.1.

6.7.3.4.1.2	MSR

For E-UTRA ACLR requirement outside the Base Station RF Bandwidth edges and the ACLR requirement applied inside sub-block gap, in addition, for non-contiguous spectrum operation or Inter RF Bandwidth gap for multi-band operation using, the test configurations defined in clause 4.8, the method of test described in clause 6.7.3.4.2 applies.

6.7.3.4.1.3	UTRA FDD

Set the AAS BS to transmit a signal modulated in accordance to TM1, in clause 4.12.2.

For an AAS BS declared to be capable of multi-carrier operation, set the base station to transmit according to TM1 on all carriers configured.

6.7.3.4.1.4	E-UTRA

For an AAS BS declared to be capable of single carrier operation only set to transmit a signal according to E-TM1.1 in clause 4.12.2.

For an AAS BS declared to be capable of multi-carrier and/or CA operation, set to transmit according to E-TM1.1 on all carriers configured.

6.7.3.4.1.5	NR

For an AAS BS declared to be capable of single carrier operation only set to transmit a signal according to NR-FR1-TM1.1 in clause 4.12.2.

For an AAS BS declared to be capable of multi-carrier and/or CA operation, set to transmit according to NR-FR1-TM1.1 on all carriers configured.

##### 6.7.3.4.2	Procedure

6.7.3.4.2.1	General Procedure

The following is the preferred procedure for measuring TRP, based on the directional power measurements within anechoic chamber (as defined in TR 37.941 [38]), as described in Annex F. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so, follow steps 1, 3, 4, 6 and 9. When calibrated and operated within the guidance of 3GPP TR 37.941 [38] the measurement methods are applicable and selected depending on availability at the test facility.

------------------------------ Next modified section ------------------------------

### 6.7.4	OTA Spectrum emission mask

#### 6.7.4.1	Definition and applicability

This requirement is applicable for AAS BS in single RAT UTRA operation only.

The spectrum emission mask minimum requirements are quoted as TRP unless otherwise stated.

#### 6.7.4.2	Minimum requirement

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], clause 9.7.4.3.

This requirement does not apply to single RAT E-UTRA operation or MSR operation.

#### 6.7.4.3	Test purpose

This test measures the emissions of the AAS BS, close to the assigned channel bandwidth of the wanted signal, while the AAS BS is in operation.

#### 6.7.4.4	Method of test

##### 6.7.4.4.1	Initial conditions

6.7.4.4.1.1	General test conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW in single-band operation; see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.12.1.

6.7.4.4.1.2	UTRA FDD

For an AAS BS declared to be capable of single carrier operation only, set to transmit a signal according to TM1, in clause 4.12.2.

For a multi-carrier capable AAS BS, set to transmit according to TM1 on all carriers configured using the applicable test configuration.

##### 6.7.4.4.2	Procedure

The following is the preferred procedure for measuring TRP, based on the directional power measurements within anechoic chamber (as defined in TR 37.941 [38]), as described in Annex F. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so, follow steps 1, 3, 4, 5, 7 and 10. When calibrated and operated within the guidance of 3GPP TR 37.941 [38] the measurement methods are applicable and selected depending on availability at the test facility.

------------------------------ Next modified section ------------------------------

### 6.7.5	OTA Operating band unwanted emission

#### 6.7.5.1	Definition and applicability

Unless otherwise stated, for E-UTRA single band and MSR the operating band unwanted emission limits are defined from ΔfOBUE below the lowest frequency of each supported downlink operating band to the lower Base Station RF Bandwidth edge located at FBW RF,low and from the upper Base Station RF Bandwidth edge located at FBW RF,high up to ΔfOBUE above the highest frequency of each supported downlink operating band.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer's specification.

For BS operating in bands n50, n51, n74, n75 and n76 additional emission limits that might be applicable outside OBUE frequency domain are specified in clause 6.7.5.5.4.6.

#### 6.7.5.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], clause 9.7.5.2

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], clause 9.7.5.4.

This requirement does not apply to single RAT UTRA operation.

#### 6.7.5.3	Test purpose

This test measures the emissions of the AAS BS, close to the assigned channel bandwidth of the wanted signal, while the AAS BS is in operation.

#### 6.7.5.4	Method of test

##### 6.7.5.4.1	Initial conditions

Test environment:

-	normal; annex G.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW in single-band operation; see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.12.1

##### 6.7.5.4.2	Procedure

The following is the preferred procedure for measuring TRP, based on the directional power measurements within anechoic chamber (as defined in TR 37.941 [38]), as described in Annex F. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so, follow steps 1, 3, 4, 6, 9 and 10. When calibrated and operated within the guidance of 3GPP TR 37.941 [38] the measurement methods are applicable and selected depending on availability at the test facility.

------------------------------ Next modified section ------------------------------

### 6.7.6	OTA Spurious emission

#### 6.7.6.1	General

The OTA spurious emissions limits are specified as TRP per cell unless otherwise specified.

The OTA transmitter spurious emission limits apply from 30 MHz to 12.75 GHz, excluding the following RAT-specific frequency ranges:

-	UTRA FDD BS as specified in TS 25.104 [2]: from 12.5 MHz below the lowest carrier frequency used up to 12.5 MHz above the highest carrier frequency used.

-	E-UTRA BS as specified in TS 36.104 [4]: from ΔfOBUE below the lowest frequency of the downlink operating band up to ΔfOBUE above the highest frequency of the downlink operating band, where ΔfOBUE is defined in clause 6.7.1.

-	MSR BS as specified in TS 37.104 [5]: from ΔfOBUE below the lowest frequency of the downlink operating band up to ΔfOBUE above the highest frequency of the downlink operating band, where ΔfOBUE is defined in clause 6.7.1. For some operating bands the upper frequency limit is higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329 [16]. In some exceptional cases, requirements apply also closer than 10 MHz from the downlink operating band; these cases are highlighted in the requirement tables in respective referenced UTRA, E-UTRA, NR or MSR specifications. For operating bands supported by multi-band RIB each supported band including the ΔfOBUE around the band are excluded from the spurious emissions requirements.

The requirements apply for both single band RIBs and multi-band RIBs (except for frequencies at which exclusion bands or other multi-band provisions apply) and for all transmission modes foreseen by the manufacturer's specification. Unless otherwise stated, all requirements are measured as mean power.

For operation in Region 2, where the FCC guidance for MIMO systems in [17] is applicable, the emissions limits are the same regardless of the number of transceiver units so the limits are equivalent to those for a single transceiver unit as specified in the as the corresponding applicable non-AAS BS per transmitter requirement specified in TS 25.104 [2], TS 25.105 [3], TS 36.104 [4], TS 37.104 [5] or TS 38.104 [33]. For E-UTRA and NR the limits will be 9dB lower and for UTRA FDD the limits will be 6dB lower, unless stated differently in regional regulation.

The AAS BS requirements for spurious emissions limits which are specified for Band 46 in TS 37.104 [5], are applicable for AAS BS.

For BS operating in bands n50, n51, n74, n75 and n76 additional emission limits that might be applicable in the spurious emissions frequency domain are specified in clause 6.7.5.5.4.6.

#### 6.7.6.2	Mandatory Requirements

##### 6.7.6.2.1	Definition and applicability

The OTA spurious emissions mandatory requirements include the CAT A, CAT B and additional minimum requirements for BC2, limits are specified as TRP per cell unless otherwise specified.

##### 6.7.6.2.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], clause 9.7.6.2.1.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], clause 9.7.6.3.1.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], clause 9.7.6.4.1.

##### 6.7.6.2.3	Test purpose

The test purpose is to verify the radiated spurious emissions from the AAS BS at the RIB are within the specified minimum requirements.

##### 6.7.6.2.4	Method of test

6.7.6.2.4.1	Initial conditions

Test environment: normal, see annex G.2.

RF channels to be tested for single carrier:

B when testing from 30 MHz to FDL_low - ΔfOBUE

T when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

RF bandwidth positions to be tested: in single-band operation, see clause 4.12.1.

BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

TRFBW when testing from FDL_high + ΔfOBUE to 12.75 GHz (or 5th harmonic)

RF bandwidth positions to be tested in multi-band operation, see clause 4.12.1.

BRFBW_T'RFBW when testing from 30 MHz to FDL_Blow_low - ΔfOBUE

B'RFBW_TRFBW when testing from FDL_Bhigh_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

BRFBW_T'RFBW and B'RFBW_TRFBW when testing from FDL_Blow_high + ΔfOBUE to FDL_Bhigh_low - ΔfOBUE

Directions to be tested

As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex F).

6.7.6.2.4.2	Procedure

The following is the preferred procedure for measuring TRP, based on the directional power measurements within anechoic chamber (as defined in TR 37.941 [38]), as described in Annex F. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so, follow steps 1, 3, 4, 5, 7 and 10. When calibrated and operated within the guidance of 3GPP TR 37.941 [38] the measurement methods are applicable and selected depending on availability at the test facility.

------------------------------ Next modified section ------------------------------

#### 6.7.6.4	Additional spurious emissions requirements

##### 6.7.6.4.1	Definition and applicability

These requirements may be applied for the protection of system operating in frequency ranges other than the BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the BS, or they may be set by local or regional regulation as a mandatory requirement for an operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in clause 4.5.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM/EDGE, CDMA, UTRA, E-UTRA, NR etc.) as listed below.

All additional spurious requirements are TRP unless otherwise stated.

##### 6.7.6.4.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], clause 9.7.6.2.3.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], clause 9.7.6.3.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], clause 9.7.6.4.3.

##### 6.7.6.4.3	Test purpose

The test purpose is to verify the radiated spurious emissions from the AAS BS at the RIB are within the specified additional spurious emissions requirements.

##### 6.7.6.4.4	Method of test

6.7.6.4.4.1	Initial conditions

Test environment: normal, see annex G.2.

RF channels to be tested for single carrier:

B when testing from 30 MHz to FDL_low - ΔfOBUE

T when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

RF bandwidth positions to be tested: in single-band operation, see clause 4.12.1.

BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

TRFBW when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

RF bandwidth positions to be tested in multi-band operation, see clause 4.12.1.

BRFBW_T'RFBW when testing from 30 MHz to FDL_Blow_low - ΔfOBUE

B'RFBW_TRFBW when testing from FDL_Bhigh_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

BRFBW_T'RFBW and B'RFBW_TRFBW when testing from FDL_Blow_high + ΔfOBUE to FDL_Bhigh_low - ΔfOBUE

Directions to be tested:

As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex F).

6.7.6.4.4.2	Procedure

The following is the preferred procedure for measuring TRP, based on the directional power measurements within anechoic chamber (as defined in TR 37.941 [38]), as described in Annex F. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so, follow steps 1, 3, 4, 5, 7 and 10. When calibrated and operated within the guidance of 3GPP TR 37.941 [38] the measurement methods are applicable and selected depending on availability at the test facility.

------------------------------ Next modified section ------------------------------

## 7.7	OTA Receiver spurious emissions

### 7.7.1	Definition and applicability

The receiver spurious emission requirement is the power of the emissions radiated from the antenna array from a receiver unit. For an OTA AAS BS operating in FDD, OTA RX spurious emissions requirement do not apply as they are superseded by the OTA TX spurious emissions requirement. This is due to the fact that TX and RX spurious emissions cannot be distinguished in OTA domain.

NOTE:	The OTA receiver spurious emission requirement applicability for the AAS BS with the RX-only capabilities are not covered by the present release of this specification..

For an OTA AAS BS operating in TDD, the OTA receiver spurious emissions requirement applies during the transmitter OFF period only.

For RX only multi-band RIB, the RX spurious emissions requirements are subject to exclusion zones in each supported operating band.

### 7.7.2	Minimum Requirement

For an MSR AAS BS the minimum requirement is in TS 37.105 [6], clause 10.7.2

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], clause 10.7.4

For single RAT UTRA AAS BS the minimum requirement is in TS 37.105 [6], clause 10.7.3

### 7.7.3	Test purpose

The test purpose is to verify the receiver radiated spurious emissions from the AAS BS at the RIB are within the specified requirements.

### 7.7.4	Method of test

#### 7.7.4.1	Initial conditions

Test environment: normal; see annex G.2.

RF channels to be tested for single carrier: M; see clause 4.12.1.

RF bandwidth positions to be tested in single-band operation: -	MRFBW, see clause 4.12.1,

RF bandwidth positions to be tested in multi-band operation, see clause 4.12.1.

-	BRFBW_T'RFBW when testing from 30 MHz to FDL_Blow_low - ΔfOBUE

-	B'RFBW_TRFBW when testing from FDL_Bhigh_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

-	BRFBW_T'RFBW and B'RFBW_TRFBW when testing from FDL_Blow_high + ΔfOBUE to FDL_Bhigh_low - ΔfOBUE

Directions to be tested: Not applicable as Rx only TRP measurement.

#### 7.7.4.2	Procedure

The following is the preferred procedure for measuring TRP, based on the directional power measurements within anechoic chamber (as defined in TR 37.941 [38]), as described in Annex F. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so, follow steps 1, 3, 4, 5, 7 and 10. When calibrated and operated within the guidance of 3GPP TR 37.941 [38] the measurement methods are applicable and selected depending on availability at the test facility.

------------------------------ Next modified section ------------------------------

Annex F (normative):
TRP measurement  procedures in anechoic chamber

# F.1	General

The annex describes various procedures for BS OTA TRP measurements within anechoic chamber (as defined in TR 37.941 [38]). These procedures can provide either an accurate or an over-estimate of TRP values. The procedures for an accurate estimate can be applied to all TRP requirements. However, if a TRP requirement does not need accurate TRP estimate then the procedures for over-estimate of TRP may be used in order to have a reasonable OTA test time. Pre-scan does not provide an accurate TRP estimate or over-estimate of TRP. Pre-scan is a fast but coarse method that is used to identify the spurious emission frequencies with emission power as described in annex F.13. A sequential measurement is then made at the emission frequencies, to assess the TRP as described in annex F.2 to annex F.9.

When making TRP measurements the alignment between EUT and measurement antenna is important to achieve expected measurement uncertainty;

1.	The measurement antenna needs to be aligned tangential to the measurement surface forming a sphere around the EUT, in order to measure the TRP properly.

2.	Test methods described in clauses F.5.1, F.5.2, F.10, F.11 and F.12 require angular alignment between the selected measurement grid and EUT radiation pattern in order to measure peak values in the main beams. Angular misalignment can lead to differences in the actual and measured angular positions of the intended maximum EIRP.

3.	Test methods described in clause F.5.3, F.6 and F.9 are designed to be independent of rotations of the angular grid, and hence angular alignment between the measurement grid and EUT is not needed.

The reference TRP measurement procedure for both in-band and out-of-band emissions is the spherical equal angle grid (see clause I.2), with angular steps not exceeding the reference angular step criteria (see clause I.2.2).

Other TRP measurement methods described e.g., in clauses I.3, I.4, I.6, can also be considered as accurate, if the reference angular step criteria (see clause I.2.2) is met. In-band measurements methods in clause I.5 (i.e., 2-3 orthogonal cuts), or clause I.10 (i.e., beam-based directions) can also be considered as accurate, if all applicability conditions for these methods are met.

# F.2	Spherical equal angle grid

## F.2.1	General

TRPEstimate is defined as:

TRPEstimate=π2NMn=0N-1m=0M-1EIRP(θn,ϕm)sinθn

when EIRP measurements is used or as:

TRPEstimate=4πd2π2NMn=1N-1m=0M-1PD(θn,ϕm)sinθn

when power density measurements are used, and d is the test distance. N and M are the number of samples in the θ and ϕ angles. Each (θn,ϕm) is a sampling point. The sampling angular intervals for θ and ϕ angles are Δθ=πN and Δϕ=2πM. The sampling intervals Δθ and Δϕ are described in F.2.2.

## F.2.2	Reference angular step criteria

The reference angular steps Δϕ and Δθ, in degrees, are defined as:

Δθref=⁡min(180∘πλD,15∘)

Δϕref= min⁡(180∘πλDcyl,15∘)

The upper limit for these reference angular steps of 15∘ ensures a low Summation Error (SE) when λ is large compared to the DUT dimensions.

Dcyl and D are calculated as:

Dcyl=d2+w2

D=d2+w2+h2

The definition of d, w and h is shown in Figure F.2.2-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure F.2.2-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

Optionally, in the case of a Uniform Linear Array (ULA), when d is negligible (d ≈0) and when the EUT is mounted along the yz plane as shown in figure F.2.2-2, the reference angular step, in degrees, can be determined by

∆θref=min(180°πarcsin⁡(λDz),15°)

Δϕref=min(180°πarcsin⁡(λDy),15°)

Where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUTalong the z-axis and  is wavelength for the measured frequency.

Figure F.2.2-2: Spherical coordinate for OTA conformance testing of EUT

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor) of the grid is defined as:

SF=max⁡(ΔθgridΔθref,ΔϕgridΔϕref)

Where Δθ and Δϕ are the actual angular steps used in the measurement.

Alternatively, when the EUT radiating dimensions are not known. For each frequency within the downlink operating band including ΔfOBUE, the reference angular steps can be specified in terms of the beamwidth of the wanted signal as

∆ϕref=λλo BeWϕ

Δθref=λλo BeWθ

where λo is the wavelength of the wanted signal, and BeWφ and BeWθ are the beamwidth of the wanted signal in the φ-axis and θ-axis, respectively.

BeWφ and BeWθ may be set to beamwidth declared for the OTA AAS BS radiated transmit power requirement provided the same beam is applied to test in-band TRP requirements.

NOTE:	Beamwidth is approximately equal to half the first-null beam width.

# F.3	Spherical equal area grid

TRPEstimate is defined as:

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

N is the total number of samples and specified as:

N≥4π∆θref∆ϕref

The sampling intervals Δθref and Δϕref are described in F.2.2. Each (θn,ϕn) is a sampling point.

# F.4	Spherical Fibonacci grid

TRPEstimate is defined as:

TRPEstimate=1Nn=0N-1EIRP(θn,ϕn)

N is the total number of samples and specified as:

N≥4π∆θref∆ϕref

The sampling intervals Δθref and Δϕref are described in F.2.2. Each (θn,ϕn) is a sampling point, where θn and ϕn, in degrees, are defined as:

θn=cos-11-2n+1N

ϕn=360°nΨ, Ψ=1+52

# F.5	Orthogonal cut grid

## F.5.1	General

Here, at least two cuts (default) shall be used, an optional third cut can be used. The alignment of the cuts must be along the symmetry planes of the antenna array. No alignment is required for spurious emissions.

When alignment is required:

1.	The first mandatory cut is a horizontal cut passing through the peak direction of the main beam.

2.	The second mandatory is a vertical cut passing through the peak direction of the main beam. Using the data from these two mandatory cuts, a conditional pattern multiplication can be used.

3.	The third optional cut is a vertical cut orthogonal to the first and the second cut.

When alignment is not required, the cuts can be aligned arbitrarily.

Once the number and the orientation of the cuts are decided, the total EIRP is measured on the orthogonal cuts and the TRP is then calculated as follows: First the contributions from each cut is calculated as:

EIRPav,cut-n=1Pj=1PEIRP(j)

where P is the number of sampling points in the cut. The final contribution for all cuts is calculated as:

TRPEstimate=1Nn=1NEIRPav,cut-n

where N is the number of cuts. Note that when orthogonal cuts are measured, the intersection points are measured multiple times and the repeated values can be removed from the samples before averaging.

When two cuts measurements are used, a conditional pattern multiplication can be applied. The following are the conditions for applying pattern multiplication:

i.	The vertical cut (and the main beam) is in the xz -plane

ii.	The frequency of the emission is within the downlink operating band.

iii.	The bandwidth of the emission is the same as the bandwidth of the in-band modulated signal

iv.	The emission appears/disappears when the Tx power is turned on/off.

v.	The antenna arrays of the EUT

1.	Have rectangular grids of antenna element positions

2.	Have symmetry planes that are vertical and horizontal.

3.	Have parallel antenna planes

The antenna array is here assumed to be placed in the yz-plane. The pattern multiplication is performed in uv-coordinates and the data in the two cuts are denoted EIRPcut1ϕ at θ=θH and a vertical cut with data EIRPcut2θ at ϕ=0. The data is split in two parts corresponding to the forward and backward hemispheres. The uv-coordinates are the projections of the angular directions onto the antenna plane, here the yz-plane. Using the spherical coordinates as depicted in figure F.2.2.-1 the u and v coordinates are defined as:

u=sinθsinϕv=cosθ

Note that only the data on the cuts are measured.

Calculate power density/EIRP values outside the two cardinal cuts as

EIRPu,v=EIRPcut1uEIRPcut2vEIRP0,vH

The pattern multiplication is applied separately for the forward (fwd) and backward (bwd) hemisphere. The TRP is then calculated as:

TRP=14πfwdEIRPfwdu,vdudv1-u2-v2+bwdEIRPbwdu,vdudv1-u2-v2

NOTE:	The numerical singularity at u2+v2=1 must be treated with care, e.g. by change of variables.

## F.5.2	Operating band unwanted emissions

The procedure is as follows:

1)	Follow steps described in annex I.5.1 for the first two mandatory cuts and calculate the TRPEstimate.

2)	Compare the TRPEstimate to the limit.

3)	If the TRPEstimate is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 2.

## F.5.3	Spurious unwanted emissions

The procedure is as follows:

1)	Follow steps described in annex F.5.1 for two cuts and calculate the preliminary TRPEstimate.

2)	Add the appropriate correction factor ΔTRP according to table F.5.3-1 to ensure overestimation with 95% confidence.

3)	Compare the corrected TRPEstimate (including ΔTRP) to the limit.

4)	If the corrected TRPEstimate is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 3.

Table F.5.3-1: The correction factor for two or three cuts dense sampling

|  | Three cuts | Two cuts |
| --- | --- | --- |
| Correction factor ΔTRP (dB) | 2.0 | 2.5 |

# F.6	Wave vector space grid

If EUT is mounted along the yz plane as shown in figure F.2.2-1, the reference step in wave vector space can be determined by:

Δuref=λDy

Δvref=λDz

where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUT along the z-axis.

According to the relationship between the normalized wave vector and spherical coordinate, the wave vector can be represented as following:

u=sinθsinϕ, v=cos⁡(θ)

The total radiated power (TRP) in the wave vector space is determined by:

TRP=ΔuΔv4πu2+v2<1cosϕ>0EIRPθn,ϕm,nsinθncosϕm,n +u2+v2<1cosϕ<0EIRPθn,ϕm,nsinθncosϕm,n

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor) of the grid is defined as:

SF=max⁡(ΔugridΔuref,ΔvgridΔvref)

Where Δugridand Δvgrid are the actual steps used in the wave vector space in the measurement and the upper bound of their value is 15 degrees.

# F.7	Orthogonal 2 cuts with pattern multiplication

This method can be used when the antenna symmetries are compatible with pattern multiplication, see Clause F.1.2.4. The procedure is as follows:

1.	Calculate the reference angular steps as described in Clause F.1.2.

2.	Align the EUT to allow for proper pattern multiplication. See Clause F.5. Measure EIRP on two orthogonal cuts with steps smaller or equal to the reference steps according to step 1.

3.	Apply pattern multiplication according to Clause F.5 to extrapolate the two cuts data to full-sphere.

4.	Apply numerical integration to obtain the TRP estimate as described in Clause F.5.

# F.8	Void

# F.9	Full sphere with sparse sampling

The procedure is as follows:

1.	Set the angular grid:

a.	Non-harmonic frequencies: choose the angular steps Δϕ and Δθ smaller than or equal to 15 degrees. Calculate the sparsity factor (SF) as

SF=maxΔϕΔϕref,ΔθΔθref

and the correction factor as:

ΔTRP=SF-1SFmax-1⋅1.0 dB,

where SFmax corresponds to 15 degrees angular step. If the sparsity factor is smaller than 1, the correction factor ΔTRP is 0 dB.

b.	Harmonic frequencies with fixed beam test signal: choose the angular steps smaller than or equal to the reference angular steps Δϕref and Δθref. Correction factor ΔTRP is 0 dB.

c.	Harmonic frequencies with beam sweeping test signal: set the angular steps to 15 degrees. Correction factor is ΔTRP 0 dB.

2.	Apply a suitable numerical integration to calculate the TRP estimate.

3.	Add the appropriate correction factor ΔTRP according to step 1 to ensure an overestimation with 95% confidence.

4.	Compare the (TRP estimate + ΔTRP) with the limit. If the (TRP estimate + ΔTRP) is above the limit, choose a smaller angular step and repeat steps 2-4. If the sparsity factor is less than one, no significant improvement of accuracy is expected.

# F.10	Beam-based directions

Beam-based direction can be used in the base station operating band only if the directivity of the radiation pattern of the emssions being measured is known.band. TRPEstimate is defined as:

TRPEstimate=EIRPpeakDEUT,

where EIRPpeak is the maximum EIRP in the beam peak direction within a particular beam direction pair and DEUT is directivity of the EUT antenna.

# F.11	Peak method

The peak method can be used when frequencies with unwanted peak emissions are identified during pre-scan. The method does not provide an estimate of TRP.

For each peak emission frequency identified during pre-scan, measure peak EIRP or power density as follows:

1.	Move EUT and test antenna to the same position where the peak emission is recorded during the pre-scan.

2.	Move the EUT around the position and test antenna orientation to find the final peak EIRP or power density.

3.	The measured peak power density or EIRP shall be used to demonstrate conformance.

NOTE:	Peak EIRP is the linear sum of two orthogonal polarised components.

# F.12	Equal sector with peak average

Equal sector with peak average can be performed on frequencies with unwanted peak emission, which are considered by the peak method for further measurements.

The spherical angle ϕ is divided into K equal sectors. If the largest dimension of EUT is less than 60 cm, then each sector is a half quadrant of 45°.

For each peak emission frequency, measure peak EIRP of beams belonging to different sectors of the sphere as follows:

1.	Move EUT and test antenna to the same position where the emission peak is recorded during the pre-scan.

2.	Move EUT around the position and test antenna orientation to find the final peak EIRP.

3.	Repeat Steps 1 to 2 until all sectors are covered.

4.	Calculate TRPEstimate as:

TRPEstimate=1Kk=1KEIRPk,

where EIRPk is the peak EIRP in the kth sector.

NOTE:	Peak EIRP is the linear sum of two orthogonal polarised components.

# F.13	Pre-scan

Pre-scan is used to identify frequencies with unwanted emission power levels above a certain threshold. The pre-scan does not provide an estimate of TRP. An emission frequency identified by a pre-scan may be further investigated by any of the TRP measurement methods in this annex.

The procedure for pre-scan is as follows:

1.	Scan the entire surface around EUT.

2.	Rotate test antenna to cover all possible polarisations of emissions to detect maximum emissions.

3.	Record the list of frequencies and corresponding unwanted emission power levels, EUT spatial positions, and test antenna polarization for which the maximum emission levels occur.

4.	Emissions which are 20 dB or more below the specified limit shall not require further measurements.

------------------------------ End of modified section -------------------------