3GPP TSG-RAN WG4 Meeting #116bis	R4-2515121

# Prague,CZ, 13th - 17th Oct, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR |  | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.104 on SBFD BS maintenance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some of RF requirement for SBFD BS is not appropriately defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the definition for SBFD; - Update the operating band for SBFD part to avoid the potential misunderstanding for exceptional bands; - To update the note for Table 5.3.2-1 to make it more clear; - To update the note for Table 5.3.5-1/2 to make it more clear; - Change the WA, MR, LA BS with more clear definition. - To add more clarification to clause 12.1.3.2; - To add more clarification to clause 12.2; - To add more clarification to clause 12.3; - To add more clarification to clause 12.5.9; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RF requirement for SBFD BS is not appropriately defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.2, 5.3.2, 12.1.3, 12.2, 12.3, 12.5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.141-1/2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change 1 >>

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aggregated BS Channel Bandwidth:	The RF bandwidth in which a Base Station transmits and receives multiple contiguously aggregated carriers. The aggregated BS channel bandwidth is measured in MHz.

antenna connector: connector at the conducted interface of the BS type 1-C

active transmitter unit: transmitter unit which is ON, and has the ability to send modulated data streams that are parallel and distinct to those sent from other transmitter units to a BS type 1-C antenna connector, or to one or more BS type 1-H TAB connectors at the transceiver array boundary

Base Station RF Bandwidth: RF bandwidth in which a base station transmits and/or receives single or multiple carrier(s) within a supported operating band

NOTE:	In single carrier operation, the Base Station RF Bandwidth is equal to the BS channel bandwidth.

Base Station RF Bandwidth edge: frequency of one of the edges of the Base Station RF Bandwidth.

basic limit: emissions limit relating to the power supplied by a single transmitter to a single antenna transmission line in ITU-R SM.329 [2] used for the formulation of unwanted emission requirements for FR1

beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

NOTE:	For certain BS antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

beam peak direction: direction where the maximum EIRP is found

beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

BS channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink

NOTE 1:	The BS channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE 2:	It is possible for the BS to transmit to and/or receive from one or more UE bandwidth parts that are smaller than or equal to the BS transmission bandwidth configuration, in any part of the BS transmission bandwidth configuration.

BS transmission bandwidth configuration: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or receiving by the BS

BS transmission bandwidth configuration for SBFD-capable BS : set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the SBFD-capable BS

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

BS type 2-O:	NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

Channel edge: lowest or highest frequency of the NR carrier, separated by the BS channel bandwidth.

Carrier aggregation: aggregation of two or more component carriers in order to support wider transmission bandwidths

Carrier aggregation configuration: a set of one or more operating bands across which the BS aggregates carriers with a specific set of technical requirements

co-location reference antenna: a passive antenna used as reference for base station to base station co-location requirements

Contiguous carriers: set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Contiguous spectrum: spectrum consisting of a contiguous block of spectrum with no sub-block gap(s).

directional requirement: requirement which is applied in a specific direction within the OTA coverage range for the Tx and when the AoA of the incident wave of a received signal is within the OTA REFSENS RoAoA or the minSENS RoAoA as appropriate for the receiver

Enhanced channel raster: channel raster with a 10 kHz granularity in bands with a 100 kHz channel raster.

equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

NOTE:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

equivalent isotropic sensitivity: sensitivity for an isotropic directivity device equivalent to the sensitivity of the discussed device exposed to an incoming wave from a defined AoA

NOTE 1:	The sensitivity is the minimum received power level at which specific requirement is met.

NOTE 2:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

fractional bandwidth: fractional bandwidth FBW is defined as FBW=200∙FFBWhigh−FFBWlowFFBWhigh+FFBWlow%

Highest Carrier: The carrier with the highest carrier frequency transmitted/received in a specified frequency band.

Inter-band carrier aggregation: carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Inter-band gap: The frequency gap between two supported consecutive operating bands.

Intra-band contiguous carrier aggregation: contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: non-contiguous carriers aggregated in the same operating band.

Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands

Lowest Carrier:	The carrier with the lowest carrier frequency transmitted/received in a specified frequency band.

Lower sub-block edge: frequency at the lower edge of one sub-block.

NOTE:	It is used as a frequency reference point for both transmitter and receiver requirements.

maximum carrier output power: mean power level measured per carrier at the indicated interface, during the transmitter ON period in a specified reference condition

maximum carrier TRP output power: mean power level measured per RIB during the transmitter ON period for a specific carrier in a specified reference condition and corresponding to the declared rated carrier TRP output power (Prated,c,TRP)

maximum total output power: mean power level measured within the operating band at the indicated interface, during the transmitter ON period in a specified reference condition

maximum total TRP output power: mean power level measured per RIB during the transmitter ON period in a specified reference condition and corresponding to the declared rated total TRP output power (Prated,t,TRP)

measurement bandwidth: RF bandwidth in which an emission level is specified

minSENS: the lowest declared EIS value for the OSDD's declared for OTA sensitivity requirement.

minSENS RoAoA: The reference RoAoA associated with the OSDD with the lowest declared EIS

multi-band connector: Antenna Connector of BS type 1-C or TAB connector of BS type 1-H associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

multi-band RIB: operating band specific RIB associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

Multi-carrier transmission configuration: set of one or more contiguous or non-contiguous carriers that a NR BS is able to transmit simultaneously according to the manufacturer’s specification.

NB-IoT operation in NR in-band: NB-IoT is operating in-band when it is located within a NR transmission bandwidth configuration plus 15 kHz at each edge but not within the NR minimum guard band GBChannel.

NB-IoT operation in NR guard band: NB-IoT is operating in guard band when it is located within a NR BS channel bandwidth but is not NB-IoT operation in NR in-band.

Non-contiguous spectrum: spectrum consisting of two or more sub-blocks separated by sub-block gap(s).

Non-SBFD operation: SBFD-capable BS operation in non-SBFD symbols.

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

NOTE:	The operating band(s) for a BS is declared by the manufacturer according to the designations in tables 5.2-1 and 5.2-2.

OTA coverage range: a common range of directions within which TX OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met

OTA peak directions set: set(s) of beam peak directions within which certain TX OTA requirements are intended to be met, where all OTA peak directions set(s) are subsets of the OTA coverage range

NOTE:     The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

OTA REFSENS RoAoA: the RoAoA determined by the contour defined by the points at which the achieved EIS is 3dB higher than the achieved EIS in the reference direction assuming that for any AoA, the receiver gain is optimized for that AoA

NOTE:	This contour will be related to the average element/sub-array radiation pattern 3dB beamwidth.

OTA sensitivity directions declaration: set of manufacturer declarations comprising at least one set of declared minimum EIS values (with BS channel bandwidth), and related directions over which the EIS applies

NOTE:	All the directions apply to all the EIS values in an OSDD.

polarization match: condition that exists when a plane wave, incident upon an antenna from a given direction, has a polarization that is the same as the receiving polarization of the antenna in that direction

radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply

NOTE:	For requirements based on EIRP/EIS, the radiated interface boundary is associated to the far-field region

Radio Bandwidth: frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier

Railway Mobile Radio: railway operations encompassing GSM-R and its successor(s), including the Future Railway Mobile Communication System (FRMCS); in the context of this specification the Railway Mobile Radio is limited to NR operation in band n100, or n101

rated beam EIRP: For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the base station is declared to radiate at the associated beam peak direction during the transmitter ON period

rated carrier output power: mean power level associated with a particular carrier the manufacturer has declared to be available at the indicated interface, during the transmitter ON period in a specified reference condition

rated carrier TRP output power: mean power level declared by the manufacturer per carrier, for BS operating in single carrier, multi-carrier, or carrier aggregation configurations that the manufacturer has declared to be available at the RIB during the transmitter ON period

rated total output power: mean power level associated with a particular operating band the manufacturer has declared to be available at the indicated interface, during the transmitter ON period in a specified reference condition

rated total TRP output power: mean power level declared by the manufacturer, that the manufacturer has declared to be available at the RIB during the transmitter ON period

reference beam direction pair: declared beam direction pair, including reference beam centre direction and reference beam peak direction where the reference beam peak direction is the direction for the intended maximum EIRP within the OTA peak directions set

receiver target: AoA in which reception is performed by BS types 1-H or BS type 1-O

receiver target redirection range: union of all the sensitivity RoAoA achievable through redirecting the receiver target related to particular OSDD

receiver target reference direction: direction inside the OTA sensitivity directions declaration declared by the manufacturer for conformance testing. For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA

reference RoAoA: the sensitivity RoAoA associated with the receiver target reference direction for each OSDD.

requirement set:	one of the NR base station requirement's set as defined for BS type 1-C, BS type 1-H, BS type 1-O, and BS type 2-O

SBFD DUD operation: SBFD operation in the carrier on which one SBFD UL subband and two DL subbands are configured, with the UL subband configured in the middle of two SBFD DL subbands.

SBFD DU operation: SBFD operation in the carrier on which one SBFD UL subband and one DL subband are configured, with the UL subband configured higher than the DL subband in frequency domain.

SBFD operation: SBFD-capable BS operating with simultaneous transmitting and receiving in non-overlapping frequency domain in SBFD symbol(s).

SBFD UD operation: SBFD operation in the carrier on which one SBFD UL subband and one DL subband are configured, with the UL subband configured lower than the DL subband in frequency domain.

SBFD-capable BS: NR base station which is capable of SBFD operation.

sensitivity RoAoA: RoAoA within the OTA sensitivity directions declaration, within which the declared EIS(s) of an OSDD is intended to be achieved at any instance of time for a specific BS direction setting

single-band connector: BS type 1-C antenna connector or BS type 1-H TAB connector supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band connector.

single-band RIB: operating band specific RIB supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band RIB.

sub-band: A sub-band of an operating band contains a part of the uplink and downlink frequency range of the operating band.

sub-block bandwidth: bandwidth of one sub-block.

sub-block: one contiguous allocated block of spectrum for transmission and reception by the same base station

NOTE:	There may be multiple instances of sub-blocks within a Base Station RF Bandwidth.

sub-block gap: frequency gap between two consecutive sub-blocks within a Base Station RF Bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation

superseding-band: A superseding-band of an operating band includes the whole of the uplink and downlink frequency range of the operating band.

TAB connector: transceiver array boundary connector

TAB connector RX min cell group: operating band specific declared group of TAB connectors to which BS type 1-H conducted RX requirements are applied

NOTE:	Within this definition, the group corresponds to the group of TAB connectors which are responsible for receiving a cell when the BS type 1-H setting corresponding to the declared minimum number of cells with reception on all TAB connectors supporting an operating band, but its existence is not limited to that condition

TAB connector TX min cell group: operating band specific declared group of TAB connectors to which BS type 1-H conducted TX requirements are applied.

NOTE:	Within this definition, the group corresponds to the group of TAB connectors which are responsible for transmitting a cell when the BS type 1-H setting corresponding to the declared minimum number of cells with transmission on all TAB connectors supporting an operating band, but its existence is not limited to that condition

total radiated power: is the total power radiated by the antenna

NOTE:	The total radiated power is the power radiating in all direction for two orthogonal polarizations.  Total radiated power is defined in both the near-field region and the far-field region

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transmission bandwidth: RF Bandwidth of an instantaneous transmission from a UE or BS, measured in resource block units

transmitter OFF period: time period during which the BS transmitter is not allowed to transmit

transmitter ON period: time period during which the BS transmitter is transmitting data and/or reference symbols

transmitter transient period: time period during which the transmitter is changing from the OFF period to the ON period or vice versa

UE transmission bandwidth configuration: set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE

upper sub-block edge: frequency at the upper edge of one sub-block.

NOTE:	It is used as a frequency reference point for both transmitter and receiver requirements.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BeWθ,REFSENS	Beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees. Applicable for FR1 only.

BeWφ,REFSENS	Beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees. Applicable for FR1 only.

BWChannel	BS channel bandwidth

BWChannel_CA	Aggregated BS Channel Bandwidth, expressed in MHz. BWChannel_CA = Fedge,high- Fedge,low.

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block = Fedge,block,high- Fedge,block,low.

BWConfig	Transmission bandwidth configuration, where BWConfig = NRB x SCS x 12

BWContiguous	Contiguous transmission bandwidth, i.e. BS channel bandwidth for single carrier or Aggregated BS channel bandwidth for contiguously aggregated carriers. For non-contiguous operation within a band the term is applied per sub-block.

BWGB,low	The minimum guard band defined in clause 5.3.3 for lowest assigned component carrier

BWGB,high	The minimum guard band defined in clause 5.3.3 for highest assigned component carrier

f	Separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

ΔfBE_offset	Separation between the edge of the last transmitted channel of the channels assigned for NR-U channel bandwidth and the nominal -3 dB point of the measuring filter closest to the carrier frequency

ΔFGlobal	Global frequency raster granularity

fmax	f_offsetmax minus half of the bandwidth of the measuring filter

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

ΔFR2_REFSENS	Offset applied to the FR2 OTA REFSENS depending on the AoA

ΔminSENS	Difference between conducted reference sensitivity and minSENS

ΔOTAREFSENS	Difference between conducted reference sensitivity and OTA REFSENS

ΔFRaster	Channel raster granularity

Δshift	Channel raster offset for SUL

EISminSENS	The EIS declared for the minSENS RoAoA

EISREFSENS	OTA REFSENS EIS value

EISREFSENS_50M	Declared OTA reference sensitivity basis level for FR2 based on a reference measurement channel with 50MHz BS channel bandwidth

FFBWhigh	Highest supported frequency within supported operating band, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported operating band, for which fractional bandwidth support was declared

FC	RF reference frequency on the channel raster, given in table 5.4.2.2-1

FC,block,high	Fc of the highest transmitted/received carrier in a sub-block.

FC,block,low	Fc of the lowest transmitted/received carrier in a sub-block.

FC,low	The Fc of the lowest carrier, expressed in MHz.

FC,high	The Fc of the highest carrier, expressed in MHz.

FC,SBFD,UL	The center frequency for SBFD UL subband

FC,SBFD,DL	The center frequency for SBFD DL subband in DU or UD case

FC,SBFD,DL,1	The center frequency for the lower assigned SBFD DL subband in DUD case

FC,SBFD,DL,2	The center frequency for the upper assigned SBFD DL subband in DUD case

FDL,low	The lowest frequency of the downlink operating band

FDL,high	The highest frequency of the downlink operating band

Fedge,low	The lower edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low.

Fedge,high	The upper edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high.

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset,low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset,high.

Ffilter	Filter centre frequency

Foffset,high	Frequency offset from FC,high to the upper Base Station RF Bandwidth edge, or from F C,block,high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower Base Station RF Bandwidth edge, or from FC,block,low to the lower sub-block edge.

f_BE_offset	Separation between the edge of the last transmitted channel of the channels assigned for NR-U channel bandwidth and the centre of the measuring

f_offset	Separation between the channel edge frequency and the centre of the measuring

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF,shift	RF reference frequency for Supplementary Uplink (SUL) bands

Fstep,X	Frequency steps for the OTA transmitter spurious emissions (Category B)

FUL,low	The lowest frequency of the uplink operating band

FUL,high	The highest frequency of the uplink operating band

GBChannel	Minimum guard band defined in clause 5.3.3

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an BS type 1-H in a particular operating band

Physical resource block number

NRB	Transmission bandwidth configuration, expressed in resource blocks

NRB,high	Transmission bandwidth configuration for the highest assigned component carrier within a sub-block in CA

NRB,low	Transmission bandwidth configuration for the lowest assigned component carrier within a sub-block in CA

NRB,SBFD	Transmission bandwidth configuration for SBFD carrier, the set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the BS, expressed in resource blocks.

NRB,SBFD,DL	Transmission bandwidth configuration for SBFD DL subband in SBFD DU or UD case, expressed in resource blocks.

NRB,SBFD,DL,1	Transmission bandwidth configuration for the lower assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

NRB,SBFD,DL,2	Transmission bandwidth configuration for the higher assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

NRB,SBFD,UL	Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks.

NRB,SBFD,GB	The size of guardband between SBFD UL subband and DL subband in SBFD DU, UD and DUD case, expressed in resource blocks.

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

NRXU,active	The number of active receiver units. The same as the number of demodulation branches to which compliance is declared for chapter 8 performance requirements

NRXU,counted	The number of active receiver units that are taken into account for conducted Rx spurious emission scaling, as calculated in clause 7.6.1

NRXU,countedpercell	The number of active receiver units that are taken into account for conducted RX spurious emissions scaling per cell, as calculated in clause 7.6.1

NTXU,counted	The number of active transmitter units as calculated in clause 6.1, that are taken into account for conducted TX output power limit in clause 6.2.1, and for unwanted TX emissions scaling

NTXU,countedpercell	The number of active transmitter units that are taken into account for conducted TX emissions scaling per cell, as calculated in clause 6.1

PEM,n50/n75,ind	Declared emission level for Band n50/n75; ind = a, b

PEM,n54,ind	Declared emission level for Band n54; ind = a, b, c, d, e, f

PEIRP,N	EIRP level for channel N

Pmax,c,AC	Maximum carrier output power measured per antenna connector

Pmax,c,cell	The maximum carrier output power per TAB connector TX min cell group

Pmax,c,TABC	The maximum carrier output power per TAB connector

Pmax,c,TRP	Maximum carrier TRP output power measured at the RIB(s), and corresponding to the declared rated carrier TRP output power (Prated,c,TRP)

Pmax,c,EIRP	The maximum carrier EIRP when the NR BS is configured at the maximum rated carrier output TRP (Prated,c,TRP)

Prated,c,AC	The rated carrier output power per antenna connector

Prated,c,cell	The rated carrier output power per TAB connector TX min cell group

Prated,c,FBWhigh	The rated carrier EIRP for the higher supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,c,FBWlow	The rated carrier EIRP for the lower supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,c,sys	The sum of Prated,c,TABC for all TAB connectors for a single carrier

Prated,c,TABC	The rated carrier output power per TAB connector

Prated,c,TRP	Rated carrier TRP output power declared per RIB

Prated,t,AC	The rated total output power declared at the antenna connector

Prated,t,TABC	The rated total output power declared at TAB connector

Prated,t,TRP	Rated total TRP output power declared per RIB

PREFSENS	Conducted Reference Sensitivity power level

SCSlow	Sub-Carrier Spacing for the lowest assigned component carrier within a sub-block in CA

SCShigh	Sub-Carrier Spacing for the highest assigned component carrier within a sub-block in CA

SSREF	SS block reference frequency position

Wgap	Sub-block gap or Inter RF Bandwidth gap size

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AA	Antenna Array

AAS	Active Antenna System

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AoA	Angle of Arrival

ATG	Air-To-Ground

AWGN	Additive White Gaussian Noise

BS	Base Station

BW	Bandwidth

CA	Carrier Aggregation

CACLR	Cumulative ACLR

CPE	Common Phase Error

CP-OFDM	Cyclic Prefix-OFDM

CW	Continuous Wave

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DM-RS	Demodulation Reference Signal

EIS	Equivalent Isotropic Sensitivity

EIRP	Effective Isotropic Radiated Power

EEIRP	Expected EIRP

E-UTRA	Evolved UTRA

EVM	Error Vector Magnitude

FBW	Fractional Bandwidth

FR	Frequency Range

FRC	Fixed Reference Channel

FSS	Fixed Satellite Service

FRMCS	Future Railway Mobile Communication System

GSCN	Global Synchronization Channel Number

GSM	Global System for Mobile communications

HAPS	High Altitude Platform Station

ITU-R	Radiocommunication Sector of the International Telecommunication Union

ICS	In-Channel Selectivity

LA	Local Area

LNA	Low Noise Amplifier

MCS	Modulation and Coding Scheme

MR	Medium Range

NB-IoT	Narrowband – Internet of Things

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

OBUE	Operating Band Unwanted Emissions

OCC	Orthogonal Covering Code

OOB	Out-of-band

OSDD	OTA Sensitivity Directions Declaration

OTA	Over-The-Air

PRB	Physical Resource Block

PT-RS	Phase Tracking Reference Signal

QAM	Quadrature Amplitude Modulation

RB	Resource Block

RDN	Radio Distribution Network

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

RIB	Radiated Interface Boundary

RMR	Railway Mobile Radio

RMS	Root Mean Square (value)

RoAoA	Range of Angles of Arrival

QAM	Quadrature Amplitude Modulation

RB	Resource Block

RX	Receiver

SBFD	Subband non-overlapping Full Duplex

SCS	Sub-Carrier Spacing

SDL	Supplementary Downlink

SS	Synchronization Symbol

SSB	Synchronization Signal Block

SUL	Supplementary Uplink

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TDL	Tapped Delay Line

TX	Transmitter

TRP	Total Radiated Power

UCI	Uplink Control Information

UEM	Unwanted Emissions Mask

UTRA	Universal Terrestrial Radio Access

WA	Wide Area

ZF	Zero Forcing

## << End of change 1 >>

## << Start of change 2 >>

5	Operating bands and channel arrangement

5.1	General

The channel arrangements presented in this clause are based on the operating bands and BS channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and BS channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to the present version of the specification are identified as described in table 5.1-1. Whenever FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be applied, unless otherwise stated.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

5.2	Operating bands

NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

NR operating band n1, n2, n3, n5, n7, n8, n20, n25, n26, n28, n34, n38, n39, n41, n67, n85 and n90 which are defined in Table 5.2-1, can be applied for HAPS operation.

NOTE:	For HAPS operation, the UL and DL frequency ranges are identifed in Nos. 5.312B, 5.314A, 5.388A and 5.409A and under the conditions specified in Resolutions 213, 218 and 221 in the ITU Radio Regulations [28].

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31,  n41, n65, n66, n70, n71, n72, n74, n85, n87, n88, n90, n106 which are defined in Table 5.2-1.

Subband non-overlapping full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 which are defined in Table 5.2-1 and NR operating bands in FR2-1 which are defined in Table 5.2-2 except bands n34, n46, n51, n53, n54, n96, n101 and n102 and band n263.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Notes |
| --- | --- | --- | --- | --- |
| n1 | 1920 – 1980 | 2110 – 2170 | FDD |  |
| n2 | 1850 – 1910 | 1930 – 1990 | FDD |  |
| n3 | 1710 – 1785 | 1805 – 1880 | FDD |  |
| n5 | 824 – 849 | 869 – 894 | FDD |  |
| n7 | 2500 – 2570 | 2620 – 2690 | FDD |  |
| n8 | 880 – 915 | 925 – 960 | FDD |  |
| n12 | 699 – 716 | 729 – 746 | FDD |  |
| n13 | 777 – 787 | 746 – 756 | FDD |  |
| n14 | 788 – 798 | 758 – 768 | FDD |  |
| n18 | 815 – 830 | 860 – 875 | FDD |  |
| n20 | 832 – 862 | 791 – 821 | FDD |  |
| n24 | 1626.5 – 1660.5 | 1525 – 1559 | FDD | Note 7 |
| n25 | 1850 – 1915 | 1930 – 1995 | FDD |  |
| n26 | 814 – 849 | 859 – 894 | FDD |  |
| n28 | 703 – 748 | 758 – 803 | FDD |  |
| n29 | N/A | 717 – 728 | SDL |  |
| n30 | 2305 – 2315 | 2350 – 2360 | FDD |  |
| n31 | 452.5 – 457.5 | 462.5 – 467.5 | FDD |  |
| n34 | 2010 – 2025 | 2010 – 2025 | TDD | Note 12 |
| n38 | 2570 – 2620 | 2570 – 2620 | TDD |  |
| n39 | 1880 – 1920 | 1880 – 1920 | TDD |  |
| n40 | 2300 – 2400 | 2300 – 2400 | TDD |  |
| n41 | 2496 – 2690 | 2496 – 2690 | TDD |  |
| n46 | 5150 – 5925 | 5150 – 5925 | TDD | Note 3, Note 12 |
| n48 | 3550 – 3700 | 3550 – 3700 | TDD |  |
| n50 | 1432 – 1517 | 1432 – 1517 | TDD |  |
| n51 | 1427 – 1432 | 1427 – 1432 | TDD | Note 12 |
| n53 | 2483.5 – 2495 | 2483.5 – 2495 | TDD | Note 12 |
| n54 | 1670 – 1675 | 1670 – 1675 | TDD | Note 12 |
| n65 | 1920 – 2010 | 2110 – 2200 | FDD |  |
| n66 | 1710 – 1780 | 2110 – 2200 | FDD |  |
| n67 | N/A | 738 – 758 | SDL |  |
| n68 | 698 – 728 | 753 – 783 | FDD |  |
| n70 | 1695 – 1710 | 1995 – 2020 | FDD |  |
| n71 | 663 – 698 | 617 – 652 | FDD |  |
| n72 | 451 – 456 | 461 – 466 | FDD |  |
| n74 | 1427 – 1470 | 1475 – 1518 | FDD |  |
| n75 | N/A | 1432 – 1517 | SDL |  |
| n76 | N/A | 1427 – 1432 | SDL |  |
| n77 | 3300 – 4200 | 3300 – 4200 | TDD | Note 11 |
| n78 | 3300 – 3800 | 3300 – 3800 | TDD |  |
| n79 | 4400 – 5000 | 4400 – 5000 | TDD |  |
| n80 | 1710 – 1785 | N/A | SUL |  |
| n81 | 880 – 915 | N/A | SUL |  |
| n82 | 832 – 862 | N/A | SUL |  |
| n83 | 703 – 748 | N/A | SUL |  |
| n84 | 1920 – 1980 | N/A | SUL |  |
| n85 | 698 – 716 | 728 – 746 | FDD |  |
| n86 | 1710 – 1780 | N/A | SUL |  |
| n87 | 410 - 415 | 420 – 425 | FDD |  |
| n88 | 412 - 417 | 422 - 427 | FDD |  |
| n89 | 824 – 849 | N/A | SUL |  |
| n90 | 2496 – 2690 | 2496 – 2690 | TDD |  |
| n91 | 832 – 862 | 1427 – 1432 | FDD | Note 2 |
| n92 | 832 – 862 | 1432 – 1517 | FDD | Note 2 |
| n93 | 880 – 915 | 1427 – 1432 | FDD | Note 2 |
| n94 | 880 – 915 | 1432 – 1517 | FDD | Note 2 |
| n95 | 2010 – 2025 | N/A | SUL | Note 1 |
| n96 | 5925 – 7125 | 5925 – 7125 | TDD | Note 3, Note 4, Note 12 |
| n97 | 2300 – 2400 | N/A | SUL | Note 5 |
| n98 | 1880 – 1920 | N/A | SUL | Note 5 |
| n99 | 1626.5 – 1660.5 | N/A | SUL | Note 6 |
| n100 | 874.4 – 880 | 919.4 – 925 | FDD | Note 10 |
| n101 | 1900 – 1910 | 1900 – 1910 | TDD | Note 10, Note 12 |
| n102 | 5925 – 6425 | 5925 – 6425 | TDD | Note 3, Note 4, Note 12 |
| n104 | 6425 – 7125 | 6425 – 7125 | TDD | Note 8 |
| n105 | 663 – 703 | 612 – 652 | FDD |  |
| n106 | 896 – 901 | 935 – 940 | FDD |  |
| n109 | 703 – 733 | 1432 – 1517 | FDD | Note 2 |
| n110 | 1390 - 1395 | 1432 - 1435 | FDD |  |
| NOTE 1: This band is applicable in China only. NOTE 2: Variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. NOTE 3: This band is restricted to operation with shared spectrum channel access as defined in TS 37.213 [20]. NOTE 4: This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions. NOTE 5: The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 6: UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48 [25]. NOTE 7: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order 20-51 [24]. NOTE 8: This band is applicable only in countries/regions designating this band for IMT licensed operation subject to country-specific conditions. NOTE 9: Operating band n200 is a reserved value. NOTE 10: This band is applicable to CEPT countries subject to ECC Decision (20)02 [21], for the RMR application. NOTE 11: In the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. In Canada this band is restricted to 3450 – 3650 MHz and 3650 – 3980 MHz. NOTE 12: This TDD band is not applicable to SBFD operation. |  |  |  |  |

Table 5.2-2: NR operating bands in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high (MHz) | Duplex mode | Notes |
| --- | --- | --- | --- |
| n257 | 26500 – 29500 | TDD |  |
| n258 | 24250 – 27500 | TDD |  |
| n259 | 39500 – 43500 | TDD |  |
| n260 | 37000 – 40000 | TDD |  |
| n261 | 27500 – 28350 | TDD |  |
| n262 | 47200 – 48200 | TDD |  |
| n263 | 57000 – 71000 | TDD | Note 1 |
| NOTE 1: This TDD band is not applicable to SBFD operation. |  |  |  |

5.3	BS channel bandwidth

5.3.1	General

The BS channel bandwidth supports a single NR RF carrier in the uplink or downlink at the Base Station. Different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS. The placement of the UE channel bandwidth is flexible but can only be completely within the BS channel bandwidth. The BS shall be able to transmit to and/or receive from one or more UE bandwidth parts that are smaller than or equal to the number of carrier resource blocks on the RF carrier, in any part of the carrier resource blocks.

The relationship between the channel bandwidth, the guardband and the transmission bandwidth configuration is shown in figure 5.3.1-1.

.

Figure 5.3.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one NR channel

For SBFD-capable BS, the transmission bandwidth configurations for SBFD DL subband(s) and UL subbands are defined over the carrier configured for SBFD operation. The relationship between the channel bandwidth, the guardband and the transmission bandwidth configuration for SBFD DL subband(s) and UL subband is shown in figures 5.3.1-2, 5.3.1-3 and 5.3.1-4, respectively for SBFD DUD, DU and UD operations.

Figure 5.3.1-2: Definition of channel bandwidth and transmission bandwidth configuration for SBFD DUD operation

Figure 5.3.1-3: Definition of channel bandwidth and transmission bandwidth configuration for SBFD DU operation

Figure 5.3.1-4: Definition of channel bandwidth and transmission bandwidth configuration for SBFD UD operation

5.3.2	Transmission bandwidth configuration

The transmission bandwidth configuration NRB for each BS channel bandwidth and subcarrier spacing is specified in table 5.3.2.-1 for FR1 and table 5.3.2-2 for FR2.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |
| NOTE 1: In FR1 bands applicable to SBFD operation, the BS channel bandwidth for SBFD operation shall be no less than 40 MHz except for band n39 and no less than 20 MHz for band n39. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |
| Note: The BS channel bandwidth for SBFD operation shall be no less than 100 MHz. |  |  |  |  |

Table 5.3.2-3: Transmission bandwidth configuration NRB for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 264 | N/A | N/A | N/A |
| 480 | N/A | 66 | 124 | 248 | N/A |
| 960 | N/A | 33 | 62 | 124 | 148 |

NOTE:	All Tx and Rx requirements are defined based on transmission bandwidth configuration specified in table 5.3.2-1 for FR1 and table 5.3.2-2 and table 5.3.2-3 for FR2.

The transmission bandwidth configuration for NB-IoT is specified in TS 36.104 [13] clause 5.6.

For SBFD operation, the transmission bandwidth configuration for SBFD UL subband NRB,SBFD,UL shall be configured according to the values provided for BS transmission bandwidth configuration NRB corresponding to bandwidth no less than 10MHz in Table 5.3.2-1 for FR1, and NRB in Table 5.3.2-2 for FR2-1. Transmission bandwidth configuration for SBFD DL subband in SBFD (i.e., NRB,SBFD,DL for SBFD DU or UD operation, and NRB,SBFD,DL,1 and NRB,SBFD,DL,2 for SBFD DUD operation) and guardband between SBFD UL subband and DL subband shall be configured by SBFD-capable BS. For SBFD DUD operation, NRB,SBFD,DL,1 and NRB,SBFD,DL,2 shall have the same size or different sizes by only 1RB.

5.3.3	Minimum guardband and transmission bandwidth configuration

The minimum guardband for each BS channel bandwidth and SCS is specified in table 5.3.3-1 for FR1 and in table 5.3.3-2 and table 5.3.3-2a for FR2.

Table 5.3.3-1: Minimum guardband (kHz) (FR1)

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 142.5 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

Table 5.3.3-2: Minimum guardband (kHz) (FR2-1)

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N/A |
| 120 | 1900 | 2420 | 4900 | 9860 |

Table 5.3.3-2a: Minimum guardband (kHz) (FR2-2)

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
| 120 | 2420 | 9860 | N/A | N/A | N/A |
| 480 | N/A | 9680 | 42640 | 85520 | N/A |
| 960 | N/A | 9440 | [42400] | 85280 | 147040 |

The minimum guardband of SCS 240 kHz SS/PBCH block for each BS channel bandwidth is specified in table 5.3.3-3 for FR2.

Table 5.3.3-3: Minimum guardband (kHz) of SCS 240 kHz SS/PBCH block (FR2)

| SCS (kHz) | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- |
| 240 | 3800 | 7720 | 15560 |

NOTE:	The minimum guardband in Table 5.3.3-3 is applicable only when the SCS 240 kHz SS/PBCH block is placed adjacent to the edge of the BS channel bandwidth within which the SS/PBCH block is located.

The number of RBs configured in any BS channel bandwidth shall ensure that the minimum guardband specified in this clause is met.

Figure 5.3.3-1: BS PRB utilization

In the case that multiple numerologies are multiplexed in the same symbol, the minimum guardband on each side of the carrier is the guardband applied at the configured BS channel bandwidth for the numerology that is transmitted/received immediately adjacent to the guard band.

For FR1, if multiple numerologies are multiplexed in the same symbol and the BS channel bandwidth is >50 MHz, the guardband applied adjacent to 15 kHz SCS shall be the same as the guardband defined for 30 kHz SCS for the same BS channel bandwidth.

For FR2, if multiple numerologies are multiplexed in the same symbol and the BS channel bandwidth is >200 MHz, the guardband applied adjacent to 60 kHz SCS shall be the same as the guardband defined for 120 kHz SCS for the same BS channel bandwidth.

Figure 5.3.3-2: Guard band definition when transmitting multiple numerologies

NOTE:	Figure 5.3.3-2 is not intended to imply the size of any guard between the two numerologies. Inter-numerology guard band within the carrier is implementation dependent.

Figure 5.3.3-3: Void

Figure 5.3.3-4: Void

Figure 5.3.3-5: Void

5.3.4	RB alignment

For each BS channel bandwidth and each numerology, BS transmission bandwidth configuration must fulfil the minimum guardband requirement specified in clause 5.3.3.

For each numerology, its common resource blocks are specified in clause 4.4.4.3 in [9], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology.

For each numerology, all UE transmission bandwidth configurations indicated to UEs served by the BS by higher layer parameter carrierBandwidth defined in TS 38.331 [11] shall fall within the BS transmission bandwidth configuration.

5.3.5	BS channel bandwidth per operating band

The requirements in this specification apply to the combination of BS channel bandwidths, SCS and operating bands shown in table 5.3.5-1 for FR1 and in table 5.3.5-2 and table 5.3.5-3 for FR2. The transmission bandwidth configuration in table 5.3.2-1, table 5.3.2-2 and table 5.3.2-3 shall be supported for each of the BS channel bandwidths within the BS capability. The BS channel bandwidths are specified for both the Tx and Rx path.

Table 5.3.5-1: BS channel bandwidths and SCS per operating band in FR1

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
| n1 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
| n2 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
| n3 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  | 15 | 3 | 5 | 10 | 15 | 20 | 257 |  |  |  |  |  |  |  |  |  |  |  |
| n5 | 30 |  |  | 10 | 15 | 20 | 257 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  | 50 |  |  |  |  |  |  |
| n7 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  | 50 |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  | 50 |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 257 | 307 | 357 |  |  |  |  |  |  |  |  |  |
| n8 | 30 |  |  | 10 | 15 | 20 | 257 | 307 | 357 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 30 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n13 | 15 |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 30 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n18 | 30 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 30 |  |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 30 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
| n25 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
| n26 | 15 | 3 | 5 | 10 | 15 | 20 | 257 | 307 |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 257 | 307 |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 3 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |  |
| n28 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n29 | 30 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 30 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n31 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 30 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |  |
| n38 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
| n39 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  | 15 |  | 54 | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n40 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  | 58 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
| n41 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  |  | 106 |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  |  |
| n46 | 30 |  |  | 106 |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 100 |  |
|  | 60 |  |  | 106 |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 100 |  |
|  | 15 |  | 52 | 10 | 15 | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n48 | 30 |  |  | 10 | 15 | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 | 15 | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  | 52 | 10 | 15 | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n50 | 30 |  |  | 10 | 15 | 20 |  | 30 |  | 40 |  | 50 | 60 |  | 80 |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 |  | 30 |  | 40 |  | 50 | 60 |  | 80 |  |  |  |
|  | 15 |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n51 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n53 | 30 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n54 | 15 |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 |  |  |  |  |  | 50 |  |  |  |  |  |  |
| n65 | 30 |  |  | 10 | 15 | 20 |  |  |  |  |  | 50 |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 |  |  |  |  |  | 50 |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
| n66 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| n67 | 30 |  |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n68 | 30 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |  |  |
| n70 | 30 |  |  | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 |  |  |  |  |  |  |  |  |  |
| n71 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n72 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 30 |  |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n75 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  | 15 |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n76 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n77 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n78 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n79 | 30 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
| n80 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| n81 | 30 |  |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| n82 | 30 |  |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |  |
| n83 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
| n84 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  | 15 | 3 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n85 | 30 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |  |
| n86 | 30 |  |  | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |  |
|  | 15 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n87 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n88 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| n89 | 30 |  |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
| n90 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  | 5 | 103 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n91 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| n92 | 30 |  |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 103 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n93 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| n94 | 30 |  |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n95 | 30 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  |  |
| n96 | 30 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 100 |  |
|  | 60 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 100 |  |
| n97 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
| n98 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
| n99 | 15 |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n100 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n101 | 15 |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  |  |
| n102 | 30 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 100 |  |
|  | 60 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 100 |  |
|  | 15 |  |  |  |  | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n104 | 30 |  |  |  |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  |  |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 257 | 307 | 357 |  |  |  |  |  |  |  |  |  |
| n105 | 30 |  |  | 10 | 15 | 20 | 257 | 307 | 357 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n106 | 15 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n109 | 15 | 5 | 5 | 10 | 15 | 20 | 25 | 30 |  | 407 |  | 507 |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 407 |  | 507 |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n110 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. NOTE 2: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 3: For this bandwidth, it only applies for UL transmission. NOTE 4: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 5: Void. NOTE 6: This bandwidth can only be applied in certain regions where the absence of non 3GPP technologies can be guaranteed on a long term basis in this version of specification. NOTE 7: For this bandwidth, it only applies for DL transmission. NOTE 8: Not all frequency positions of 5 MHz carriers are possible due limitations of the SSB position relative to the 5 MHz channels. 5 MHz channels with Fc such that 2499+N*1.2 ≤Fc<2499.3+N*1.2MHz for 0≤N<157 are not compatible with SSB positions and cannot be used for 5 MHz n41. NOTE 9: In FR1 bands applicable to SBFD operation, the BS channel bandwidth for SBFD operation shall be no less than 40 MHz except for band n39 and no less than 20 MHz for band n39. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.5-2: BS channel bandwidths and SCS per operating band in FR2-1

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n257 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n258 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n259 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n260 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n261 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n262 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| NOTE 1: In FR2-1 bands applicable to SBFD operation, the BS channel bandwidth for SBFD operation shall be no less than 100 MHz. |  |  |  |  |  |

Table 5.3.5-3: BS channel bandwidths and SCS per operating band in FR2-2

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 100 | 400 | 800 | 1600 | 2000 |
| n263 | 120 | 100 | 400 |  |  |  |
|  | 480 |  | 400 | 800 | 1600 |  |
|  | 960 |  | 400 | 800 | 1600 | 2000 |

## << End of change 2 >>

## << Start of change 3 >>

12	Requirements for SBFD-capable BS

12.1	General

12.1.1	Applicability of requirements for SBFD-capable BS

In table 12.1.1-1, the requirement applicability for each requirement set is defined for SBFD-capable BS. For each requirement, the applicable requirement clause in the specification is identified. Requirements not included in a requirement set is marked not applicable (NA).

Table 12.1.1-1: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols/slots |  |  | Requirement Set for SBFD symbols/slots |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| BS output power | 6.2 |  |  | 12.2.2 |  |  |
| Output power dynamics | 6.3 |  |  | 12.2.3 |  |  |
| Transmit ON/OFF power | 6.4 |  |  | 12.2.4 |  |  |
| Transmitted signal quality | 6.5 |  |  | 12.2.5 |  |  |
| Occupied bandwidth | 6.6.2 |  |  | 12.2.6 |  |  |
| ACLR | 6.6.3 |  |  | 12.2.6 |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  | 12.2.6 |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  | 12.2.6 |  |  |
| Transmitter intermodulation | 6.7 | NA | NA | 12.2.7 | NA | NA |
| Transient period between SBFD and non-SBFD | NA |  |  | 12.2.8 |  |  |
| In-channel adjacent subband leakage | NA |  |  | 12.2.9 |  |  |
| Reference sensitivity level | 7.2 |  |  | 12.3.2 |  |  |
| Dynamic range | 7.3 |  |  | 12.3.3 |  |  |
| In-band selectivity and blocking | 7.4 |  |  | 12.3.4 |  |  |
| Out-of-band blocking | 7.5 |  |  | 12.3.5 |  |  |
| Receiver spurious emissions | 7.6 |  |  | 12.3.6 |  |  |
| Receiver intermodulation | 7.7 |  |  | N/A |  |  |
| In-channel selectivity | 7.8 |  |  | 12.3.7 |  |  |
| In-channel adjacent subband blocking | NA |  |  | 12.3.8 |  |  |
| Performance requirements | 8 |  |  | 12.4 |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | 12.5.2 | 12.5.2 | 12.5.2 |
| OTA base station output power |  | 9.3 | 9.3 |  | 12.5.3 | 12.5.3 |
| OTA output power dynamics |  | 9.4 | 9.4 |  | 12.5.4 | 12.5.4 |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |  | 12.5.5 | 12.5.5 |
| OTA transmitted signal quality |  | 9.6 | 9.6 |  | 12.5.6 | 12.5.6 |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |  | 12.5.7 | 12.5.7 |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | NA | 12.5.7 | 12.5.7 |
| OTA operating band unwanted emission |  | 9.7.4 | 9.7.4 |  | 12.5.7 | 12.5.7 |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |  | 12.5.7 | 12.5.7 |
| OTA transmitter intermodulation |  | 9.8 | NA |  | 12.5.8 | NA |
| OTA spatial emission |  | 9.9 | NA |  | 12.5.9 | NA |
| OTA transient period between SBFD and non-SBFD |  | NA | NA |  | 12.5.10 | 12.5.10 |
| OTA in-channel adjacent subband leakage |  | NA | NA |  | 12.5.11 | 12.5.11 |
| OTA sensitivity | 10.2 | 10.2 | NA | 12.6.2 | 12.6.2 | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |  | 12.6.3 | 12.6.3 |
| OTA dynamic range |  | 10.4 | NA |  | 12.6.4 | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |  | 12.6.5 | 12.6.5 |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | NA | 12.6.6 | 12.6.6 |
| OTA receiver spurious emission |  | 10.7 | 10.7 |  | 12.6.7 | 12.6.7 |
| OTA receiver intermodulation |  | 10.8 | 10.8 |  | N/A | N/A |
| OTA in-channel selectivity |  | 10.9 | 10.9 |  | 12.6.8 | 12.6.8 |
| OTA in-channel adjacent subband blocking |  | NA | NA |  | 12.6.9 | 12.6.9 |
| Radiated performance requirements |  | 11 | 11 |  | 12.7 | 12.7 |

### 12.1.2	General SBFD-capable BS Operating Requirements

An SBFD-capable BS shall operate only SBFD subband configurations (i.e., DL/UL subbands) that have been declared in accordance with the provisions in this specification.

The RF requirements for SBFD-capable BS have been derived based on the simulation assumption that only SBFD-capable BSs are deployed with the identical SBFD configuration under each BS class in the same geographical area.

### 12.1.3	SBFD-capable BS Types and Configuration Constraints

#### 12.1.3.1 General

SBFD-capable BSs are categorized as either configuration-limited or configuration-flexible according to the classification defined in Table 12.1.3-1. The classification is based on BS class and rated output power specified in clause 12.5.2.

Table 12.1.3.1-1: SBFD-capable BS classification

| BS Class | Rated Output Power (Prated,c,sys and Prated,c,TRP ) | Sub-band configuration |
| --- | --- | --- |
| FR1 Wide Area BS | -- | Configuration-limited |
| FR1 Medium Range BS | ≥ 36 dBm | Configuration-limited |
| FR1 Medium Range BS | < 36 dBm | Configuration-flexible |
| FR1 Local Area BS | -- | Configuration-flexible |
| FR2 Wide Area BS | -- | Configuration-flexible |
| FR2 Local Area BS | -- | Configuration-flexible |
| NOTE 1: A power level threshold of Prated,c,sys = 36 dBm is used to distinguish between configuration-limited and configuration-flexible SBFD-capable BSs in the FR1 FR1 Medium Range BS class. |  |  |

#### 12.1.3.2 Configuration-limited SBFD-capable BS

For configuration-limited SBFD-capable BSs, the following requirements apply:

For each channel bandwidth supported for SBFD operation, a maximum of three SBFD subband configurations may be declared. The three configurations shall cover all supported SBFD patterns, including both DUD and DU/UD cases. A single UL subband size used in both DU and UD configurations shall be counted as two configurations. For example, a UD 20-80 MHz and a DU 80-20 MHz configuration shall be counted as two SBFD subband configurations.

For each supported channel bandwidth and for each supported UL subband size: only one DL subband size may be declared and tested for the DU/UD pattern and only one DL subband size or two DL subband sizes with 1PRB difference may be declared and tested for the DUD pattern.

A configuration-limited SBFD-capable BS shall not operate in any declared SBFD configuration unless it has been tested.

#### 12.1.3.3 Configuration-flexible SBFD-capable BS

For configuration-flexible SBFD-capable BSs, the following requirements apply:

There is no restriction on the number of UL subband sizes that may be declared.

There is no restriction on the number of DL subband sizes that may be declared.

For each declared channel bandwidth and for each declared SBFD pattern, the following configurations shall be tested:

- The configuration with the narrowest UL subband size, and the corresponding worst-case SBFD DL transmission bandwidth:

  - For DU/UD: the configuration with the largest NRB,SBFD,DL.

  - For DUD: the configuration with the largest NRB,SBFD,DL,1/2.

- The configuration with the widest UL subband size, and the corresponding worst-case SBFD DL transmission bandwidth:

  - For DU/UD: the configuration with the largest NRB,SBFD,DL.

  - For DUD: the configuration with the largest NRB,SBFD,DL,1/2.

12.2 Conducted transmitter characteristics for SBFD-capable BS

12.2.1	General

Conducted transmitter characteristics requirements apply on the SBFD-capable BS type 1-H including all its functional components active for SBFD operation.

12.2.2	Base station output power for SBFD

12.2.2.1	General

The BS conducted output power requirement for SBFD operation is specified over SBFD symbol(s), in which the rated carrier output power for SBFD symbol(s) may be different from the rated carrier output powers over non-SBFD symbol(s) for transmitter ON period as specified in clause 6.2.1.

12.2.2.2	Minimum requirement for BS type 1-H

The same requirement as clause 6.2.3 is applicable to BS output power requirement for SBFD operation over SBFD symbol(s).

12.2.3	Output power dynamics for SBFD

12.2.3.1	General

The requirements in clause 12.2.3 apply during SBFD symbol(s). Transmitted signal quality as specified in clause 12.2.5 shall be maintained for the output power dynamics requirements of this clause.

12.2.3.2		RE power control dynamic range

12.2.3.2.1	General

The RE power control dynamic range for SBFD operation is the difference between the power of an RE and the average RE power for a BS at maximum output power ( Pmax,c,TABC) during SBFD symbol(s) for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

12.2.3.2.2	Minimum requirement for BS type 1-H

The same requirement as clause 6.3.2.2 is applicable to RE power control dynamic range requirement for SBFD operation.

12.2.3.3	Total power dynamic range for SBFD

12.2.3.3.1	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

NOTE 1:	The upper limit of the total power dynamic range for SBFD is the OFDM symbol power for a BS when transmitting on all DL RBs in transmission bandwidth configuration for SBFD at maximum output power. The lower limit of the total power dynamic range is the average power for single RB transmission in transmission bandwidth configuration for SBFD. The OFDM symbol shall carry PDSCH and not contain RS or SSB.

NOTE 2:	The requirement does not apply to operation with shared spectrum channel access.

12.2.3.3.2	Minimum requirement for BS type 1-H

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log10(NRB,SBFD,DL) if one SBFD DL subband is configured, i.e., DU or UD pattern, and as 10log10(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured, i.e., DUD pattern.

12.2.4	Transmit ON/OFF power for SBFD

12.2.4.1	Transmitter OFF power for SBFD

12.2.4.1.1		General

The general description for transmitter OFF power as clause 6.4 shall be applied.

12.2.4.1.2		Minimum requirement for BS type 1-H

For the transmitter OFF power requirements for non-SBFD symbols, the same requirement as specified in clause 6.4 shall be applied. Requirement is not applicable for SBFD symbol(s).

12.2.5	Transmitted signal quality for SBFD

12.2.5.1	General

The general description for transmitted signal quality as clause 6.5 shall be applied.

12.2.5.2		Minimum requirement for BS type 1-H

For frequency error, modulation quality (EVM) and time alignment error (TAE), the same requirements as specified in clause 6.5 shall be applied.

12.2.6	Unwanted emissions for SBFD

12.2.6.1	General

The general description for unwanted emissions as clause 6.6 shall be applied.

12.2.6.2	Minimum requirement for BS type 1-H

For occupied bandwidth, adjacent channel leakage power ratio (ACLR), operating band unwanted emissions (OBUE), and transmitter spurious emissions, the same requirements as specified in clause 6.6 shall be applied.

Requirements related with multi-band and non-contiguous spectrum are not applied to SBFD-capable BS.

12.2.7	Transmitter intermodulation for SBFD

12.2.7.1	General

The general description for transmitted signal intermodulation as clause 6.7 shall be applied.

12.2.7.2	Minimum requirements for BS type 1-H

For the transmitter intermodulation requirement, the same requirement as specified in clause 6.7 shall be applied. Receiver requirement is not specified during the transmitter intermodulation test.

12.2.8	Transient period between SBFD and non-SBFD operation

12.2.8.1	General

Transmitter transient period between SBFD and non-SBFD operation requirements apply only to SBFD-capable BS.

The transmitter transient period between SBFD and non-SBFD operation is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period between SBFD and non-SBFD operation is illustrated in figure 12.2.8.1-1.

Transmitter output power frequency regions for transition

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 12.2.8.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period between SBFD and non-SBFD operation

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases:

Table 12.2.8.1-1: Transmitter transient period between SBFD and non-SBFD operation

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| SBFD symbol to DL symbol | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| SBFD symbol to UL symbol | SBFD DL subband | BS transmitter ON-to-OFF |
| DL symbol to SBFD symbol | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| UL symbol to SBFD symbol | SBFD DL subband | BS transmitter OFF-to-ON |

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

12.2.8.2	Minimum requirement for BS type 1-H

For BS type 1-H, the transmitter transient period between SBFD and non-SBFD operation shall be shorter than the values listed in the minimum requirement table 12.2.8.2-1.

Table 12.2.8.2-1: Minimum requirement for the transmitter transient period between SBFD and non-SBFD operation for BS type 1-H

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

12.2.9	In-channel adjacent subband leakage for SBFD

12.2.9.1	General

In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of SBFD UL subband within NR carrier.

The requirement shall apply during the transmitter ON period.

12.2.9.2	Limits and Basic limits

For SBFD operation in the unpaired spectrum, the ICASL shall be less than or equal to the value specified in table 12.2.9.2-1.

Table 12.2.9.2-1: SBFD Base station ICASL absolute basic limit

| SBFD BS category / BS class | ICASL absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

Table 12.2.9.2-2: Filter parameters for the UL subband

| RAT | Filter on the assigned UL subband and corresponding filter bandwidth |
| --- | --- |
| NR | Square (the declared bandwidth of UL subband) |

12.2.9.3	Minimum requirement for SBFD BS type 1-H

The ICASL requirements are that for each TAB connector TX min cell group and each applicable basic limit in clause 12.2.9.2, the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed a BS limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell).

NOTE:	Conformance to the in-channel adjacent subband leakage requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the in-channel adjacent subband leakage power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The  in-channel adjacent subband leakage power at each TAB connector shall be less than or equal to the limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

12.3	Conducted receiver characteristics for SBFD-capable BS

12.3.1	General

Conducted receiver characteristics are specified at the TAB connector for SBFD-capable BS type 1-H, with full complement of transceivers configured for SBFD operation in normal operating condition.

12.3.2	Reference sensitivity level

12.3.2.1	General

The reference sensitivity power level is the minimum mean power received at the TAB connector for BS type 1-H inside the received SBFD UL subband for SBFD operation at which a throughput requirement shall be met for a specified reference measurement channel.

12.3.2.1	Minimum requirements for BS type 1-H

The same requirements as clause 7.2.2 is applicable to reference sensitivity power level for SBFD operation, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

12.3.3	Dynamic range

#### 12.3.3.1	General

The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal at the TAB connector for BS type 1-H inside the received SBFD UL subband for SBFD operation. In this condition, a throughput requirement shall be met for a specified reference measurement channel. The interfering signal for the dynamic range requirement is an AWGN signal.

#### 12.3.3.2	Minimum requirement for BS type 1-H

The same requirements as clause 7.3.2 is applicable to dynamic range requirement for SBFD operation, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

12.3.4	In-band selectivity and blocking for SBFD

#### 12.3.4.1	Adjacent Channel Selectivity (ACS) for SBFD

12.3.4.1.1	General

Adjacent channel selectivity (ACS) is a measure of the receiver’s ability to receive a wanted signal at its assigned channel frequency at the TAB connector for BS type 1-H in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

NOTE: SBFD UL subband is not configured adjacent to the Radio Bandwidth edge where cross link interference from adjacent channel may appear.

12.3.4.1.2	Minimum requirement for SBFD capable BS type 1-H

For SBFD operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS type 1-H TAB connector using the parameters in table 12.3.4.1.2-1 for ACS. The reference measurement channel for the wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

Table 12.3.4.1.2-1: Base station ACS requirement for SBFD

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | PREFSENS + 6 dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -35 | ±10 | 20 MHz CP-OFDM NR signal 30 kHz SCS, 51 RBs |
| NOTE 1: PREFSENS for SBFD UL subband bandwidth depends on the BS channel bandwidth as specified in clause 12.3.2. NOTE 2: For lowest/highest carrier is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. |  |  |  |  |

#### 12.3.4.2	In-band blocking for SBFD

12.3.4.2.1	General

The in-band blocking characteristics is a measure of the receiver's ability to receive a wanted signal at its assigned channel at the TAB connector for BS type 1-H in the presence of an unwanted interferer, which is an NR signal for general blocking.

NOTE 1: For FR1 Wide Area BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz with 288 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements.

NOTE 2: For FR1 Medium Range BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz outdoor deployment with 167 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements or in the context of indoor.

NOTE 3: SBFD UL subband is not configured adjacent to the Radio Bandwidth edge where cross link interference from adjacent channel may appear.

12.3.4.2.2	Minimum requirement for SBFD capable BS type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS type 1-H TAB connector using the parameters in table 12.3.4.2.2-1 for general blocking for SBFD. The reference measurement channel for the wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

Minimum conducted requirement is defined at the TAB connector for BS type 1-H.

Table 12.3.4.2.2-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | PREFSENS + 6 dB | Wide Area BS: -25 Medium Range BS: -32 Local Area BS: -35 | ±30 | 20 MHz CP-OFDM NR signal 30 kHz SCS, 51 RBs |
| NOTE 1: PREFSENS for SBFD UL subband bandwidth depends on the BS channel bandwidth as specified in clause 12.3.2. NOTE 2: For lowest/highest carrier is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. |  |  |  |  |

12.3.5	Out-of-band blocking for SBFD capable BS

12.3.5.1 General

The general requirement of out-of-band blocking for SBFD capable BS is the same as general requirement specified in clause 7.5.1.

12.3.5.2 Minimum requirement for SBFD capable BS type 1-H

The minimum requirements are the same as those for the BS type 1-H specified in clause 7.5.2, and the PREFSENS depends on the SBFD UL subband as specified in clause 12.3.2.12.3.5.3 Co-location minimum requirements for SBFD capable BS type 1-H

The co-location minimum requirements are the same as those for the BS type 1-H specified in clause 7.5.3, and the PREFSENS depends on the SBFD UL subband as specified in clause 12.3.2.

12.3.6	Receiver spurious emissions

12.3.6.1	General

The general description given in Clause 7.6.1 for BS type 1-H shall be applied.

12.3.6.2	Minimum requirement for SBFD-capable BS type 1-H

The same requirements as specified in Clause 7.6.4 shall be applied.

12.3.7	In-channel selectivity for SBFD

The same requirements as specified in clause 7.8 shall be applied to SBFD-capable BS type 1-H during SBFD symbol(s), except for the note in table 7.8.2-1, table 7.8.2-1b,  table 7.8.2-2, table 7.8.2-2b, table 7.8.2-2d, table 7.8.2-3 and table 7.8.2-3d, which shall be adjusted to: Wanted and interfering signal are placed adjacently around FC,SBFD,UL. The aggregated wanted and interferer signal shall be centred in the SBFD UL subband bandwidth of the wanted signal. For DU/UD case, place the wanted signal closer to the DL subband.

12.3.8	In-channel adjacent subband blocking for SBFD

12.3.8.1	General

The in-channel adjacent subband blocking requirement is a measure of the receiver’s ability to receive a wanted signal at its assigned channel frequency at the TAB connector for SBFD-capable BS type 1-H in the presence of an interfering signal from other sector(s) and/or site(s) within the same operator’s network, which is an NR signal.

12.3.8.2	Minimum requirement for SBFD-capable BS type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal and an interfering signal coupled to the SBFD-capable BS type 1-H TAB connector using the parameters specified in table 12.3.8.2-1. The reference measurement channel for the wanted signal is identified in table 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each SBFD UL subband bandwidth and are further specified in annex A.1. The characteristic of the interfering signal is further specified in annex D.

The in-channel adjacent subband blocking requirement shall apply to SBFD UL subband bandwidth. The interfering signal(s) shall be positioned at the centre frequency of the SBFD DL subband bandwidth(s).

Minimum conducted requirement is defined at the TAB connector for BS type 1-H.

Table 12.3.8.2-1: In-channel adjacent subband blocking requirement for SBFD-capable BS type 1-H

| SBFD UL subband bandwidth (MHz) | Wanted signal mean power (dBm) (NOTE 1) | Interfering signal mean power (dBm) (NOTE 5) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) （NOTE 2, 3, 4） |
| --- | --- | --- | --- | --- | --- |
| 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | PREFSENS + 6 dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | FC,SBFD,DL | CP-OFDM NR signal 30 kHz SCS, BW = X (in the unit of RB) |
|  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | CP-OFDM NR signal 30 kHz SCS, BW1 = X1 and BW2 = X2 (in the unit of RB) |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2. NOTE 2: For DU or UD case, interfering signal is placed at FC,SBFD,DL. For DUD case, two interfering signals are placed at FC,SBFD,DL,1 and FC,SBFD,DL,2, respectively. NOTE 3: For DU or UD case, X is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL. For DUD case, X1 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,1 and X2 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,2. NOTE 4: In case the SBFD BS is configured with intra-band carrier aggregation, the set of interfering signals consists of the interfering signal(s) in this table and the signals from other intra-band contiguous carrier(s) operated in non-SBFD operation. The power spectral density of each interfering signal is set to be the same level. NOTE 5: For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dBc coupling loss between interfering BS transmitter and SBFD BS receiver. |  |  |  |  |  |

12.5 Radiated transmitter characteristics for SBFD-capable BS

12.5.1	General

Radiated transmitter characteristics requirements apply on the SBFD-capable BS type 1-H, BS type 1-O, or BS type 2-O including all its functional components active and for SBFD operation.

12.5.2	Radiated transmit power for SBFD

12.5.2.1	General

The BS radiated transmit power for SBFD operation is specified over SBFD symbol, in which the EIRP for SBFD symbol may be different from the EIRP for transmitter ON period as specified in clause 9.2.1.

12.5.2.2	Minimum requirement for BS type 1-H and BS type 1-O

The same requirement as clause 9.2.2 is applicable to BS radiated transmit power requirement for SBFD operation.

12.5.2.3	Minimum requirement for BS type 2-O

The same requirement as clause 9.2.3 is applicable to BS radiated transmit power requirement for SBFD operation.

12.5.3	OTA base station output power for SBFD

12.5.3.1	General

The OTA base station output power for SBFD operation is specified over SBFD symbol, in which the TRP for SBFD symbol may be different from the TRP for transmitter ON period as specified in clause 9.3.1.

12.5.3.2	Minimum requirement for BS type 1-O

The same requirement as clause 9.3.2 is applicable to OTA base station output power requirement for SBFD operation.

12.5.3.3	Minimum requirement for BS type 2-O

The same requirement as clause 9.3.3 is applicable to OTA base station output power requirement for SBFD operation.

12.5.4	OTA output power dynamics for SBFD

12.5.4.1	General

The requirements in clause 12.5.4 apply during SBFD symbol. Transmit signal quality (as specified in clause 12.5.6) shall be maintained for the output power dynamics requirements of this clause.

The OTA output power requirements are directional requirements and apply to the beam peak directions over the OTA peak directions set.

12.5.4.2		OTA RE power control dynamic range

12.5.4.2.1	General

The RE power control dynamic range for SBFD operation is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,AC or Pmax,c,TABC) during SBFD symbol for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

12.5.4.2.2	Minimum requirement for BS type 1-O

The OTA RE power control dynamic range for SBFD operation is specified the same as the conducted RE power control dynamic range requirement for BS type 1-H in clause 12.2.3.2.

12.5.4.3	OTA total power dynamic range

12.5.4.3.1	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE 1:	The upper limit of the OTA total power dynamic range for SBFD is the BS maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all DL RBs in transmission bandwidth configuration for SBFD at maximum output power. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbol carries PDSCH and not contain RS or SSB.

12.5.4.3.2	Minimum requirement for BS type 1-O and 2-O

The OTA total power dynamic range minimum requirement for each NR carrier shall be larger than or equal to the level calculated as 10log10(NRB,SBFD,DL) if one SBFD DL subband is configured, i.e., DU or UD pattern, and as 10log10(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured, i.e., DUD pattern.

12.5.5	OTA transmit ON/OFF power for SBFD

12.5.5.1	OTA transmitter OFF power for SBFD

12.5.5.1.1		General

The general description for OTA transmitter OFF power as clause 9.5 shall be applied.

12.5.5.1.2		Minimum requirement for BS type 1-O and 2-O

For the OTA transmitter OFF power requirements for non-SBFD symbols, the same requirement as specified in clause 9.5 shall be applied. Requirement is not applicable for SBFD symbols.

12.5.6	OTA transmitted signal quality for SBFD

12.5.6.1	General

The general description for OTA transmitted signal quality as clause 9.6 shall be applied.

12.5.6.2		Minimum requirement for BS type 1-O and 2-O

For frequency error, modulation quality (EVM) and time alignment error (TAE), the same requirements as specified in clause 9.6 shall be applied.

12.5.7	OTA unwanted emissions

12.5.7.1	General

The general description for OTA unwanted emissions as clause 9.7 shall be applied.

12.5.7.2	Minimum requirement for BS type 1-O and 2-O

For OTA occupied bandwidth, OTA adjacent channel leakage power ratio (ACLR), OTA operating band unwanted emissions (OBUE), and OTA transmitter spurious emissions the same requirements as specified in clause 9.7 shall be applied.

Requirements related with multi-band and non-contiguous spectrum are not applied to SBFD.

12.5.8	OTA Transmitter intermodulation for SBFD

12.5.8.1	General

The general description for OTA transmitter intermodulation as clause 9.8 shall be applied.

12.5.8.2	Minimum requirements for BS type 1-O and 2-O

For the OTA transmitter intermodulation requirement, the same requirement as specified in clause 9.8 shall be applied. Receiver requirement is not specified during the OTA transmitter intermodulation test.

12.5.9	OTA spatial emission for SBFD

12.5.9.1	General

OTA spatial emission requirements apply to SBFD-capable BS during SBFD symbol(s).

12.5.9.2	Protection of FSS UL

For SBFD-capable BS type 1-H and BS type 1-O operating in band n104 during SBFD symbol(s), the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, shall not exceed the limits specified in table 9.9.2.2-1.

12.5.10	OTA transient period between SBFD and non-SBFD

12.5.10.1	General

The transmitter transient period between SBFD and non-SBFD operation is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period between SBFD and non-SBFD operation is illustrated in figure 12.5.10.1-1.

Transmitter output power frequency regions for transition

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 12.5.10.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period between SBFD and non-SBFD operation

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases:

Table 12.5.10.1-1: Transmitter transient period between SBFD and non-SBFD operation

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| SBFD symbol to DL symbol | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| SBFD symbol to UL symbol | SBFD DL subband | BS transmitter ON-to-OFF |
| DL symbol to SBFD symbol | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| UL symbol to SBFD symbol | SBFD DL subband | BS transmitter OFF-to-ON |

This requirement shall be applied at each RIB supporting transmission in the operating band.

12.5.10.2	Minimum requirement for BS type 1-O

For BS type 1-O, the OTA transmitter transient period between SBFD and non-SBFD operation shall be shorter than the values listed in the minimum requirement table 12.5.10.2-1.

Table 12.5.10.2-1: Minimum requirement for the OTA transmitter transient period between SBFD and non-SBFD operation for BS type 1-O

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

12.5.10.3	Minimum requirement for BS type 2-O

For BS type 2-O, the OTA transmitter transient period between SBFD and non-SBFD operation shall be shorter than the values listed in the minimum requirement table 12.5.10.3-1.

Table 12.5.10.3-1: Minimum requirement for the OTA transmitter transient period between SBFD and non-SBFD operation for BS type 2-O

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 3 |
| ON to OFF | 3 |

12.5.11	OTA In-channel adjacent subband leakage for SBFD

12.5.11.1 General

OTA In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of UL subband configuration within NR carrier. The measured power is TRP.

The requirement shall be applied per RIB during the transmitter ON period.

12.5.11.2 Minimum requirement for SBFD BS type 1-O

The ICASL shall be less than or equal to a basic limit defined in table 12.2.9.2-1 + X (where X = 9 dB).

12.5.11.3 Minimum requirement for SBFD BS type 2-O

The ICASL shall be less than or equal to the value specified in table 12.5.11.3-1.

Table 12.5.11.3-1: SBFD Base station ICASL absolute basic limit

| SBFD BS category / BS class | ICASL absolute basic limit |
| --- | --- |
| Wide Area BS | -13 dBm/MHz |
| Medium Range BS | -20 dBm/MHz |
| Local Area BS | -20 dBm/MHz |

Table 12.5.11.3-2: Filter parameters for the UL subband

| RAT | Filter on the assigned UL subband and corresponding filter bandwidth |
| --- | --- |
| NR | Square (the declared bandwidth of UL subband) |

12.6 Radiated receiver characteristics for SBFD-capable BS

12.6.1	General

Radiated receiver characteristics are specified at RIB for BS type 1-H, BS type 1-O, or BS type 2-O, with full complement of transceivers configured for SBFD operation in normal operating condition.

12.6.2	OTA sensitivity for SBFD

12.6.2.1	BS type 1-H and BS type 1-O

12.6.2.1.1	General

The general description of OTA sensitivity requirement as clause 10.2.1.1 shall also be applied for SBFD operation.

12.6.2.1.2	Minimum requirement

For a received signal whose AoA of the incident wave is within the active sensitivity RoAoA of an OSDD, the error rate criterion as described in clause 7.2 shall be met when the level of the arriving signal is equal to the minimum EIS level with 0.7dB degradation allowed in the respective declared set of EIS level and UL subband bandwidth, in SBFD symbols with the transmitted signals in SBFD DL subband(s) present.

12.6.3	OTA reference sensitivity level

12.6.3.1	General

The general description of OTA reference sensitivity requirement as clause 10.3.1 shall also be applied for SBFD operation.

12.6.3.2	Minimum requirement for BS type 1-O

The OTA reference sensitivity requirement as specified in clause 10.3.2 with 0.7dB degradation allowed is applicable for SBFD operation in SBFD symbols with the transmitted signals in SBFD DL subband(s) present, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

12.6.3.3	Minimum requirement for BS type 2-O

The OTA reference sensitivity requirement as specified in clause 10.3.3 with 0.7dB degradation allowed is applicable for SBFD operation in SBFD symbols with the transmitted signals in SBFD DL subband(s) present, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

12.6.4	OTA dynamic range

#### 12.6.4.1	General

The general description of OTA dynamic range requirement as clause 10.4.1 shall also be applied for SBFD operation.

#### 12.6.4.2	Minimum requirement for BS type 1-O

The same requirement as clause 10.4.2 is applicable to dynamic range requirement for SBFD operation, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

12.6.5	OTA in-band selectivity and blocking for SBFD

#### 12.6.5.1	OTA Adjacent Channel Selectivity for SBFD

12.6.5.1.1	General

OTA Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

NOTE: SBFD UL subband is not configured adjacent to the Radio Bandwidth edge where cross link interference from adjacent channel may appear.

12.6.5.1.2	Minimum requirement for SBFD capable BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For SBFD operation, the OTA wanted and the interfering signal are specified in table 12.6.5.1.2-1 for OTA ACS. The reference measurement channel for the OTA wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

Table 12.6.5.1.2-1: OTA ACS requirement for BS type 1-O for SBFD

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge(MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB | Wide Area BS: -28 - ΔOTAREFSENS Medium Range BS: -32 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS | ±10 | 20 MHz CP-OFDM NR signal 30 kHz SCS, 51 RBs |
| NOTE 1: EISREFSENS is given in clause 12.6.3. NOTE 2: For lowest/highest carrier is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. |  |  |  |  |

12.6.5.1.3	Minimum requirement for SBFD capable BS type 2-O

The general minimum requirements are the same as those specified in clause 10.5.1.3, except for EISREFSENS. The EISREFSENS depends on the SBFD capable BS class and the SBFD UL subband, and it is specified in clause 12.6.3.

#### 12.6.5.2	OTA in-band blocking for SBFD

12.6.5.2.1	General

The OTA in-band blocking characteristics is a measure of the receiver's ability to receive an OTA wanted signal at its assigned channel in the presence of an unwanted OTA interferer, which is an NR signal for general blocking or an NR signal with one RB for narrowband blocking.

NOTE 1: For FR1 WA BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz with 288 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements.

NOTE 2: For FR1 MR BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz outdoor deployment with 167 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements or in the context of indoor.

NOTE 3: SBFD UL subband is not configured adjacent to the Radio Bandwidth edge where cross link interference from adjacent channel may appear.

12.6.5.2.2	Minimum requirement for SBFD capable BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with OTA wanted and OTA interfering signal specified in tables 12.6.5.2.2-1 for general OTA blocking requirements. The reference measurement channel for the OTA wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The OTA in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

Table 12.6.5.2.2-1: General OTA blocking requirement for BS type 1-O for SBFD

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB | Wide Area BS: -25 - ΔOTAREFSENS Medium Range BS: -32 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS | ±30 | 20 MHz CP-OFDM NR signal 30 kHz SCS, 51 RBs |
| NOTE 1: EISREFSENS is given in clause 12.6.3. NOTE 2: For lowest/highest carrier is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. |  |  |  |  |

12.6.5.2.2	Minimum requirement for SBFD capable BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For BS type 2-O, the OTA wanted and OTA interfering signals are provided at RIB using the parameters in table 12.6.5.2.2-1 for general OTA blocking requirements. The reference measurement channel for the wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The OTA blocking requirements are applicable outside the Base Station RF Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges.

Table 12.6.5.2.2-1: General OTA blocking requirement for BS type 2-O

| Frequency Range | BS channel bandwidth of the lowest/highest carrier received (MHz) | OTA wanted signal mean power (dBm) | OTA interfering signal mean power (dBm) | OTA interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of OTA interfering signal |
| --- | --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200, 400 | EISREFSENS + 6 dB | -44 | ±75 | 50 MHz DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs |
| NOTE 1: EISREFSENS and EISREFSENS_50M are given in clause 12.6.3. NOTE 2: For lowest/highest carrier is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. |  |  |  |  |  |

12.6.6	OTA out-of-band blocking for SBFD capable BS

12.6.6.1 General

The general requirement of OTA out-of-band blocking for SBFD capable BS is the same as general requirement specified in clause 10.6.1.

12.6.6.2	Minimum requirement for SBFD capable BS type 1-O

12.6.6.2.1	General minimum requirement

The general minmum requirements are the same as those specified in clause 10.6.2.1, except for EISminSENS. The EISminSENS depends on the SBFD capable BS class and the SBFD UL subband, and it is specified in clause 12.6.2.

12.6.6.2.2	Co-location minimum requirement

The co-locatioon minmum requirements are the same as those specified in clause 10.6.2.2, except for EISminSENS. The EISminSENS depends on the SBFD capable BS class and the SBFD UL subband, and it is specified in clause 12.6.2.

12.6.6.3	Minimum requirement for SBFD capable BS type 2-O

12.6.6.3.1	General minimum requirement

The general minmum requirements are the same as those specified in clause 10.6.3.1, except for EISREFSENS. The EISREFSENS depends on the SBFD capable BS class and the SBFD UL subband, and it is specified in clause 12.6.3.

12.6.7	OTA receiver spurious emissions

12.6.7.1	General

The general description given in Clause 10.7.1 shall be applied.

12.6.7.2	Minimum requirement for SBFD-capable BS type 1-O

The same requirements as specified in Clause 10.7.2 shall be applied.

12.6.7.3	Minimum requirement for SBFD-capable BS type 2-O

The same requirements as specified in Clause 10.7.3 shall be applied.

12.6.8	OTA in-channel selectivity for SBFD

The same requirements as specified in clause 10.9 shall be applied to SBFD-capable BS type 1-O and BS type 2-O during SBFD symbol, except for the note in table 10.9.2-1, table 10.9.2-1a, table 10.9.2-2, table 10.9.2-2a, table 10.9.2-3 and table 10.9.2-3a, which shall be adjusted to: Wanted and interfering signal are placed adjacently around FC,SBFD,UL. The aggregated wanted and interferer signal shall be centred in the SBFD UL subband bandwidth of the wanted signal. For DU/UD pattern, place the wanted signal closer to the DL subband.

12.6.9	OTA in-channel adjacent subband blocking for SBFD

12.6.9.1	General

OTA in-channel adjacent subband blocking requirement is a measure of the receiver’s ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA interfering signal from other sector(s) and/or site(s) within the same operator’s network, which is an NR signal.

12.6.9.2	Minimum requirement for SBFD-capable BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with an OTA wanted signal and OTA interfering signal(s) using the parameters specified in table 12.6.9.2-1. The reference measurement channel for the wanted signal is identified in clause 10.3.2 for each SBFD UL subband bandwidth and are further specified in annex A.1. The characteristic of the interfering signal(s) is further specified in annex D.

The OTA in-channel adjacent subband blocking requirement shall apply to SBFD UL subband bandwidth. The interfering signal(s) shall be positioned at the centre frequency of the SBFD DL subband bandwidth(s).

Table 12.6.9.2-1: In-channel adjacent subband blocking requirement for SBFD-capable BS type 1-O

| SBFD UL subband bandwidth (MHz) | Wanted signal mean power (dBm) (NOTE 1) | Interfering signal mean power (dBm) (NOTE 5) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) （NOTE 2, 3, 4） |
| --- | --- | --- | --- | --- | --- |
| 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | EISREFSENS + 6 dB | Wide Area BS: -28- ΔOTAREFSENS Medium Range BS: -32- ΔOTAREFSENS Local Area BS: -34- ΔOTAREFSENS | DU or UD | FC,SBFD,DL | CP-OFDM NR signal 30 kHz SCS, BW = X (in the unit of RB) |
|  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | CP-OFDM NR signal 30 kHz SCS, BW1 = X1 and BW2 = X2 (in the unit of RB) |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2. NOTE 2: For DU or UD case, interfering signal is placed at FC,SBFD,DL. For DUD case, two interfering signals are placed at FC,SBFD,DL,1 and FC,SBFD,DL,2, respectively. NOTE 3: For DU or UD case, X is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL. For DUD case, X1 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,1 and X2 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,2. NOTE 4: In case the SBFD BS is configured with intra-band carrier aggregation, the set of interfering signals consists of the interfering signal(s) in this table and the signals from other intra-band contiguous carrier(s) operated in non-SBFD operation. The power spectral density of each interfering signal is set to be the same level. NOTE 5: For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dBc coupling loss between interfering BS transmitter and SBFD BS receiver. |  |  |  |  |  |

12.6.9.3	Minimum requirement for SBFD-capable BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with an OTA wanted signal and OTA interfering signal(s) using the parameters specified in table 12.6.9.3-1. The reference measurement channel for the wanted signal is further specified in annex A.1. The characteristic of the interfering signal(s) is further specified in annex D.

The OTA in-channel adjacent subband blocking requirement shall apply to SBFD UL subband bandwidth. The interfering signal(s) shall be positioned at the centre frequency of the SBFD DL subband bandwidth(s).

Table 12.6.9.3-1: In-channel adjacent subband blocking requirement for SBFD-capable BS type 2-O

| Frequency Range | SBFD UL subband bandwidth (MHz) | OTA wanted signal mean power (dBm) (NOTE 1) | OTA interfering signal mean power (dBm) (NOTE 5) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) （NOTE 2, 3, 4） |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200 | EISREFSENS + 6 dB | Wide Area BS: -52 | DU or UD | FC,SBFD,DL | CP-OFDM NR signal 60 kHz SCS, BW = X (in the unit of RB) |
|  |  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | CP-OFDM NR signal 60 kHz SCS, BW1 = X1 and BW2 = X2 (in the unit of RB) |
| NOTE 1: EISREFSENS depends on the SBFD BS UL subband bandwidth as specified in clause 12.6.3. NOTE 2: For DU or UD case, interfering signal is placed at FC,SBFD,DL. For DUD case, two interfering signals are placed at FC,SBFD,DL,1 and FC,SBFD,DL,2, respectively. NOTE 3: For DU or UD case, X is the largest transmission bandwidth configuration as specified in table 5.3.2-2 that is not larger than NRB, SBFD, DL. For DUD case, X1 is the largest transmission bandwidth configuration as specified in table 5.3.2-2 that is not larger than NRB, SBFD, DL,1 and X2 is the largest transmission bandwidth configuration as specified in table 5.3.2-2 that is not larger than NRB, SBFD, DL,2. NOTE 4: In case the SBFD BS is configured with intra-band carrier aggregation, the set of interfering signals consists of the interfering signal(s) in this table and the signals from other intra-band contiguous carrier(s) operated in non-SBFD operation. The power spectral density of each interfering signal is set to be the same level. NOTE 5: For FR2-1 WA BS, in-channel adjacent subband blocking requirement assumes 87 dBc coupling loss between interfering BS transmitter and SBFD BS receiver. |  |  |  |  |  |  |

## << End of change 3 >>

## << Start of change 4 >>

B.8	Averaged EVM for FR1 SBFD-capable BS

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval.

For SBFD slot and non-SBFD slot, the EVM measurement are conducted separately. For SBFD slot, only resource blocks allocated to downlink subband(s) shall be measured.

For SBFD, let NdlTDD be the number of slots with downlink non-SBFD symbols or SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from  NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c−W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×NslotNdlTDD

and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

## << End of change 4 >>

## << Start of change 5 >>

C.8	Averaged EVM for FR2-1 SBFD-capable BS

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of  Ndl slots where Ndl is the number of slots in a 10 ms measurement interval.

For SBFD slot and non-SBFD slot, the EVM measurement are conducted separately. For SBFD slot, only resource blocks allocated to downlink subband(s) shall be measured.

For SBFD, let NdlTDD be the number of slots with downlink non-SBFD symbols or SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement intervals. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c−W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×NslotNdlTDD

and Nslot=4 for 60 kHz SCS and Nslot=8 for 120 kHz SCS.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

## << End of change 5 >>

## << Start of change 6 >>

Annex D (normative): 
Characteristics of the interfering signals

The interfering signal shall be a PUSCH containing data and DM-RS symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 6 of TS38.211 [9]. Mapping of PUSCH modulation to receiver requirement are specified in table D-1.

Table D-1: Modulation of the interfering signal

| Receiver requirement | Modulation |
| --- | --- |
| In-channel selectivity | 16QAM |
| Adjacent channel selectivity and narrow-band blocking | QPSK |
| General blocking | QPSK |
| Receiver intermodulation | QPSK |
| In-channel adjacent subband blocking for SBFD | QPSK |

## << End of change 6 >>