3GPP TSG-RAN4 Meeting #117	R4-2520539

# Dallas, United States, 17th - 21st November, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 1440 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (HPUE_FR1_DC_LTE_NR_R18-Core) DC_19A-21A_n79A PC2 REFSENS MSD test points correction on DL frequency for band 19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_FR1_DC_LTE_NR_R18-Core |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MSD test points definition for Scell due to dual uplink operation for EN-DC in NR FR1 in the case of DC_19A-21A_n79A defines a wrong TX-RX frequency separation for band 19 (44.7 MHz instead of 45 MHz according to TS 36.101 clause 5.7.4) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of DL frequency for band 19 to 882.5 MHz equal to UL frequency 837.5 MHz + 45 MHz TX-RX frequency separation) in Table 7.3B.2.3.5.2-1a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement will remain incorrect for such frequency band combination. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-3 CR 2014 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The error has been spread from R4-2309387. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

###### 7.3B.2.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.3.5.2-0: MSD test points for Pcell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A-(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |  |
| DC_1A_n1A-n78A | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2125 | 12.0 | IMD4 |  |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |  |
| DC_1A-3A_n1A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
| DC_1A-3A-3A_n1A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2150 | 23 | IMD3 |  |
| DC_1A-3A_n28A DC_1A-3C_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1818.5 | 4.0 | IMD5 |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2139 | 11.0 | IMD4 |  |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
| DC_1A-3A_n71A DC_1A-3A_n71B | 1 | N/A | 5 | N/A | 2150 | 5 | IMD4 |  |
| DC_1A-3C_n71A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |  |
| DC_1A_n3A-n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1818.5 | 4.0 | IMD5 |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
| DC_1A_n3A-n41A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2507.5 | 5.0 | IMD5 |  |
| DC_1A_n3A-n75A | n75 | N/A | 5 | N/A | 1480 | 15.2 | IMD34 |  |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |  |
| DC_1A_n3A-n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4950 | 4.7 | IMD5 |  |
| DC_1A_n5A-n40A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |  |
|  | n40 | N/A | 10 | N/A | 2305 | 9.0 | IMD4 |  |
|  | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A | 880 | 8.5 | IMD4 |  |
|  | n40 | 2385 | 5 | 25 | 2385 | N/A | N/A |  |
| DC_1A-7A_n28A DC_1A-7C_n28A DC_1A-7A-7A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A | 2653 | 30.0 | IMD2 |  |
| DC_1A-7A_n40A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
| DC_1A-7A-7A_n40A | 7 | N/A | 5 | N/A | 2630 | 23 | IMD3 |  |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2120 | 16.4 | IMD3 |  |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |  |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |  |
| DC_1A-8A_n41A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 927.5 | 1.0 | IMD5 |  |
|  | n41 | 2502.5 | 5 | 25 | 2502.5 | N/A | N/A |  |
| DC_1A_n8A-n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |  |
| DC_1A_n8A-n77(2A) | n8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3410 | 1.5 | IMD5 |  |
| DC_1A_n8A-n77A | n8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
| DC_1A_n8A-n77(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3960 | 8.8 | IMD3 |  |
| DC_1A_n8A-n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |  |
| DC_1A_n8A-n77(2A) | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |  |
|  | n8 | N/A | 5 | N/A | 955 | 3.3 | IMD5 |  |
| DC_1A-8A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_1A_n8A-n77(2A) | 8 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_1A-3A_n77A DC_1A-3A_n77(2A) DC_1A-3A_n77(3A) DC_1A-3C_n77A DC_1A-3A_n77C DC_1A-3C_n77(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1807.5 | 31.5 | IMD2 |  |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1870 | 8.5 | IMD4 |  |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2140 | 31.0 | IMD2 |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |  |
| DC_1A-3A_n78A DC_1A-3A-3A_n78A DC_1A-3C_n78A DC_1A-3A_n78C DC_1A-3A_n78(2A) DC_1A-3C_n78(2A) DC_1A-3A_n78(A-C) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1807.5 | 31.2 | IMD2 |  |
|  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2125 | 2.8 | IMD5 |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |  |
| DC_1A_n3A-n77A DC_1A_n3A-n77(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3700 | 28.4 | IMD2 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3360 | 11.2 | IMD4 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1807.5 | 31.5 | IMD2 |  |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1870 | 8.5 | IMD4 |  |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |  |
| DC_1A_n3A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3700 | 28.4 | IMD2 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1830 | 27.9 | IMD2 |  |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |  |
| DC_1A-3A_n105A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1855 | 4 | IMD5 |  |
|  | n105 | 695 | 5 | 25 | 644 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2160 | 5 | IMD4 |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | n105 | 695 | 5 | 25 | 644 | N/A | N/A |  |
| DC_1A-5A_n77A DC_1A-5A_n77(2A) DC_1A-5A_n77(3A) | 1 | N/A | 5 | N/A | 2122 | 18.1 | IMD3 |  |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |  |
|  | n77 | 3780 | 10 | 50 | 3780 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 885 | 3.1 | IMD5 |  |
|  | n77 | 3405 | 10 | 50 | 3405 | N/A | N/A |  |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1807.5 | 37.5 | IMD21 |  |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1870 | 20.5 | IMD41 |  |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2140 | 37.0 | IMD21 |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |  |
| DC_1A-5A_n78A DC_1A-5A_n78C DC_1A-5A_n78(A-C) | 1 | N/A | 5 | N/A | 2122 | 18.1 | IMD3 |  |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |  |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 885 | 3.1 | IMD5 |  |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |  |
| DC_1A_n5A-n78A | 1 | 1932 | 5 | 25 | 2122 | N/A | N/A |  |
|  | n5 | 829 | 5 | 25 | 874 | N/A | N/A |  |
|  | n78 | 3583 | 10 | 50 | 3583 | 18.1 | IMD3 |  |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | n5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |  |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |  |
| DC_1A-7A_n77A DC_1A-7A_n77(2A) DC_1A-7A_n77(3A) DC_1A-7A-7A_n77A DC_1A-7A-7A_n77(2A) DC_1A-7A-7A_n77(3A) | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2627.5 | 9.1 | IMD44 |  |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2140 | 8.7 | IMD4 |  |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |  |
|  | n77 | 3580 | 10 | 50 | 3580 | N/A | N/A |  |
| DC_1A-7A_n78A DC_1A-7C_n78A DC_1A-7A_n78(2A) DC_1A-7C_n78(2A) DC_1A-7A_n78C DC_1A-7A_n78(A-C) DC_1A-1A-7A_n78A DC_1A-7A-7A_n78C DC_1A-7A-7A_n78(A-C) | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2627.5 | 9.1 | IMD4 |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2140 | 8.7 | IMD4 |  |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |  |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |  |
| DC_1A_n7A-n78A DC_1A_n7B-n78A DC_1A_n7A-n78(2A) | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2627.5 | 9.1 | IMD4 |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3390 | 10.1 | IMD4 |  |
| DC_1A-3A_n79A | 1 | N/A | 5 | N/A | 2140 | 3.6 | IMD5 |  |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | N/A |  |
| DC_1A-5A_n28A | 1 | N/A | 5 | N/A | 2123 | 4 | IMD5 |  |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |  |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |  |
|  | 1 | 1965 | 5 | 25 | 2155 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 875 | 4.6 | IMD5 |  |
|  | n28 | 710 | 5 | 25 | 765 | N/A | N/A |  |
| DC_1A-5A_n40A | 1 | N/A | 5 | N/A | 2144 | 4.0 | IMD5 |  |
|  | 5 | 832 | 5 | 25 | 877 | N/A | N/A |  |
|  | n40 | 2320 | 5 | 25 | 2320 | N/A | N/A |  |
|  | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 880 | 8.0 | IMD4 |  |
|  | n40 | 2385 | 5 | 25 | 2385 | N/A | N/A |  |
| DC_1A-5A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 882.5 | 18.3 | IMD3 |  |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 882.5 | 8.9 | IMD4 |  |
|  | n79 | 4907.5 | 40 | 216 | 4907.5 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2140 | 8.1 | IMD4 |  |
|  | 5 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |  |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |  |
| DC_1A-7A_n105A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2673 | 30 | IMD2 |  |
|  | n105 | 698 | 5 | 25 | 647 | N/A | N/A |  |
| DC_1A-8A_n7A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | n7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 927.5 | 1.0 | IMD5 |  |
| DC_1A-8A_n28A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n28 | 730 | 5 | 25 | 785 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 950 | 3.3 | IMD5 |  |
| DC_1A-8A_n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 930 | 8.0 | IMD4 |  |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2135 | 5.3 | IMD5 |  |
|  | 8 | 885 | 5 | 25 | 930 | N/A | N/A |  |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |  |
| DC_1A-8A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |  |
| DC_1A-8A_n77(2A) DC_1A-8A_n77(3A) DC_1A-8B_n77A DC_1A-8B_n77(2A) | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 955 | 3.3 | IMD5 |  |
| DC_1A-8A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
| DC_1A-8A_n77(2A) DC_1A-8A_n77(3A) | n77 | 3960 | 10 | 50 | 3960 | N/A | N/A |  |
| DC_1A-8B_n77A DC_1A-8B_n77(2A) | 1 | N/A | 5 | N/A | 2140 | 14.4 | IMD3 |  |
| DC_1A-8A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
|  | n79 | 4815 | 40 | 216 | 4815 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 945 | 15.8 | IMD3 |  |
| DC_1A-8A_n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n79 | 4845 | 40 | 216 | 4845 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2145 | 8.2 | IMD4 |  |
| DC_1A_n8A-n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n8 | N/A | 5 | N/A | 930 | 8.0 | IMD4 |  |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |  |
| DC_1A_n8A-n78A | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |  |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3745 | 14.9 | IMD3 |  |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
|  | n8 | N/A | 5 | N/A | 940 | 3.3 | IMD5 |  |
|  | n78 | 3380 | 10 | 50 | 3330 | N/A | N/A |  |
| DC_1A-11A_n3A | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |  |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A | 1480 | 15.2 | IMD3 |  |
| DC_1A-11A_n28A | 11 | 1440 | 5 | 25 | 1488 | N/A | N/A |  |
|  | n28 | 710 | 5 | 25 | 765 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2150 | 28.3 | IMD21 |  |
| DC_1A-11A_n41A | 11 | 1442 | 5 | 25 | 1490 | N/A | N/A |  |
|  | n41 | 2520 | 10 | 50 | 2520 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2156 | 10.2 | IMD4 |  |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A | 1490 | 10.6 | IMD4 |  |
| DC_1A-11A_n77A DC_1A-11A_n77(2A) DC_1A-11A_n77(3A) | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A | 1486 | 31.4 | IMD2 |  |
|  | n77 | 3441 | 10 | 50 | 3441 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2140 | 30.8 | IMD2 |  |
|  | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |  |
|  | n77 | 3578 | 10 | 50 | 3578 | N/A | N/A |  |
| DC_1A-11A_n78A DC_1A-11A_n78(2A) | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A | 1486 | 31.4 | IMD2 |  |
|  | n78 | 3441 | 10 | 50 | 3441 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2140 | 30.8 | IMD2 |  |
|  | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |  |
|  | n78 | 3578 | 10 | 50 | 3578 | N/A | N/A |  |
| DC_1A-11A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A | 1483 | 10.2 | IMD4 |  |
|  | n79 | 4427 | 40 | 216 | 4427 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2118 | 15.6 | IMD3 |  |
|  | 11 | 1431 | 5 | 25 | 1479 | N/A | N/A |  |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |  |
| DC_1A-18A_n77A DC_1A-18A_n77(2A) | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | 18 | N/A | 5 | N/A | 870 | 3.5 | IMD5 |  |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2120 | 16.4 | IMD3 |  |
|  | 18 | 825 | 5 | 25 | 870 | N/A | N/A |  |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |  |
| DC_1A-18A_n78A DC_1A-18A_n78(2A) | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | 18 | N/A | 5 | N/A | 870 | 3.5 | IMD5 |  |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2120 | 16.4 | IMD3 |  |
|  | 18 | 819 | 5 | 25 | 864 | N/A | N/A |  |
|  | n78 | 3758 | 10 | 50 | 3758 | N/A | N/A |  |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
|  | 18 | N/A | 5 | N/A | 867.5 | 18.3 | IMD3 |  |
|  | n79 | 4737.5 | 40 | 216 | 4737.5 | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 18 | N/A | 5 | N/A | 865 | 8.9 | IMD4 |  |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2125 | 8.1 | IMD4 |  |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A | N/A |  |
|  | n79 | 4592.5 | 40 | 216 | 4592.5 | N/A | N/A |  |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | N/A | 5 | N/A | 2130 | 17.8 | IMD3 |  |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |  |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |  |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
|  | 19 | N/A | 5 | N/A | 880 | 5.1 | IMD5 |  |
|  | n77, n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |  |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 19 | N/A | 5 | N/A | 882.5 | 18.3 | IMD3 |  |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2140 | 8.1 | IMD4 |  |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |  |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |  |
| DC_1A-20A_n1A | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 20 | 850 | 5 | 25 | 809 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2160 | 6 | IMD4 |  |
| DC_1A_n28A-n41A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A | 2653 | 30.1 | IMD2 |  |
|  | 1 | 1923 | 5 | 25 | 2113 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 762 | 29.3 | IMD2 |  |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |  |
|  | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
|  | n28 | N/A | 10 | N/A | 785 | 4.5 | IMD5 |  |
|  | n41 | 2510 | 10 | 50 | 2510 | N/A | N/A |  |
| DC_1A-20A_n7A | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
|  | 20 | N/A | 10 | N/A | 800 | 4.5 | IMD5 |  |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |  |
| DC_1A-20A_n8A | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A | 805 | 11.5 | IMD4 |  |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
| DC_1A-20A_n38A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 20 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | n38 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_1A-20A_n78A DC_1A-1A-20A_n78A | 1 | N/A | 5 | N/A | 2120 | 20.3 | IMD3 |  |
| DC_1A-20A_n78(2A) | 20 | 835 | 5 | 25 | 794 | N/A | N/A |  |
| DC_1A-20A_n78C | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A | 810 | 3.0 | IMD5 |  |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |  |
| DC_1A-21A_n28A10 | 1 | N/A | 5 | N/A | 2165.3 | 16.1 | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |  |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | N/A | 5 | N/A | 2154.6 | 30.6 | IMD2 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n77, n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2154.6 | 3.6 |  | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |
|  | n77, n78 | 3647 | 10 | 50 | 3647 | N/A |  | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 21 | N/A | 5 | N/A | 1500 | 31.5 | IMD2 |  |
|  | n77, n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 21 | N/A | 5 | N/A | 1500 | 2.9 | IMD5 |  |
|  | n77, n78 | 3675 | 10 | 50 | 3675 | N/A | N/A |  |
| DC_1A-21A_n79A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD4 |  |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_1A-26A_n78A | 1 | N/A | 5 | N/A | 2122 | 18.1 | IMD3 |  |
|  | 26 | 829 | 5 | 25 | 874 | N/A | N/A |  |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | 26 | N/A | 5 | N/A | 885 | 3.1 | IMD5 |  |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |  |
| DC_1A_n26A-n78A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n26 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3610 | 15.7 | IMD3 |  |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | n26 | N/A | 5 | N/A | 885 | 3.1 | IMD5 |  |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |  |
| DC_1A-28A_n3A | 1 | N/A | 5 | N/A | 2139 | 11.0 | IMD4 |  |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
| DC_1A-28A_n7A DC_1A-1A-28A_n7A DC_1A-28A_n7B DC_1A-1A-28A_n7B | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
|  | 28 | N/A | 10 | N/A | 785 | 4.5 | IMD5 |  |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |  |
| DC_1A-28A_n40A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
| DC_1A-28C_n40A | 28 | N/A | 5 | N/A | 780 | 8.9 | IMD4 |  |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |  |
| DC_1A-28A_n38A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 765 | 4.5 | IMD5 |  |
|  | n38 | 2580 | 5 | 25 | 2580 | N/A | N/A |  |
| DC_1A-28A_n71A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 779.5 | 7.5 | IMD5 |  |
|  | n71 | 675.5 | 5 | 25 | 629.5 | N/A | N/A |  |
| DC_1A-28A_n77A | 1 | N/A | 5 | N/A | 2150 | 15.7 | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |  |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | N/A |  |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 794 | 4.2 | IMD5 |  |
|  | n77 | 3352 | 10 | 50 | 3352 | N/A | N/A |  |
| DC_1A-28A_n78A | 1 | N/A | 5 | N/A | 2150 | 15.7 | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |  |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |  |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 794 | 4.2 | IMD5 |  |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | N/A |  |
| DC_1A-28A_n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 788 | 15.2 | IMD3 |  |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |  |
|  | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 795 | 10.0 | IMD4 |  |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2167.5 | 1.2 | IMD4 |  |
|  | 28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2125 | 4.5 | IMD5 |  |
|  | 28 | 718 | 5 | 25 | 773 | N/A | N/A |  |
|  | n79 | 4807 | 40 | 216 | 4807 | N/A | N/A |  |
| DC_1A_n28A-n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A | 2374 | 10.1 | IMD4 |  |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 768 | 8.6 | IMD4 |  |
|  | n40 | 2314 | 5 | 25 | 2314 | N/A | N/A |  |
| DC_1A_n28A-n77A DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |  |
|  | n77/n78 | N/A | 10 | N/A | 3416 | 15.7 | IMD3 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n77/n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 790 | 4.2 | IMD5 |  |
| DC_1A_n28A-n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 788 | 15.2 | IMD39 |  |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n28 | 730 | 5 | 25 | 785 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4630 | 14.9 | IMD34 |  |
| DC_1A-32A_n3A | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A | 1480 | 15.2 | IMD34 |  |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |  |
| DC_1A-32A_n78A DC_1A-32A_n78C DC_1A-32A_n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A | 1470 | 31.8 | IMD2 |  |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A | 1470 | 0 | IMD5 |  |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |  |
| DC_1A-38A_n78A DC_1A-38A_n78(2A) | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | 38 | N/A | 5 | N/A | 2590 | 12.7 | IMD4 |  |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |  |
| DC_1A_n38A-n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n38 | N/A | 10 | N/A | 2590 | 12.7 | IMD4 |  |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |  |
| DC_1A-40A_n28A | 1 | 1920 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A | 2374 | 10.1 | IMD4 |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
| DC_1A_n40A-n71A | 1 | 1977 | 5 | 25 | 2167 | N/A | N/A |  |
|  | n40 | 2305 | 10 | 50 | 2305 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 649 | 1 | IMD4 |  |
| DC_1A_n40A-n77A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
| DC_1A_n40A-n77(2A) | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3450 | 9.8 | IMD4 |  |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A | 2360 | 10.6 | IMD4 |  |
|  | n77 | 3520 | 10 | 50 | 3520 | N/A | N/A |  |
| DC_1A-40A_n78A DC_1A-40C_n78A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A | 2340 | 10.6 | IMD4 |  |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2140 | 9.1 | IMD4 |  |
|  | 40 | 2360 | 5 | 25 | 2360 | N/A | N/A |  |
|  | n78 | 3430 | 10 | 50 | 3430 | N/A | N/A |  |
| DC_1A_n40A-n78A DC_1A_n40A-n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
| DC_1A_n40A-n78C | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3450 | 9.8 | IMD4 |  |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A | 2360 | 10.6 | IMD4 |  |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |  |
| DC_1_n40-n105 | 1 | 1977 | 5 | 25 | 2167 | N/A | N/A |  |
|  | n40 | 2305 | 10 | 50 | 2305 | N/A | N/A |  |
|  | n105 | 700 | 5 | 25 | 649 | 1 | IMD4 |  |
| DC_1A-41A_n3A DC_1A-41C_n3A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2507.5 | 5.0 | IMD5 |  |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
|  | 41 | N/A | 10 | N/A | 2653 | 30 | IMD2 |  |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |  |
| DC_1A-41A_n77A DC_1A-41C_n77A DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 11.0 | IMD4 |  |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2140 | 9.3 | IMD4 |  |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |  |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 3.6 | IMD5 |  |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A | N/A |  |
| DC_1A_n41A-n77A DC_1A_n41A-n77(2A) | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A | 2515 | 11.5 | IMD44 |  |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |  |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n41 | 2650 | 10 | 25 | 2650 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3330 | 19.6 | IMD34,9 |  |
| DC_1A-41A_n78A DC_1A-41C_n78A DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | 1 | N/A | 5 | N/A | 2140 | 9.3 | IMD4 |  |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |  |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2515 | 12 | IMD4 |  |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |  |
| DC_1A_n41A-n78A DC_1A_n41A-n78(2A) | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A | 2515 | 11.5 | IMD44 |  |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |  |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n41 | 2650 | 10 | 25 | 2650 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3330 | 19.6 | IMD34,9 |  |
| DC_1A-41A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2530 | 29.4 | IMD2 |  |
|  | n79 | 4500 | 40 | 216 | 4500 | N/A | N/A |  |
| DC_1A-42A_n3A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
|  | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A | 3425 | 13.0 | IMD4 |  |
| DC_1A-42A_n28A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A | 3416 | 15.7 | IMD3 |  |
| DC_1A-42A_n28A | 42 | 3580 | 5 | 25 | 3580 | N/A | N/A |  |
|  | n28 | 723 | 5 | 25 | 778 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2134 | 15.7 | IMD3 |  |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A | 3490 | 4.8 | IMD5 |  |
|  | 42 | 3402.5 | 5 | 25 | 3402.5 | N/A | N/A |  |
|  | n79 | 4640 | 40 | 216 | 4640 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2165 | 15.5 | IMD3 |  |
|  | 42 | 3450 | 5 | 25 | 3450 | N/A | N/A |  |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A | 2140 | 9.3 | IMD4 |  |
| DC_1A_n71A-n77A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 635 | 15.2 | IMD3 |  |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n71 | 686 | 5 | 25 | 640 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3342 | 15.7 | IMD3 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n71 | 680 | 5 | 25 | 634 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3990 | 9.4 | IMD4 |  |
| DC_1A_SUL_n77A-n80A | 1 | N/A | 5 | N/A | 2140 | 23 | IMD3 |  |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |  |
| DC_1A_SUL_n77A-n80A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3425 | 13.0 | IMD4 |  |
| DC_1A_n75A-n78A DC_1A_n75A-n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n75 | N/A | 5 | N/A | 1470 | 30.4 | IMD2 |  |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |  |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4870 | 15.9 | IMD3 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3490 | 4.6 | IMD5 |  |
| DC_1A_SUL_n78A-n80A DC_1A_SUL_n78C-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |  |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |  |
|  | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |  |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |  |
| DC_1_n78-n105 | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | n105 | 686 | 5 | 25 | 635 | 15.2 | IMD3 |  |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3342 | 15.7 | IMD3 |  |
|  | n105 | 686 | 5 | 25 | 635 | N/A | N/A |  |
| DC_2A-(n)66AA | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2145 | 2.8 | IMD5 |  |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |  |
| DC_2A_n2A-n66A | 2 | 1875 | 5 | 25 | 1955 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1975 | 20 | IMD3 |  |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |  |
| DC_2A_n2A-n77A | 2 | 1875 | 5 | 25 | 1955 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1935 | 26 | IMD2 |  |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |  |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1965 | 8.0 | IMD44 |  |
|  | n77 | 3735 | 10 | 50 | 3735 | N/A | N/A |  |
| DC_2A_n2A-n78A | 2 | 1852.5 | 5 | 25 | 1932.5 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1942.5 | 26 | IMD24 |  |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |  |
| DC_2A-4A_n28A | 2 | N/A | 5 | N/A | 1960 | 11.0 | IMD4 |  |
|  | 4 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n28 | 740 | 5 | 25 | 795 | N/A | N/A |  |
| DC_2A-4A_n41A | 2 | N/A | 5 | N/A | 1940 | 11.0 | IMD4 |  |
|  | 4 | 1715 | 5 | 25 | 2115 | N/A | N/A |  |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |  |
| DC_2A-4A_n78A | 2 | 1875 | 5 | 25 | 1955 | N/A | N/A |  |
|  | 4 | N/A | 5 | N/A | 2145 | 10.3 | IMD4 |  |
|  | n78 | 3480 | 10 | 50 | 3480 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A | 1960 | 32.1 | IMD2 |  |
|  | 4 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A | 1940 | 9.1 | IMD4 |  |
|  | 4 | 1750 | 5 | 25 | 2150 | N/A | N/A |  |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A | 1950 | 2.1 | IMD5 |  |
|  | 4 | 1750 | 5 | 25 | 2150 | N/A | N/A |  |
|  | n78 | 3600 | 10 | 50 | 3600 | N/A | N/A |  |
| DC_2A_n5A-n7A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |  |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2685 | 30.0 | IMD2 |  |
| DC_2A-5A_n12A8 | 2 | N/A | 5 | N/A | 1980 | 5.9 | IMD5 |  |
|  | 5 | 840 | 5 | 25 | 885 | N/A | N/A |  |
|  | n12 | 705 | 5 | 25 | 735 | N/A | N/A |  |
| DC_2A-5A_n30A | 2 | 1870 | 5 | 25 | 1959 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 880 | 9.7 | IMD4 |  |
|  | n30 | 2310 | 10 | 50 | 2355 | N/A | N/A |  |
| DC_2A-5A_n48A DC_2A-5A_n48B | 2 | N/A | 5 | N/A | 1962 | 15.6 | IMD3 |  |
|  | 5 | 839 | 5 | 25 | 884 | N/A | N/A |  |
|  | n48 | 3640 | 5 | 25 | 3640 | N/A | N/A |  |
| DC_2A-5A_n71A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |  |
|  | n71 | 686.5 | 5 | 25 | 640.5 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 891.5 | 4.2 | IMD5 |  |
| DC_2A_n5A-n77A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3540 | 16.0 | IMD3 |  |
| DC_2A_n5A-n77A11 | 2 | 1907 | 5 | 25 | 1987 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A | 889 | 3.8 | IMD5 |  |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
| DC_2A-5A_n77A11 | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
| DC_2A-5A_n77C11 DC_2A-5A_n77(2A)11 DC_2A-2A-5A_n77A11 | 5 | N/A | 5 | N/A | 887.5 | 3.8 | IMD5 |  |
| DC_2A-2A-5A_n77C11 DC_2A-2A-5A_n77(2A)11 | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A | 1987 | 16.5 | IMD3 |  |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |  |
|  | n77 | 3680 | 10 | 50 | 3680 | N/A | N/A |  |
| DC_2A-5A_n78A DC_2A-2A-5A_n78A DC_2A-5A_n78(2A) | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 887.5 | 3.8 | IMD5 |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A | 1987 | 16.5 | IMD3 |  |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |  |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |  |
| DC_2A-7A_n5A DC_2A-7C_n5A DC_2A-7A-7A_n5A | 2 | 1855 | 10 | 50 | 1935 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A | 2685 | 30.0 | IMD2 |  |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
| DC_2A-7A_n12A | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
| DC_2A-2A-7A_n12A | 7 | N/A | 5 | N/A | 2622.5 | 30.8 | IMD2 |  |
|  | n12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |  |
| DC_2A_n7A-n12A | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | N/A | 30.8 | IMD2 |  |
|  | n12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |  |
|  | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
|  | n7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |  |
|  | n12 | N/A | 5 | N/A | 731.5 | 4.5 | IMD5 |  |
| DC_2A-7A_n28A DC_2A-7C_n28A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2120 | 29.0 | IMD2 |  |
|  | n28 | 740 | 5 | 25 | 795 | N/A | N/A |  |
| DC_2A_n7A-n71A | 2 | 1865 | 5 | 25 | 1945 | N/A | N/A |  |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 645 | 28.7 | IMD2 |  |
|  | 2 | 1890 | 5 | 25 | 1970 | N/A | N/A |  |
|  | n7 | 2520 | 10 | 50 | 2640 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 630 | 1 | IMD5 |  |
| DC_2A-7A_n77A DC_2A-2A-7A_n77A DC_2A-7C_n77A DC_2A-7A-7A_n77A DC_2A-7A_n77(2A) DC_2A-7C_n77(2A) DC_2A-7A-7A_n77(2A) | 2 | N/A | 5 | N/A | 1950 | 8.6 | IMD4 |  |
|  | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n77 | 3525 | 10 | 50 | 3475 | N/A | N/A |  |
|  | 2 | 1860 | 5 | 25 | 1940 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2660 | 3.4 | IMD5 |  |
|  | n77 | 4120 | 10 | 50 | 4120 | N/A | N/A |  |
| DC_2A_n7A-n77A | 2 | 1860 | 5 | 25 | 1940 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2660 | 3.4 | IMD5 |  |
|  | n77 | 4120 | 10 | 50 | 4120 | N/A | N/A |  |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3775 | 4.2 | IMD5 |  |
| DC_2A-7A_n78A DC_2A-2A-7A_n78A DC_2A-7C_n78A DC_2A-7A-7A_n78A DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) DC_2A-7A-7A_n78(2A) | 2 | N/A | 5 | N/A | 1950 | 8.6 | IMD4 |  |
|  | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n78 | 3525 | 10 | 50 | 3475 | N/A | N/A |  |
| DC_2A_n7A-n78A, DC_2A_n7(2A)-n78A DC_2A_n7A-n78(2A) DC_2A_n7(2A)-n78(2A) | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3775 | 4.2 | IMD5 |  |
| DC_2-8_n2 | 2 | N/A | 5 | N/A | 1940 | 4 | IMD4 |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
| DC_2A-12A_n5A DC_2A-2A-12A_n5A | 2 | N/A | 5 | N/A | 1980 | 5.9 | IMD5 |  |
|  | 12 | 705 | 5 | 25 | 735 | N/A | N/A |  |
|  | n5 | 840 | 5 | 25 | 885 | N/A | N/A |  |
| DC_2A-12A_n7A DC_2A-12A_n7(2A) | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
| DC_2A-2A-12A_n7A | 12 | N/A | 5 | N/A | 731.5 | 4.5 | IMD5 |  |
|  | n7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |  |
| DC_2A-12A_n41A DC_2A-2A-12A_n41A | 2 | N/A | 5 | N/A | 1952 | 26 | IMD2 |  |
|  | 12 | 708 | 5 | 50 | 738 | N/A | N/A |  |
|  | n41 | 2660 | 10 | 50 | 2660 | N/A | N/A |  |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | 12 | N/A | 5 | N/A | 738 | 28.7 | IMD24 |  |
|  | n41 | 2638 | 10 | 50 | 2638 | N/A | N/A |  |
| DC_2A-12A_n66A | 2 | N/A | N/A | N/A | N/A | N/A | IMD4 |  |
|  | 12 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_2A-12A_n77A DC_2A-12A_n77(2A) | 2 | N/A | 5 | N/A | 1960 | 16.5 | IMD39,11 |  |
| DC_2A-2A-12A_n77A DC_2A-2A-12A_n77(2A) | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |  |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | N/A |  |
| DC_2A_n12A-n77A DC_2A-2A_n12A-n77A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3315 | 16.0 | IMD34,9,11 |  |
| DC_2A-12A_n78A DC_2A-2A-12A_n78A DC_2A-12A_n78(2A) | 2 | N/A | 5 | N/A | 1954 | 16.5 | IMD3 |  |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |  |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |  |
| DC_2A_n12A-n78A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |  |
|  | n78 | 3315 | 10 | 50 | 3315 | 16.0 | IMD3 |  |
| DC_2A-13A_n48A DC_2A-13A_n48B | 2 | N/A | 5 | N/A | 1983.5 | 15.6 | IMD3 |  |
|  | 13 | 784.5 | 5 | 25 | 753.5 | N/A | N/A |  |
|  | n48 | 3552.5 | 5 | 25 | 3552.5 | N/A | N/A |  |
| DC_2A-13A_n66A DC_2A-2A-13A_n66A | 2 | N/A | 5 | N/A | 1940 | 6.2 | IMD4 |  |
|  | 13 | 780 | 10 | 50 | 749 | N/A | N/A |  |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | N/A |  |
| DC_2A-13A_n77A | 2 | N/A | 5 | N/A | 1944 | 16.0 | IMD3 |  |
| DC_2A-13A_n77C | 13 | 783 | 10 | 50 | 752 | N/A | N/A |  |
| DC_2A-2A-13A_n77A DC_2A-2A-13A_n77C | n77 | 3510 | 10 | 50 | 3510 | N/A | N/A |  |
| DC_2A-14A_n77A DC_2A-14A_n77(2A) | 2 | N/A | 5 | N/A | 1954 | 16.5 | IMD3 |  |
| DC_2A-2A-14A_n77A DC_2A-2A-14A_n77(2A) | 14 | 793 | 5 | 25 | 763 | N/A | N/A |  |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |  |
| DC_2_n25-n66 | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A | 1935 | 20 | IMD3 |  |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |  |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A | 1963.3 | 4 | IMD5 |  |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | N/A |  |
| DC_2A_n38A-n71A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n38 | N/A | 10 | N/A | 2586 | 29.2 | IMD2 |  |
|  | n71 | 686 | 5 | 25 | 640 | N/A | N/A |  |
| DC_2A_n38A-n78A | 2 | 1870 | 5 | 25 | 1950 | N/A | N/A |  |
|  | n38 | 2610 | 10 | 50 | 2610 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3350 | 14.8 | IMD3 |  |
| DC_2A-14A_n66A | 2 | N/A | 5 | N/A | 1954 | 7.2 | IMD4 |  |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |  |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |  |
| DC_2A-28A_n66A | 2 | N/A | 5 | N/A | 1980 | 11 | IMD4 |  |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |  |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
| DC_2A-28A_n78A | 2 | 1904 | 5 | 25 | 1984 | 16.5 | IMD3 |  |
|  | 28 | 708 | 5 | 25 | 763 | N/A | N/A |  |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |  |
| DC_2A-30A_n77A DC_2A-30A_n77(2A) | 2 | N/A | 5 | N/A | 1986 | 8.6 | IMD411 |  |
| DC_2A-2A-30A_n77A DC_2A-2A-30A_n77(2A) | 30 | 2312 | 5 | 25 | 2357 | N/A | N/A |  |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 2 | 1905 | 5 | 25 | 1985 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A | 2354 | 10.6 | IMD411 |  |
|  | n77 | 3361 | 10 | 50 | 3361 | N/A | N/A |  |
|  | 2 | 1860 | 5 | 25 | 1940 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A | 2354 | 3.4 | IMD5 |  |
|  | n77 | 3967 | 10 | 50 | 3967 | N/A | N/A |  |
| DC_2A-38A_n78A | 2 | N/A | 5 | N/A | 1932.5 | 16 | IMD39 |  |
|  | 38 | 2617.5 | 5 | 25 | 2617.5 | N/A | N/A |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
| DC_2A_n41A-n71A DC_2A-2A_n41A-n71A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 630 | 28.7 | IMD2 |  |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A | 2586 | 29.2 | IMD2 |  |
|  | n71 | 686 | 5 | 50 | 640 | N/A | N/A |  |
| DC_2A_n41A-n77A | 2 | 1870 | 5 | 25 | 1950 | N/A | N/A |  |
|  | n41 | 2670 | 5 | 25 | 2670 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3470 | 14.8 | IMD34 |  |
|  | 2 | 1870 | 5 | 25 | 1950 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2640 | 5.3 | IMD55 |  |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | N/A |  |
| DC_2A_n41A-n78A | 2 | 1870 | 5 | 25 | 1950 | N/A | N/A |  |
|  | n41 | 2610 | 5 | 25 | 2610 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3350 | 14.8 | IMD34 |  |
| DC_2A-46A_n5A5 DC_2A-46C_n5A5 DC_2A-46D_n5A5 DC_2A-46E_n5A5 | 2 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_2A-2A-46A_n5A5 DC_2A-2A-46C_n5A5 DC_2A-2A-46D_n5A5 | 46 | N/A | N/A | N/A | N/A | N/A | IMD4, IMD5 |  |
|  | n5 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_2A-46A_n66A5 DC_2A-46C_n66A5 DC_2A-46D_n66A5 DC_2A-46E_n66A5 | 2 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD3, IMD5 |  |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_2A-46A_n77A5 DC_2A-46A-46A_n77A5 | 2 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD3 |  |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | n2 | 1853 | 5 | 25 | 1933 | N/A | N/A |  |
|  | 48 | 3590 | 20 | 100 | 3590 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A | 1969 | 12 | IMD4 |  |
| DC_2A-48A_n5A | 2 | N/A | 5 | N/A | 1950 | 16.9 | IMD3 |  |
| DC_2A-48C_n5A | 48 | 3610 | 10 | 50 | 3610 | N/A | N/A |  |
| DC_2A-48D_n5A | n5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
| DC_2A-48E_n5A | 2 | 1890 | 5 | 25 | 1970 | N/A | N/A |  |
|  | 48 | N/A | 5 | N/A | 3570 | 16.2 | IMD3 |  |
|  | n5 | 840 | 5 | 25 | 885 | N/A | N/A |  |
| DC_2A-48A_n66A DC_2A-48C_n66A DC_2A-48D_n66A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | 48 | N/A | 10 | N/A | 3620 | 29.4 | IMD2 |  |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A | 1960 | 28.3 | IMD2 |  |
|  | 48 | 3695 | 5 | 25 | 3695 | N/A | N/A |  |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | N/A |  |
| DC_2A_n48A-n66A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
| DC_2A-48E_n66A | n48 | N/A | 10 | N/A | 3620 | 29.4 | IMD2 |  |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A | 1980 | 20 | IMD3 |  |
| DC_2A-66A_n2A | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |  |
| DC_2A-66A-66A_n2A | n2 | 1855 | 5 | 25 | 1935 | N/A | N/A |  |
| DC_2A-66A_n5A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2140 | 7.2 | IMD4 |  |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
| DC_2A-66A_n25A | 2 | N/A | 5 | N/A | 1935 | 20 | IMD3 |  |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A | 1935 | 20 | IMD3 |  |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2150 | 4 | IMD5 |  |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |  |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2112.5 | 23 | IMD3 |  |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |  |
| DC_2A-66A_n28A | 2 | N/A | 5 | N/A | 1960 | 11.0 | IMD4 |  |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n28 | 740 | 5 | 25 | 795 | N/A | N/A |  |
| DC_2A-66A_n41A DC_2A-66A_n41C DC_2A-66A_n41(2A) | 2 | N/A | 5 | N/A | 1940 | 11.0 | IMD4 |  |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |  |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | N/A |  |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1905 | 5 | 25 | 1985 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2155 | 12.1 | IMD4 |  |
|  | n48 | 3560 | 5 | 25 | 3560 | N/A | N/A |  |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | N/A | 5 | N/A | 1960 | 28.3 | IMD5 |  |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |  |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |  |
| DC_2A-66A_n77A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |  |
| DC_2A-66A_n77C DC_2A-66A_n77(2A) DC_2A-2A-66A_n77A DC_2A-2A-66A_n77C DC_2A-2A-66A_n77(2A) DC_2A-66A-66A_n77A DC_2A-66A-66A_n77C DC_2A-66A-66A_n77(2A) DC_2A-2A-66A-66A_n77A DC_2A-2A-66A-66A_n77C | 66 | N/A | 5 | N/A | 2115 | 29.2 | IMD2 |  |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | N/A |  |
|  | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2140 | 10.4 | IMD4 |  |
|  | n77 | 3500 | 10 | 50 | 3500 | N/A | N/A |  |
|  | 2 | 1885 | 5 | 25 | 1965 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2175 | 4.0 | IMD5 |  |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A | 1960 | 32.1 | IMD2 |  |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |  |
| DC_2A-66A_n77A11 DC_2A-66A_n77C11 DC_2A-66A_n77(2A)11 DC_2A-2A-66A_n77A11 DC_2A-2A-66A_n77C11 DC_2A-66A-66A_n77A11 DC_2A-66A-66A_n77C11 DC_2A-2A-66A-66A_n77A11 DC_2A-2A-66A-66A_n77C11 | 2 | N/A | 5 | N/A | 1960 | 32.1 | IMD2 |  |
|  | 66 | 1745 | 5 | 25 | 2145 | N/A | N/A |  |
|  | n77 | 3705 | 10 | 50 | 3705 | N/A | N/A |  |
| DC_2A_n66A-n77A11 DC_2A-2A_n66A-n77A11 | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2115 | 29.2 | IMD2 |  |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | N/A |  |
|  | 2 | 1853 | 5 | 25 | 1933 | N/A | N/A |  |
|  | n66 | 1713 | 5 | 25 | 2113 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3566 | 29.4 | IMD2 |  |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
| DC_2A-2A-66A_n78A | 66 | N/A | 5 | N/A | 2160 | 10.3 | IMD4 |  |
|  | n78 | 3480 | 10 | 50 | 3480 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A | 1960 | 32.1 | IMD2 |  |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A | 1960 | 9.1 | IMD4 |  |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A | 1960 | 2.1 | IMD5 |  |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | N/A |  |
| DC_2A_n66A-n78A DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A) | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3620 | 29.4 | IMD2 |  |
|  | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2140 | 10.3 | IMD4 |  |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |  |
|  | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3340 | 8.9 | IMD4 |  |
| DC_2A-71A_n7A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
| DC_2A-2A-71A_n7A | 71 | N/A | 5 | N/A | 630 | 28.7 | IMD24 |  |
|  | n7 | 2530 | 10 | 50 | 2650 | N/A | N/A |  |
| DC_2A-71A_n38A DC_2A-2A-71A_n38A | 2 | N/A | 5 | N/A | 1942 | 26 | IMD2 |  |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |  |
|  | n38 | 2610 | 10 | 50 | 2610 | N/A | N/A |  |
| DC_2A-71A_n41A DC_2A-2A-71A_n41A | 2 | N/A | 5 | N/A | 1942 | 26 | IMD2 |  |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |  |
|  | n41 | 2610 | 10 | 50 | 2610 | N/A | N/A |  |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | 71 | N/A | 5 | N/A | 630 | 28.7 | IMD24 |  |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |  |
| DC_2A-71A_n77A DC_2A-2A-71A_n77A DC_2A-71A_n77(2A) | 2 | N/A | 5 | N/A | 1954 | 16.5 | IMD39 |  |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |  |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | N/A |  |
| DC_2A_n71A-n77A DC_2A-2A_n71A-n77A DC_2A_n71A-n77(2A) | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3305 | 8 | IMD3 |  |
| DC_2A-71A_n78A DC_2A-2A-71A_n78A | 2 | N/A | 5 | N/A | 1954 | 16.5 | IMD3 |  |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |  |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |  |
| DC_2A_n71A-n78A DC_2A-2A_n71A-n78A | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3305 | 8 | IMD3 |  |
| DC_3A_n1A-n28A DC_3C_n1A-n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2139 | 11.0 | IMD4 |  |
| DC_3A_n1A-n40A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A | 2380 | 8.0 | IMD5 |  |
| DC_3A_n1A-n41A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
|  | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2507.5 | 5.0 | IMD5 |  |
| DC_3A_n1A-n75A | n75 | N/A | 5 | N/A | 1480 | 15.2 | IMD34,19 |  |
| DC_3C_n1A-n75A | n1 | 1960 | 5 | 25 | 2150 | N/A | N/A |  |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
| DC_3A_n1A-n77A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3700 | 28.4 | IMD2 |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2140 | 31.0 | IMD2 |  |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |  |
| DC_3A_n1A-n78A DC_3C_n1A-n78A DC_3A-3A_n1A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3700 | 28.4 | IMD2 |  |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2130 | 3.5 | IMD5 |  |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |  |
| DC_3A_n1A-n79A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n79 | 4950 | 40 | 216 | 4950 | 4.7 | IMD5 |  |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | n1 | 1950 | 40 | 216 | 2140 | 3.6 | IMD5 |  |
|  | n79 | 4860 | 5 | 25 | 4860 | N/A | N/A |  |
| DC_(n)3AA-n8A | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1837.5 | 4.5 | IMD5 |  |
|  | n3 | 1747.5 | 5 | 25 | 1842.5 | 6.4 | IMD5 |  |
| DC_3A_n3A-n41A | 3 | 1725 | 5 | 25 | 1820 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1865 | 8.2 | IMD4 |  |
|  | n41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |  |
| DC_(n)3AA-n77A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD24 |  |
| DC_(n)3CA-n77A | n3 | N/A | 5 | N/A | 1840 | [28.9] | IMD24 |  |
| DC_(n)3AA-n77(2A) DC_(n)3CA-n77(2A) | n77 | 3575 | 10 | 50 | 3575 | N/A | N/A |  |
| DC_(n)3AA-n78A DC_(n)3CA-n78A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD24 |  |
| DC_(n)3AA-n78(2A) DC_(n)3CA-n78(2A) | n3 | N/A | 5 | N/A | 1840 | [28.9] | IMD24 |  |
|  | n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |  |
| DC_3A-5A_n28A | 3 | N/A | 5 | N/A | 1829.5 | 8.7 | IMD4 |  |
|  | 5 | 845 | 5 | 25 | 890 | N/A | N/A |  |
|  | n28 | 705.5 | 5 | 25 | 760.5 | N/A | N/A |  |
| DC_3A-5A_n77A DC_3A-5A_n77(2A) DC_3A-5A_n77(3A) | 3 | N/A | 5 | N/A | 1820 | 17.3 | IMD3 |  |
|  | 5 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n77 | 3510 | 10 | 50 | 3510 | N/A | N/A |  |
| DC_3A-5A_n78A DC_3A-5A_n78(A-C) | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |  |
|  | 5 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_3A_n5A-n78A DC_3C_n5A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |  |
|  | n78 | 3420 | 10 | 52 | 3420 | 16.1 | IMD3 |  |
| DC_3A-5A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 885 | 18.5 | IMD3 |  |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1877.5 | 0.2 | IMD4 |  |
|  | 5 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
| DC_3A-7A_n5A | 3 | 1780 | 10 | 50 | 1875 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A | 2625 | 30.0 | IMD21 |  |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |  |
| DC_3A-(n)7AA DC_3C-(n)7AA | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2647.5 | 6.9 | IMD4 |  |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | IMD4 |  |
| DC_3A-7A_n8A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | n8 | 890 | 5 | 25 | 935 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A | 2670 | 29.0 | IMD2 IMD33 |  |
| DC_3A-7A_n26A | 3 | 1780 | 10 | 50 | 1875 | N/A | N/A |  |
| DC_3A-7C_n26A | 7 | N/A | 10 | N/A | 2625 | 30.0 | IMD2 |  |
| DC_3C-7A_n26A DC_3C-7C_n26A | n26 | 845 | 5 | 25 | 890 | N/A | N/A |  |
|  | 3 | 1760 | 5 | 25 | 1855 | N/A | N/A |  |
|  | 7 | 2555 | 10 | N/A | 2675 | 16.9 | IMD319 |  |
|  | n26 | 845 | 5 | 25 | 890 | N/A | N/A |  |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
| DC_3A-7A-7A_n28A | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A | 2682 | 16.9 | IMD3 |  |
|  | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1832.5 | 26.0 | IMD2 |  |
| DC_3A_n8A-n77A | 3 | 1740 | 5 | 25 | 1835 | N/A | N/A |  |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3540 | 16.3 | IMD34 |  |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |  |
|  | n8 | N/A | 5 | N/A | 955 | 9.7 | IMD4 |  |
| DC_3A-18A_n3A | 3 | N/A | 5 | N/A | 1814 | 4 | IMD4 ¦2*fn3-2*fB18¦ |  |
|  | 18 | 823 | 5 | 25 | 868 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
| DC_3-18_n41 | 18 | N/A | 5 | N/A | 865 | 28.9 | IMD2 |  |
|  | 3 | 1765 | 5 | 25 | 1860 | N/A | N/A |  |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | N/A |  |
|  | 18 | N/A | 5 | N/A | 865 | 19.0 | IMD3 |  |
|  | 3 | 1725 | 5 | 25 | 1820 | N/A | N/A |  |
|  | n41 | 2585 | 5 | 25 | 2585 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1850 | 28.8 | IMD2 |  |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |  |
|  | 18 | 820 | 5 | 25 | 865 | MSD | N/A |  |
| DC_3A-18A_n77A DC_3A-18A_n77(2A) DC_3A-18A_n78A DC_3A-18A_n78(2A) | 3 | N/A | 5 | N/A | 1865 | 15.7 | IMD3 |  |
|  | 18 | 820 | 5 | 25 | 865 | N/A | N/A |  |
|  | n77, n78 | 3505 | 10 | 50 | 3505 | N/A | N/A |  |
| DC_3A-19A_n77A DC_3A-19A_n78A | 3 | N/A | 5 | N/A | 1850 | 17.3 | IMD3 |  |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n77, n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |  |
| DC_3A_n7A-n28A | 3 | 1747 | 5 | 25 | 1842 | N/A | N/A |  |
| DC_3C_n7A-n28A | n7 | 2543 | 5 | 25 | 2663 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 796.0 | 20.0 | IMD2 |  |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2682 | 17.0 | IMD3 |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
| DC_3A-7A_n40A | 3 | N/A | 5 | N/A | 1866.6 | 3.4 | IMD5 |  |
| DC_3A-7A-7A_n40A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |  |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |  |
| DC_3A-7A_n77A | 3 | N/A | 5 | N/A | 1820 | 17.6 | IMD3 |  |
| DC_3A-7A_n77(2A) DC_3A-7A_n77(3A) | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
| DC_3A-7A-7A_n77(2A) | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |  |
| DC_3A-7A-7A_n77(3A) | 3 | N/A | 5 | N/A | 1820 | 8.6 | IMD4 |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n77 | 3475 | 10 | 50 | 3475 | N/A | N/A |  |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2670 | 5.2 | IMD5 |  |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |  |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2640 | 3.4 | IMD5 |  |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |  |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A DC_3A-3A-7A_n78A DC_3A-3A-7A-7A_n78A DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A DC_3A-7A_n78(2A) DC_3C-7A_n78(2A) DC_3A-7C_n78(2A) DC_3C-7C_n78(2A) DC_3A-7A_n78C DC_3A-7A_n78(A-C) DC_3A-7A-7A_n78C | 3 | N/A | 5 | N/A | 1820 | 17.6 | IMD3 |  |
| DC_3A-7A-7A_n78(A-C) | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1820 | 8.6 | IMD4 |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | N/A |  |
| DC_3A-7A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
| DC_3A-3A-7A_n79A | n79 | 4440 | 10 | 50 | 4440 | N/A | N/A |  |
| DC_3A-7A-7A_n79A | 7 | N/A | 5 | N/A | 2670 | 30.2 | IMD2 |  |
| DC_3A-3A-7A-7A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n79 | 4440 | 10 | 50 | 4440 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2640 | 5.0 | IMD5 |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n79 | 4420 | 10 | 50 | 4420 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1855 | 29.4 | IMD2 |  |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
|  | n79 | 4745 | 10 | 50 | 4745 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1840 | 4.8 | IMD5 |  |
| DC_3A-7A_n105A | 3 | N/A | 5 | N/A | 1875 | 16.5 | IMD2 |  |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
|  | n105 | 675 | 5 | 25 | 624 | N/A | N/A |  |
| DC_3A-8A_n7A | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |  |
|  | n7 | 2530 | 10 | 50 | 2650 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 940 | 18.0 | IMD3 |  |
| DC_3A-8A_n40A | 3 | N/A | 5 | N/A | 1874 | 4 | IMD5 |  |
| DC_3C-8A_n40A | 8 | 912 | 5 | 25 | 957 | N/A | N/A |  |
|  | n40 | 2305 | 5 | 25 | 2305 | N/A | N/A |  |
| DC_3A-8A_n41A | 3 | 1725 | 5 | 25 | 1820 | N/A | N/A |  |
| DC_3A-3A-8A_n41A | 8 | N/A | 5 | N/A | 945 | 26.0 | IMD215 |  |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1807.5 | 25 | IMD2x |  |
|  | 8 | 882.5 | 5 | 25 | 927.5 | N/A | N/A |  |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |  |
| DC_3A_n8A-n41A | 3 | 1722.5 | 5 | 25 | 1817.5 | N/A | N/A |  |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A | 2610 | 28.0 | IMD216 |  |
|  | 3 | 1725 | 5 | 25 | 1820 | N/A | N/A |  |
|  | n8 | N/A | 5 | N/A | 945 | 26.0 | IMD216 |  |
|  | n41 | 2516 | 10 | 50 | 2516 | N/A | N/A |  |
| DC_3A-8A_n71A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
| DC_3C-8A_n71A | 8 | N/A | 5 | N/A | 932 | 5 | IMD5 |  |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1870 | 5 | IMD5 |  |
|  | 8 | 890 | 5 | 25 | 935 | N/A | N/A |  |
|  | n71 | 690 | 5 | 25 | 644 | N/A | N/A |  |
| DC_3A-8A_n77A DC_3A-8A_n77(2A) DC_3A-8A_n77(3A) DC_3C-8A_n77A DC_3C-8A_n77(2A) | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
| DC_3A-8B_n77A | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 955 | 9.7 | IMD4 |  |
| DC_3A-8A_n77A DC_3A-8A_n77(2A) DC_3A-8A_n77(3A) DC_3C-8A_n77A DC_3C-8A_n77(2A) | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
| DC_3A-8B_n77A | n77 | 3640 | 10 | 50 | 3640 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1820 | 16.5 | IMD3 |  |
| DC_3A-8A_n78A DC_3A-3A-8A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
| DC_3A-8B_n78A DC_3A-3A-8B_n78A DC_3A-8A_n78(2A) DC_3C-8A_n78(2A) | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1820 | 16.5 | IMD319 |  |
| DC_3A_n8A-n78A | 3 | 1740 | 5 | 25 | 1835 | N/A | N/A |  |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3540 | 16.3 | IMD3 |  |
| DC_3A-8A_n79A | 3 | 1755 | 5 | 25 | 1850 | N/A | N/A |  |
|  | n79 | 4465 | 40 | 216 | 4465 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 955 | 15.3 | IMD3 |  |
| DC_3A-8A_n79A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1850 | 8.8 | IMD4 |  |
| DC_3A_n7A-n78A DC_3A_n7B-n78A DC_3C_n7A-n78A DC_3C_n7B-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
| DC_3A_n7A-n78(2A) | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
| DC_3C_n7A-n78(2A) | n78 | N/A | 10 | N/A | 3390 | 16.1 | IMD3 |  |
| DC_3A-11A_n77A DC_3A-11A_n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n77 | 3675 | 10 | 50 | 3675 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A | 1491 | 8.8 | IMD4 |  |
|  | 11 | 1435.4 | 5 | 25 | 1483.4 | N/A | N/A |  |
|  | n77 | 3905 | 10 | 50 | 3905 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1848 | 3.4 | IMD57 |  |
| DC_3A-11A_n79A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | 11 | N/A | 5 | 25 | N/A | 25.1 | IMD3 |  |
|  | n79 | 4920 | 40 | 216 | 4920 | N/A | N/A |  |
| DC_3A-19A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | 19 | N/A | 5 | N/A | 885 | 18.5 | IMD3 |  |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1877.5 | 5.5 | IMD4 |  |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
| DC_3A-20A_n3A | 3 | N/A | 5 | N/A | 1870 | 4 | IMD4 |  |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |  |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | N/A |  |
| DC_3A-20A_n7A DC_3C-20A_n7A | 3 | 1737 | 5 | 25 | 1832 | N/A | N/A |  |
|  | 20 | N/A | 10 | N/A | 806 | 10.5 | IMD2 |  |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | N/A |  |
| DC_3A-20A_n8A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A | 810 | 27 | IMD2 |  |
|  | 3 | N/A | 5 | N/A | 1860 | 14.5 | IMD4 |  |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |  |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |  |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1828 | 9.4 | IMD4 |  |
| DC_3A-20A_n38A | 3 | 1779 | 5 | 25 | 1874 | N/A | N/A |  |
|  | 20 | N/A | 10 | N/A | 811 | 26.0 | IMD21 |  |
|  | n38 | 2590 | 10 | 50 | 2590 | N/A | N/A |  |
| DC_3A-20A_n41A DC_3C-20A_n41A DC_3A-3A-20A-n41A | 3 | N/A | 5 | N/A | 1839 | 26.0 | IMD2 |  |
|  | n41 | 2680 | 10 | 50 | 2680 | N/A | N/A |  |
|  | 20 | 841 | 10 | 50 | 800 | N/A | N/A |  |
|  | 3 | 1779 | 5 | 25 | 1874 | N/A | N/A |  |
|  | n41 | 2590 | 10 | 50 | 2590 | N/A | N/A |  |
|  | 20 | N/A | 10 | N/A | 811 | 26.0 | IMD2 |  |
|  | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n41 | 2660 | 10 | 50 | 2660 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A | 800 | 12.5 | IMD3 |  |
| DC_3_n20-n67 | 3 | 1765 | 5 | 25 | 1860 | N/A | N/A |  |
|  | n20 | 837 | 5 | 25 | 796 | N/A | N/A |  |
|  | n67 | N/A | 5 | N/A | 746 | 10.1 | IMD4 |  |
| DC_3A_20A_SUL_n78A-n80A DC_3C_20A_SUL_n78A-n80A | 3 | N/A | 5 | N/A | 1820 | 17.3 | IMD3 |  |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |  |
| DC_3A_n20A-n78A DC_3A-3A_n20A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3420 | 16.1 | IMD3 |  |
| DC_3A-20A_n78A DC_3C-20A_n78A DC_3A-20A_n78(2A) | 3 | N/A | 5 | N/A | 1820 | 17.3 | IMD3 |  |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |  |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |  |
|  | 21 | N/A | 5 | N/A | 1507.5 | 8.8 | IMD4 |  |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1862.5 | 30.8 | IMD2 |  |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | N/A | N/A |  |
|  | n77, n78 | 3322 | 10 | 50 | 3322 | N/A | N/A |  |
| DC_3A-21A_n77A | 3 | N/A | 5 | N/A | 1866.6 | 3.4 | IMD5 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | N/A |  |
| DC_3A-21A_n79A | 3 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD3 |  |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1869.2 | 17.8 | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |  |
| DC_3A-26A_n78A DC_3C-26A_n78A | 3 | N/A | 5 | N/A | 1862 | 15.7 | IMD3 |  |
|  | 26 | 839 | 5 | 25 | 884 | N/A | N/A |  |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | N/A |  |
| DC_3A-28A_n1A DC_3C-28A_n1A | 3 | N/A | 5 | N/A | 1820 | 4 | IMD5 |  |
|  | 28 | 710 | 5 | 25 | 765 | N/A | N/A |  |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
| DC_3A-28A_n5A DC_3C-28A_n5A | 3 | N/A | 5 | N/A | 1830 | 8.7 | IMD4 |  |
|  | 28 | 705 | 5 | 25 | 798 | N/A | N/A |  |
|  | n5 | 845 | 5 | 25 | 874 | N/A | N/A |  |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 785 | 9.4 | IMD4 |  |
|  | n5 | 845 | 5 | 25 | 874 | N/A | N/A |  |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-3A-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B DC_3A-3A-28A_n7B | 3 | N/A | 5 | N/A | 1832.5 | 26.0 | IMD2 |  |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | N/A |  |
|  | 3 | 1747 | 5 | 25 | 1842 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 796.0 | 20.0 | IMD2 |  |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | N/A |  |
| DC_3A-28A_n77A DC_3C-28A_n77(2A) DC_3A-28C_n77(2A) DC_3C-28C_n77(2A) | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 770 | 15.3 | IMD3 |  |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1850 | 17.0 | IMD3 |  |
|  | 28 | 735 | 5 | 25 | 790 | N/A | N/A |  |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | N/A |  |
| DC_3A_n28A-n75A DC_3C_n28A-n75A | B3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | n28 | 708 | 5 | 25 | 763 | N/A | N/A |  |
|  | n75 | N/A | - | N/A | 1436 | 3.3 | IMD5 |  |
| DC_3A_n28A-n77A DC_3C_n28A-n77A DC_3C_n28A-n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | 28 | 733 | 5 | 25 | 788 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 4173 | 15.9 | IMD3 |  |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 770 | 15.3 | IMD3 |  |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |  |
| DC_3A-28A_n38A | 3 | N/A | 5 | N/A | 1870 | 26.0 | IMD2 |  |
|  | 28 | 710 | 5 | 25 | 765 | N/A | N/A |  |
|  | n38 | 2580 | 5 | 25 | 2580 | N/A | N/A |  |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 800 | 20.0 | IMD21 |  |
|  | n38 | 2580 | 5 | 25 | 2580 | N/A | N/A |  |
| DC_3A-28A_n41A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 790 | 26.0 | IMD21 |  |
|  | 3 | N/A | 5 | N/A | 1832.5 | 26.0 | IMD2 |  |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | N/A |  |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
| DC_3A_n28A-n41A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 790 | 261 | IMD2 ¦fn41-fB3¦ |  |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | N/A |  |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2518 | 27.4 | IMD2 ¦fB3+fn28¦ |  |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2687 | 15.9 | IMD3 ¦2*fB3-fn28¦ |  |
| DC_3A_n26A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
| DC_3C_n26A-n78A | n26 | 839 | 5 | 25 | 884 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3408 | 16.1 | IMD3 |  |
|  | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n26 | 839 | 5 | 25 | 884 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3512 | 4.5 | IMD5 |  |
| DC_3A-28A_n78A DC_3C-28A_n78A DC_3A-3A-28A_n78A | 3 | N/A | 5 | N/A | 1870 | 17.3 | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 760 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | N/A |  |
| DC_3A-28A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 780 | 10.3 | IMD4 |  |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1870 | 5.7 | IMD5 |  |
|  | 28 | 725 | 5 | 25 | 780 | N/A | N/A |  |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |  |
| DC_3A_n28A-n78A DC_3C_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3764 | 4.5 | IMD5 |  |
| DC_3A_n28A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 780 | 10.3 | IMD4 |  |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |  |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4585 | 9.4 | IMD44 |  |
| DC_3A_n40A-n71A | 3 | 1745 | 5 | 25 | 1840 | N/A | N/A |  |
| DC_3C_n40A-n71A | n40 | 2380 | 10 | 50 | 2380 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 635 | 26.0 | IMD2 |  |
|  | 3 | 1777.5 | 5 | 25 | 1872.5 | N/A | N/A |  |
|  | n40 | 2350 | 10 | 50 | 2350 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 632.5 | 4.5 | IMD5 |  |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n40 | N/A | 10 | N/A | 2388 | 26.0 | IMD2 |  |
|  | n71 | 668 | 5 | 25 | 622 | N/A | N/A |  |
| DC_3A_n40A-n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
| DC_3A_n40A-n77(2A) | n40 | 2350 | 5 | 25 | 2350 | N/A | N/A |  |
| DC_3C_n40A-n77A | n77 | N/A | 10 | N/A | 4070 | 30.3 | IMD2 |  |
|  | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n40 | 2360 | 5 | 25 | 2360 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3620 | 4.8 | IMD5 |  |
|  | 3 | 1745 | 5 | 25 | 1840 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A | 2355 | 29,2 | IMD2 |  |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | N/A |  |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A | 2360 | 4.4 | IMD5 |  |
|  | n77 | 3760 | 10 | 50 | 3760 | N/A | N/A |  |
| DC_3A_SUL_n77A-n84A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |  |
|  | n84 | 1922.5 | 5 | 25 |  | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3425 | 13.0 | IMD4 |  |
| DC_3A_n40A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
| DC_3A_n40A-n78C | n40 | 2360 | 5 | 25 | 2360 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3620 | 4.8 | IMD5 |  |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A | 2360 | 4.4 | IMD5 |  |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | N/A |  |
| DC_3A_n40A-n79A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n40 | 2330 | 5 | 25 | 2330 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4550 | 4.7 | IMD5 |  |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A | 2330 | 3.2 | IMD5 |  |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |  |
| DC_3_n40-n105 | 3 | 1745 | 5 | 25 | 1840 | N/A | N/A |  |
|  | n40 | 2380 | 10 | 50 | 2380 | N/A | N/A |  |
|  | n105 | N/A | 5 | N/A | 635 | 26.0 | IMD2 |  |
|  | 3 | 1777.5 | 5 | 25 | 1872.5 | N/A | N/A |  |
|  | n40 | 2350 | 10 | 50 | 2350 | N/A | N/A |  |
|  | n105 | N/A | 5 | N/A | 632.5 | 4.5 | IMD5 |  |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n40 | N/A | 10 | N/A | 2388 | 26.0 | IMD2 |  |
|  | n105 | 668 | 5 | 25 | 617 | N/A | N/A |  |
| DC_3A_n41A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4440 | 30.8 | IMD24 |  |
| DC_3A-42A_n1A DC_3A-42C_n1A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A | 3425 | 13.0 | IMD4 |  |
|  | n1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
| DC_3A_n71A-n77A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
| DC_3C_n71A-n77A | n71 | 680 | 5 | 25 | 634 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 4140 | 15.9 | IMD31 |  |
|  | 3 | 1747 | 5 | 25 | 1842 | N/A | N/A |  |
|  | n71 | 680 | 5 | 25 | 634 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3787 | 10.1 | IMD4 |  |
|  | 3 | 1748 | 5 | 25 | 1843 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 632 | 15.3 | IMD3 |  |
|  | n77 | 4128 | 10 | 50 | 4128 | N/A | N/A |  |
| DC_3A_n75A-n78A DC_3C_n75A-n78A DC_3A_n75A-n78(2A) | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | n75 | N/A | - | N/A | 1514.5 | 10.0 | IMD2 |  |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
| DC_3A-3A_n78A-n79A | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4910 | 16.3 | IMD3 |  |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n79 | 4510 | 40 | 216 | 4510 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3710 | 4.2 | IMD5 |  |
| DC_3A_SUL_n78A-n82A | 3 | N/A | 5 | N/A | 1870 | 4 | IMD4 |  |
|  | n82 | 840 | 5 | 25 |  | N/A | N/A |  |
| DC_3A_SUL_n78A-n84A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |  |
|  | n84 | 1922.5 | 5 | 25 |  | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3425 | 13.0 | IMD4 |  |
| DC_3A-32A_n1A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
| DC_3C-32A_n1A | 32 | N/A | 5 | N/A | 1480 | 15.2 | IMD34, 19 |  |
|  | n1 | 1960 | 5 | 25 | 2150 | N/A | N/A |  |
| DC_3A-32A_n7A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A | 1470 | 10.5 | IMD4 |  |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |  |
| DC_3A-32A_n78A DC_3C-32A_n78A DC_3A-32A_n78C DC_3A-32A_n78(2A) | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A | 1470 | 4.9 | IMD4 |  |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A | 1475 | 0 | IMD5 |  |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |  |
| DC_3A-38A_n28A DC_3C-38A_n28A | 38 | 2575 | 5 | 25 | 2575 | N/A | N/A |  |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1850 | 26 | IMD2 |  |
| DC_3A-38A_n78A DC_3C-38A_n78A | 3 | N/A | 5 | N/A | 1830 | 16.4 | IMD35 |  |
|  | 38 | 2615 | 5 | 25 | 2615 | N/A | N/A |  |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |  |
| DC_3A-40A_n1A DC_3A-40C_n1A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A | 2380 | 8.0 | IMD5 |  |
| DC_3A-40A_n77A DC_3A-40C_n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | 40 | 2310 | 5 | 25 | 2310 | 29.4 | IMD2 |  |
|  | n77 | 4030 | 10 | 50 | 4030 | N/A | N/A |  |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | 40 | 2350 | 5 | 25 | 2350 | 5.3 | IMD5 |  |
|  | n77 | 3755 | 10 | 50 | 3755 | N/A | N/A |  |
|  | 3 | 1725 | 5 | 25 | 1820 | 29.9 | IMD29 |  |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |  |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | N/A |  |
| DC_3A-40A_n78A DC_3A-40C_n78A | 3 | N/A | 5 | N/A | 1870 | 9.1 | IMD4 |  |
|  | 40 | 2390 | 5 | 25 | 2390 | N/A | N/A |  |
|  | n78 | 3325 | 10 | 50 | 3325 | N/A | N/A |  |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A | 2360 | 4.4 | IMD5 |  |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | N/A |  |
| DC_3A-41A_n1A DC_3A-41C_n1A DC_3A-3A-41A_n1A DC_3A-3A-41C_n1A | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2507.5 | 5.0 | IMD5 |  |
| DC_3A-41A_n3A DC_3A-41C_n3A | 3 | N/A | 5 | N/A | 1865 | 8.2 | IMD4 ¦2*fB41-2*fn3¦ |  |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |  |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | N/A |  |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2543 | 10 | 50 | 2543 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1832.5 | 26 | IMD2 |  |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2518 | 27.4 | IMD2 |  |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2687 | 15.9 | IMD3 |  |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) DC_3A_n41A-n77A DC_3A_n41A-n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |  |
|  | 41/n41 | N/A | 5 | N/A | 2640 | 5.3 | IMD5 |  |
|  | 41/n41 | 2620 | 5 | 25 | 2620 | N/A | N/A |  |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1840 | 16.4 | IMD3 |  |
| DC_3A-41A_n78A DC_3A-41C_n78A DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |  |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1840 | 16.4 | IMD3 |  |
| DC_3A_n41A-n78A DC_3A_n41A-n78(2A) | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n41 | 2560 | 10 | 50 | 2560 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3390 | 16.4 | IMD3 |  |
| DC_3A-41A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2670 | 30.2 | IMD2 |  |
|  | 41 | 2570 | 5 | 25 | 2570 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A | 1850 | 29.4 | IMD2 |  |
| DC_3A_n71A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n71 | 666 | 5 | 25 | 620 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3728 | 13.0 | IMD4 |  |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
|  | n71 | 685 | 5 | 25 | 639 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3775 | 3.8 | IMD5 |  |
| DC_3_n78-n105 | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3725 | 13 | IMD44 |  |
|  | n105 | 670 | 5 | 25 | 619 | N/A | N/A |  |
| DC_4A-5A_n78A | 4 | N/A | 5 | N/A | 2122 | 18.1 | IMD3 |  |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |  |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |  |
| DC_4A-7A_n28A | 4 | 1715 | 5 | 25 | 2115 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2685 | 18.0 | IMD3 |  |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |  |
| DC_4A-7A_n78A | 4 | N/A | 5 | N/A | 2150 | 8.7 | IMD4 |  |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | N/A |  |
| DC_5A_n1A-n78A | 5 | 829 | 5 | 25 | 874 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2122 | 18.1 | IMD3 |  |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |  |
|  | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3610 | 15.7 | IMD3 |  |
| DC_5A_n1A-n28A | 5 | 829 | 5 | 25 | 874 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2123 | 4 | IMD5 |  |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |  |
| DC_5A_n2A-n7A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n2 | 1855 | 5 | 25 | 1935 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2685 | 30.0 | IMD2 |  |
| DC_5A_n2A-n41A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n2 | 1855 | 10 | 50 | 1935 | N/A | N/A |  |
|  | n41 | 2685 | 10 | 50 | 2685 | 30.0 | IMD2 |  |
| DC_5A_n2A-n66A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n66 | 1740 | 5 | 25 | 2140 | 7.2 | IMD4 |  |
| DC_5A_n2A-n77A11 | n2 | N/A | 5 | N/A | 1987 | 16.5 | IMD3 |  |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |  |
|  | n77 | 3680 | 10 | 50 | 3680 | N/A | N/A |  |
| DC_5A_n3A-n28A | 5 | 845 | 5 | 25 | 890 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1829.5 | 8.7 | IMD4 |  |
|  | n28 | 705.5 | 5 | 25 | 760.5 | N/A | N/A |  |
|  | 5 | 827 | 5 | 25 | 872 | N/A | N/A |  |
|  | n3 | 1713 | 5 | 25 | 1808 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 768 | 9.4 | IMD4 |  |
| DC_5A_n5A-n77A11 | 5 | 834 | 5 | 25 | 879 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A | 889 | 8.3 | IMD4 |  |
|  | n77 | 3391 | 10 | 50 | 3391 | N/A | N/A |  |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A | 882 | 5.5 | IMD5 |  |
|  | n77 | 4188 | 10 | 50 | 4188 | N/A | N/A |  |
| DC_5A-7A_n7A | 5 | N/A | 5 | N/A | 879 | 12 | IMD34 |  |
|  | 7 | 2527 | 10 | 50 | 2647 | N/A | N/A |  |
|  | n7 | 2547 | 10 | 50 | 2667 | N/A | N/A |  |
| DC_5A_n2A-n78A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3540 | 16.0 | IMD3 |  |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1987 | 16.5 | IMD3 |  |
|  | n78 | 3680 | 10 | 25 | 3680 | N/A | N/A |  |
| DC_5A_n3A-n78A | 5 | 839 | 5 | 25 | 884 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3408 | 16.1 | IMD3 |  |
|  | 5 | 839 | 5 | 25 | 884 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3512 | 4.5 | IMD5 |  |
|  | 5 | 839 | 5 | 25 | 884 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1862 | 15.7 | IMD3 |  |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | N/A |  |
| DC_5A-7A_n25A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | 7 | 2565 | 5 | 25 | 2685 | 30.0 | IMD2 |  |
|  | n25 | 1855 | 5 | 25 | 1935 | N/A | N/A |  |
| DC_5A_n7A-n25A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2685 | 30.0 | IMD2 |  |
|  | n25 | 1855 | 5 | 25 | 1935 | N/A | N/A |  |
| DC_5A-7A_n28A | 5 | 842 | 5 | 25 | 887 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2640 | 5.9 | IMD5 |  |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |  |
| DC_5A-7A_n66A DC_5A-7C_n66A DC_5A-7A-7A_n66A | 5 | N/A | 5 | N/A | 880 | 17.8 | IMD3 |  |
|  | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2624 | 29.0 | IMD21 |  |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | N/A |  |
| DC_5A_n7A-n66A | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2624 | 29.0 | IMD2 |  |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | N/A |  |
|  | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2670 | 13 | IMD3 |  |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | N/A |  |
| DC_5A-7A_n71A | 5 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2660 | 6.5 | IMD5 |  |
|  | n71 | 680 | 5 | 25 | 634 | N/A | N/A |  |
|  | 5 | 844 | 5 | 25 | 889 | N/A | N/A |  |
| DC_5A-7A_n77A | 7 | N/A | 5 | N/A | 2645 | 30.1 | IMD2 |  |
| DC_5A-7A_n77(2A) DC_5A-7A_n77(3A) | n77 | 3489 | 10 | 50 | 3489 | N/A | N/A |  |
| DC_5A-7A-7A_n77A | 5 | N/A | 5 | N/A | 879 | 30.2 | IMD21 |  |
| DC_5A-7A-7A_n77(2A) DC_5A-7A-7A_n77(3A) | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
|  | n77 | 3429 | 10 | 50 | 3429 | N/A | N/A |  |
| DC_5A_n7A-n77A | 5 | 844 | 5 | 25 | 889 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2645 | 30.1 | IMD2 |  |
|  | n77 | 3489 | 10 | 50 | 3489 | N/A | N/A |  |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n7 | 2540 | 5 | 25 | 2660 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3375 | 29.7 | IMD2 |  |
|  | 5 | 829 | 5 | 25 | 885 | N/A | N/A |  |
|  | n7 | 2505 | 5 | 25 | 2625 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 4163 | 16.1 | IMD3 |  |
| DC_5A-7A_n78A DC_5A-7A_n78C DC_5A-7A_n78(A-C) DC_5A-7A-7A_n78C | 5 | 844 | 5 | 25 | 889 | N/A | N/A |  |
| DC_5A-7A-7A_n78(A-C) | 7 | N/A | 10 | N/A | 2645 | 30.1 | IMD2 |  |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 879 | 30.2 | IMD2 |  |
|  | 7 | 2550 | 10 | 50 | 2670 | N/A | N/A |  |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 875 | 3.3 | IMD5 |  |
|  | 7 | 2525 | 10 | 50 | 2645 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |  |
| DC_5A_n7A-n78A DC_5A_n7(2A)-n78A DC_5A_n7A-n78(2A) DC_5A_n7(2A)-n78(2A) | 5 | 844 | 5 | 25 | 889 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2645 | 30.1 | IMD2 |  |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |  |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n7 | 2540 | 5 | 25 | 2660 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3375 | 29.7 | IMD2 |  |
| DC_5A-13A_n66A | 5 | 840 | 5 | 25 | 885 | N/A | N/A |  |
|  | 13 | N/A | 5 | N/A | 750 | 9.4 | IMD4 |  |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | N/A |  |
| DC_5A-13A_n77A11 | 5 | 840 | 5 | 25 | 885 | N/A | N/A |  |
| DC_5A-13A_n77C11 | n77 | 4110 | 10 | 50 | 4110 | N/A | N/A |  |
|  | 13 | N/A | 5 | N/A | 750 | 4.4 | IMD5 |  |
|  | 13 | 782 | 5 | 20 | 751 | N/A | N/A |  |
|  | n77 | 4013 | 10 | 50 | 4013 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 885 | 4.5 | IMD5 |  |
| DC_5A_n28A-n77A DC_5A_n28A-n77C | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | 11.6 | IMD4 |  |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n28 | 710 | 5 | 25 | 765 | 4.4 | IMD5 |  |
|  | n77 | 4105 | 10 | 50 | 4105 | N/A | N/A |  |
| DC_5A_n28A-n78A DC_5A_n28A-n78C | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |  |
|  | n28 | 715.5 | 5 | 25 | 765.5 | 11.6 | IMD4 |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n28 | 707 | 5 | 25 | 762 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3658 | 4.0 | IMD5 |  |
| DC_5A-30A_n2A | 5 | N/A | 5 | N/A | 880 | 8 | IMD4 |  |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |  |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | N/A |  |
| DC_5A-30A_n77A DC_5A-30A_n77(2A) | 5 | 835 | 5 | 25 | 880 | 15.2 | IMD34 |  |
|  | 30 | N/A | 5 | N/A | 2355 | N/A | N/A |  |
|  | n77 | 3740 | 10 | 50 | 3740 | N/A | N/A |  |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A | 2355 | 13.2 | IMD311 |  |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | N/A |  |
| DC_5A_n38A-n66A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n38 | N/A | 10 | N/A | 2590 | 28.9 | IMD2 |  |
| DC_5A-40A_n77A | 5 | 835 | 5 | 25 | 880 | N/A | N/A |  |
| DC_5A-40C_n77A | 40 | 2355 | 5 | 25 | 2355 | 13.2 | IMD3 |  |
| DC_5A-40A_n77C | n77 | 4025 | 10 | 50 | 4025 | N/A | N/A |  |
| DC_5A-40C_n77C | 5 | 835 | 5 | 25 | 880 | 15.2 | IMD34 |  |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |  |
|  | n77 | 3740 | 10 | 50 | 3740 | N/A | N/A |  |
| DC_5A_n41A-n66A | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |  |
|  | n41 | 2624 | 10 | 50 | 2624 | 29.0 | IMD2 |  |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | N/A |  |
|  | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n41 | 2600 | 10 | 50 | 2600 | 18 | IMD3 |  |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | N/A |  |
| DC_5A_n40A-n77A | 5 | 840 | 5 | 25 | 885 | N/A | N/A |  |
| DC_5A_n40A-n77(2A) | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3780 | 16.1 | IMD3 |  |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A | 2355 | 13.2 | IMD3 |  |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | N/A |  |
| DC_5A-40A_n78A DC_5A-40C_n78A | 5 | 835 | 5 | 25 | 880 | 15.2 | IMD3 |  |
| DC_5A-40A_n78C | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |  |
| DC_5A-40C_n78C | n78 | 3740 | 10 | 50 | 3740 | N/A | N/A |  |
| DC_5A_n40A-n78A | 5 | 840 | 5 | 25 | 885 | N/A | N/A |  |
| DC_5A_n40A-n78C | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3780 | 16.1 | IMD3 |  |
| DC_5A_n41A-n77A | 5 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n41 | 2540 | 5 | 25 | 2540 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3375 | 29.7 | IMD22 |  |
|  | 5 | 840 | 5 | 25 | 885 | N/A | N/A |  |
|  | n41 | 2500 | 5 | 25 | 2500 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 4160 | 16.1 | IMD3 |  |
|  | 5 | 844 | 5 | 25 | 889 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2645 | 30.1 | IMD2 |  |
|  | n77 | 3489 | 10 | 50 | 3489 | N/A | N/A |  |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2510 | 13.2 | IMD3 |  |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | N/A |  |
| DC_5A_41A_n78A | 5 | N/A | 5 | N/A | 885 | 30.2 | IMD2 |  |
|  | 41 | 2615 | 5 | 25 | 2615 | N/A | N/A |  |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 881.5 | 3.1 | IMD5 |  |
|  | 41 | 2620.5 | 5 | 25 | 2620.5 | N/A | N/A |  |
|  | n78 | 3490 | 10 | 50 | 3490 | N/A | N/A |  |
| DC_5A_n41A-n78A | 5 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n41 | 2540 | 5 | 25 | 2540 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3375 | 29.7 | IMD22 |  |
|  | 5 | 844 | 5 | 25 | 889 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2645 | 30.1 | IMD2 |  |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |  |
| DC_5A-41A_n79A | 5 | N/A | 5 | N/A | 880 | 23.9 | IMD3 |  |
|  | 41 | 2665 | 20 | 100 | 2665 | N/A | N/A |  |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | N/A |  |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |  |
|  | 41 | N/A | 20 | N/A | 2517.5 | 1.8 | IMD4 |  |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |  |
| DC_5A-46A_n66A | 5 | 847 | 5 | 25 | 892 | N/A | N/A |  |
|  | 46 | N/A | 10 | N/A | 5163 | 9.04 | IMD4 ¦2*fB5+2*fn66¦ |  |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |  |
| DC_5A-48A_n12A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | 48 | N/A | 5 | N/A | 3650 | 4.4 | IMD5 |  |
|  | n12 | 705 | 5 | 25 | 735 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 875 | 5.9 | IMD5 |  |
|  | 48 | 3695 | 5 | 25 | 3695 | N/A | N/A |  |
|  | n12 | 705 | 5 | 25 | 735 | N/A | N/A |  |
| DC_5A-48A_n71A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | 48 | N/A | 5 | N/A | 3590 | 4.4 | IMD5 |  |
|  | n71 | 690 | 5 | 25 | 644 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 880 | 5.9 | IMD5 |  |
|  | 48 | 3600 | 5 | 25 | 3600 | N/A | N/A |  |
|  | n71 | 680 | 5 | 25 | 634 | N/A | N/A |  |
| DC_5A-66A_n2A DC_5B-66A_n2A DC_5A-5A-66A_n2A DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A DC_5A-5A-66A-66A_n2A | 5 | 834 | 5 | 25 | 879 | N/A | N/A |  |
| DC_5A-66B_n2A DC_5A-66A_n2(2A) | 66 | N/A | 5 | N/A | 2132 | 7.2 | IMD4 |  |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
| DC_5A-66A_n7A DC_5A-66A-66A_n7A | 5 | N/A | 5 | N/A | 880 | 18.0 | IMD3 |  |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
| DC_5A-66A_n25A | 5 | 834 | 5 | 25 | 879 | N/A | N/A |  |
|  | 66 | 1732 | 5 | 25 | 2132 | 7.2 | IMD4 |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
| DC_5A-66A_n30A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2125 | 4 | IMD5 |  |
|  | n30 | 2307.5 | 5 | 50 | 2352.5 | N/A | N/A |  |
| DC_5A-66A_n41A | 5 | 830 | 5 | 25 | 875 | 28.9 | IMD2 |  |
|  | 66 | 1765 | 5 | 25 | 2165 | N/A | N/A |  |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | N/A |  |
|  | 5 | 835 | 5 | 25 | 880 | 18.0 | IMD3 |  |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n41 | 2560 | 10 | 50 | 2560 | N/A | N/A |  |
| DC_5A-66A_n71A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2161 | 13 | IMD3 |  |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 891.5 | 4.2 | IMD5 |  |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |  |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |  |
| DC_5A-66A_n77A | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |  |
| DC_5A-66A_n77C DC_5A-66A_n77(2A) DC_5A-66A-66A_n77A DC_5A-66A-66A_n77C | 66 | N/A | 5 | N/A | 2142 | 13.2 | IMD3 |  |
| DC_5A-66A-66A_n77(2A) | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |  |
| DC_5A-66A_n78A DC_5A-66A_n78(2A) | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |  |
| DC_5A-66A-66A_n78A | 66 | N/A | 5 | N/A | 2142 | 13.2 | IMD3 |  |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |  |
| DC_5A_n66A-n77A | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2142 | 13.2 | IMD3 |  |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |  |
|  | 5 | 845 | 5 | 25 | 890 | N/A | N/A |  |
|  | n66 | 1785 | 5 | 25 | 2185 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3475 | 16.1 | IMD3 |  |
| DC_5A_n66A-n78A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3420 | 16.6 | IMD3 |  |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2142 | 13.2 | IMD3 |  |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |  |
| DC_7A_n1A-n28A | 7 | 2535 | 5 | 25 | 2655 | N/A | N/A |  |
| DC_7C-n1A-n28A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 780 | 4.3 | IMD5 |  |
| DC_7A_n1A-n40A | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |  |
|  | n40 | 2335 | 5 | 25 | 2335 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2130 | 15.2 | IMD3 |  |
| DC_7A_n1A-n75A | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | 7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |  |
|  | 75 | N/A | 5 | N/A | 1454.5 | 15.2 | IMD3 |  |
| DC_7A_n1A-n78A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |  |
| DC_7C_n1A-n78A | n1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
| DC_7A_n1A-n78(2A) | n78 | N/A | 10 | N/A | 3390 | 10.1 | IMD4 |  |
| DC_7C_n1A-n78(2A) | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2160 | 9.0 | IMD4 |  |
|  | n78 | 3610 | 10 | 50 | 3610 | N/A | N/A |  |
| DC_7A_n2A-n71A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 630 | 28.7 | IMD2 |  |
| DC_7A_n2A-n77A | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n2 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |  |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | N/A |  |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n77 | 3775 | 10 | 50 | 3775 | 4.2 | IMD5 |  |
| DC_7A_n2A-n78A | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1950 | 8.6 | IMD4 |  |
|  | n78 | 3525 | 10 | 50 | 3525 | N/A | N/A |  |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3775 | 4.2 | IMD5 |  |
| DC_7A_n3A-n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
| DC_7C_n3A-n78A | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
| DC_7A_n3A-n78(2A) | n78 | N/A | 10 | N/A | 3390 | 16.1 | IMD3 |  |
| DC_7C_n3A-n78(2A) | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1820 | 15.6 | IMD3 |  |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |  |
| DC_7A_n8A-n40A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |  |
|  | n8 | 905 | 5 | 25 | 950 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A | 2345 | 3.0 | IMD5 |  |
| DC_7A-8A_n3A | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |  |
|  | 7 | 2530 | 10 | 50 | 2650 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 940 | 18.0 | IMD3 |  |
| DC_7A-8A_n3A | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | 8 | 890 | 5 | 25 | 935 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A | 2670 | 29.0 | IMD2+IMD33 |  |
| DC_7A-8A_n20A | 7 | N/A | 5 | N/A | 2640 | 21.1 | IMD34,15 |  |
|  | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n20 | 840 | 5 | 25 | 799 | N/A | N/A |  |
|  | 7 | 2503 | 5 | 25 | 2623 | N/A | N/A |  |
|  | n20 | 859 | 5 | 25 | 818 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 933 | 4.4 | IMD5 |  |
| DC_7A-8A_n77A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 940 | 3.1 | IMD5 |  |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |  |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 940 | 30.5 | IMD2 |  |
|  | n77 | 3470 | 10 | 50 | 3470 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2650 | 28 | IMD2 |  |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |  |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | N/A |  |
| DC_7A-8A_n78A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |  |
| DC_7A-8B_n78A | 8 | N/A | 5 | N/A | 940 | 30.5 | IMD2 |  |
| DC_7A-7A-8B_n78A | n78 | 3470 | 10 | 50 | 3470 | N/A | N/A |  |
|  | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 940 | 3.1 | IMD5 |  |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2650 | 28 | IMD2 |  |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |  |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | N/A |  |
| DC_7A_n8A-n78A | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |  |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3455 | 28.5 | IMD2 |  |
|  | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |  |
|  | n8 | N/A | 5 | N/A | 945 | 29.7 | IMD2 |  |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |  |
| DC_7A-12A_n2A | 7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |  |
| DC_7A-12A_n2(2A) | 12 | N/A | 5 | N/A | 731.5 | 5.3 | IMD5 |  |
|  | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2621 | 30.8 | IMD2 |  |
|  | 12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |  |
|  | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
| DC_7A-12A_n25A | 7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |  |
|  | 12 | N/A | 5 | N/A | 731.5 | 5.3 | IMD5 |  |
|  | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2622.5 | 30.8 | IMD2 |  |
|  | 12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |  |
|  | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
| DC_7A-12A_n66A | 7 | 2515 | 5 | 25 | 2635 | N/A | N/A |  |
|  | 12 | N/A | 5 | N/A | 742 | 31 | IMD2 |  |
|  | n66 | 1773 | 5 | 25 | 2173 | N/A | N/A |  |
| DC_7A_n12A-n77A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n12 | N/A | 5 | N/A | 740 | 30.8 | IMD2 |  |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 7 | 2505 | 5 | 25 | 2625 | N/A | N/A |  |
|  | n12 | 702 | 5 | 25 | 732 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3909 | 16.0 | IMD3 |  |
| DC_7A-12A_n77A DC_7A-12A_n77(2A) | 7 | N/A | 5 | N/A | 2662 | 29.6 | IMD21 |  |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |  |
|  | n77 | 3370 | 10 | 50 | 3370 | N/A | N/A |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | 12 | N/A | 5 | N/A | 740 | 30.8 | IMD2 |  |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
| DC_7A-12A_n78A DC_7A-12A_n78(2A) | 7 | N/A | 5 | N/A | 2662 | 29.6 | IMD2 |  |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |  |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | 12 | N/A | 5 | N/A | 740 | 30.8 | IMD24 |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
| DC_7A_n12A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n12 | 710 | 5 | 25 | 740 | 30.8 | IMD24 |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 7 | 2505 | 5 | 25 | 2625 | N/A | N/A |  |
|  | n12 | 673 | 5 | 25 | 732 | N/A | N/A |  |
|  | n78 | 3664 | 10 | 50 | 3664 | 10.3 | IMD4 |  |
| DC_7A-13A_n66A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |  |
|  | 13 | N/A | 5 | N/A | 750 | 31 | IMD2 |  |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2660 | 18 | IMD3 |  |
|  | 13 | 780 | 5 | 25 | 749 | N/A | N/A |  |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
| DC_7A-13A_n25A DC_7A-7A-13A_n25A DC_7C-13A_n25A | 7 | N/A | 10 | N/A | 2662 | 27.6 | IMD2 |  |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
| DC_7A-20A_n1A DC_7C-20A_n1A | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |  |
|  | 20 | N/A | 10 | N/A | 800 | 4.5 | IMD5 |  |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
| DC_7A-20A_n3A | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |  |
|  | 20 | N/A | 10 | N/A | 806 | 10.5 | IMD2 |  |
|  | n3 | 1737 | 5 | 25 | 1832 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A | 2630 | 26.0 | IMD21 |  |
|  | 20 | 855 | 5 | 25 | 896 | N/A | N/A |  |
|  | n3 | 1775 | 10 | 50 | 1870 | N/A | N/A |  |
| DC_7A-20A_n7A | 7 | N/A | 5 | N/A | 2631.5 | 2.5 | IMD5 |  |
|  | 20 | 834.5 | 5 | 25 | 793.5 | N/A | N/A |  |
|  | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | N/A |  |
| DC_7A-20A_n8A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n8 | 885 | 5 | 25 | 930 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A | 795 | 17.4 | IMD3 |  |
|  | 7 | N/A | 5 | N/A | 2640 | 21.1 | IMD3 |  |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2624 | 18.8 | IMD3 |  |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | 20 | 857 | 5 | 25 | 816 | N/A | N/A |  |
| DC_7A-20A_n28A DC_7C-20A_n28A | 20 | 842 | 5 | 25 | 801 | N/A | N/A |  |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A | 2640 | 5.9 | IMD5 |  |
| DC_7A-20A_n78A DC_7A-7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
| DC_7A-20A_n78(2A) | 20 | N/A | 5 | N/A | 810 | 30.5 | IMD2 |  |
| DC_7A-20A_n78C | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |  |
|  | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A | 810 | 3.0 | IMD5 |  |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2675 | 30.8 | IMD2 |  |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |  |
| DC_7A_n25A-n71A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 630 | 28.7 | IMD2 |  |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
|  | n25 | 1910 | 5 | 25 | 1990 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 630 | 5 | IMD5 |  |
| DC_7A-25A_n77A DC_7A-7A-25A_n77A DC_7C-25A_n77A DC_7C-25A-25A_n77A DC_7A-25A-25A_n77A DC_7A-7A-25A-25A_n77A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A | 1950 | 8.6 | IMD4 |  |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2660 | 3.4 | IMD5 |  |
|  | 25 | 1860 | 5 | 25 | 1940 | N/A | N/A |  |
|  | n77 | 4120 | 10 | 50 | 4120 | N/A | N/A |  |
| DC_7A-25A_n78A DC_7A-7A-25A_n78A DC_7C-25A_n78A DC_7A-25A-25A_n78A DC_7A-7A-25A-25A_n78A DC_7C-25A-25A_n78A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A | 1950 | 8.6 | IMD4 |  |
|  | n78 | 3525 | 10 | 50 | 3525 | N/A | N/A |  |
| DC_7A-26A_n78A | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |  |
| DC_7C-26A_n78A | 26 | 844 | 5 | 25 | 889 | N/A | N/A |  |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |  |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
|  | 26 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |  |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |  |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |  |
|  | 26 | 830 | 5 | 25 | 875 | 3.3 | IMD5 |  |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |  |
| DC_7A_n26A-n78A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
| DC_7C_n26A-n78A | n26 | N/A | 5 | N/A | 879 | 30.2 | IMD2 |  |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |  |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |  |
|  | n26 | N/A | 5 | N/A | 875 | 3.3 | IMD5 |  |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |  |
|  | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |  |
|  | n26 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3375 | 29.7 | IMD2 |  |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
|  | n26 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3430 | 9.7 | IMD4 |  |
| DC_7A-28A_n1A | 7 | 2535 | 5 | 25 | 2655 | N/A | N/A |  |
| DC_7A-7A-28A_n1A | 28 | N/A | 5 | N/A | 780 | 4.3 | IMD5 |  |
|  | n1 | 1950 | 5 | 25 | 2165 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2665 | 29.0 | IMD2 |  |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |  |
|  | n1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
| DC_7A-28A_n2A | 7 | N/A | 10 | N/A | 2630 | 27.6 | IMD2 |  |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
| DC_7A-28A_n3A DC_7C-28A_n3A | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 796.0 | 20.0 | IMD2 |  |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2685 | 18 | IMD3 |  |
|  | 28 | 745 | 5 | 25 | 800 | N/A | N/A |  |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 | 5 | 25 | 2725 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 776 | 4.4 | IMD5 |  |
|  | n5 | 829 | 5 | 25 | 854 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2630 | 5.9 | IMD5 |  |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |  |
|  | n5 | 840 | 5 | 25 | 874 | N/A | N/A |  |
| DC_7A-28A_n20A | 7 | N/A | 5 | N/A | 2640 | 5.9 | IMD5 |  |
|  | 28 | 728 | 5 | 25 | 783 | N/A | N/A |  |
|  | n20 | 842 | 5 | 25 | 801 | N/A | N/A |  |
|  | 7 | 2505 | 5 | 25 | 2625 | N/A | N/A |  |
|  | n20 | 859 | 5 | 25 | 818 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 787 | 17.4 | IMD3 |  |
| DC_7A-28A_n40A | 7 | N/A | 5 | N/A | 2630 | 5.9 | IMD5 |  |
|  | 28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |  |
| DC_7A-28A_n66A DC_7C-28A_n66A | 7 | N/A | 10 | N/A | 2682 | 16.9 | IMD3 |  |
|  | 28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
|  | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 796 | 20.0 | IMD2 |  |
|  | n66 | 1747 | 5 | 25 | 2147 | N/A | N/A |  |
| DC_7A-28A_n78A | 7 | 2567.5 | 5 | 25 | 2687.5 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 782.5 | 28.8 | IMD2 |  |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |  |
|  | 7 | 2567.5 | 5 | 25 | 2687.5 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 782.5 | 3.0 | IMD5 |  |
|  | n78 | 3460 | 10 | 50 | 3460 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2650 | 30.5 | IMD2 |  |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |  |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |  |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3310 | 29.7 | IMD2 |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 800 | 28.8 | IMD2 |  |
| DC_7A-29A_n78A DC_7C-29A_n78A DC_7A-7A-29A_n78A | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |  |
|  | 29 | N/A | N/A | N/A | 720 | 3.0 | IMD5 |  |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |  |
| DC_7A-32A_n1A | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | 7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A | 1454.5 | 15.2 | IMD3 |  |
| DC_7A-32A_n3A | 7 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | n3 | 2510 | 10 | 50 | 2630 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A | 1470 | 10.5 | IMD4 |  |
| DC_7A-32A_n78A | n78 | 3560.5 | 10 | 50 | 3560.5 | N/A | N/A |  |
|  | 7 | 2517.5 | 5 | 25 | 2637.5 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A | 1474.5 | 17.6 | IMD3 |  |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A | 1492 | 4.9 | IMD4 |  |
| DC_7A-40A_n1A DC_7A-40C_n1A | n1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2650 | 32.1 | IMD3 |  |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |  |
| DC_7A_n40A-n77A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |  |
| DC_7A_n40A-n77(2A) | n40 | N/A | 5 | N/A | 2360 | 9.2 | IMD4 |  |
| DC_7A-7A_n40A-n77A DC_7A-7A_n40A-n77(2A) | n77 | 3700 | 10 | 50 | 3700 | N/A | N/A |  |
| DC_7A-40A_n78A DC_7A-40C_n78A | 7 | N/A | 5 | N/A | 2630 | 10.1 | IMD4 |  |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |  |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | N/A |  |
|  | 7 | 2510 | 5 | 25 | 2630 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A | 2310 | 8.7 | IMD4 |  |
|  | n78 | 3785 | 10 | 50 | 3785 | N/A | N/A |  |
| DC_7A_n40A-n78A | 7 | 2520 | 10 | 50 | 2640 | N/A | N/A |  |
| DC_7A_n40A-n78C | n40 | N/A | 5 | N/A | 2360 | 8.7 | IMD4 |  |
| DC_7A-7A_n40A-n78A DC_7A-7A_n40A-n78C | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |  |
| DC_7A-46A_n78A6 | 7 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD5 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_7A-66A_n5A DC_7C-66A_n5A DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A DC_7A-7A-66A_n5A DC_7A-7A-66A-66A_n5A | 7 | N/A | 10 | N/A | 2625 | 30.0 | IMD26 |  |
|  | 66 | 1775 | 10 | 50 | 2175 | N/A | N/A |  |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |  |
| DC_7A-66A_n7A DC_7A-66A-66A_n7A | 7 | N/A | 10 | N/A | 2675 | 15 | IMD4 |  |
|  | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |  |
|  | n7 | 2515 | 10 | 50 | 2635 | N/A | N/A |  |
| DC_7A-66A_n28A | 7 | N/A | 5 | N/A | 2685 | 18.0 | IMD3 |  |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |  |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |  |
| DC_7A-66A_n77A DC_7A-7A-66A_n77A DC_7A-7A-66A_n77(2A) DC_7A-66A_n77(2A) DC_7C-66A_n77A DC_7C-66A_n77(2A) | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2150 | 8.7 | IMD4 ¦2*fB7-2*fn77¦ |  |
|  | n77 | 3625 | 10 | 50 | 3475 | N/A | N/A |  |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2670 | 5.2 | IMD5 |  |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |  |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A | 2640 | 3.4 | IMD5 |  |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |  |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2150 | 8.7 | IMD4 |  |
|  | n77 | 3625 | 10 | 50 | 3625 | N/A | N/A |  |
|  | 7 | 2542 | 5 | 25 | 2662 | N/A | N/A |  |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3344 | 16.0 | IMD3 |  |
|  | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |  |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 4040 | 4.2 | IMD5 |  |
| DC_7A-66A_n78A DC_7C-66A_n78A DC_7A-7A-66A_n78A DC_7A-66A-66A_n78A DC_7A-7A-66A-66A_n78A DC_7C-66A-66A_n78A DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) DC_7A-7A-66A_n78(2A) DC_7A-66A-66A_n78(2A) DC_7A-7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |  |
|  | 66/n66 | N/A | 5 | N/A | 2150 | 8.7 | IMD4 |  |
|  | n78 | 3625 | 10 | 50 | 3475 | N/A | N/A |  |
| DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A | 7 | 2542 | 5 | 25 | 2662 | N/A | N/A |  |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3344 | 16.0 | IMD3 |  |
| DC_7A-71A_n2A | n2 | 1859 | 5 | 25 | 1933 | N/A | N/A |  |
| DC_7A-71A_n2(2A) | 7 | 2505 | 5 | 25 | 2625 | N/A | N/A |  |
|  | 71 | N/A | 5 | N/A | 646 | 30.8 | IMD2 |  |
| DC_7A-71A_n25A | 7 | 2530 | 5 | 25 | 2530 | N/A | N/A |  |
|  | 71 | N/A | 5 | N/A | 630 | 28.7 | IMD24 |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
| DC_7A-71A_n77A DC_7A-71A_n77(2A) | 7 | N/A | 5 | N/A | 2670 | 29.6 | IMD21 |  |
|  | 71 | 680 | 5 | 25 | 634 | N/A | N/A |  |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | N/A |  |
| DC_7A_n71A-n77A | 7 | 2505 | 5 | 25 | 2625 | N/A | N/A |  |
|  | n71 | 666 | 5 | 25 | 620 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3837 | 16.0 | IMD3 |  |
| DC_7A-71A_n78A DC_7A-71A_n78(2A) | 7 | N/A | 5 | N/A | 2670 | 29.6 | IMD2 |  |
|  | 71 | 680 | 5 | 25 | 634 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |  |
|  | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |  |
|  | 71 | N/A | 5 | N/A | 640 | 3.0 | IMD5 |  |
|  | n78 | 3490 | 10 | 50 | 3490 | N/A | N/A |  |
| DC_7A_n71A-n78A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
|  | n71 | 693 | 5 | 25 | 647 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3714 | 9.7 | IMD4 |  |
|  | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |  |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 625 | 3.9 | IMD5 |  |
| DC_7A_n75A-n78A | n78 | 3560.5 | 10 | 50 | 3560.5 | N/A | N/A |  |
|  | 7 | 2517.5 | 5 | 25 | 2637.5 | N/A | N/A |  |
|  | n75 | N/A | 5 | N/A | 1474.5 | 17.6 | IMD3 |  |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n75 | N/A | 5 | N/A | 1492 | 4.9 | IMD4 |  |
| DC_7A_n78A-n79A DC_7A_n78A-n79C DC_7A-7A_n78A-n79A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |  |
|  | n78 | 3600 | 10 | 50 | 3600 | N/A | N/A |  |
|  | n79 | N/A | 10 | N/A | 4680 | 20.6 | IMD34,9,13 |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3770 | 6.4 | IMD413 |  |
|  | n79 | 4450 | 10 | 50 | 4450 | N/A | N/A |  |
| DC_7A_SUL_n78A-n80A | n80 | 1730 | 5 | 25 |  | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A | 2655 | 13 | IMD4 |  |
| DC_7_n78-n105 | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3700 | 9.7 | IMD4 |  |
|  | n105 | 670 | 5 | 25 | 619 | N/A | N/A |  |
|  | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |  |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |  |
|  | n105 | N/A | 5 | N/A | 625 | 3.9 | IMD5 |  |
| DC_8A_n1A-n28A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n1 | 1965 | 5 | 25 | 2155 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 765 | 11.6 | IMD4 |  |
| DC_8A_n1A-n40A | 8 | 885 | 5 | 25 | 930 | N/A | N/A |  |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2135 | 3.3 | IMD5 |  |
| DC_8A_n1A-n77A DC_8B_n1A-n77A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n1 | 1945 | 5 | 25 | 2135 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3745 | 14.9 | IMD31 |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n77 | 3960 | 10 | 50 | 3960 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2140 | 14.4 | IMD3 |  |
| DC_8A_n1A-n78A DC_8B_n1A-n78A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n1 | 1945 | 5 | 25 | 2135 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3745 | 14.9 | IMD3 |  |
| DC_8A_n1A-n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n1 | 1955 | 5 | 25 | 2145 | 8.2 | IMD4 |  |
|  | n79 | 4845 | 40 | 216 | 4845 | N/A | N/A |  |
| DC_8A-(n)3AA | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |  |
| DC_8A-(n)3CA | 3 | N/A | 5 | N/A | 1835 | 4.5 | IMD5 |  |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |  |
| DC_8A_n3A-n28A | 8 | 912.5 | 5 | 25 | 957.5 | N/A | N/A |  |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 800 | 30.4 | IMD2 |  |
| DC_8A_n3A-n77A DC_8A_n3A-n77(2A) | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
| DC_8B_n3A-n77A | n3 | 1740 | 5 | 25 | 1835 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3540 | 16.3 | IMD3 |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1820 | 16.5 | IMD3 |  |
| DC_8A_n3A-n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
| DC_8A_n3A-n78(2A) | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3550 | 16.1 | IMD3 |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3370 | 4.5 | IMD5 |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1820 | 15.7 | IMD3 |  |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |  |
| DC_8A_n3A-n79A | 8 | 885 | 5 | 25 | 930 | N/A | N/A |  |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4425 | 15.7 | IMD39 |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1850 | 8.8 | IMD4 |  |
| DC_8A_n7A-n78A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n7 | 2555 | 5 | 25 | 2675 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3455 | 28.5 | IMD21 |  |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2650 | 28 | IMD2 |  |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | N/A |  |
| DC_8A-11A_n1A | 11 | 1435 | 5 | 25 | 1483 | N/A | N/A |  |
| DC_8B-11A_n1A | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 930 | 16.6 | IMD35 |  |
| DC_8A-11A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
| DC_8A-11A_n77(2A) DC_8B-11A_n77A DC_8B-11A_n77(2A) | n77 | 3311 | 10 | 50 | 3311 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A | 1491 | 18.8 | IMD3 |  |
|  | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |  |
|  | n77 | 3791 | 10 | 50 | 3791 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 930 | 18.2 | IMD3 |  |
| DC_8A-11A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A | 1491 | 18.8 | IMD3 |  |
|  | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |  |
|  | n78 | 3791 | 10 | 50 | 3791 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 930 | 18.2 | IMD3 |  |
| DC_8A-11A_n79A | 8 | 882.5 | 5 | 25 | 927.5 | N/A | N/A |  |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A | 1478.4 | 1.2 | IMD5 |  |
|  | 11 | 1435 | 5 | 25 | 1483 | N/A | N/A |  |
|  | n79 | 4810 | 40 | 216 | 4810 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 930 | 2.8 | IMD5 |  |
| DC_8-20_n1 | n1 | 1925 | 5 | 25 | 2115 | N/A | N/A |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A | 805 | 11.5 | IMD4 |  |
| DC_8-20_n3 | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A | 810 | 27 | IMD24 |  |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | 8 | 890 | 5 | 25 | 930 | 27 | IMD24 |  |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |  |
| DC_8A-20A_n28A | 8 | N/A | 5 | N/A | 946 | [23.5] | IMD3 |  |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |  |
|  | n28 | 728 | 5 | 25 | 773 | N/A | N/A |  |
| DC_8A-20A_n78A | 8 | 890 | 5 | 25 | 935 | N/A | N/A |  |
|  | n78 | 3470 | 10 | 50 | 3470 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A | 800 | 12.1 | IMD4 |  |
|  | 8 | N/A | 5 | N/A | 940 | 12.1 | IMD4 |  |
|  | n78 | 3481 | 10 | 50 | 3481 | N/A | N/A |  |
|  | 20 | 847 | 5 | 25 | 806 | N/A | N/A |  |
| DC_8A-28A_n1A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 780 | 9.4 | IMD4 |  |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 942 | 3.3 | IMD5 |  |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |  |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
| DC_8A-28A_n3A | 8 | 912.5 | 5 | 25 | 957.5 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 800 | 30.4 | IMD24 |  |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
| DC_8A-28A_n40A | 8 | N/A | 5 | N/A | 928 | 17.0 | IMD3 |  |
| DC_8A-28C_n40A | 28 | 706 | 5 | 25 | 761 | N/A | N/A |  |
|  | n40 | 2340 | 10 | 50 | 2340 | N/A | N/A |  |
| DC_8A_n28A-n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
| DC_8A-28A_n77A | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
| DC_8A-28A_n77(2A) | n77 | N/A | 10 | N/A | 3473 | 10.3 | IMD4 |  |
| DC_8A-28C_n77(2A) | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | 28/n28 | N/A | 5 | N/A | 765 | 11.6 | IMD4 |  |
|  | n77 | 3495 | 10 | 50 | 3495 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 935 | 4.3 | IMD5 |  |
|  | 28 | 713 | 5 | 25 | 768 | N/A | N/A |  |
|  | n77 | 3787 | 10 | 50 | 3787 | N/A | N/A |  |
| DC_8A_n28A-n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
| DC_8A_n28A-n78(2A) | n28 | 725 | 5 | 25 | 780 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3455 | 10.3 | IMD4 |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 765 | 11.6 | IMD4 |  |
|  | n78 | 3495 | 10 | 50 | 3495 | N/A | N/A |  |
| DC_8A_n28A-n79A | 8 | 912.5 | 5 | 25 | 957.5 | N/A | N/A |  |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4420 | 0.0 | IMD5 |  |
|  | 8 | 905 | 5 | 25 | 950 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 800 | 3.9 | IMD5 |  |
| DC_8A-32A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A | 1491 | 18.8 | IMD3 |  |
| DC_8A-38A_n28A | 8 | 912.5 | 5 | 25 | 957.5 | N/A | N/A |  |
|  | 38 | N/A | 10 | N/A | 2575 | 14.4 | IMD3 |  |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |  |
| DC_8A-38A_n78A | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |  |
|  | 38 | N/A | 5 | N/A | 2602.5 | 28 | IMD2 |  |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 927.5 | 29 | IMD2 |  |
|  | 38 | 2572.5 | 5 | 25 | 2572.5 | N/A | N/A |  |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 937.5 | 3.1 | IMD5 |  |
|  | 38 | 2572.5 | 5 | 25 | N/A | N/A | N/A |  |
|  | n78 | 3390 | 10 | 50 | N/A | N/A | N/A |  |
| DC_8A-39A_n40A | 8 | 895 | 5 | 25 | 940 | 8.6 | IMD4 |  |
|  | 39 | 1900 | 5 | 25 | 1900 | N/A | N/A |  |
|  | n40 | 2370 | 5 | 25 | 2370 | N/A | N/A |  |
|  | 8 | 885 | 5 | 25 | 930 | 4.9 | IMD5 |  |
|  | 39 | 1890 | 5 | 25 | 1890 | N/A | N/A |  |
|  | n40 | 2370 | 5 | 25 | 2370 | N/A | N/A |  |
| DC_8-39_n79 | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |  |
|  | 39 | N/A | 5 | N/A | 1907.5 | 13.8 | IMD4 |  |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | N/A |  |
|  | 8 | 895 | 5 | 25 | 940 | 15.1 | IMD3 |  |
|  | 39 | 1900 | 10 | 50 | 1900 | N/A | N/A |  |
|  | n79 | 4740 | 40 | 216 | 4740 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 940 | 7.1 | IMD4 |  |
|  | 39 | 1900 | 5 | 25 | 1900 | N/A | N/A |  |
|  | n79 | 4760 | 40 | 216 | 4760 | N/A | N/A |  |
| DC_8A_n39A-n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4680 | 15.9 | IMD3 |  |
|  | 8 | 890 | 5 | 25 | 935 | N/A | N/A |  |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4560 | 12.1 | IMD4 |  |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |  |
|  | n39 | N/A | 10 | N/A | 1907.5 | 13.8 | IMD4 |  |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | N/A |  |
| DC_8A-40A_n1A DC_8A-40C_n1A | 8 | N/A | 5 | N/A | 930 | 8.0 | IMD4 |  |
|  | 40 | 2395 | 5 | 25 | 2395 | N/A | N/A |  |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
| DC_8A-40A_n28A | 8 | 895 | 5 | 25 | 940 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A | 2335 | 18.8 | IMD3 |  |
|  | n28 | 720 | 5 | 25 | 775 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 928 | 17.0 | IMD3 |  |
|  | 40 | 2340 | 5 | 25 | 2340 | N/A | N/A |  |
|  | n28 | 706 | 5 | 25 | 761 | N/A | N/A |  |
| DC_8A_n40A-n41A DC_8A_n40A-n41C | 8 | 895 | 5 | 25 | 940 | N/A | N/A |  |
|  | n40 | 2355 | 5 | 25 | 2355 | 4.9 | IMD5 |  |
|  | n41 | N/A | 10 | N/A | 2520 | N/A | N/A |  |
| DC_8_n40-n71 | 8 | 882.5 | 5 | 25 | 927.5 | N/A | N/A |  |
|  | n40 | 2397.5 | 5 | 25 | 2397.5 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 632.5 | 17 | IMD3 |  |
|  | 8 | 912.5 | 5 | 25 | 957.5 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A | 2302.5 | 18 | IMD3 |  |
|  | n71 | 695 | 5 | 25 | 649 | N/A | NA |  |
| DC_28A_n40A-n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
| DC_28C_n40A-n77A | n40 | N/A | 10 | N/A | 2395 | 28 | IMD21 |  |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n40 | 2395 | 10 | 50 | 2395 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3305 | 28.8 | IMD21 |  |
| DC_8A-40A_n78A DC_8A-40C_n78A | 8 | N/A | 5 | N/A | 950 | 30.5 | IMD2 |  |
|  | 40 | 2380 | 5 | 25 | 2380 | N/A | N/A |  |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 935 | 19.8 | IMD3 |  |
|  | 40 | 2320 | 5 | 25 | 2320 | N/A | N/A |  |
|  | n78 | 3705 | 10 | 50 | 3705 | N/A | N/A |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A | 2395 | 28 | IMD2 |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
| DC_8A_n40A-n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |  |
|  | n78 | 3305 | 10 | 50 | 3307.5 | 28.7 | IMD21 |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n40 | 2395 | 5 | 25 | 2395 | 27.3 | IMD2 |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
| DC_8A_n40A-n79A | 8 | 885 | 5 | 25 | 930 | N/A | N/A |  |
|  | n40 | 2305 | 5 | 25 | 2305 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4960 | 10.7 | IMD4 |  |
|  | 8 | 885 | 5 | 25 | 930 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A | 2305 | 9.2 | IMD4 |  |
|  | n79 | 4960 | 40 | 216 | 4960 | N/A | N/A |  |
| DC_8A-41A_n1A | 41 | 2500 | 5 | 25 | 2500 | N/A | N/A |  |
| DC_8A-41C_n1A | n1 | 1977 | 5 | 25 | 2167 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 931 | 4.5 | IMD5 |  |
| DC_8A-41A_n3A | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
| DC_8A-41C_n3A | 8 | 885 | 5 | 25 | 930 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2665 | 27.4 | IMD21 |  |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 950 | 28.9 | IMD21 |  |
|  | 41 | 2665 | 5 | 25 | 2665 | N/A | N/A |  |
| DC_8A-41A_n77A | 8 | N/A | 5 | N/A | 950 | 29.1 | IMD21, 4 |  |
| DC_8A-41C_n77A | 41 | 2630 | 10 | 50 | 2630 | N/A | N/A |  |
|  | n77 | 3580 | 10 | 50 | 3580 | N/A | N/A |  |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2650 | 28.0 | IMD2 |  |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | N/A |  |
| DC_8A-41A_n78A | 8 | N/A | 5 | N/A | 950 | 29.1 | IMD24 |  |
| DC_8A-41C_n78A | 41 | 2630 | 5 | 25 | 2630 | N/A | N/A |  |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |  |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2650 | 28.0 | IMD2 |  |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | N/A |  |
| DC_8A_n41A-n78A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n41 | 2555 | 5 | 25 | 2555 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3455 | 28.5 | IMD21 |  |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2650 | 28 | IMD2 |  |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | N/A |  |
| DC_8A_n41A-n79A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4470 | 16.3 | IMD3 |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A | 2650 | 15.5 | IMD3 |  |
|  | n79 | 4470 | 40 | 216 | 4470 | N/A | N/A |  |
| DC_8A-42A_n1A | 42 | 3405 | 10 | 50 | 3405 | N/A | N/A |  |
| DC_8A-42C_n1A | n1 | 1955 | 5 | 25 | 2145 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 945 | 3.3 | IMD5 |  |
| DC_8A-42A_n3A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A | 3540 | 16.3 | IMD3 |  |
| DC_8A-42A_n28A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A | 3443 | 8.7 | IMD4 |  |
| DC_8A-42A_n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n79 | 4470 | 40 | 216 | 4470 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A | 3570 | 34.8 | IMD2 |  |
| DC_8A_n71A-n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n71 | 674 | 5 | 25 | 628 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 340524 | 10.3 | IMD4 |  |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n71 | 674 | 5 | 25 | 628 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 36056 | 4 | IMD5 |  |
|  | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 634.5 | 11.6 | IMD4 |  |
|  | n77 | 3334.5 | 10 | 50 | 3334.5 | N/A | N/A |  |
|  | 8 | 882.5 | 5 | 25 | 927.5 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 640 | 14.49 | IMD5 |  |
|  | n77 | 418570 | 10 | 50 | 418570 | N/A | N/A |  |
| DC_8A_SUL_n78A-n80A | n80 | 1755 | 10 | 50 |  | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A | 945 | 8 | IMD4 |  |
|  | n80 | 1750 | 10 | 50 |  | N/A | N/A |  |
|  | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3550 | 8 | IMD33 |  |
| DC_11A_n1A-n3A | 11 | 1430 | 5 | 25 | 1478 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2130 | 17.7 | IMD3 |  |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
| DC_11A_n1A-n77A DC_11A_n1A-n77(2A) | 11 | 1435 | 5 | 25 | 1483 | N/A | N/A |  |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3375 | 29.6 | IMD21 |  |
|  | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |  |
|  | n77 | 3578 | 10 | 50 | 3578 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2140 | 30.8 | IMD21 |  |
| DC_11A_n3A-n28A | 11 | 1435 | 5 | 25 | 1483 | N/A | N/A |  |
|  | n3 | 1753 | 5 | 25 | 1848 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 800 | 3.0 | IMD5 |  |
| DC_11A_n3A-n77A DC_11A_n3A-n77(2A) | 11 | 1440 | 5 | 25 | 1488 | N/A | N/A |  |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3780 | 10.8 | IMD4 |  |
|  | 11 | 1440 | 5 | 25 | 1488 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1870 | 29.0 | IMD2 |  |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |  |
| DC_11A_n3A-n79A | 11 | 1435 | 5 | 25 | 1483 | N/A | N/A |  |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4640 | 16.2 | IMD3 |  |
|  | 11 | 1435 | 5 | 25 | 1483 | N/A | N/A |  |
|  | n79 | 4735 | 40 | 216 | 4735 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1865 | 17.8 | IMD3 |  |
| DC_11A-18A_n77A | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |  |
| DC_11A-18A_n77(2A) | n77 | 3706 | 10 | 50 | 3706 | N/A | N/A |  |
|  | 18 | N/A | 5 | N/A | 865 | 18.7 | IMD3 |  |
| DC_11A-18A_n78A | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |  |
| DC_11A-18A_n78(2A) | n78 | 3706 | 10 | 50 | 3706 | N/A | N/A |  |
|  | 18 | N/A | 5 | N/A | 865 | 18.7 | IMD3 |  |
| DC_11A_n28A-n77A DC_11A_n28A-n77(2A) | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3629 | 17.5 | IMD3 |  |
|  | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |  |
|  | n77 | 3684 | 10 | 50 | 3684 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 798 | 15.8 | IMD3 |  |
| DC_12A_n2A-n7A | 12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |  |
|  | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2622.5 | 30.8 | IMD2 |  |
| DC_12A_n2A-n38A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n38 | N/A | 10 | N/A | 2608 | 28.7 | IMD2 |  |
| DC_12A_n2A-n41A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2608 | 28.7 | IMD2 |  |
| DC_12A_n2A-n66A | 12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |  |
|  | n2 | 1907.5 | 5 | 25 | 1987.5 | 2 | IMD4 |  |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
| DC_12A_n2A-n77A | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |  |
|  | n2 | 1880 | 5 | 25 | 1960 | 16.5 | IMD3 |  |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | N/A |  |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |  |
|  | n2 | 1890 | 5 | 25 | 1970 | 12 | IMD4 |  |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | N/A |  |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n77 | 3315 | 10 | 50 | 3315 | 16.0 | IMD34, |  |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |  |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | N/A |  |
|  | n77 | 4000 | 10 | 50 | 4000 | 12 | IMD4 |  |
| DC_12_n2-n78 | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1960 | 16.5 | IMD3 |  |
|  | n78 | 3375 | 10 | 50 | 3375 | N/A | N/A |  |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3315 | 16.0 | IMD3 |  |
| DC_12A_n7A-n25A | 12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2626 | 30.8 | IMD2 |  |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |  |
| DC_12A_n7A-n77A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2662 | 29.6 | IMD2 |  |
|  | n77 | 3370 | 10 | 50 | 3370 | N/A | N/A |  |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2670 | 13.2 | IMD3 |  |
|  | n77 | 4086 | 10 | 50 | 4086 | N/A | N/A |  |
|  | 12 | 702 | 5 | 25 | 732 | N/A | N/A |  |
|  | n7 | 2505 | 5 | 25 | 2625 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3909 | 16.0 | IMD3 |  |
|  | 12 | 673 | 5 | 25 | 732 | N/A | N/A |  |
|  | n7 | 2505 | 5 | 25 | 2625 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3664 | 10.3 | IMD4 |  |
| DC_12A_n7A-n78A, DC_12A_n7(2A)-n78A DC_12A_n7A-n78(2A) DC_12A_n7(2A)-n78(2A) | 12 | 708 | 5 | 25 | 738 | N/A | N/A |  |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3624 | 9 | IMD4 |  |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |  |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2662 | 29.6 | IMD2 |  |
| DC_12A_n25A-n41A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2608 | 28.7 | IMD2 |  |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A | 1970 | 26 | IMD2 |  |
|  | n41 | 2680 | 10 | 50 | 2680 | N/A | N/A |  |
| DC_12A_n25A-n77A | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A | 1960 | 16.5 | IMD3 |  |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | N/A |  |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A | 1970 | 12.5 | IMD4 |  |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | N/A |  |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3315 | 16.0 | IMD3 |  |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |  |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 4000 | 12 | IMD4 |  |
| DC_12A-30A_n2A | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A | 2353 | 12.0 | IMD4 |  |
|  | n2 | 1885 | 5 | 25 | 1965 | N/A | N/A |  |
| DC_12A-30A_n5A | 12 | 702 | 5 | 25 | 732 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A | 2355 | 18.8 | IMD3 |  |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |  |
| DC_12A-30A_n77A DC_12A-30A_n77(2A) | 12 | N/A | 5 | N/A | 740 | 15.2 | IMD34 |  |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |  |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | N/A |  |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A | 2355 | 13.2 | IMD3 |  |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |  |
| DC_12A_n41A-n66A | 12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |  |
|  | n41 | 2501 | 10 | 50 | 2501 | 20.0 | IMD218 |  |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | N/A |  |
| DC_12A-66A_n5A DC_12A-66A-66A_n5A | 12 | N/A | 5 | N/A | 742 | 9.4 | IMD4 |  |
|  | 66 | 1745 | 5 | 25 | 2145 | N/A | N/A |  |
|  | n5 | 829 | 5 | 25 | 874 | N/A | N/A |  |
| DC_12A-66A_n7A | 12 | N/A | 5 | N/A | 742 | 31 | IMD2 |  |
|  | 66 | 1773 | 5 | 25 | 2173 | N/A | N/A |  |
|  | n7 | 2515 | 5 | 25 | 2635 | N/A | N/A |  |
| DC_12A-66A_n25A | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |  |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A | 1935 | 20 | IMD3 |  |
|  | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2150 | 4 | IMD5 |  |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |  |
|  | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2112.5 | 23 | IMD3 |  |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |  |
| DC_12A-66A_n41A | 12 | N/A | 5 | N/A | 742 | 31 | IMD2 |  |
|  | 66 | 1773 | 5 | 25 | 2173 | N/A | N/A |  |
|  | n41 | 2515 | 5 | 25 | 2515 | N/A | N/A |  |
| DC_12A-66A_n77A DC_12A-66A_n77(2A) | 12 | N/A | 5 | N/A | 740 | 15.2 | IMD311 |  |
| DC_12A-66A-66A_n77A | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
| DC_12A-66A-66A_n77(2A) | n77 | 4180 | 10 | 50 | 4180 | N/A | N/A |  |
|  | 12 | 707 | 5 | 25 | 737 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2126 | 13.2 | IMD3 |  |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |  |
| DC_12A_n66A-n77A | 12 | 707 | 5 | 25 | 737 | N/A | N/A |  |
|  | n66 | 1726 | 5 | 25 | 2126 | 13.2 | IMD3 |  |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |  |
|  | 12 | 704 | 5 | 25 | 734 | N/A | N/A |  |
|  | n66 | 1723 | 5 | 25 | 2123 | N/A | N/A |  |
|  | n77 | 4150 | 10 | 50 | 4150 | 16.0 | IMD32,4 |  |
|  | 12 | 709 | 5 | 25 | 739 | N/A | N/A |  |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | N/A |  |
|  | n77 | 3842 | 10 | 50 | 3842 | 9 | IMD4 |  |
| DC_12A-66A_n78A | 12 | 710 | 5 | 25 | 740 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2160 | 17.1 | IMD3 |  |
|  | n78 | 3580 | 5 | 25 | 3580 | N/A | N/A |  |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | 12 | 703 | 5 | 25 | 733 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2140 | 16.5 | IMD3 |  |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | N/A |  |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | 12 | 703 | 5 | 25 | 733 | N/A | N/A |  |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3754 | 4.1 | IMD5 |  |
| DC_13A_n2A-n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
|  | n2 | 1896 | 5 | 25 | 1976 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3460 | 17.3 | IMD3 |  |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1960 | 16.0 | IMD3 |  |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | N/A |  |
| DC_13A_n5A-n77A11 | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
|  | n77 | 4013 | 10 | 50 | 4013 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A | 885 | 4.5 | IMD5 |  |
| DC_13A_n7A-n78A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
|  | n78 | 3432 | 10 | 50 | 3432 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2650 | 27.9 | IMD2 |  |
|  | 13 | 749 | 5 | 25 | 780 | N/A | N/A |  |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3622 | 9 | IMD4 |  |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
|  | n7 | 2530 | 5 | 25 | 2650 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3312 | 29.0 | IMD2 |  |
| DC_13A_n25A-n66A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
|  | n25 | 1860 | 5 | 25 | 1940 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2156 | 7.2 | IMD4 |  |
| DC_13A_n25A-n66A | 13 | 780 | 5 | 25 | 749 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A | 1940 | 6.2 | IMD4 |  |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | N/A |  |
| DC_13A-46A_n2A5 | 13 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD4 |  |
|  | n2 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_13A-46A_n66A5 | 13 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD4, IMD5 |  |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_13A-46A_n77A5 DC_13A-46A-46A_n77A5 | 13 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD3, IMD4, IMD5 |  |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_13A_n48A-n66A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
|  | n48 | N/A | 5 | N/A | 3584 | 2.8 | IMD5 |  |
|  | n66 | 1716 | 5 | 25 | 2116 | N/A | N/A |  |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2131 | 17.1 | IMD3 |  |
| DC_13A-66A_n2A DC_13A-66A-66A_n2A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
| DC_13A-66B_n2A | 66 | N/A | 5 | N/A | 2156 | 7..2 | IMD4 |  |
| DC_13A-66C_n2A | n2 | 1860 | 5 | 25 | 1940 | N/A | N/A |  |
| DC_13A-66A_n5A | 13 | N/A | 5 | N/A | 750 | 9.4 | IMD4 |  |
| DC_13A-66A-66A_n5A | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |  |
|  | n5 | 840 | 5 | 25 | 885 | N/A | N/A |  |
| DC_13A-66A_n48A DC_13A-66A_n48B DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2131 | 17.1 | IMD3 |  |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |  |
| DC_13A-66A_n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
| DC_13A-66A_n77C DC_13A-66A-66A_n77A DC_13A-66A-66A_n77C | 66 | N/A | 5 | N/A | 2156 | 17.1 | IMD3 |  |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |  |
| DC_13A-66A_n77A11 | 13 | N/A | 5 | N/A | 750 | 15.2 | IMD3 |  |
| DC_13A-66A_n77C11 DC_13A-66A-66A_n77A11 DC_13A-66A-66A_n77C11 | 66 | 1710 | 5 | 25 | 2110 | N/A | N/A |  |
|  | n77 | 4170 | 10 | 50 | 4170 | N/A | N/A |  |
| DC_13A_n66A-n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
|  | n66 | 1731 | 5 | 25 | 2131 | 17.1 | IMD3 |  |
|  | n77 | 3695 | 10 | 50 | 3695 | N/A | N/A |  |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |  |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n77 | 3324 | 10 | 50 | 3324 | 16.4 | IMD34,9 |  |
| DC_14A-30A_n5A | 14 | 795 | 5 | 25 | 765 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A | 2353 | 5.9 | IMD5 |  |
|  | n5 | 827 | 5 | 25 | 872 | N/A | N/A |  |
| DC_14A-30A_n77A DC_14A-30A_n77(2A) | 14 | N/A | 5 | N/A | 763 | 15.2 | IMD34 |  |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |  |
|  | n77 | 3857 | 10 | 50 | 3857 | N/A | N/A |  |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A | 2355 | 13.2 | IMD3 |  |
|  | n77 | 3941 | 10 | 50 | 3941 | N/A | N/A |  |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 | 763 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2162 | 7.6 | IMD4 |  |
|  | n2 | 1874 | 5 | 25 | 1954 | N/A | N/A |  |
| DC_14A-66A_n5A | 14 | N/A | 5 | N/A | 762 | 9.4 | IMD4 |  |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n5 | 834 | 5 | 25 | 879 | N/A | N/A |  |
| DC_14A-66A_n77A DC_14A-66A_n77(2A) | 14 | N/A | 5 | N/A | 763 | 15.2 | IMD311 |  |
| DC_14A-66A-66A_n77A | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
| DC_14A-66A-66A_n77(2A) | n77 | 4188 | 10 | 50 | 4188 | N/A | N/A |  |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2155 | 13.2 | IMD3 |  |
|  | n77 | 3741 | 10 | 50 | 3741 | N/A | N/A |  |
| DC_18A_n3A-n41A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |  |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A | 2540 | 29.4 | IMD2 |  |
|  | 18 | 820 | 5 | 25 | 865 | N/A | N/A |  |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1850 | 28.2 | IMD2 |  |
| DC_18A_n3A-n77A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |  |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3410 | 16.3 | IMD3 |  |
|  | 18 | 820 | 5 | 25 | 865 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1865 | 15.7 | IMD3 |  |
|  | n77 | 3505 | 10 | 50 | 3505 | N/A | N/A |  |
| DC_18A_n3A-n78A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |  |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3390 | 15.2 | IMD33 |  |
| DC_18A-28A_n77A DC_18A_n28A-n77A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |  |
|  | 28/n28 | N/A | 5 | N/A | 778 | 4.4 | IMD5 |  |
|  | n77 | 4058 | 10 | 50 | 4058 | N/A | N/A |  |
| DC_18A-28A_n77A | 18 | N/A | 5 | N/A | 865 | 3.9 | IMD5 |  |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |  |
|  | n77 | 3757 | 10 | 50 | 3757 | N/A | N/A |  |
| DC_18A-28A_n78A | 18 | N/A | 5 | N/A | 864 | 3.8 | IMD5 |  |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |  |
|  | n78 | 3756 | 10 | 50 | 3756 | N/A | N/A |  |
| DC_18A_n28A-n77A DC_18A_n28A-n78A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |  |
|  | n28 | 710 | 5 | 25 | 765 | N/A | N/A |  |
|  | n77/n78 | N/A | 10 | N/A | 3770 | 4.0 | IMD5 |  |
| DC_18A-41A_n3A DC_18A-41C_n3A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |  |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2630 | 16.0 | IMD3 |  |
|  | 18 | N/A | 5 | N/A | 865 | 28.9 | IMD21 |  |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | N/A |  |
|  | 41 | 2630 | 5 | 25 | 2630 | N/A | N/A |  |
| DC_18A_n41A-n77A DC_18A_n41A-n77(2A) DC_18A_n41A-n78A DC_18A_n41A-n78(2A) | 18 | N/A | 5 | N/A | 865 | 3.4 | IMD5 |  |
|  | n77 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |  |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |  |
| DC_18A_n41A-n77A DC_18A_n41A-n78A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |  |
|  | n41 | 2570 | 5 | 25 | 2570 | N/A | N/A |  |
|  | n77/n78 | N/A | 10 | N/A | 3390 | 30.1 | IMD2 |  |
|  | 18 | 820 | 5 | 25 | 865 | N/A | N/A |  |
|  | n77/n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2630 | 28.5 | IMD2 |  |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | N/A | 5 | N/A | 865 | 3.4 | IMD5 |  |
|  | n78 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |  |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |  |
| DC_19A_n1A-n77A DC_19A_n1A-n78A | 19 | 840 | 5 | 25 | 885 | N/A | N/A |  |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | n77/n78 | N/A | 10 | N/A | 3655 | [21.4] | IMD3 |  |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2130 | 17.8 | IMD3 |  |
|  | n77/n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |  |
| DC_19A_n1A-n79A20 |  |  |  | N/A |  |  |  |  |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | N/A | 5 | N/A | 882.5 | 18.7 | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n77, n78 | 3783.3 | 10 | 50 | 3783.3 | N/A | N/A |  |
|  | 19 | N/A | 5 | N/A | 882.5 | 13.2 | IMD4 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n77, n78 | 3468.7 | 10 | 50 | 3468.7 | N/A | N/A |  |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |  |
|  | 21 | N/A | 5 | N/A | 1502.5 | 9.0 | IMD4 |  |
|  | n77 | 4015 | 10 | 50 | 4015 | N/A | N/A |  |
| DC_19A-21A_n79A | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 19 | 837.5 | 5 | 25 | 882.2 | N/A | N/A |  |
|  | 21 | N/A | 5 | N/A | 1500 | 3.8 | IMD5 |  |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | N/A |  |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4515 | 29.3 | IMD2 |  |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3715 | 28.8 | IMD2 |  |
| DC_20A-n1A_n75A | n1 | 1950.5 | 5 | 50 | 2140.5 | N/A | N/A |  |
|  | 20 | 852.5 | 5 | 25 | 811.5 | N/A | N/A |  |
|  | n75 | N/A | 5 | N/A | 1459.5 | 4.0 | IMD5 |  |
| DC_20A_n1A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3630 | 16.0 | IMD3 |  |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2120 | 15.3 | IMD3 |  |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |  |
| DC_20A-(n)3AA | 3 | N/A | 5 | N/A | 1865 | 3 | IMD4 |  |
|  | n3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |  |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |  |
| DC_20_n3-n67 | 20 | 837 | 5 | 25 | 796 | N/A | N/A |  |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | N/A |  |
|  | n67 | N/A | 5 | N/A | 746 | 9.4 | IMD4 |  |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3420 | 16.1 | IMD3 |  |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1860 | 15.7 | IMD3 |  |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | N/A |  |
| DC_20A_n7A-n28A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |  |
|  | n7 | 2512 | 5 | 25 | 2632 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 798 | 13.9 | IMD3 |  |
|  | 20 | 852 | 5 | 25 | 811 | N/A | N/A |  |
|  | n7 | N/A | 10 | N/A | 2670 | 5.9 | IMD5 |  |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |  |
| DC_20A_n7A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2675 | 30.8 | IMD2 |  |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |  |
|  | 20 | 850 | 5 | 25 | 809 | N/A | N/A |  |
|  | n7 | 2550 | 10 | 50 | 2675 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3400 | 28.8 | IMD21 |  |
| DC_20A_n8A-n78A | n8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3567 | 10.3 | IMD4 |  |
|  | n8 | N/A | 5 | N/A | 940 | 12.1 | IMD4 |  |
|  | n78 | 3481 | 10 | 50 | 3481 | N/A | N/A |  |
|  | 20 | 847 | 5 | 25 | 806 | N/A | N/A |  |
| DC_20A-28A_n3A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 785 | 9.4 | IMD4 |  |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
| DC_20A-28A_n7A | n7 | 2505 | 5 | 25 | 2625 | N/A | N/A |  |
|  | 20 | 859 | 5 | 25 | 818 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 787 | 17.4 | IMD34 |  |
| DC_20A-28A_n78A | 20 | 837 | 5 | 25 | 796 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 799 | 9.4 | IMD4 |  |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A | 808 | 3.8 | IMD5 |  |
|  | 28 | 705.5 | 5 | 25 | 760.5 | N/A | N/A |  |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |  |
| DC_20A_n28A-n78A, DC_20A_SUL_n78A-n83A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |  |
|  | n28, n83 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3314 | 8.7 | IMD4 |  |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |  |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 799 | 9.4 | IMD4 |  |
| DC_20A-32A_n1A | n1 | 1950.5 | 5 | 50 | 2140.5 | N/A | N/A |  |
|  | 20 | 852.5 | 5 | 25 | 811.5 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A | 1459.5 | 4.0 | IMD5 |  |
| DC_20A-38A_n1A | n1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 20 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | 38 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_20A-38A_n3A | 20 | 850 | 5 | 25 | 809 | N/A | N/A |  |
|  | 38 | N/A | 5 | N/A | 2610 | 28.4 | IMD21 |  |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |  |
| DC_20A-38A_n28A | 20 | 835.5 | 5 | 25 | 793.5 | N/A | N/A |  |
|  | 38 | N/A | 5 | N/A | 2612 | 5.9 | IMD5 |  |
|  | n28 | 730 | 5 | 25 | 785 | N/A | N/A |  |
| DC_20A-38A_n78A DC_20A-38A_n78(2A | 20 | N/A | N/A | N/A | N/A | N/A | IMD2 |  |
|  | 38 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 20 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 38 | N/A | N/A | N/A | N/A | N/A | IMD2 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_20A_n38A-n78A | 20 | 850 | 5 | 25 | 809 | N/A | N/A |  |
|  | n38 | N/A | 10 | N/A | 2600 | 30.9 | IMD2 |  |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |  |
| DC_20A-40A_n1A DC_20A-40C_n1A | 20 | N/A | 5 | N/A | 800 | 8.0 | IMD4 |  |
|  | 40 | 2330 | 5 | 25 | 2330 | N/A | N/A |  |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
| DC_20A-40A_n28A | 20 | 835 | 5 | 25 | 794 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A | 2385 | 18.8 | IMD3 |  |
|  | n28 | 715 | 5 | 25 | 770 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A | 815 | 17.0 | IMD3 |  |
|  | 40 | 2305 | 5 | 25 | 2305 | N/A | N/A |  |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |  |
| DC_20A-40A_n78A DC_20A-40C_n78A DC_20A-40A_n78(2A) DC_20A-40C_n78(2A) | 20 | N/A | 5 | N/A | 815 | 19.8 | IMD3 |  |
|  | 40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |  |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |  |
| DC_20A-41A_n1A | 20 | N/A | 5 | N/A | 800 | 4.5 | IMD5 |  |
| DC_20A-41C_n1A | 41 | 2510 | 10 | 50 | 2510 | N/A | N/A |  |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
| DC_20A-41A_n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
| DC_20A-41C_n78A | 41 | N/A | 10 | N/A | 2675 | 29.8 | IMD2 |  |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A | 798 | 30.8 | IMD24 |  |
|  | 41 | 2642 | 10 | 50 | 2642 | N/A | N/A |  |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | N/A |  |
| DC_20A_n41A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A | 2675 | 29.8 | IMD2 |  |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |  |
|  | 20 | 850 | 5 | 25 | 809 | N/A | N/A |  |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3400 | 28.8 | IMD2 |  |
| DC_20A-67A_n3A | 20 | 837 | 5 | 25 | 796 | N/A | N/A |  |
|  | 67 | N/A | 5 | N/A | 746 | 9.4 | IMD4 |  |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | N/A |  |
| DC_20A_SUL_n78A-n80A | 20 | N/A | 5 | N/A | 806 | 9 | IMD4 |  |
|  | n80 | 1735 | 5 | 25 |  | N/A | N/A |  |
| DC_21A_n1A-n77A DC_21A_n1A-n78A | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2154.6 | 30.6 | IMD24 |  |
|  | n77/n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |  |
| DC_21A_n1A-n79A20 |  |  |  | N/A |  |  |  |  |
| DC_21A-28A_n77A DC_21A-28A_n78A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 785.5 | 16.9 | IMD3 |  |
|  | n77/n78 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |  |
|  | 21 | N/A | 5 | N/A | 1498.5 | 9.9 | IMD4 |  |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |  |
|  | n77/n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |  |
| DC_21A-28A_n79A | 21 | N/A | 5 | N/A | 1498 | 5.2 | IMD5 |  |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
| DC_21A_n28A-n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |  |
| DC_21A_n28A-n78A | n28 | N/A | 5 | N/A | 785.5 | 16.9 | IMD39 |  |
|  | n77/n78 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |  |
|  | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |  |
|  | n28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |  |
|  | n77/n78 | N/A | 10 | N/A | 3634.5 | 17.3 | IMD39 |  |
| DC_21A_n28A-n79A 17 | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 790.5 | 2.8 | IMD5 |  |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |  |
|  | 21 | 1460.4 | 5 | 25 | 1508.4 | N/A | N/A |  |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4420 | [6.3] | IMD44 |  |
| DC_21A-42A_n1A | 21 | N/A | 5 | N/A | 1500 | 31.4 | IMD2 |  |
|  | 42 | 3450 | 10 | 50 | 3450 | N/A | N/A |  |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |  |
|  | n78 | 3420 | 10 | 50 | 3420 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4873 | 30.1 | IMD2 |  |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |  |
|  | n79 | 4940 | 40 | 216 | 4940 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3487 | 29.8 | IMD2 |  |
| DC_25A-41A_n41A DC_25A-41C_n41A DC_25A-41D_n41A | 25 | N/A | 5 | N/A | 1992.5 | 8.5 | IMD7 |  |
| DC_25A-25A-41A_n41A DC_25A-25A-41C_n41A DC_25A-25A-41D_n41A | 41 | 2502.5 | 5 | 1 (RBstart=0) | 2502.5 | N/A | N/A |  |
| DC_25A-(n)41CA DC_25A-(n)41DA DC_25A-25A-(n)41CA DC_25A-25A-(n)41DA | n41 | 2670 | 5 | 1 (RBstart=9) | 2670 | N/A | N/A |  |
| DC_25A-66A_n77A DC_25A-25A-66A_n77A | 25 | 1855 | 5 | 25 | 1935 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2115 | 29.2 | IMD2 |  |
|  | n77 | 3970 | 10 | 25 | 3970 | N/A | N/A |  |
|  | 25 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2140 | 10.4 | IMD4 |  |
|  | n77 | 3500 | 10 | 25 | 3500 | N/A | N/A |  |
|  | 25 | 1885 | 5 | 25 | 1965 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2175 | 4.0 | IMD5 |  |
|  | n77 | 3915 | 10 | 25 | 3915 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A | 1960 | 32.1 | IMD2 |  |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n77 | 3720 | 10 | 25 | 3720 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A | 1940 | 9.1 | IMD411 |  |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |  |
|  | n77 | 3385 | 10 | 25 | 3385 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A | 1935 | 4.2 | IMD5 |  |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |  |
|  | n77 | 3540 | 10 | 25 | 3540 | N/A | N/A |  |
| DC_25A-66A_n78A DC_25A-25A-66A_n78A | 25 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2160 | 10.4 | IMD4 |  |
|  | n78 | 3480 | 10 | 50 | 3480 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A | 1960 | 32.1 | IMD2 |  |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A | 1960 | 9.1 | IMD4 |  |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A | 1980 | 4.2 | IMD5 |  |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |  |
|  | n78 | 3645 | 10 | 25 | 3645 | N/A | N/A |  |
| DC_28A_n1A-n5A | 28 | 708 | 5 | 25 | 763 | N/A | N/A |  |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A | 882 | 4.6 | IMD5 |  |
|  | 28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2136 | 4 | IMD5 |  |
|  | n5 | 836 | 5 | 25 | 881 | N/A | N/A |  |
| DC_28A_n1A-n40A | 28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A | 2374 | 10.1 | IMD4 |  |
| DC_28A_n1A-n78A | 28 | 733 | 5 | 25 | 788 | N/A | N/A |  |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3416 | 15.7 | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2150 | 15.7 | IMD3 |  |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |  |
| DC_28A_n3A-n77A | 28 | 735 | 5 | 25 | 790 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1850 | 17.0 | IMD3 |  |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | N/A |  |
|  | 28 | 733 | 5 | 25 | 788 | N/A | N/A |  |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 4173 | 15.9 | IMD3 |  |
| DC_28A_n3A-n78A | 28 | 735 | 5 | 25 | 790 | N/A | N/A |  |
|  | n3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 |  |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |  |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | 28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | IMD5 |  |
| DC_28A_n5A-n40A | 28 | 712 | 5 | 25 | 767 | N/A | N/A |  |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A | 2365 | 18.8 | IMD3 |  |
|  | 28 | 720 | 5 | 25 | 775 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A | 880 | 17.0 | IMD3 |  |
|  | n40 | 2320 | 5 | 25 | 2320 | N/A | N/A |  |
| DC_28A_n5A-n78A | 28 | 707 | 5 | 25 | 762 | N/A | N/A |  |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3781 | 4.0 | IMD5 |  |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A | 874 | 3.8 | IMD5 |  |
|  | n78 | 3766 | 10 | 50 | 3756 | N/A | N/A |  |
| DC_28A_n5A-n105A | 28 | 735 | 5 | 25 | 790 | N/A | N/A |  |
|  | n5 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n105 | N/A | 5 | N/A | 635 | 25 | IMD3 |  |
| DC_28A_n7A-n78A DC_28A_n7B-n78A | 28 | 745 | 5 | 25 | 800 | N/A | N/A |  |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3310 | 29.7 | IMD2 |  |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2650 | 30.5 | IMD2 |  |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |  |
| DC_28A_n8A-n78A | 28 | 728 | 5 | 25 | 783 | N/A | N/A |  |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3458 | 9.1 | IMD4 |  |
|  | 28 | 713 | 5 | 25 | 768 | N/A | N/A |  |
|  | n8 | N/A | 5 | N/A | 935 | 4.3 | IMD5 |  |
|  | n78 | 3787 | 10 | 50 | 3787 | N/A | N/A |  |
| DC_28A-38A_n1A | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 775 | 4.5 | IMD5 |  |
|  | 38 | 2575 | 5 | 25 | 2575 | N/A | N/A |  |
| DC_28A-40A_n1A | 28 | 740 | 5 | 25 | 795 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A | 2365 | 10.1 | IMD4 |  |
|  | n1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 780 | 8 | IMD4 |  |
|  | 40 | 2340 | 5 | 25 | 2340 | N/A | N/A |  |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
| DC_28_n40-n71 | 28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |  |
|  | n40 | 2362.5 | 5 | 25 | 2362.5 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 619.5 | [3.3] | IMD5 |  |
| DC_28A-38A_n78A | 28 | 738 | 5 | 25 | 793 | N/A | N/A |  |
|  | 38 | N/A | 5 | N/A | 2582 | 29.5 | IMD2 |  |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 793 | 30.8 | IMD24 |  |
|  | 38 | 2582 | 5 | 25 | 2582 | N/A | N/A |  |
|  | n78 | 3375 | 10 | 50 | 3375 | N/A | N/A |  |
| DC_28A_n40A-n77A | 28 | 708 | 5 | 25 | 763 | N/A | N/A |  |
|  | n40 | 2310 | 10 | 50 | 2310 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3726 | [28.6] | IMD31 |  |
|  | 28 | 708 | 5 | 25 | 763 | N/A | N/A |  |
|  | n40 | N/A | 10 | N/A | 2390 | [12.3] | IMD3 |  |
|  | n77 | 3806 | 10 | 50 | 3806 | N/A | N/A |  |
| DC_28A-40A_n78A DC_28A-40C_n78A | 28 | N/A | 5 | N/A | 800.5 | 11 | IMD3 |  |
|  | 40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |  |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |  |
|  | 28 | 715 | 5 | 25 | 770 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A | 2320 | 15.7 | IMD3 |  |
|  | n78 | 3750 | 10 | 50 | 3750 | N/A | N/A |  |
| DC_28A-41A_n77A | 28 | 738 | 5 | 25 | 793 | N/A | N/A |  |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2642 | 29.5 | IMD2 |  |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |  |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 798 | 30.8 | IMD2 |  |
|  | 41 | 2567.5 | 10 | 50 | 2567.5 | N/A | N/A |  |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 782.5 | 3.0 | IMD5 |  |
| DC_28A-41A_n78A | 28 | 738 | 5 | 25 | 793 | N/A | N/A |  |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2642 | 29.5 | IMD2 |  |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |  |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 798 | 30.8 | IMD2 |  |
| DC_28A-41A_n79A | 28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n79 | 4739 | 40 | 216 | 4739 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A | 2510 | 8.6 | IMD4 |  |
|  | 41 | 2650 | 5 | 25 | 2650 | N/A | N/A |  |
|  | n79 | 4502 | 40 | 216 | 4502 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 798 | 15.9 | IMD3 |  |
| DC_28A-42A_n79A DC_28A-42A_n79C DC_28A-42C_n79A DC_28A-42C_n79C | 28 | 730 | 5 | 25 | 785 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A | 3420 | 15.3 | IMD3 |  |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A | 800 | 16.2 | IMD2 |  |
|  | 42 | 3597.5 | 5 | 25 | 3597.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
| DC_28A-66A_n7A | 28 | N/A | 5 | N/A | 790 | 27.6 | IMD2 |  |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |  |
|  | n7 | 2505 | 5 | 50 | 2625 | N/A | N/A |  |
| DC_28A-66A_n66A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2129 | 11.0 | IMD4 |  |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |  |
| DC_28A_n71A-n77A | 28 | 705.5 | 5 | 25 | 760.5 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 628 | 3.8 | IMD5 |  |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | N/A |  |
| DC_28A_n78A-n105A | 28 | 705.5 | 5 | 25 | 760.5 | N/A | N/A |  |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |  |
|  | n105 | N/A | 5 | N/A | 628 | 3.9 | IMD5 |  |
| DC_29A-30A_n66A | 29 | N/A | 5 | 25 | 719.5 | 4.5 | IMD5 |  |
|  | 30 | 2307.5 | 5 | 25 | 2352.5 | N/A | N/A |  |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | N/A |  |
| DC_29A-30A_n77A | 29 | N/A | 5 | N/A | 722 | 15.2 | IMD34 |  |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |  |
|  | n77 | 3898 | 10 | 50 | 3898 | N/A | N/A |  |
| DC_29A-66A_n77A | 29 | N/A | 5 | N/A | 722 | 15.2 | IMD311 |  |
| DC_29A-66A-66A_n77A | 66 | 1734 | 5 | 25 | 2134 | N/A | N/A |  |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |  |
| DC_30A-66A_n5A, DC_30A-66A-66A_n5A, DC_30A-66A-66A-66A_n5A | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2130 | 2.5 | IMD5 |  |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |  |
| DC_30A-66A_n77A DC_30A-66A_n77(2A) | 30 | N/A | 5 | N/A | 2355 | 29.2 | IMD211 |  |
| DC_30A-66A-66A_n77A | 66 | 1745 | 5 | 25 | 2145 | N/A | N/A |  |
| DC_30A-66A-66A_n77(2A) | n77 | 4100 | 10 | 50 | 4100 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A | 2355 | 3.4 | IMD5 |  |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |  |
|  | n77 | 3780 | 10 | 50 | 3780 | N/A | N/A |  |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2160 | 8.7 | IMD411 |  |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | N/A |  |
| DC_38A_n3A-n78A | 38 | 2560 | 10 | 50 | 2560 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |  |
|  | 38 | 2620 | 5 | 25 | 2620 | N/A | N/A |  |
|  | n3 | 1745 | 5 | 25 | 1840 | 17.6 | IMD39 |  |
|  | n78 | 3400 | 10 | 52 | 3400 | N/A | N/A |  |
| DC_38A_n28A-n78A | 38 | 2615 | 5 | 25 | 2615 | N/A | N/A |  |
|  | n28 | 745 | 5 | 25 | 798 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3360 | 28.2 | IMD29 |  |
|  | 38 | 2615 | 5 | 25 | 2615 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 785 | 30.8 | IMD24 |  |
|  | n78 | 3400 | 10 | 25 | 3400 | N/A | N/A |  |
| DC_38A-40A_n28A | 38 | N/A | 5 | N/A | 2576 | 5.3 | IMD5 |  |
|  | 40 | 2350 | 5 | 25 | 2350 | N/A | N/A |  |
|  | n28 | 708 | 5 | 25 | 763 | N/A | N/A |  |
| DC_39A_n40A-n41A DC_39A_n40A-n41C | 39 | 1917.5 | 5 | 25 | 1917.5 | N/A | N/A |  |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A | 2685 | 30.3 | IMD3 |  |
| DC_39A_n40A-n79A | 39 | 1917.5 | 5 | 25 | 1917.5 | N/A | N/A |  |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4980 | 5.8 | IMD4 |  |
| DC_39A_n41A-n79A | 39 | 1900 | 5 | 25 | 1900 | N/A | N/A |  |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4520 | 29.8 | IMD24 |  |
|  | 39 | 1900 | 5 | 25 | 1900 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A | 2620 | 30.2 | IMD24 |  |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |  |
| DC_40A_n1A-n78A | 40 | 2340 | 5 | 25 | 2340 | N/A | N/A |  |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3450 | 9.8 | IMD4 |  |
|  | 40 | 2360 | 5 | 25 | 2360 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A | 2140 | 9.1 | IMD4 |  |
|  | n78 | 3430 | 10 | 50 | 3430 | N/A | N/A |  |
| DC_40A_n41A-n79A | 40 | 2340 | 5 | 25 | 2340 | N/A | N/A |  |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A | 4940 | 30.5 | IMD2 |  |
|  | 40 | 2340 | 5 | 25 | 2340 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A | 2600 | 29.4 | IMD24 |  |
|  | n79 | 4880 | 40 | 216 | 4940 | N/A | N/A |  |
| DC_41A_n1A-n77A | 41 | 2650 | 5 | 25 | 2650 | N/A | TDD |  |
| DC_41C_n1A-n77A | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD |  |
|  | n77 | N/A | 10 | N/A | 3330 | 19.6 | TDD |  |
|  | 41 | 2510 | 5 | 25 | 2510 | N/A | TDD |  |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A | TDD |  |
|  | n1 | N/A | 5 | N/A | 2120 | 11.0 | FDD |  |
| DC_41A_n3A-n77A DC_41C_n3A-n77A DC_41A_n3A-n78A DC_41C_n3A-n78A | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A | 1840 | 16.4 | IMD3 |  |
|  | n77/n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |  |
|  | 41 | 2580 | 5 | 25 | 2580 | N/A | N/A |  |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n77/n78 | N/A | 10 | N/A | 3440 | 16.8 | IMD34 |  |
| DC_41A_n28A-n77A DC_41C_n28A-n77A DC_41A_n28A-n78A DC_41C_n28A-n78A | 41 | 2580 | 5 | 25 | 2580 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n77/n78 | N/A | 10 | N/A | 3323 | 28.2 | IMD21 |  |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A | 798 | 30.8 | IMD21 |  |
|  | n77/n78 | 3440 | 10 | 50 | 3440 | N/A | N/A |  |
| DC_46A-48A_n5A5 DC_46C-48A_n5A5 DC_46D-48A_n5A5 DC_46E-48A_n5A5 | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD3 |  |
|  | 48 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n5 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_46A-48A_n66A5 DC_46C-48A_n66A5 DC_46D-48A_n66A5 DC_46E-48A_n66A5 | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD3 |  |
|  | 48 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_46A-66A_n5A | 46 | N/A | 10 | N/A | 5163 | 9.0 | IMD4 |  |
| DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A DC_46A-66A-66A_n5A DC_46C-66A-66A_n5A DC_46D-66A-66A_n5A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |  |
|  | n5 | 847 | 5 | 25 | 892 | N/A | N/A |  |
| DC_46A-66A_n25A4 DC_46C-66A_n25A4 DC_46D-66A_n25A4 | 46 | N/A | 10 | N/A | 5505 | 16.1 | IMD3 |  |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A | 1935 | 20 | IMD3 |  |
|  | 46 | N/A | 10 | N/A | 5505 | 16.1 | IMD3 |  |
|  | 66 | N/A | 5 | N/A | 2150 | 4 | IMD5 |  |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |  |
|  | 46 | N/A | 10 | N/A | 5505 | 16.1 | IMD3 |  |
|  | 66 | N/A | 5 | N/A | 2112.5 | 23 | IMD3 |  |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |  |
| DC_46A-66A_n77A5 DC_46A-46A-66A_n77A5 | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD3 |  |
|  | 66 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_48A-(n)12AA | 48 | 3557.5 | 10 | 50 | 3557.5 | N/A | N/A |  |
|  | 12 | N/A | 5 | N/A | 740.5 | 5.5 | IMD5 |  |
|  | n12 | 705.5 | 5 | 25 | 735.5 | 5.5 | IMD5 |  |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | 48 | N/A | 10 | N/A | 3620 | 29.4 | IMD2 |  |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 48 | 3560 | 5 | 25 | 3560 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2155 | 12.1 | IMD4 |  |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | N/A |  |
| DC_48A-66A_n12A | 48 | 3580 | 5 | 25 | 3580 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2160 | 17.1 | IMD3 |  |
|  | n12 | 710 | 5 | 25 | 740 | N/A | N/A |  |
| DC_48A-66A_n25A DC_48C-66A_n25A DC_48D-66A_n25A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2130 | 8.3 | IMD4 |  |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |  |
|  | 48 | N/A | 10 | N/A | 3620 | 29.4 | IMD2 |  |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
| DC_48A-66A_n66A DC_48C-66A_n66A | 48 | 3660 | 20 | 100 | 3660 | N/A | N/A |  |
| DC_48D-66A_n66A | 66 | N/A | 5 | N/A | 2175 | 4.0 | IMD5 |  |
| DC_48E-66A_n66A | n66 | 1715 | 5 | 25 | 2115 | N/A | N/A |  |
| DC_48A-66A_n71A | 48 | 3560 | 5 | 25 | 3560 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A | 2174 | 15.8 | IMD3 |  |
|  | n71 | 693 | 5 | 25 | 647 | N/A | N/A |  |
|  | 48 | N/A | 5 | N/A | 3697.5 | 13.0 | IMD4 |  |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |  |
| DC_66A_n2A-n41A | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |  |
|  | n2 | 1860 | 5 | 25 | 1940 | 11.0 | IMD4 |  |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | N/A |  |
| DC_66A_n2A-n66A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1935 | 20 | IMD3 |  |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2170 | 4.0 | IMD5 |  |
| DC_66A_n2A-n77A | n2 | N/A | 5 | N/A | 1960 | 32.1 | IMD2 |  |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |  |
|  | n2 | 1853 | 5 | 25 | 1933 | N/A | N/A |  |
|  | 66 | 1713 | 5 | 25 | 2113 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3566 | 29.4 | IMD24 |  |
| DC_66A_n2A-n78A | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1960 | 32.1 | IMD2 |  |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |  |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3620 | 34.9 | IMD2 |  |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3340 | 20.9 | IMD4 |  |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1960 | 21.1 | IMD4 |  |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |  |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1960 | 2.1 | IMD5 |  |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | N/A |  |
| DC_66A-(n)5AA | 66 | 1721 | 5 | 25 | 2121 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A | 878 | 25 | IMD2 |  |
|  | n5 | 838 | 5 | 25 | 883 | 30 | IMD2 |  |
| DC_66A_n5A-n48A | 66 | 1750 | 5 | 25 | 2150 | N/A | N/A |  |
|  | n5 | 834 | 5 | 25 | 879 | N/A | N/A |  |
|  | n48 | N/A | 5 | N/A | 3582 | 3.3 | IMD5 |  |
| DC_66A_n5A-n77A | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |  |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3460 | 16.6 | IMD39 |  |
| DC_66A_n7A-n12A | 66 | 1773 | 5 | 25 | 2173 | N/A | N/A |  |
|  | n7 | 2515 | 5 | 25 | 2635 | N/A | N/A |  |
|  | n12 | N/A | 5 | N/A | 742 | 31 | IMD2 |  |
| DC_66A_n7A-n77A | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n7 | 2542 | 5 | 25 | 2662 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3344 | 16.0 | IMD34 |  |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2640 | 3.4 | IMD5 |  |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |  |
| DC_66A_n7A-n78A, DC_66A-66A_n7A-n78 DC_66A_n7(2A)-n78A DC_66A-66A_n7(2A)-n78A DC_66A_n7A-n78(2A) DC_66A-66A_n7A-n78(2A) DC_66A-66A_n7(2A)-n78(2A) | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |  |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3390 | 16.1 | IMD3 |  |
| DC_66A_n12A-n77A | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n12 | N/A | 5 | N/A | 740 | 15.2 | IMD311 |  |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | N/A |  |
|  | 66 | 1723 | 5 | 25 | 2123 | N/A | N/A |  |
|  | n12 | 704 | 5 | 25 | 734 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 4150 | 16.0 | IMD34,9,11 |  |
| DC_66A_n25A-n41A | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |  |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |  |
|  | n25 | 1860 | 5 | 25 | 1940 | 5 | 11.0 |  |
| DC_66A_n25A-n48A | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | N/A |  |
|  | n48 | N/A | 10 | N/A | 3620 | 29.4 | IMD2 |  |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A | 1960 | 28.3 | IMD2 |  |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |  |
| DC_66A_n25A-n66A | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2117.5 | 23 | IMD3 |  |
|  | 66 | 1750 | 5 | 25 | 2150 | N/A | N/A |  |
|  | n25 | 1873 | 5 | 25 | 1953 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2119 | 4 | IMD5 |  |
| DC_66A_n38A-n78A | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n38 | 2610 | 10 | 50 | 2610 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3460 | 15.0 | IMD3 |  |
| DC_66A_n41A-n77A | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |  |
|  | n41 | 2600 | 5 | 25 | 2600 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3470 | 16.1 | IMD31 |  |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2670 | 5.2 | IMD55 |  |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |  |
| DC_66A_n41A-n78A | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |  |
|  | n41 | 2560 | 5 | 25 | 2560 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3390 | 16.1 | IMD3 |  |
| DC_66A_n66A-n71A | 66 | 1752 | 5 | 25 | 2152 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2118 | 5.0 | IMD4 |  |
|  | n71 | 693 | 5 | 25 | 647 | N/A | N/A |  |
| DC_66A_n66A-n77A | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2170 | 31 | IMD2 |  |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |  |
| DC_66A_n66A-n78A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2125 | 2.8 | IMD5 |  |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |  |
| DC_66A-71A_n77A DC_66A-71A_n77(2A) | 66 | N/A | 5 | N/A | 2160 | 15.5 | IMD39 |  |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |  |
|  | n77 | 3546 | 10 | 50 | 3546 | N/A | N/A |  |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 71 | N/A | 5 | N/A | 640 | 15.3 | IMD311 |  |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | N/A |  |
| DC_66A_n71A-n77A | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n71 | 668 | 5 | 25 | 622 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 4108 | 15.9 | IMD34,9,11 |  |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A | 640 | 15.3 | IMD311 |  |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | N/A |  |
| DC_66A_n71A-n78A | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3709 | 13.0 | IMD4 |  |
| DC_71A_n2A-n41A | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2586 | 29.2 | IMD2 |  |
|  | 71 | 686 | 5 | 50 | 640 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1942 | 26 | IMD2 |  |
|  | n41 | 2610 | 5 | 25 | 2610 | N/A | N/A |  |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |  |
| DC_71A_n2A-n78A | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
|  | 71 | 695.5 | 5 | 25 | 649.5 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3305 | 8.0 | IMD3 |  |
|  | n2 | N/A | 5 | N/A | 1954 | 16.5 | IMD3 |  |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |  |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |  |
| DC_71A_n2A-n77A | 71 | 695.5 | 5 | 25 | 649.5 | N/A | N/A |  |
|  | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3305 | 8.0 | IMD34,9,11 |  |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A | 1954 | 16.5 | IMD39,11 |  |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | N/A |  |
| DC_71A_n7A-n77A | 71 | 680 | 5 | 25 | 634 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2670 | 29.6 | IMD2 |  |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | N/A |  |
|  | 71 | 680 | 5 | 25 | 634 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A | 2640 | 13.2 | IMD3 |  |
|  | n77 | 4000 | 10 | 50 | 4000 | N/A | N/A |  |
|  | 71 | 666 | 5 | 25 | 620 | N/A | N/A |  |
|  | n7 | 2505 | 5 | 25 | 2625 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3837 | 16.0 | IMD3 |  |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |  |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3714 | 9.7 | IMD4 |  |
| DC_71A_n25A-n41A | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A | 2586 | 29.2 | IMD29 |  |
|  | 71 | 686 | 5 | 50 | 640 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A | 1942 | 26 | IMD2 |  |
|  | n41 | 2610 | 5 | 25 | 2610 | N/A | N/A |  |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |  |
| DC_71A_n25A-n77A | 71 | 693 | 5 | 25 | 647 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A | 1954 | 16.5 | IMD3 |  |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | N/A |  |
|  | 71 | 666 | 5 | 25 | 620 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A | 1982 | 12.5 | IMD4 |  |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |  |
|  | 71 | 695.5 | 5 | 25 | 649.5 | N/A | N/A |  |
|  | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3298.5 | 16 | IMD34 |  |
|  | 71 | 666 | 5 | 25 | 620 | N/A | N/A |  |
|  | n25 | 1890 | 5 | 25 | 1970 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 3888 | 12 | IMD4 |  |
| DC_71A_n38A-n78A | 71 | 693 | 5 | 25 | 647 | N/A | N/A |  |
|  | n38 | 2615 | 10 | 50 | 2615 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3308 | 29.1 | IMD2 |  |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |  |
|  | n78 | 3308 | 10 | 50 | 3308 | N/A | N/A |  |
|  | n38 | N/A | 10 | N/A | 2615 | 28.7 | IMD2 |  |
| DC_71A_n66A-n77A | 71 | 668 | 5 | 25 | 622 | N/A | N/A |  |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A | 4108 | 15.9 | IMD34,9,11 |  |
|  | 71 | 690 | 5 | 25 | 644 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2150 | 15.5 | IMD39,11 |  |
|  | n77 | 3530 | 10 | 50 | 3530 | N/A | N/A |  |
| DC_71A_n66A-n78A | 71 | 693 | 5 | 25 | 647 | N/A | N/A |  |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A | 2160 | 15.5 | IMD3 |  |
|  | 71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A | 3697.5 | 13.0 | IMD4 |  |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 2: For DC_3A_n3A-n77A, DC_3A_n3A-n78A paired with UL_DC_3A_n3A, the 3rd DL bands n77/n78 are subject to IMD2 which MSD is not specified NOTE 3: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. DC_2A-46A_n77A; DC_2A_n77A; fc_2A + fc_n77A; BW_2A + BW_n77A. DC_2A-46A_n77A; DC_2A_n77A; -fc_2A + 2*fc_n77A; -BW_2A + 2*BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 2*fc_13A + fc_n77A; 2*BW_13A + BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 3*fc_13A + fc_n77A; 3*BW_13A + BW_n77A. DC_13A-46A_n2A; DC_13A_n2A; 2*fc_n2A + 2*fc_13A; 2*BW_n2A+2*BW_13A. DC_13A-46A_n77A; DC_13A_n77A; -3*fc_13A + 2*fc_n77A; -3*BW_13A + 2*BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; fc_66A + fc_n77A; BW_66A + BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; -fc_66A + 2*fc_n77A; -BW_66A + 2*BW_n77A. DC_13A-46A_n66A; DC_13A_n66A; 3*fc_13A + fc_n66A; BW_13A + 2*BW_n66A. DC_13A-46A_n66A; DC_13A_n66A; 2*fc_13A + 3*fc_n66A; BW_13A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; fc_48A + fc_n66A; BW_48A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; 2*fc_48A + fc_n66A; 2*BW_48A + BW_n66A. DC_2A-46_n5A; DC_2A_n5A; 2*fc_2A + 2*fc_n5A; BW_2A + 2*BW_n5A. DC_2A-46_n5A; DC_2A_n5A; fc_2A + 4*fc_n5A; BW_2*2A + BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + fc_n5A; BW_48A + 2*BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + 2*fc_n5A; BW_2*48A + BW_n5A. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: This band is also subject to IMD2 which is not specified. The frequency range below 3400MHz in n77 is not used for this combination. NOTE 8: Band 5 is also affected by IMD5 from UL DC_2A_n12A, but MSD value is not specified as there is only partial overlap of IMD5 with DL carrier. NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 10: The frequency range in band n28 is restricted for this band combination to 728 – 738 MHz for the UL and 783 – 793 MHz for the DL. This band is subject to IMD2 fall in B1 also which MSD is not specified. NOTE 11: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 12: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 13: Void NOTE 14: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 15: This band is subject to additional IMD3 for which MSD is not specified. NOTE 16: This band is subject to IMD3 also which MSD is not specified. NOTE 17: The frequency range in band n28 is restricted for this band combination to 728 – 738 MHz for the UL and 783 – 793 MHz for the DL. NOTE 18: In the MSD test configuration, the IMD center does not fall into the DL victim Fc. NOTE 19: This band is subject to 1st order triple-beat IMD3 where MSD is not specified when the UL configuration includes intra-band uplink CCs. NOTE 20: No MSD test points are specified for this combination and verification of IMD impact is not required. |

Table 7.3B.2.3.5.2-1a: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration |  | EUTRA / NR band |  | UL Fc (MHz) |  | UL/DL BW (MHz) | UL LCRB |  |  |  | DL Fc (MHz) |  | MSD (dB) |  | IMD order |  |
| DC_1A-3A_n77A DC_1A-3A_n77(2A) DC_1A-3A_n77(3A) DC_1A-3C_n77A DC_1A-3C_n77(2A) |  | 1 |  | 1950 |  | 5 | 25 |  |  |  | 2140 |  | N/A |  | N/A |  |
|  |  | 3 |  | N/A |  | 5 | N/A |  |  |  | 1807.5 |  | 37.5 |  | IMD21 |  |
|  |  | n77 |  | 3757.5 |  | 10 | 50 |  |  |  | 3757.5 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1950 |  | 5 | 25 |  |  |  | 2140 |  | N/A |  | N/A |  |
|  |  | 3 |  | N/A |  | 5 | N/A |  |  |  | 1870 |  | 20.5 |  | IMD41 |  |
|  |  | n77 |  | 3980 |  | 10 | 50 |  |  |  | 3980 |  | N/A |  | N/A |  |
|  |  | 1 |  | N/A |  | 5 | N/A |  |  |  | 2140 |  | 37.0 |  | IMD21 |  |
|  |  | 3 |  | 1775 |  | 5 | 25 |  |  |  | 1870 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3915 |  | 10 | 50 |  |  |  | 3915 |  | N/A |  | N/A |  |
| DC_1A-3A_n78A DC_1A-3A_n78(2A) DC_1A-3C_n78A DC_1A-3C_n78(2A) |  | 1 |  | 1950 |  | 5 | 25 |  |  |  | 2140 |  | N/A |  | N/A |  |
|  |  | 3 |  | N/A |  | 5 | N/A |  |  |  | 1807.5 |  | 37.2 |  | IMD21 |  |
|  |  | n78 |  | 3757.5 |  | 10 | 50 |  |  |  | 3757.5 |  | N/A |  | N/A |  |
|  |  | 1 |  | N/A |  | 5 | N/A |  |  |  | 2125 |  | 17.8 |  | IMD5 |  |
|  |  | 3 |  | 1775 |  | 5 | 25 |  |  |  | 1870 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3725 |  | 10 | 50 |  |  |  | 3725 |  | N/A |  | N/A |  |
| DC_1A-3A_n79A |  | 1 |  | N/A |  | 5 | N/A |  |  |  | 2140 |  | 24.6 |  | IMD5 |  |
|  |  | 3 |  | 1750 |  | 5 | 25 |  |  |  | 1845 |  | N/A |  | N/A |  |
|  |  | n79 |  | 4860 |  | 10 | 50 |  |  |  | 4860 |  | N/A |  | N/A |  |
| DC_1A-5A_n78A |  | 1 |  | N/A |  | 5 | N/A |  |  |  | 2120 |  | 19.2 |  | IMD4 |  |
|  |  | 5 |  | 844 |  | 5 | 25 |  |  |  | 889 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3670 |  | 10 | 52 |  |  |  | 3670 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1950 |  | 5 | 25 |  |  |  | 2140 |  | N/A |  | N/A |  |
|  |  | 5 |  | N/A |  | 5 | N/A |  |  |  | 889 |  | 19.2 |  | IMD4 |  |
|  |  | n78 |  | 3421 |  | 10 | 52 |  |  |  | 3421 |  | N/A |  | N/A |  |
|  |  | 1 |  | N/A |  | 5 | N/A |  |  |  | 2122 |  | 27.0 |  | IMD3 |  |
|  |  | 5 |  | 829 |  | 5 | 25 |  |  |  | 874 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3780 |  | 10 | 52 |  |  |  | 3780 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1975 |  | 5 | 25 |  |  |  | 2165 |  | N/A |  | N/A |  |
|  |  | 5 |  | N/A |  | 5 | N/A |  |  |  | 885 |  | 13.2 |  | IMD5 |  |
|  |  | n78 |  | 3405 |  | 10 | 52 |  |  |  | 3405 |  | N/A |  | N/A |  |
| DC_1A-7A_n78A |  | 1 |  | N/A |  | 5 | N/A |  |  |  | 2120 |  | 19.2 |  | IMD4 |  |
|  |  | 7 |  | 2550 |  | 5 | 25 |  |  |  | 2670 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3670 |  | 10 | 52 |  |  |  | 3670 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1977.5 |  | 5 | 25 |  |  |  | 2167.5 |  | N/A |  | N/A |  |
|  |  | 7 |  | N/A |  | 5 | N/A |  |  |  | 2627.5 |  | 20.2 |  | IMD4 |  |
|  |  | n78 |  | 3305 |  | 10 | 52 |  |  |  | 3305 |  | N/A |  | N/A |  |
|  |  | 1 |  | N/A |  | 5 | N/A |  |  |  | 2140 |  | 19.7 |  | IMD4 |  |
|  |  | 7 |  | 2510 |  | 10 | 50 |  |  |  | 2630 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3580 |  | 10 | 52 |  |  |  | 3580 |  | N/A |  | N/A |  |
| DC_1A-8A_n77A |  | 1 |  | 1955 |  | 5 | 25 |  |  |  | 2145 |  | N/A |  | N/A |  |
| DC_1A-8A_n77(2A) DC_1A-8A_n77(3A) |  | 8 |  | 910 |  | 5 | 25 |  |  |  | 955 |  | 15.7 |  | IMD5 |  |
|  |  | n77 |  | 3410 |  | 10 | 50 |  |  |  | 3410 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1950 |  | 5 | 25 |  |  |  | 2140 |  | 23.4 |  | IMD3 |  |
|  |  | 8 |  | 910 |  | 5 | 25 |  |  |  | 955 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3960 |  | 10 | 50 |  |  |  | 3960 |  | N/A |  | N/A |  |
| DC_1A-8A_n78A |  | 1 |  | 1955 |  | 5 | 25 |  |  |  | 2145 |  | N/A |  | N/A |  |
| DC_1A-8A_n78(2A) |  | 8 |  | 910 |  | 5 | 25 |  |  |  | 955 |  | 15.7 |  | IMD5 |  |
|  |  | n78 |  | 3410 |  | 10 | 50 |  |  |  | 3410 |  | N/A |  | N/A |  |
| DC_1A-11A_n77A |  | 1 |  | 1955 |  | 5 | 25 |  |  |  | 2145 |  | N/A |  | N/A |  |
|  |  | 11 |  | N¦/A |  | 5 | N/A |  |  |  | 1486 |  | 37.6 |  | IMD2 |  |
|  |  | n77 |  | 3441 |  | 10 | 50 |  |  |  | 3441 |  | N/A |  | N/A |  |
| DC_1A-11A_n79A |  | 1 |  | 1970 |  | 5 | 25 |  |  |  | 2160 |  | N/A |  | N/A |  |
|  |  | 11 |  | N/A |  | 5 | N/A |  |  |  | 1483 |  | 22.2 |  | IMD4 |  |
|  |  | n79 |  | 4427 |  | 40 | 216 |  |  |  | 4427 |  | N/A |  | N/A |  |
| DC_1A-18A_n77A |  | 1 |  | 1970 |  | 5 | 25 |  |  |  | 2160 |  | N/A |  | N/A |  |
|  |  | 18 |  | N/A |  | 5 | N/A |  |  |  | 870 |  | 15.8 |  | IMD5 |  |
|  |  | n77 |  | 3390 |  | 10 | 50 |  |  |  | 3390 |  | N/A |  | N/A |  |
|  |  | 1 |  | N/A |  | 5 | N/A |  |  |  | 2120 |  | 25.0 |  | IMD3 |  |
|  |  | 18 |  | 825 |  | 5 | 25 |  |  |  | 870 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3770 |  | 10 | 50 |  |  |  | 3770 |  | N/A |  | N/A |  |
| DC_1A-19A_n77A DC_1A-19A_n77(2A) |  | 1 |  | N/A |  | 5 |  |  | N/A |  | 2130 |  | 26.7 |  | IMD3 |  |
|  |  | 19 |  | 832.5 |  | 5 |  |  | 25 |  | 877.5 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3795 |  | 10 |  |  | 50 |  | 3795 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1940 |  | 5 |  |  | 25 |  | 2130 |  | N/A |  | N/A |  |
|  |  | 19 |  | N/A |  | 5 |  |  | N/A |  | 880 |  | 18.5 |  | IMD5 |  |
|  |  | n77 |  | 3350 |  | 10 |  |  | 50 |  | 3350 |  | N/A |  | N/A |  |
| DC_1A-19A_n78A DC_1A-19A_n78(2A) |  | 1 |  | N/A |  | 5 |  |  | N/A |  | 2130 |  | 26.7 |  | IMD3 |  |
|  |  | 19 |  | 832.5 |  | 5 |  |  | 25 |  | 877.5 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3795 |  | 10 |  |  | 50 |  | 3795 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1940 |  | 5 |  |  | 25 |  | 2130 |  | N/A |  | N/A |  |
|  |  | 19 |  | N/A |  | 5 |  |  | N/A |  | 880 |  | 18.5 |  | IMD5 |  |
|  |  | n78 |  | 3350 |  | 10 |  |  | 50 |  | 3350 |  | N/A |  | N/A |  |
| DC_1A-19A_n79A |  | 1 |  | 1950 |  | 5 |  |  | 25 |  | 2140 |  | N/A |  | N/A |  |
|  |  | 19 |  | N/A |  | 5 |  |  | N/A |  | 882.5 |  | 33.3 |  | IMD35 |  |
|  |  | n79 |  | 4782.5 |  | 10 |  |  | 50 |  | 4782.5 |  | N/A |  | N/A |  |
|  |  | 1 |  | N/A |  | 5 |  |  | N/A |  | 2140 |  | 26.1 |  | IMD4 |  |
|  |  | 19 |  | 837.5 |  | 5 |  |  | 25 |  | 882.5 |  | N/A |  | N/A |  |
|  |  | n79 |  | 4652.5 |  | 10 |  |  | 50 |  | 4652.5 |  | N/A |  | N/A |  |
| DC_1A-21A_n77A DC_1A-21A_n77(2A) |  | 1 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | 21 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | IMD2 |  |
|  |  | n77 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | 1 |  | 1950 |  | 5 |  |  | 25 |  | 2140 |  | N/A |  | N/A |  |
|  |  | 21 |  | N/A |  | 5 |  |  | N/A |  | 1500 |  | 17.9 |  | IMD5 |  |
|  |  | n77 |  | 3605 |  | 10 |  |  | 50 |  | 3605 |  | N/A |  | N/A |  |
|  |  | 1 |  | N/A |  | 5 |  |  | N/A |  | 2154.6 |  | 36.6 |  | IMD21 |  |
|  |  | 21 |  | 1450.4 |  | 5 |  |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3605 |  | 10 |  |  | 50 |  | 3605 |  | N/A |  | N/A |  |
| DC_1A-21A_n78A DC_1A-21A_n78(2A) |  | 1 |  | N/A |  | 5 |  |  | N/A |  | 2154.6 |  | 36.6 |  | IMD2 |  |
|  |  | 21 |  | 1450.4 |  | 5 |  |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3605 |  | 10 |  |  | 50 |  | 3605 |  | N/A |  | N/A |  |
|  |  | 1 |  | N/A |  | 5 |  |  | N/A |  | 2154.6 |  | 16.2 |  | IMD5 |  |
|  |  | 21 |  | 1450.4 |  | 5 |  |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3647 |  | 10 |  |  | 50 |  | 3647 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1950 |  | 5 |  |  | 25 |  | 2140 |  | N/A |  | N/A |  |
|  |  | 21 |  | N/A |  | 5 |  |  | N/A |  | 1500 |  | 37.5 |  | IMD2 |  |
|  |  | n78 |  | 3450 |  | 10 |  |  | 50 |  | 3450 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1950 |  | 5 |  |  | 25 |  | 2140 |  | N/A |  | N/A |  |
|  |  | 21 |  | N/A |  | 5 |  |  | N/A |  | 1500 |  | 14.9 |  | IMD5 |  |
|  |  | n78 |  | 3675 |  | 10 |  |  | 50 |  | 3675 |  | N/A |  | N/A |  |
| DC_1A-21A_n79A7,8 |  | 1 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | 21 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | IMD4 |  |
|  |  | n79 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
| DC_1A_n28A-n77A DC_1A_n28A-n77(2A) |  | 1 |  | 1950 |  | 5 |  |  | 25 |  | 2140 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3320 |  | 10 |  |  | 50 |  | 3320 |  | N/A |  | N/A |  |
|  |  | n28 |  | N/A |  | 5 |  |  | 25 |  | 790 |  | 18.7 |  | IMD5 |  |
| DC_1A-41A_n77A |  | 1 |  | 1970 |  | 5 |  |  | 25 |  | 2160 |  | N/A |  | N/A |  |
| DC_1A-41C_n77A |  | 41 |  | N/A |  | 5 |  |  | N/A |  | 2510 |  | 22.5 |  | IMD4 |  |
|  |  | n77 |  | 3400 |  | 10 |  |  | 50 |  | 3400 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1930 |  | 5 |  |  | 25 |  | 2120 |  | N/A |  | N/A |  |
|  |  | 41 |  | N/A |  | 5 |  |  | N/A |  | 2510 |  | 15.6 |  | IMD5 |  |
|  |  | n77 |  | 4150 |  | 10 |  |  | 50 |  | 4150 |  | N/A |  | N/A |  |
| DC_1A_n41A-n77A |  | 1 |  | 1975 |  | 5 |  |  | 25 |  | 2165 |  | N/A |  | N/A |  |
|  |  | n41 |  | N/A |  | 10 |  |  | N/A |  | 2515 |  | 22.0 |  | IMD41 |  |
|  |  | n77 |  | 3410 |  | 10 |  |  | 50 |  | 3410 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1970 |  | 5 |  |  | 25 |  | 2160 |  | N/A |  | N/A |  |
|  |  | n41 |  | 2650 |  | 10 |  |  | 25 |  | 2650 |  | N/A |  | N/A |  |
|  |  | n77 |  | N/A |  | 10 |  |  | N/A |  | 3330 |  | 28.2 |  | IMD31,5 |  |
| DC_1A-42A_n79A DC_1A-42C_n79A DC_1A-42D_n79A DC_1A-42E_n79A |  | 1 |  | 1977.5 |  | 5 |  |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |
|  |  | 42 |  | N/A |  | 5 |  |  | N/A |  | 3490 |  | 25.8 |  | IMD5 |  |
|  |  | n79 |  | 4420 |  | 10 |  |  | 50 |  | 4420 |  | N/A |  | N/A |  |
| DC_1A_n78A-n79A |  | 1 |  | 1950 |  | 5 |  |  | 25 |  | 2140 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3410 |  | 10 |  |  | 50 |  | 3410 |  | N/A |  | N/A |  |
|  |  | n79 |  | N/A |  | 10 |  |  | N/A |  | 4870 |  | 24.9 |  | IMD31 |  |
|  |  | 1 |  | 1950 |  | 5 |  |  | 25 |  | 2140 |  | N/A |  | N/A |  |
|  |  | n78 |  | N/A |  | 10 |  |  | N/A |  | 3490 |  | 22.6 |  | IMD5 |  |
|  |  | n79 |  | 4670 |  | 10 |  |  | 50 |  | 4670 |  | N/A |  | N/A |  |
| DC_2A_n2A-n77A DC_2A_n2A-n77C |  | 2 |  | 1875 |  | 5 |  |  | 25 |  | 1955 |  | N/A |  | N/A |  |
|  |  | n2 |  | 1855 |  | 5 |  |  | 25 |  | 1935 |  | 32.0 |  | IMD2 |  |
|  |  | n77 |  | 3810 |  | 10 |  |  | 50 |  | 3810 |  | N/A |  | N/A |  |
|  |  | 2 |  | 1895 |  | 5 |  |  | 25 |  | 1975 |  | N/A |  | N/A |  |
|  |  | n2 |  | N/A |  | 5 |  |  | N/A |  | 1975 |  | 20.0 |  | IMD41 |  |
|  |  | n77 |  | 3710 |  | 10 |  |  | 50 |  | 3710 |  | N/A |  | N/A |  |
| DC_2A-5A_n77A2 DC_2A-5A_n77(2A)2 DC_2A-2A-5A_n77A2 DC_2A-2A-5A_n77(2A)2 DC_2A-5A_n77C2 DC_2A-2A-5A_n77C2 |  | 2 |  | 1907.5 |  | 5 |  |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |
|  |  | 5 |  | N/A |  | 5 |  |  | N/A |  | 887.5 |  | 13.6 |  | IMD5 |  |
|  |  | n77 |  | 3305 |  | 10 |  |  | 50 |  | 3305 |  | N/A |  | N/A |  |
|  |  | 2 |  | N/A |  | 5 |  |  | N/A |  | 1987 |  | 24.8 |  | IMD3 |  |
|  |  | 5 |  | 846.5 |  | 5 |  |  | 25 |  | 891.5 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3680 |  | 10 |  |  | 50 |  | 3680 |  | N/A |  | N/A |  |
| DC_2A_n5A-n77A2 DC_2A-2A_n5A-n77A2 DC_2A_n5A-n77C2 DC_2A-2A_n5A-n77C2 |  | 2 |  | 1907 |  | 5 |  |  | 25 |  | 1987 |  | N/A |  | N/A |  |
|  |  | n5 |  | N/A |  | 5 |  |  | N/A |  | 889 |  | 13.6 |  | IMD52 |  |
|  |  | n77 |  | 3305 |  | 10 |  |  | 50 |  | 3305 |  | N/A |  | N/A |  |
| DC_2A-7A_n78A |  | 2 |  | 1870 |  | 5 |  |  | 25 |  | 1950 |  | 20.0 |  | IMD4 |  |
|  |  | 7 |  | 2550 |  | 5 |  |  | 25 |  | 2685 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3525 |  | 10 |  |  | 50 |  | 3525 |  | N/A |  | N/A |  |
| DC_2A-12A_n77A DC_2A-12A_n77(2A) DC_2A-2A-12A_n77A DC_2A-2A-12A_n77(2A) |  | 2 |  | N/A |  | 5 |  |  | N/A |  | 1960 |  | 24.8 |  | IMD32, 5 |  |
|  |  | 12 |  | 707.5 |  | 5 |  |  | 25 |  | 737.5 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3375 |  | 10 |  |  | 50 |  | 3375 |  | N/A |  | N/A |  |
| DC_2A-13A_n77A DC_2A-2A-13A_n77A DC_2A-13A_n77C DC_2A-2A-13A_n77C |  | 2 |  | N/A |  | 5 |  |  | N/A |  | 1944 |  | 24.2 |  | IMD3 |  |
|  |  | 13 |  | 783 |  | 5 |  |  | 25 |  | 752 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3510 |  | 10 |  |  | 50 |  | 3510 |  | N/A |  | N/A |  |
| DC_2A-14A_n77A DC_2A-14A_n77(2A) DC_2A-2A-14A_n77A DC_2A-2A-14A_n77(2A) |  | 2 |  | N/A |  | 5 |  |  | N/A |  | 1954 |  | 24.8 |  | IMD3 |  |
|  |  | 14 |  | 793 |  | 5 |  |  | 25 |  | 763 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3540 |  | 10 |  |  | 50 |  | 3540 |  | N/A |  | N/A |  |
| DC_2A-30A_n77A DC_2A-30A_n77(2A) DC_2A-2A-30A_n77A DC_2A-2A-30A_n77(2A) |  | 2 |  | N/A |  | 5 |  |  | N/A |  | 1986 |  | 19.3 |  | IMD42 |  |
|  |  | 30 |  | 2312 |  | 5 |  |  | 25 |  | 2357 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3305 |  | 10 |  |  | 50 |  | 3305 |  | N/A |  | N/A |  |
|  |  | 2 |  | 1905 |  | 5 |  |  | 25 |  | 1985 |  | N/A |  | N/A |  |
|  |  | 30 |  | N/A |  | 5 |  |  | N/A |  | 2354 |  | 22.2 |  | IMD42 |  |
|  |  | n77 |  | 3361 |  | 10 |  |  | 50 |  | 3361 |  | N/A |  | N/A |  |
|  |  | 2 |  | 1860 |  | 5 |  |  | 25 |  | 1940 |  | N/A |  | N/A |  |
|  |  | 30 |  | N/A |  | 5 |  |  | N/A |  | 2354 |  | 12.9 |  | IMD5 |  |
|  |  | n77 |  | 3967 |  | 10 |  |  | 50 |  | 3967 |  | N/A |  | N/A |  |
| DC_2A-66A_n41A |  | 2 |  | N/A |  | 5 |  |  | N/A |  | 1940 |  | 22.6 |  | IMD4 |  |
|  |  | 66 |  | 1715 |  | 5 |  |  | 25 |  | 2115 |  | N/A |  | N/A |  |
|  |  | n41 |  | 2685 |  | 5 |  |  | 25 |  | 2685 |  | N/A |  | N/A |  |
| DC_2A-66A_n77A DC_2A-66A_n77(2A) DC_2A-2A-66A_n77A DC_2A-2A-66A_n77(2A) DC_2A-66A-66A_n77A DC_2A-66A-66A_n77(2A) DC_2A-2A-66A-66A_n77A DC_2A-66A_n77C DC_2A-66A-66A_n77C DC_2A-2A-66A-66A_n77C |  | 2 |  | 1855 |  | 5 |  |  | 25 |  | 1935 |  | N/A |  | N/A |  |
|  |  | 66 |  | N/A |  | 5 |  |  | N/A |  | 2115 |  | 34.7 |  | IMD2 |  |
|  |  | n77 |  | 3970 |  | 10 |  |  | 50 |  | 3970 |  | N/A |  | N/A |  |
|  |  | 2 |  | 1880 |  | 5 |  |  | 25 |  | 1960 |  | M/A |  | N/A |  |
|  |  | 66 |  | N/A |  | 5 |  |  | N/A |  | 2140 |  | 21.1 |  | IMD41 |  |
|  |  | n77 |  | 3500 |  | 10 |  |  | 50 |  | 3500 |  | N/A |  | N/A |  |
|  |  | 2 |  | N/A |  | 5 |  |  | N/A |  | 1960 |  | 37.6 |  | IMD2 |  |
|  |  | 66 |  | 1760 |  | 5 |  |  | 25 |  | 2160 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3720 |  | 10 |  |  | 50 |  | 3720 |  | N/A |  | N/A |  |
|  |  | 2 |  | N/A |  | 5 |  |  | N/A |  | 1940 |  | 19.8 |  | IMD41,2 |  |
|  |  | 66 |  | 1775 |  | 5 |  |  | 25 |  | 2195 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3385 |  | 10 |  |  | 50 |  | 3385 |  | N/A |  | N/A |  |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A DC_2A_n66A-n77C DC_2A-2A_n66A-n77C |  | 2 |  | 1855 |  | 5 |  |  | 25 |  | 1935 |  | N/A |  | N/A |  |
|  |  | n66 |  | N/A |  | 5 |  |  | N/A |  | 2115 |  | 35.2 |  | IMD2 |  |
|  |  | n77 |  | 3970 |  | 10 |  |  | 50 |  | 3970 |  | N/A |  | N/A |  |
|  |  | 2 |  | 1900 |  | 5 |  |  | 25 |  | 1980 |  | N/A |  | N/A |  |
|  |  | n66 |  | N/A |  | 5 |  |  | N/A |  | 2160 |  | 22.3 |  | IMD43 |  |
|  |  | n77 |  | 3540 |  | 10 |  |  | 50 |  | 3540 |  | N/A |  | N/A |  |
| DC_2A-66A_n78A |  | 2 |  | 1880 |  | 5 |  |  | 25 |  | 1960 |  | M/A |  | N/A |  |
|  |  | 66 |  | 1740 |  | 5 |  |  | 25 |  | 2140 |  | 21.1 |  | IMD4 |  |
|  |  | n78 |  | 3500 |  | 10 |  |  | 50 |  | 3500 |  | N/A |  | N/A |  |
|  |  | 2 |  | 1880 |  | 5 |  |  | 25 |  | 1960 |  | 37.6 |  | IMD2 |  |
|  |  | 66 |  | 1760 |  | 5 |  |  | 25 |  | 2160 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3720 |  | 10 |  |  | 50 |  | 3720 |  | N/A |  | N/A |  |
|  |  | 2 |  | 1860 |  | 5 |  |  | 25 |  | 1940 |  | 19.8 |  | IMD4 |  |
|  |  | 66 |  | 1775 |  | 5 |  |  | 25 |  | 2195 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3385 |  | 10 |  |  | 50 |  | 3385 |  | N/A |  | N/A |  |
|  |  | 2 |  | 1880 |  | 5 |  |  | 25 |  | 1960 |  | 13.2 |  | IMD5 |  |
|  |  | 66 |  | 1760 |  | 5 |  |  | 25 |  | 2160 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3620 |  | 10 |  |  | 50 |  | 3620 |  | N/A |  | N/A |  |
| DC_3A_n1A-n78A DC_3A-3A_n1A-n78A |  | 3 |  | 1770 |  | 5 |  |  | 25 |  | 1865 |  | N/A |  | N/A |  |
|  |  | n1 |  | N/A |  | 5 |  |  | N/A |  | 2130 |  | 17.8 |  | IMD5 |  |
|  |  | n78 |  | 3720 |  | 10 |  |  | 50 |  | 3720 |  | N/A |  | N/A |  |
| DC_3A_n1A-n79A |  | n1 |  | N/A |  | 5 |  |  | N/A |  | 2140 |  | 18.7 |  | IMD5 |  |
|  |  | 3 |  | 1750 |  | 5 |  |  | 25 |  | 1845 |  | N/A |  | N/A |  |
|  |  | n79 |  | 4860 |  | 40 |  |  | 216 |  | 4860 |  | N/A |  | N/A |  |
| DC_3A-7A_n78A DC_3A-3A-7A_n78A DC_3A-7A-7A_n78A DC_3A-3A-7A-7A_n78A |  | 3 |  | N/A |  | 5 |  |  | N/A |  | 1820 |  | 26.5 |  | IMD35 |  |
|  |  | 7 |  | 2565 |  | 5 |  |  | 25 |  | 2685 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3310 |  | 10 |  |  | 50 |  | 3310 |  | N/A |  | N/A |  |
| DC_3A-8A_n77A |  | 3 |  | 1715 |  | 5 |  |  | 25 |  | 1810 |  | N/A |  | N/A |  |
| DC_3C-8A_n77A |  | 8 |  | 910 |  | 5 |  |  | 25 |  | 955 |  | 21.2 |  | IMD4 |  |
| DC_3A-8A_n77(2A) |  | n77 |  | 4190 |  | 10 |  |  | 50 |  | 4190 |  | N/A |  | N/A |  |
| DC_3C-8A_n77(2A) DC_3A-8A_n77(3A) |  | 3 |  | 1725 |  | 5 |  |  | 25 |  | 1820 |  | 24.8 |  | IMD3 |  |
|  |  | 8 |  | 910 |  | 5 |  |  | 25 |  | 955 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3640 |  | 10 |  |  | 50 |  | 3640 |  | N/A |  | N/A |  |
| DC_3A-8A_n78A DC_3A-8B_n78A DC_3A-3A-8A_n78A DC_3A-3A-8B_n78A DC_3C-8A_n78A DC_3A-8A_n78(2A) |  | 8 |  | 910 |  | 5 |  |  | 25 |  | 955 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3640 |  | 10 |  |  | 50 |  | 3640 |  | N/A |  | N/A |  |
|  |  | 3 |  | N/A |  | 5 |  |  | N/A |  | 1820 |  | 24.8 |  | IMD3 |  |
| DC_3A-8A_n79A |  | 8 |  | 910 |  | 5 |  |  | 25 |  | 955 |  | N/A |  | N/A |  |
|  |  | n79 |  | 4580 |  | 40 |  |  | 216 |  | 4580 |  | N/A |  | N/A |  |
|  |  | 3 |  | N/A |  | 5 |  |  | N/A |  | 1850 |  | 21.2 |  | IMD4 |  |
| DC_3A-11A_n77A |  | 3 |  | 1720 |  | 5 |  |  | 25 |  | 1815 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3675 |  | 10 |  |  | 50 |  | 3675 |  | N/A |  | N/A |  |
|  |  | 11 |  | N/A |  | 5 |  |  | N/A |  | 1491 |  | 20.2 |  | IMD4 |  |
| DC_3A-18A_n77A |  | 3 |  | N/A |  | 5 |  |  | N/A |  | 1865 |  | 24.2 |  | IMD3 |  |
|  |  | 18 |  | 820 |  | 5 |  |  | 25 |  | 865 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3505 |  | 10 |  |  | 50 |  | 3505 |  | N/A |  | N/A |  |
| DC_3A-19A_n77A DC_3A-19A_n77(2A) |  | 3 |  | N/A |  | 5 |  |  | N/A |  | 1850 |  | 26.3 |  | IMD3 |  |
|  |  | 19 |  | 835 |  | 5 |  |  | 25 |  | 880 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3520 |  | 10 |  |  | 50 |  | 3520 |  | N/A |  | N/A |  |
| DC_3A-19A_n78A DC_3A-19A_n78(2A) |  | 3 |  | N/A |  | 5 |  |  | N/A |  | 1850 |  | 26.3 |  | IMD3 |  |
|  |  | 19 |  | 835 |  | 5 |  |  | 25 |  | 880 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3520 |  | 10 |  |  | 50 |  | 3520 |  | N/A |  | N/A |  |
| DC_3A-19A_n79A |  | 3 |  | 1775 |  | 5 |  |  | 25 |  | 1870 |  | N/A |  | N/A |  |
|  |  | 19 |  | N/A |  | 5 |  |  | N/A |  | 885 |  | 27.5 |  | IMD35 |  |
|  |  | n79 |  | 4435 |  | 40 |  |  | 216 |  | 4435 |  | N/A |  | N/A |  |
|  |  | 3 |  | N/A |  | 5 |  |  | N/A |  | 1877.5 |  | 16.2 |  | IMD4 |  |
|  |  | 19 |  | 842.5 |  | 5 |  |  | 25 |  | 887.5 |  | N/A |  | N/A |  |
|  |  | n79 |  | 4420 |  | 40 |  |  | 216 |  | 4420 |  | N/A |  | N/A |  |
| DC_3A-21A_n77A DC_3A-21A_n77(2A) |  | 3 |  | 1767.5 |  | 5 |  |  | 25 |  | 1862.5 |  | N/A |  | N/A |  |
|  |  | 21 |  | N/A |  | 5 |  |  | N/A |  | 1507.5 |  | 20.8 |  | IMD4 |  |
|  |  | n77 |  | 3795 |  | 10 |  |  | 50 |  | 3795 |  | N/A |  | N/A |  |
|  |  | 3 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | IMD2 |  |
|  |  | 21 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | n77 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | 3 |  | N/A |  | 5 |  |  | N/A |  | 1866.6 |  | 18.4 |  | IMD5 |  |
|  |  | 21 |  | 1450.4 |  | 5 |  |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3935 |  | 10 |  |  | 50 |  | 3935 |  | N/A |  | N/A |  |
| DC_3A-21A_n78A DC_3A-21A_n78(2A) |  | 3 |  | N/A |  | 5 |  |  | N/A |  | 1862.5 |  | 36.6 |  | IMD2 |  |
|  |  | 21 |  | 1459.5 |  | 5 |  |  | 25 |  | 1507.5 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3322 |  | 10 |  |  | 50 |  | 3322 |  | N/A |  | N/A |  |
|  |  | 3 |  | 1767.5 |  | 5 |  |  | 25 |  | 1862.5 |  | N/A |  | N/A |  |
|  |  | 21 |  | 1459.5 |  | 5 |  |  | 25 |  | 1507.5 |  | 23.2 |  | IMD4 |  |
|  |  | n78 |  | 3795 |  | 10 |  |  | 50 |  | 3795 |  | N/A |  | N/A |  |
|  |  | 3 |  | 1767.5 |  | 5 |  |  | 25 |  | 1862.5 |  | N/A |  | N/A |  |
|  |  | 21 |  | N/A |  | 5 |  |  | N/A |  | 1503.5 |  | 9.5 |  | IMD5 |  |
|  |  | n78 |  | 3403 |  | 10 |  |  | 50 |  | 3403 |  | N/A |  | N/A |  |
| DC_3A-21A_n79A7 |  | 3 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | 21 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | IMD3 |  |
|  |  | n79 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | 3 |  | N/A |  | 5 |  |  | N/A |  | 1869.2 |  | 32.8 |  | IMD3 |  |
|  |  | 21 |  | 1450.4 |  | 5 |  |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |
|  |  | n79 |  | 4770 |  | 10 |  |  | 50 |  | 4770 |  | N/A |  | N/A |  |
| DC_3A-28A_n41A |  | 3 |  | 1720 |  | 5 |  |  | 25 |  | 1815 |  | N/A |  | N/A |  |
|  |  | n41 |  | 2510 |  | 5 |  |  | 25 |  | 2510 |  | N/A |  | N/A |  |
|  |  | 28 |  | N/A |  | 5 |  |  | N/A |  | 790 |  | 32 |  | IMD211 |  |
|  |  | 3 |  | N/A |  | 5 |  |  | N/A |  | 1832.5 |  | 32 |  | IMD2 |  |
|  |  | n41 |  | 2543 |  | 10 |  |  | 50 |  | 2543 |  | N/A |  | N/A |  |
|  |  | 28 |  | 710.5 |  | 5 |  |  | 25 |  | 765.5 |  | N/A |  | N/A |  |
| DC_3A-28A_n77A |  | 3 |  | 1712.5 |  | 5 |  |  | 25 |  | 1807.5 |  | N/A |  | N/A |  |
|  |  | 28 |  | N/A |  | 5 |  |  | N/A |  | 770 |  | 24.2 |  | IMD3 |  |
|  |  | n77 |  | 4195 |  | 10 |  |  | 50 |  | 4195 |  | N/A |  | N/A |  |
|  |  | 3 |  | N/A |  | 5 |  |  | N/A |  | 1850 |  | 25.8 |  | IMD35 |  |
|  |  | 28 |  | 735 |  | 5 |  |  | 25 |  | 790 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3320 |  | 10 |  |  | 50 |  | 3320 |  | N/A |  | N/A |  |
| DC_3A_n28A-n77A DC_3A_n28A-n77(2A) |  | 3 |  | 1712.5 |  | 5 |  |  | 25 |  | 1807.5 |  | N/A |  | N/A |  |
|  |  | n28 |  | 715 |  | 5 |  |  | 25 |  | 770 |  | 24.2 |  | IMD3 |  |
|  |  | n77 |  | 4195 |  | 10 |  |  | 50 |  | 4195 |  | N/A |  | N/A |  |
| DC_3A-28A_n78A |  | 3 |  | N/A |  | 5 |  |  | N/A |  | 1850 |  | 25.9 |  | IMD3 |  |
|  |  | 28 |  | 735 |  | 5 |  |  | 25 |  | 790 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3320 |  | 10 |  |  | 50 |  | 3320 |  | N/A |  | N/A |  |
| DC_3A_n41A-n77A |  | 3 |  | 1720 |  | 5 |  |  | 25 |  | 1815 |  | N/A |  | N/A |  |
|  |  | n41 |  | 2580 |  | 5 |  |  | 25 |  | 2580 |  | N/A |  | N/A |  |
|  |  | n77 |  | N/A |  | 10 |  |  | N/A |  | 3440 |  | 25.6 |  | IMD31 |  |
|  |  | 3 |  | 1720 |  | 5 |  |  | 25 |  | 1815 |  | N/A |  | N/A |  |
|  |  | n41 |  | N/A |  | 5 |  |  | N/A |  | 2640 |  | 13 |  | IMD5 |  |
|  |  | n77 |  | 3900 |  | 10 |  |  | 50 |  | 3900 |  | N/A |  | N/A |  |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) |  | 3 |  | 1720 |  | 5 |  |  | 25 |  | 1815 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3900 |  | 10 |  |  | 50 |  | 3900 |  | N/A |  | N/A |  |
|  |  | 41 |  | N/A |  | 5 |  |  | N/A |  | 2640 |  | 13 |  | IMD5 |  |
| DC_3A-42A_n79A9 DC_3A-42C_n79A9 DC_3A-42D_n79A9 DC_3A-42E_n79A9 |  | 3 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | 42 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | IMD5 |  |
|  |  | n79 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
| DC_3A_n78A-n79A |  | 3 |  | 1770 |  | 5 |  |  | 25 |  | 1865 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3340 |  | 10 |  |  | 50 |  | 3340 |  | N/A |  | N/A |  |
|  |  | n79 |  | N/A |  | 10 |  |  | N/A |  | 4910 |  | 25.3 |  | IMD3 |  |
|  |  | 3 |  | 1770 |  | 5 |  |  | 25 |  | 1865 |  | N/A |  | N/A |  |
|  |  | n78 |  | N/A |  | 10 |  |  | N/A |  | 3710 |  | 25.2 |  | IMD5 |  |
|  |  | n79 |  | 4510 |  | 10 |  |  | 50 |  | 4510 |  | N/A |  | N/A |  |
| DC_5A_n2A-n77A2 DC_5A_n2A-n77C2 |  | n2 |  | N/A |  | 5 |  |  | N/A |  | 1987 |  | 25.5 |  | IMD3 |  |
|  |  | 5 |  | 846.5 |  | 5 |  |  | 25 |  | 891.5 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3680 |  | 10 |  |  | 50 |  | 3680 |  | N/A |  | N/A |  |
| DC_5A_n5A-n77A2 DC_5A_n5A-n77C2 |  | 5 |  | 834 |  | 5 |  |  | 25 |  | 879 |  | N/A |  | N/A |  |
|  |  | n5 |  | N/A |  | 5 |  |  | N/A |  | 889 |  | 20.3 |  | IMD41 |  |
|  |  | n77 |  | 3391 |  | 10 |  |  | 50 |  | 3391 |  | N/A |  | N/A |  |
| DC_5A-13A_n77A2 DC_5A-13A_n77C2 |  | 5 |  | 840 |  | 5 |  |  | 25 |  | 885 |  | N/A |  | N/A |  |
|  |  | 13 |  | N/A |  | 5 |  |  | N/A |  | 750 |  | 19.4 |  | IMD5 |  |
|  |  | n77 |  | 4110 |  | 10 |  |  | 50 |  | 4110 |  | N/A |  | N/A |  |
|  |  | 5 |  | N/A |  | 5 |  |  | N/A |  | 885 |  | 19.5 |  | IMD5 |  |
|  |  | 13 |  | 782 |  | 5 |  |  | 20 |  | 751 |  | N/A |  | N/A |  |
|  |  | n77 |  | 4013 |  | 10 |  |  | 50 |  | 4013 |  | N/A |  | N/A |  |
| DC_5A-30A_n77A DC_5A-30A_n77(2A) |  | 5 |  | N/A |  | 5 |  |  | N/A |  | 880 |  | 23.5 |  | IMD31 |  |
|  |  | 30 |  | 2310 |  | 5 |  |  | 25 |  | 2355 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3740 |  | 10 |  |  | 50 |  | 3740 |  | N/A |  | N/A |  |
|  |  | 5 |  | 835 |  | 5 |  |  | 25 |  | 880 |  | N/A |  | N/A |  |
|  |  | 30 |  | N/A |  | 5 |  |  | N/A |  | 2355 |  | 21.4 |  | IMD32 |  |
|  |  | n77 |  | 4025 |  | 10 |  |  | 50 |  | 4025 |  | N/A |  | N/A |  |
| DC_5A-66A_n77A DC_5A-66A_n77(2A) DC_5A-66A-66A_n77A DC_5A-66A-66A_n77(2A) |  | 5 |  | 826.5 |  | 5 |  |  | 25 |  | 871.5 |  | N/A |  | N/A |  |
|  |  | 66 |  | N/A |  | 5 |  |  | N/A |  | 2142 |  | 22.2 |  | IMD3 |  |
|  |  | n77 |  | 3795 |  | 10 |  |  | 50 |  | 3795 |  | N/A |  | N/A |  |
| DC_5A_n66A-n77A DC_5A_n66A-n77C |  | 5 |  | 826.5 |  | 5 |  |  | 25 |  | 871.5 |  | N/A |  | N/A |  |
|  |  | n66 |  | N/A |  | 5 |  |  | N/A |  | 2142 |  | 22.2 |  | IMD3 |  |
|  |  | n77 |  | 3795 |  | 10 |  |  | 50 |  | 3795 |  | N/A |  | N/A |  |
| DC_7A_n1A-n78A DC_7A-7A_n1A-n78A |  | 1 |  | N/A |  | 5 |  |  | N/A |  | 2140 |  | 19.7 |  | IMD4 |  |
|  |  | 7 |  | 2510 |  | 10 |  |  | 50 |  | 2630 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3580 |  | 10 |  |  | 50 |  | 3580 |  | N/A |  | N/A |  |
| DC_7A_n5A-n78A |  | 7 |  | 2555 |  | 5 |  |  | 25 |  | 2675 |  | N/A |  | N/A |  |
|  |  | n5 |  | N/A |  | 5 |  |  | N/A |  | 881 |  | 34.7 |  | IMD21 |  |
|  |  | n78 |  | 3436 |  | 10 |  |  | 50 |  | 3436 |  | N/A |  | N/A |  |
| DC_7A-8A_n78A DC_7A-8B_n78A DC_7A-7A-8A_n78A DC_7A-7A-8B_n78A |  | 7 |  | 2530 |  | 5 |  |  | 25 |  | 2650 |  | N/A |  | N/A |  |
|  |  | 8 |  | N/A |  | 5 |  |  | N/A |  | 940 |  | 35.5 |  | IMD21 |  |
|  |  | n78 |  | 3470 |  | 10 |  |  | 50 |  | 3470 |  | N/A |  | N/A |  |
|  |  | 7 |  | N/A |  | 5 |  |  | N/A |  | 2650 |  | 33 |  | IMD2 |  |
|  |  | 8 |  | 895 |  | 5 |  |  | 25 |  | 940 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3545 |  | 10 |  |  | 50 |  | 3545 |  | N/A |  | N/A |  |
| DC_7A_n8A-n78A DC_7A-7A_n8A-n78A |  | 7 |  | 2530 |  | 5 |  |  | 25 |  | 2650 |  | N/A |  | N/A |  |
|  |  | n8 |  | N/A |  | 5 |  |  | N/A |  | 940 |  | 35.5 |  | IMD21 |  |
|  |  | n78 |  | 3470 |  | 10 |  |  | 50 |  | 3470 |  | N/A |  | N/A |  |
| DC_7A-28A_n78A |  | 7 |  | 2567.5 |  | 5 |  |  | 25 |  | 2687.5 |  | N/A |  | N/A |  |
|  |  | 28 |  | N/A |  | 5 |  |  | N/A |  | 782.5 |  | 33.8 |  | IMD21 |  |
|  |  | n78 |  | 3350 |  | 10 |  |  | 50 |  | 3350 |  | N/A |  | N/A |  |
|  |  | 7 |  | N/A |  | 5 |  |  | N/A |  | 2650 |  | 35.5 |  | IMD2 |  |
|  |  | 28 |  | 740 |  | 5 |  |  | 25 |  | 795 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3390 |  | 10 |  |  | 50 |  | 3390 |  | N/A |  | N/A |  |
| DC_7A_n28A-n78A |  | 7 |  | 2565 |  | 5 |  |  | 25 |  | 2685 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3365 |  | 10 |  |  | 50 |  | 3365 |  | N/A |  | N/A |  |
|  |  | n28 |  | N/A |  | 5 |  |  | N/A |  | 800 |  | 33.8 |  | IMD21 |  |
| DC_7A-66A_n78A |  | 7 |  | 2540 |  | 5 |  |  | 25 |  | 2660 |  | N/A |  | N/A |  |
|  |  | 66 |  | 1760 |  | 5 |  |  | 25 |  | 2160 |  | 20.5 |  | IMD4 |  |
|  |  | n78 |  | 3620 |  | 10 |  |  | 50 |  | 3620 |  | N/A |  | N/A |  |
| DC_8A_n1A-n77A DC_8A_n1A-n77(2A) |  | 8 |  | 910 |  | 5 |  |  | 25 |  | 955 |  | N/A |  | N/A |  |
|  |  | n1 |  | N/A |  | 5 |  |  | N/A |  | 2140 |  | 27.5 |  | IMD3 |  |
|  |  | n77 |  | 3960 |  | 10 |  |  | 50 |  | 3960 |  | N/A |  | N/A |  |
| DC_8A_n1A-n79A |  | 8 |  | 900 |  | 5 |  |  | 25 |  | 945 |  | N/A |  | N/A |  |
|  |  | n1 |  | N/A |  | 5 |  |  | 25 |  | 2145 |  | 25.7 |  | IMD4 |  |
|  |  | n79 |  | 4845 |  | 40 |  |  | 216 |  | 4845 |  | N/A |  | N/A |  |
| DC_8A_n3A-n77A DC_8A_n3A-n77(2A) |  | 8 |  | 910 |  | 5 |  |  | 25 |  | 955 |  | N/A |  | N/A |  |
|  |  | n3 |  | N/A |  | 5 |  |  | N/A |  | 1820 |  | 24.5 |  | IMD3 |  |
|  |  | n77 |  | 3640 |  | 10 |  |  | 50 |  | 3640 |  | N/A |  | N/A |  |
| DC_8A_n3A-n79A |  | 8 |  | 910 |  | 5 |  |  | 25 |  | 955 |  | N/A |  | N/A |  |
|  |  | n3 |  | N/A |  | 5 |  |  | N/A |  | 1850 |  | 22.7 |  | IMD4 |  |
|  |  | n79 |  | 4580 |  | 40 |  |  | 216 |  | 4580 |  | N/A |  | N/A |  |
| DC_8A-11A_n79A |  | 8 |  | 882.5 |  | 5 |  |  | 25 |  | 927.5 |  | N/A |  | N/A |  |
|  |  | n79 |  | 4980 |  | 40 |  |  | 216 |  | 4980 |  | N/A |  | N/A |  |
|  |  | 11 |  | N/A |  | 5 |  |  | N/A |  | 1478.4 |  | 16.2 |  | IMD5 |  |
| DC_8A_n28A-n77A DC_8A_n28A-n77(2A) |  | 8 |  | 910 |  | 5 |  |  | 25 |  | 955 |  | N/A |  | N/A |  |
|  |  | n28 |  | N/A |  | 5 |  |  | N/A |  | 765 |  | 23 |  | IMD4 |  |
|  |  | n77 |  | 3495 |  | 10 |  |  | 50 |  | 3495 |  | N/A |  | N/A |  |
| DC_8A_n28A-n78A |  | 8 |  | 910 |  | 5 |  |  | 25 |  | 955 |  | N/A |  | N/A |  |
|  | n28 |  | N/A |  | 5 |  |  | N/A |  | 765 |  | 23 |  | IMD4 |  |
|  | n78 |  | 3495 |  | 10 |  |  | 50 |  | 3495 |  | N/A |  | N/A |  |
| DC_8A_n28A-n79A | 8 |  | 905 |  | 5 |  |  | 25 |  | 950 |  | N/A |  | N/A |  |
|  | n79 |  | 4420 |  | 40 |  |  | 216 |  | 4420 |  | N/A |  | N/A |  |
|  | n28 |  | N/A |  | 5 |  |  | N/A |  | 800 |  | 24.0 |  | IMD5 |  |
| DC_8A-41A_n77A DC_8A-41C_n77A | 8 |  | 895 |  | 5 |  |  | 25 |  | 940 |  | N/A |  | N/A |  |
|  | 41 |  | N/A |  | 5 |  |  | N/A |  | 2650 |  | 34.0 |  | IMD2 |  |
|  | n77 |  | 3545 |  | 10 |  |  | 50 |  | 3545 |  | N/A |  | N/A |  |
| DC_12A-30A_n77A DC_12A-30A_n77(2A) | 12 |  | N/A |  | 5 |  |  | N/A |  | 740 |  | 23.5 |  | IMD31 |  |
|  | 30 |  | 2310 |  | 5 |  |  | 25 |  | 2355 |  | N/A |  | N/A |  |
|  | n77 |  | 3880 |  | 10 |  |  | 50 |  | 3880 |  | N/A |  | N/A |  |
|  | 12 |  | 707.5 |  | 5 |  |  | 25 |  | 737.5 |  | N/A |  | N/A |  |
|  | 30 |  | N/A |  | 5 |  |  | N/A |  | 2355 |  | 21.4 |  | IMD3 |  |
|  | n77 |  | 3770 |  | 10 |  |  | 50 |  | 3770 |  | N/A |  | N/A |  |
| DC_12A-66A_n77A DC_12A-66A_n77(2A) DC_12A-66A-66A_n77A DC_12A-66A-66A_n77(2A) | 12 |  | N/A |  | 5 |  |  | N/A |  | 740 |  | 23.5 |  | IMD32 |  |
|  | 66 |  | 1720 |  | 5 |  |  | 25 |  | 2120 |  | N/A |  | N/A |  |
|  | n77 |  | 4180 |  | 10 |  |  | 50 |  | 4180 |  | N/A |  | N/A |  |
|  | 12 |  | 707 |  | 5 |  |  | 25 |  | 737 |  | N/A |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A |  | 2126 |  | 21.4 |  | IMD3 |  |
|  | n77 |  | 3540 |  | 10 |  |  | 50 |  | 3540 |  | N/A |  | N/A |  |
| DC_12A-71A_n2A | 12 |  | 713.5 |  | 5 |  |  | 25 |  | 743.5 |  | 4.2 |  | IMD5 |  |
|  | 71 |  | 665.5 |  | 5 |  |  | 25 |  | 619.5 |  | N/A |  | N/A |  |
|  | n2 |  | 1907.5 |  | 5 |  |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |
| DC_12A-71A_n77A | 12 |  | 702 |  | 5 |  |  | 25 |  | 732 |  | 4.4 |  | IMD5 |  |
|  | 71 |  | 667 |  | 5 |  |  | 25 |  | 621 |  | N/A |  | N/A |  |
|  | n77 |  | 3400 |  | 10 |  |  | 50 |  | 3400 |  | N/A |  | N/A |  |
|  | 12 |  | 701.5 |  | 5 |  |  | 25 |  | 731.5 |  | N/A |  | N/A |  |
|  | 71 |  | 690 |  | 5 |  |  | 25 |  | 644 |  | 3.9 |  | IMD5 |  |
|  | n77 |  | 3450 |  | 10 |  |  | 50 |  | 3450 |  | N/A |  | N/A |  |
| DC_13A_n2A-n77A DC_13A_n2A-n77C | 13 |  | 782 |  | 5 |  |  | 25 |  | 751 |  | N/A |  | N/A |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A |  | 1960 |  | 25.0 |  | IMD3 |  |
|  | n77 |  | 3524 |  | 10 |  |  | 50 |  | 3524 |  | N/A |  | N/A |  |
| DC_13A_n5A-n77A2 DC_13A_n5A-n77C2 | n5 |  | 840 |  | 5 |  |  | 25 |  | 885 |  | 19.5 |  | IMD5 |  |
|  | 13 |  | 782 |  | 5 |  |  | 20 |  | 751 |  | N/A |  | N/A |  |
|  | n77 |  | 4013 |  | 10 |  |  | 50 |  | 4013 |  | N/A |  | N/A |  |
| DC_13A-66A_n77A DC_13A-66A-66A_n77A DC_13A-66A_n77C DC_13A-66A-66A_n77C | 13 |  | 782 |  | 5 |  |  | 25 |  | 751 |  | N/A |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A |  | 2156 |  | 25.3 |  | IMD3 |  |
|  | n77 |  | 3720 |  | 10 |  |  | 50 |  | 3720 |  | N/A |  | N/A |  |
|  | 13 |  | N/A |  | 5 |  |  | N/A |  | 750 |  | 23.4 |  | IMD32 |  |
|  | 66 |  | 1720 |  | 5 |  |  | 25 |  | 2120 |  | N/A |  | N/A |  |
|  | n77 |  | 4190 |  | 10 |  |  | 50 |  | 4190 |  | N/A |  | N/A |  |
| DC_13A_n66A-n77A DC_13A_n66A-n77C | 13 |  | 782 |  | 5 |  |  | 25 |  | 751 |  | N/A |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A |  | 2156 |  | 26.1 |  | IMD3 |  |
|  | n77 |  | 3720 |  | 10 |  |  | 50 |  | 3720 |  | N/A |  | N/A |  |
| DC_14A-30A_n77A DC_14A-30A_n77(2A) | 14 |  | N/A |  | 5 |  |  | N/A |  | 763 |  | 23.5 |  | IMD31 |  |
|  | 30 |  | 2310 |  | 5 |  |  | 25 |  | 2355 |  | N/A |  | N/A |  |
|  | n77 |  | 3857 |  | 10 |  |  | 50 |  | 3857 |  | N/A |  | N/A |  |
|  | 14 |  | 793 |  | 5 |  |  | 25 |  | 763 |  | N/A |  | N/A |  |
|  | 30 |  | N/A |  | 5 |  |  | N/A |  | 2355 |  | 21.4 |  | IMD3 |  |
|  | n77 |  | 3941 |  | 10 |  |  | 50 |  | 3941 |  | N/A |  | N/A |  |
| DC_14A-66A_n77A DC_14A-66A_n77(2A) DC_14A-66A-66A_n77A DC_14A-66A-66A_n77(2A) | 14 |  | N/A |  | 5 |  |  | N/A |  | 763 |  | 23.5 |  | IMD32 |  |
|  | 66 |  | 1712.5 |  | 5 |  |  | 25 |  | 2112.5 |  | N/A |  | N/A |  |
|  | n77 |  | 4188 |  | 10 |  |  | 50 |  | 4188 |  | N/A |  | N/A |  |
|  | 14 |  | 793 |  | 5 |  |  | 25 |  | 763 |  | N/A |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A |  | 2155 |  | 21.4 |  | IMD3 |  |
|  | n77 |  | 3741 |  | 10 |  |  | 50 |  | 3741 |  | N/A |  | N/A |  |
| DC_18A_n28A-n77A | 18 |  | 820 |  | 5 |  |  | 25 |  | 865 |  | N/A |  | N/A |  |
|  | n28 |  | 723 |  | 5 |  |  | 25 |  | 778 |  | 17.5 |  | IMD5 |  |
|  | n77 |  | 4058 |  | 10 |  |  | 50 |  | 4058 |  | N/A |  | N/A |  |
| DC_19A-21A_n77A DC_19A-21A_n77(2A) | 19 |  | N/A |  | 5 |  |  | N/A |  | 882.5 |  | 27.7 |  | IMD3 |  |
|  | 21 |  | 1450.4 |  | 5 |  |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |
|  | n77 |  | 3783.3 |  | 10 |  |  | 50 |  | 3783.3 |  | N/A |  | N/A |  |
|  | 19 |  | N/A |  | 5 |  |  | N/A |  | 882.5 |  | 25.2 |  | IMD4 |  |
|  | 21 |  | 1450.4 |  | 5 |  |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |
|  | n77 |  | 3468.7 |  | 10 |  |  | 50 |  | 3468.7 |  | N/A |  | N/A |  |
|  | 19 |  | 837.5 |  | 5 |  |  | 25 |  | 882.5 |  | N/A |  | N/A |  |
|  | 21 |  | N/A |  | 5 |  |  | N/A |  | 1502.5 |  | 21.0 |  | IMD4 |  |
|  | n77 |  | 4015 |  | 10 |  |  | 50 |  | 4015 |  | N/A |  | N/A |  |
| DC_19A-21A_n78A DC_19A-21A_n78(2A) | 19 |  | N/A |  | 5 |  |  | N/A |  | 882.5 |  | 27.7 |  | IMD3 |  |
|  | 21 |  | 1450.4 |  | 5 |  |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |
|  | n78 |  | 3783.3 |  | 10 |  |  | 50 |  | 3783.3 |  | N/A |  | N/A |  |
|  | 19 |  | N/A |  | 5 |  |  | N/A |  | 882.5 |  | 25.2 |  | IMD4 |  |
|  | 21 |  | 1450.4 |  | 5 |  |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |
|  | n78 |  | 3468.7 |  | 10 |  |  | 50 |  | 3468.7 |  | N/A |  | N/A |  |
| DC_19A-21A_n79A7 | 19 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | IMD5 |  |
|  | 21 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  | n79 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  | 19 |  | 837.5 |  | 5 |  |  | 25 |  | 882.5 |  | N/A |  | N/A |  |
|  | 21 |  | N/A |  | 5 |  |  | N/A |  | 1500 |  | 24.8 |  | IMD5 |  |
|  | n79 |  | 4850 |  | 10 |  |  | 50 |  | 4850 |  | N/A |  | N/A |  |
| DC_19A-42A_n79A10 DC_19A-42C_n79A10 | 19 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  | 42 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | IMD2 |  |
|  | n79 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
| DC_19A_n78A-n79A | 19 |  | 835 |  | 5 |  |  | 25 |  | 880 |  | N/A |  | N/A |  |
|  | n78 |  | 3680 |  | 10 |  |  | 50 |  | 3680 |  | N/A |  | N/A |  |
|  | n79 |  | N/A |  | 40 |  |  | N/A |  | 4515 |  | 35.3 |  | IMD2 |  |
|  | 19 |  | 835 |  | 5 |  |  | 25 |  | 880 |  | N/A |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A |  | 3715 |  | 34.8 |  | IMD2 |  |
|  | n79 |  | 4550 |  | 40 |  |  | 216 |  | 4550 |  | N/A |  | N/A |  |
| DC_21A-42A_n79A10 DC_21A-42C_n79A10 | 21 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  | 42 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | IMD2 |  |
|  | n79 |  | N/A |  | N/A |  |  | N/A |  | N/A |  | N/A |  | N/A |  |
| DC_21A_n78A-n79A | 21 |  | 1453 |  | 5 |  |  | 25 |  | 1501 |  | N/A |  | N/A |  |
|  | n78 |  | 3420 |  | 10 |  |  | 50 |  | 3420 |  | N/A |  | N/A |  |
|  | n79 |  | N/A |  | 10 |  |  | N/A |  | 4873 |  | 36.1 |  | IMD25 |  |
|  | 21 |  | 1453 |  | 5 |  |  | 25 |  | 1501 |  | N/A |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A |  | 3487 |  | 38.8 |  | IMD2 |  |
|  | n79 |  | 4940 |  | 10 |  |  | 50 |  | 4940 |  | N/A |  | N/A |  |
| DC_29A-30A_n77A | 29 |  | N/A |  | 5 |  |  | N/A |  | 722 |  | 23.5 |  | IMD31 |  |
|  | 30 |  | 2310 |  | 5 |  |  | 25 |  | 2355 |  | N/A |  | N/A |  |
|  | n77 |  | 3898 |  | 10 |  |  | 50 |  | 3898 |  | N/A |  | N/A |  |
| DC_29A-66A_n77A DC_29A-66A-66A_n77A | 29 |  | N/A |  | 5 |  |  | N/A |  | 722 |  | 23.5 |  | IMD32 |  |
|  | 66 |  | 1734 |  | 5 |  |  | 25 |  | 2134 |  | N/A |  | N/A |  |
|  | n77 |  | 4190 |  | 10 |  |  | 50 |  | 4190 |  | N/A |  | N/A |  |
| DC_30A-66A_n77A DC_30A-66A_n77(2A) DC_30A-66A-66A_n77A DC_30A-66A-66A_n77(2A) | 30 |  | N/A |  | 5 |  |  | N/A |  | 2355 |  | 34.2 |  | IMD22 |  |
|  | 66 |  | 1745 |  | 5 |  |  | 25 |  | 2145 |  | N/A |  | N/A |  |
|  | n77 |  | 4100 |  | 10 |  |  | 50 |  | 4100 |  | N/A |  | N/A |  |
|  | 30 |  | N/A |  | 5 |  |  | N/A |  | 2355 |  | 12.9 |  | IMD5 |  |
|  | 66 |  | 1735 |  | 5 |  |  | 25 |  | 2135 |  | N/A |  | N/A |  |
|  | n77 |  | 3780 |  | 10 |  |  | 50 |  | 3780 |  | N/A |  | N/A |  |
|  | 30 |  | 2310 |  | 5 |  |  | 25 |  | 2355 |  | N/A |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A |  | 2160 |  | 19.2 |  | IMD42 |  |
|  | n77 |  | 3390 |  | 10 |  |  | 50 |  | 3390 |  | N/A |  | N/A |  |
| DC_41A_n28A-n77A | n28 |  | 743 |  | 5 |  |  | 25 |  | 798 |  | 36.8 |  | IMD21,11 |  |
|  | 41 |  | 2642 |  | 5 |  |  | 25 |  | 2642 |  | N/A |  | N/A |  |
|  | n77 |  | 3440 |  | 10 |  |  | 50 |  | 3440 |  | N/A |  | N/A |  |
| DC_66A_n2A-n77A DC_66A-66A_n2A-n77A DC_66A_n2A-n77C | n2 |  | N/A |  | 5 |  |  | N/A |  | 1960 |  | 37.6 |  | IMD2 |  |
|  | 66 |  | 1760 |  | 5 |  |  | 25 |  | 2160 |  | N/A |  | N/A |  |
|  | n77 |  | 3720 |  | 10 |  |  | 50 |  | 3720 |  | N/A |  | N/A |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A |  | 1960 |  | 21.1 |  | IMD41,2 |  |
|  | 66 |  | 1770 |  | 5 |  |  | 25 |  | 2170 |  | N/A |  | N/A |  |
|  | n77 |  | 3350 |  | 10 |  |  | 50 |  | 3350 |  | N/A |  | N/A |  |
| DC_66A_n66A-n77A | 66 |  | 1750 |  | 5 |  |  | 25 |  | 2150 |  | N/A |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A |  | 2150 |  | 37 |  | IMD2 |  |
|  | n77 |  | 3900 |  | 10 |  |  | 50 |  | 3900 |  | N/A |  | N/A |  |
|  | 66 |  | 1750 |  | 5 |  |  | 25 |  | 2150 |  | N/A |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A |  | 2170 |  | 20 |  | IMD5 |  |
|  | n77 |  | 3710 |  | 10 |  |  | 50 |  | 3710 |  | N/A |  | N/A |  |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: This UE channel bandwidth is optional in this release of the specification NOTE 4: Void NOTE 5: This band is subject to IMD4 also which MSD is not specified. NOTE 6: E-UTRA carrier shall be set to min(+23 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+23 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 7: The frequency range in band n79 is restricted for this band combination to 4400 - 4900 MHz for both the UL and the DL. NOTE 8: The frequency range in band 1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130 - 2150 MHz for the DL. NOTE 9: The frequency range in band n79 is restricted for this band combination to 4500 - 5000 MHz for both the UL and the DL NOTE 10: The frequency range in band n79 is restricted for this band combination to 4500 - 4600 MHz for both the UL and the DL NOTE 11: This band is subject to IMD3 also which MSD is not specified |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 7.3B.2.3.5.3	Void

==============End of change==============