3GPP TSG-RAN4 Meeting #117	R4-2522033

# Dallas, United States, 17th - 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0252 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections based on NR_UE_OTA_Enh Testability Discussions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-10-31 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Various clarifications are needed to the spec to clarify the radiated interface boundary and to define certain metrics untestable |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Declare the radiated interface boundary to be associated to the far-field region - Consider vibration environmental condition and pointing stability not testable from a conformance perspective - Consider the antenna gain not mandatory for conformance testing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion which test systems are applicable, requirement for VSAT vendors to provide conducted ports for all VSAT types, introduction of environmental conditions such as 100 km/h maximum wind speed would require completely different test systems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.3, 9.6.1.2.2, 9.6.2, 10.8, E.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Cat F CR in R4-2523047; Discussion papers in R4-2522035 and R4-2513347 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

"Carrier-off" state: radio state in which the NTN VSAT may transmit but does not transmit any carrier.

NOTE:	"NTN VSAT may transmit" means that all the conditions for transmission are satisfied (e.g. in a state where transmissions are permitted, no failure detected, and the NTN VSAT is correctly pointed towards the satellite).

NOTE:	The existence of a "Carrier-off" radio state depends on the system of transmission used. For NTN VSATs designed for continuous transmission mode there may be no "Carrier-off" state.

"Carrier-on" state: Radio state in which the NTN VSAT may transmit and transmits a carrier.

Co-polarized transmission: when the DUT transmission antenna polarization is aligned with test antenna polarization.

Cross-polarized transmission: when the DUT transmission antenna polarization is such with respect to the test antenna polarization that an incident wave from the DUT transmission antenna results in lowest available power at the test antenna.

"Emissions disabled" state: Radio state in which the Mobile VSAT is not emitting (e.g. before system monitoring pass, before the control channel is received, when a failure is detected, when a Mobile VSAT is commanded to disable, and when the Mobile VSAT is in a location requiring cessation of emissions).

Enhanced channel raster: channel raster with a 10 kHz granularity in bands with a 100 kHz channel raster.

eRedCap UE: The UE with enhanced reduced capabilities as defined in clause 4.2.22.1 from TS38.306 [11].

EIRP (Link=TX beam peak direction, Meas=Link angle): measurement of the EIRP of the VSAT such that the measurement angle is aligned with the beam peak direction within an acceptable measurement error uncertainty. EIRP (indicator to be measured) can be replaced by Frequency, EVM, carrier Leakage, In-band emission and OBW

NOTE:	For requirements measured along the TX beam peak direction, the radiated interface boundary is associated to the far-field region

EIS (Link=RX beam peak direction, Meas=Link angle): measurement of the EIS of the VSAT such that the measurement angle is aligned with the RX beam peak direction within an acceptable measurement error uncertainty.

NOTE 1:	The sensitivity is the minimum received power level at which specific requirement is met.

NOTE 2:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

NOTE 3:	For requirements measured along the RX beam peak direction, the radiated interface boundary is associated to the far-field region

Feeder link: A radio link from an earth station at a given location to a space station, or vice versa, conveying information for a space radiocommunication service other than for the fixed-satellite service. The given location may be at a specified fixed point, or at any fixed point within specified areas.

Fixed Satellite Service: A radiocommunication service between earth stations at given positions, when one or more satellites are used; the given position may be a specified fixed point or any fixed point within specified areas; in some cases this service includes satellite-to-satellite links, which may also be operated in the inter-satellite service; the fixed-satellite service may also include feeder links for other space radiocommunication services.

Fixed VSAT: VSAT used in FSS system at given position; the given position may be a specified fixed point or any fixed point within specified areas.

NOTE:	Mobile VSAT is excluded from this definition.

Geostationary satellite: A geosynchronous satellite whose circular and direct orbit lies in the plane of the Earth’s equator and which thus remains fixed relative to the Earth; by extension, a geosynchronous satellite which remains approximately fixed relative to the Earth.

Geostationary-Satellite Orbit: The orbit of a geosynchronous satellite whose circular and direct orbit lies in the plane of the Earth's equator.

Geosynchronous Earth Orbit: Earth-centered orbit at approximately 35786 kilometres above Earth's surface and synchronised with Earth's rotation. A geostationary orbit is a non-inclined geosynchronous orbit, i.e. in the Earth’s equator plane.

Geosynchronous satellite: An earth satellite whose period of revolution is equal to the period of rotation of the Earth about its axis.

Low Earth Orbit: Orbit around the Earth with an altitude between 300 km, and 1500 km.

Mobile VSAT: VSAT on moving platform, and which can be further declined in three types: airborne, maritime or land based.

NOTE:	Mobile VSAT can be also referred to as ESIM or ESOMP.

Non-terrestrial networks: Networks, or segments of networks, using an airborne or space-borne vehicle to embark a transmission equipment relay node or  SAN.

NTN VSAT: a UE operating in FR2-NTN which could be a Fixed VSAT or a Mobile VSAT.

Plane perpendicular to the GSO arc: The plane that is perpendicular to the “plane tangent to the GSO arc,” as defined below, and includes a line between the earth station in question and the GSO space station that it is communicating with (FCC 47 CFR 25.103).

Plane tangent to the GSO arc: The plane defined by the location of an earth station's transmitting antenna and a line in the equatorial plane that is tangent to the GSO arc at the location of the GSO space station that the earth station is communicating with (FCC 47 CFR 25.103).

RedCap UE: The UE with reduced capabilities as defined in clause 4.2.21.1 from TS38.306 [11].

Satellite: A space-borne vehicle embarking a transparent payload, or a regenerative payload telecommunication transmitter, placed into Low-Earth Orbit (LEO), Medium-Earth Orbit (MEO), or Geostationary Earth Orbit (GEO).

Satellite Access Node: node providing NR user plane and control plane protocol terminations towards NTN satellite capable UE, and connected via the NG interface to the 5GC. It encompasses a transparent payload on board a NTN platform, with satellite-gateway and gNB functions.

TRP (Link=TX beam peak direction, Meas=TRP grid): measurement of the TRP of the VSAT such that the measurement angles are aligned with the directions of the TRP grid points within an acceptable measurement uncertainty while the link angle is aligned with the TX beam peak direction

NOTE:	For requirements based on EIRP, the radiated interface boundary is associated to the far-field region

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

<<< Skip Unchanged Sections >>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

BW	Bandwidth

BWP	Bandwidth Part

CP-OFDM	Cyclic Prefix-OFDM

CW	Continuous Wave

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DM-RS	Demodulation Reference Signal

DTX	Discontinuous Transmission

EIRP	Equivalent Isotropic Radiated Power

EIS	Effective Isotropic Sensitivity

(e)RedCap	Redcap or eRedCap

eRedCap	enhanced Reduced Capability

ESIM	Earth Station in Motion

ESOMP	Earth Stations on Mobile Platforms

EVM	Error Vector Magnitude

FR	Frequency Range

FRC	Fixed Reference Channel

FSS	Fixed Satellite Service

GEO	Geosynchronous Earth Orbit

GSCN	Global Synchronization Channel Number

GSO	Geostationary-Satellite Orbit

IBB	In-band Blocking

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LEO	Low Earth Orbiting

MBW	Measurement bandwidth defined for the protected band

MOP	Maximum Output Power

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

NGEO	Non-Geostationary Earth Orbiting

NGSO	Non-Geostationary-Satellite Orbit

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

NS	Network Signalling

NTN	Non-Terrestrial Network

OCNG	OFDMA Channel Noise Generator

OOB	Out-of-band

PRB	Physical Resource Block

QAM	Quadrature Amplitude Modulation

RAN	Radio Access Network

RE	Resource Element

RedCap	Reduced Capability

REFSENS	REFerence SENSitivity

RF	Radio Frequency

RMS	Root Mean Square (value)

RSRP	Reference Signal Receive Power

RSRQ	Reference Signal Receive Quality

RX	Receiver

SAN	Satellite Access Node

SC	Single Carrier

SCS	Subcarrier spacing

SEM	Spectrum Emission Mask

SNR	Signal-to-Noise Ratio

SRS	Sounding Reference Symbol

SS	Synchronization Symbol

TN	Terrestrial Network

TRP	Total Radiated Power

TX	Transmitter

TxD	Tx Diversity

UE	User Equipment

VSAT	Very Small Aperture Terminal

<<< Skip Unchanged Sections >>>

##### 9.6.1.2.2	Pointing Stability

Editor’s Note: The pointing stability is a system level requirement and is considered not testable from a conformance perspective.

Pointing stability: Under the condition of 100 km/h maximum wind speed, with gusts of 130 km/h lasting 3 seconds, the installation shall not show any sign of permanent distortion and shall not need repointing after the application of the wind load.

<<< Skip Unchanged Sections >>>

### 9.6.2	Antenna performance

Editor’s Note: The antenna gain is not mandatory for conformance testing.

#### 9.6.2.1	Minimum requirements for bands n510 and n511

The following requirements are applicable to NTN VSAT type 1, type 2, type 4 or type 5 operating in band n511 or in band n510 and communicating with Geostationary Satellite Orbit (GSO) SAN.

The co-polarization gain in the plane tangent to the GSO arc shall not exceed the levels specified in Table 9.6.2.1-1. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2.1-1: Co-polarization gain limit in the plane tangent to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 2° ≤ θ ≤ 7° | 29 – 25log10(θ) |
| 7° ≤ θ ≤ 9.2° | 8 |
| 9.2° ≤ θ ≤ 19.1° | 32 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 0 |

The co-polarization gain in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2.1-2. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2.1-2: Co-polarization gain limit in the plane perpendicular to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 3.5° ≤ θ ≤ 7° | 32 – 25log10(θ) |
| 7° ≤ θ ≤ 9.2° | 10.9 |
| 9.2° ≤ θ ≤ 19.1° | 35 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 3 |

The off-axis cross-polarization gain in the plane tangent and in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2.1-3.

Table 9.6.2.1-3: Cross-polarization gain limit

| θ value | Cross-polarization gain (dBi) |
| --- | --- |
| 2° ≤ θ ≤ 7° | 19 – 25log10(θ) |

#### 9.6.2.2	Requirement for network signalling value “NS_203N”

When "NS_203N" is indicated in the cell, the following requirements shall apply to NTN VSAT.

The co-polarization gain in the plane tangent to the GSO arc shall not exceed the levels specified in Table 9.6.2.2-1. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2.2-1: Co-polarization gain limit in the plane tangent to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 1.5° ≤ θ ≤ 7° | 29 – 25log10(θ) |
| 7° ≤ θ ≤ 9.2° | 8 |
| 9.2° ≤ θ ≤ 19.1° | 32 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 0 |

The co-polarization gain in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2.2-2. This envelope may be exceeded by up to 6 dB in 10% of the range of θ angles from ±3–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2.2-2: Co-polarization gain limit in the plane perpendicular to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 3° ≤ θ ≤ 19.1° | 32 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 0 |

The off-axis cross-polarization gain in the plane tangent to the GSO arc shall not exceed the levels specified in Table 9.6.2.2-3.

Table 9.6.2.2-3: Cross-polarization gain limit in the plane tangent to the GSO arc

| θ value | Cross-polarization gain (dBi) |
| --- | --- |
| 1.8° ≤ θ ≤ 7° | 19 – 25log10(θ) |

The off-axis cross-polarization gain in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2.2-4.

Table 9.6.2.2-4: Cross-polarization gain limit in the plane perpendicular to the GSO arc

| θ value | Cross-polarization gain (dBi) |
| --- | --- |
| 3° ≤ θ ≤ 7° | 19 – 25log10(θ) |

#### 9.6.2.3	Requirement for network signalling value “NS_207N”

When "NS_207N" is indicated in the cell, the following requirements shall apply to NTN VSAT.

The co-polarization gain in the plane tangent to the GSO arc shall not exceed the levels specified in Table 9.6.2.3-1. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2.3-1: Co-polarization gain limit in the plane tangent to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 1.5° ≤ θ ≤ 7° | 29 – 25log10(θ) |
| 7° ≤ θ ≤ 9.2° | 8 |
| 9.2° ≤ θ ≤ 48° | 32 – 25log10(θ) |
| 48° < θ ≤ 180° | -10 |

The co-polarization gain in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2.3-2. This envelope may be exceeded by up to 6 dB in 10% of the range of θ angles from ±3–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2.3-2: Co-polarization gain limit in the plane perpendicular to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 3° ≤ θ ≤ 48° | 32 – 25log10(θ) |
| 48° < θ ≤ 180° | -10 |

The off-axis cross-polarization gain in the plane tangent to the GSO arc shall not exceed the levels specified in Table 9.6.2.3-3.

Table 9.6.2.3-3: Cross-polarization gain limit in the plane tangent to the GSO arc

| θ value | Cross-polarization gain (dBi) |
| --- | --- |
| 1.8° ≤ θ ≤ 7° | 19 – 25log10(θ) |

The off-axis cross-polarization gain in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2.3-4.

Table 9.6.2.3-4: Cross-polarization gain limit in the plane perpendicular to the GSO arc

| θ value | Cross-polarization gain (dBi) |
| --- | --- |
| 3° ≤ θ ≤ 7° | 19 – 25log10(θ) |

<<< Skip Unchanged Sections >>>

## 10.8	Receiver antenna off-axis performance

Editor’s Note: The antenna gain is not mandatory for conformance testing.

<<< Skip Unchanged Sections >>>

## E.2.3	Vibration

Editor’s Note: The vibration environmental condition is a system level requirement and is considered not testable from a conformance perspective.

The UE shall fulfil all the requirements when vibrated at the following frequency/amplitudes.

Table E.2.3-1: Vibration conditions

| Frequency | ASD (Acceleration Spectral Density) random vibration |
| --- | --- |
| 5 Hz to 20 Hz | 0,96 m2/s3 |
| 20 Hz to 500 Hz | 0,96 m2/s3 at 20 Hz, thereafter –3 dB/Octave |

Outside the specified frequency range the UE, if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in TS 38.101-1[6] for extreme operation.

<<< Skip Unchanged Sections >>>

<<< END OF CHANGES >>>