3GPP TSG-RAN WG4 Meeting #117		   				 		  								R4-2522219

Dallas, USA, 17th – 21th Nov., 2025

Agenda item:			6.23

Source:	Moderator (MediaTek inc.)

Title:	Topic summary for [117][310] Rel-19 RRM performance_Part2_NTN

Document for:	Information

# Introduction

This document provides topic summary for NTN RRM performance part 2, including the following topics:

- Topic#1: RRM performance requirements for NR NTN Ku band (AI 5.10.2)

- Topic#2: NTN testing for NGSO (AI 6.6.3.2)

- Topic#3: RRM performance requirements for IoT NTN phase 3 (AI 6.20.3)

- Topic#4: RRM performance requirements for IoT-NTN TDD mode (AI 6.21.2)

Recommended issues for online discussion:

- Issue 1-1-1, 1-2-3

- Issue 2-1-1

- Issue 4-1-1, 4-2-1, 4-2-2, 4-2-3, 4-2-4, 4-2-5

# Topic# 1: NR NTN Ku band (AI 5.10.2)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520034 | MediaTek inc. | Observation 1: For NTN VSAT OTA, minimum Io/SSB_RP is derived from vendor-declared EIS rather than set per band group. Proposal 1: Extend the current FR2-NTN measurement requirement to cover FR1-NTN Ku band VSAT. Proposal 2: For Test cases for Ku bands in FR1-NTN, reuse existing test cases defined for FR2-NTN unless otherwise specified and introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology (Option 2). |
| R4-2521216 | Ericsson | Proposal 1: Adding the clauses for VSAT UE test cases under the corresponding clauses for SAN FR1. The content can either refer to existing test cases defined for FR2-NTN or introduce new configurations as needed. Proposal 2: Support Option 1, VSAT UE supporting both GSO and NGSO shall pass two test configurations: one with FR1 numerology and one with FR2 numerology. |
| R4-2521543 | ZTECorporation,Sanechips | Proposal 1: RAN4 shall not add the new band group. Proposal 2: Reuse existing test cases defined for FR2-NTN and specify the test configuration with FR1 numerology. Proposal 3: For test cases of UL timing requirements, VSAT UE shall be at least to pass two test configurations. Otherwise, only considering test configuration with FR2 numerology. |
| R4-2521633 | Huawei, HiSilicon | Proposal 1: Do not include band n247 and n248 in the band group for FR1-NTN. Proposal 2: Do not emulate UE mobility in the UL timing test. Proposal 3: Re-use existing test cases defined for FR2-NTN for Ku band in FR1-NTN, and introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology. Proposal 4: VSAT supporting Ku bands in both FR1-NTN and FR2-NTN should pass the test with one configuration (either FR1 or FR2 numerology), except for the UL timing test. |
| R4-2521737 | Nokia | Proposal 1: For introducing the test cases for FR1-NTN numerology, reuse the FR2-NTN test cases with the necessary modifications in the reference channels on the test case description. Proposal 2: For FR1-NTN numerology, reuse the reference channels used for 15 kHz SCS and 10 MHz of bandwidth existent for FR1. Proposal 3: For NTN UEs that support both FR1-NTN and FR2-NTN, the UE is required to pass the test cases in both configurations. Observation 1: If the mobility aspect of a VSAT UE is relevant enough to be distinguished via VSAT Type whether a UE is mobile or fixed, it is important that they are tested according to the type reported. Proposal 4: For the transmit timing test case, RAN4 to discuss how to emulate the mobility of the mobile VSAT UE. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions..

### Sub-topic 1-1: Measurement Accuracy

#### Issue 1-1-1: Measurement accuracy requirement with new band ground

Background

Table 10.1.3C.1.1-1: SS-RSRP Intra-frequency absolute accuracy in FR2-NTN

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| 7 | 10 | ≥-6 | Same value as SSB_RP derived from annex B.2.1.7, according to UE VSAT type |  | N/A | -70 |
| 9 | 12 |  | N/A |  | -70 | -50 |
| NOTE 1: Values based on EIS as defined in clause 10.3 of TS 38.101-5 [42]. Applicable side condition selected depending on angle of arrival. NOTE 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

Proposals from companies:

R4-2520034	MediaTek inc.

- Proposal 1: Extend the current FR2-NTN measurement accuracy requirement to cover FR1-NTN Ku band VSAT.

R4-2521543	ZTECorporation,Sanechips

- Proposal 1: RAN4 shall not add the new band group.

R4-2521633	Huawei, HiSilicon

- Proposal 1: Do not include band n247 and n248 in the band group for FR1-NTN.

Recommended WF:

- Not add the new band group for FR1-NTN Ku band accuracy requirement

- Further discuss the following proposal

  - Extend the current FR2-NTN measurement accuracy requirement to cover FR1-NTN Ku band VSAT.

### Sub-topic 1-2: Test cases

#### Issue 1-2-1: Test cases for Ku bands in FR1-NTN

Proposals from companies:

R4-2520034	MediaTek inc.

- Proposal 2: For Test cases for Ku bands in FR1-NTN, reuse existing test cases defined for FR2-NTN unless otherwise specified and introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology (Option 2).

R4-2521216	Ericsson

- Proposal 1: Adding the clauses for VSAT UE test cases under the corresponding clauses for SAN FR1. The content can either refer to existing test cases defined for FR2-NTN or introduce new configurations as needed.

R4-2521543	ZTECorporation,Sanechips

- Proposal 2: Reuse existing test cases defined for FR2-NTN and specify the test configuration with FR1 numerology.

R4-2521633	Huawei, HiSilicon

- Proposal 3: Re-use existing test cases defined for FR2-NTN for Ku band in FR1-NTN, and introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

R4-2521737	Nokia

- Proposal 1: For introducing the test cases for FR1-NTN numerology, reuse the FR2-NTN test cases with the necessary modifications in the reference channels on the test case description.

Recommended WF:

- Reuse the existing test cases defined for FR2-NTN for Ku band with FR1-NTN numerology with the necessary modifications

  - Introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

#### Issue 1-2-2: Reference Channel for Ku band with FR1-NTN numerology

R4-2521737	Nokia

- Proposal 2: For FR1-NTN numerology, reuse the reference channels used for 15 kHz SCS and 10 MHz of bandwidth existent for FR1.

Recommended WF:

  - Agree with the above proposal.

#### Issue 1-2-3: Test principle for VSAT supporting both FR1 and FR2 numerology.

Proposals from companies:

R4-2521216	Ericsson

- Proposal 2: Support Option 1, VSAT UE supporting both GSO and NGSO shall pass two test configurations: one with FR1 numerology and one with FR2 numerology.

R4-2521543	ZTECorporation,Sanechips

- Proposal 3: For test cases of UL timing requirements, VSAT UE shall be at least to pass two test configurations. Otherwise, only considering test configuration with FR2 numerology.

R4-2521633	Huawei, HiSilicon

- Proposal 4: VSAT supporting Ku bands in both FR1-NTN and FR2-NTN should pass the test with one configuration (either FR1 or FR2 numerology), except for the UL timing test.

R4-2521737	Nokia

- Proposal 3: For NTN UEs that support both FR1-NTN and FR2-NTN, the UE is required to pass the test cases in both configurations.

Recommended WF:

- For VSAT UE supporting Ku bands in both FR1-NTN and FR2-NTN,

  - For UL timing test, the UE shall pass two test configurations: one with FR1 numerology and one with FR2 numerology.

  - For tests other than UL timing,

    - Option 1: the UE shall pass the test with one configuration (either FR1 or FR2 numerology)

    - Option 2: the UE shall pass two test configurations: one with FR1 numerology and one with FR2 numerology.

#### Issue 1-2-4: UE transmit timing test for NGSO VSAT

Proposals from companies:

R4-2521633	Huawei, HiSilicon

- Proposal 2: Do not emulate UE mobility in the UL timing test.

R4-2521737	Nokia

- Proposal 4: For the transmit timing test case, RAN4 to discuss how to emulate the mobility of the mobile VSAT UE.

Recommended WF

  - Option 1: Do not emulate UE mobility in the UL timing test.

  - Option 2: For the transmit timing test case, RAN4 to discuss how to emulate the mobility of the mobile VSAT UE.

### Sub-topic 1-3: draftCR

A separate email thread will be initiated to collect comments for the following draftCR.

| R4-2521560 | Test case on satellite switching on Ku band | ZTECorporation,Sanechips |
| --- | --- | --- |
| R4-2521736 | DraftCR to add FR1-NTN configuration for test cases involving VSAT UEs in Ku Band | Nokia |

# Topic# 2: NTN testing for NGSO (AI 6.6.3.2)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520033 | MediaTek inc. | Proposal 1: Apply the dynamic channel to R19 UL transmit timing test cases and do not apply it to the timing advance test. |
| R4-2521630 | Huawei, HiSilicon | Proposal 1: The TE-emulated channel model with varying Doppler and delay shifts is applied to UL transmit timing test cases only. Proposal 2: The TE-emulated channel model with varying Doppler and delay shifts is applied to R19 UE only. |
| R4-2522027 | Samsung | Proposal #1: Apply the new FR1-NTN channel model for R19 UL timing advance adjustment accuracy test cases - No impacts for the test procedure or test requirements; apply the following adaptation - Change the “Propagation condition” from “AWGN” to “AWGN with time varying Doppler and delay shifts” - During the tests, UE position is needed to be added and set by AT command at the beginning of T1 or other suitable method. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself. |

## Open issues summary

### Sub-topic 2-1: New channel model in FR1-NTN for R19 NGSO uplink timing test

Background: RP-241281, Revised WID

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

The relevant UL timing tests of NTN is provided for reference:

For NR NTN FR1 tests in 36.133

## A.14.3.1 UE transmit timing for Satellite Access A.14.3.1.1 NR UE Transmit Timing Test for FR1 ## A.14.3.2 Timing advance for satellite access ## A.14.3.2.1 SA FR1 timing advance adjustment accuracy

Iot-NTN tests in 36.133

## A.13.4 Timing and signalling characteristics for satellite access ### A.13.4.1 UE transmit timing for satellite access A.13.4.1.1 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access #### A.13.4.1.2 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access #### A.13.4.1.3 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access ### A.13.4.2 UE timing advance for satellite access A.13.4.2.1 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access A.13.4.2.2 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access

Issue 2-1-1: R19 UL timing tests and test requirement impact

Agreement

- Apply the new FR1-NTN channel model for R19 UL transmit timing test cases

  - No impacts for the test procedure or test requirements; apply the following adaptation

    - Change the “Propagation condition” from “AWGN” to “AWGN with time varying Doppler and delay shifts”

    - Clarify that NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts.

    - During the tests, UE position is needed to be added and set by AT command at the beginning of T1 or other suitable method. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

  - Do not add new additional margins in the test case. Test tolerance is left for RAN5

- FFS applicability of the timing advance test and its additional margin

Agreement in RAN4#116bis

#### Issue 2-1-1: R19 UL timing tests and test requirement impact

Proposals from companies:

R4-2520033	MediaTek inc.

- Proposal 1: Apply the dynamic channel to R19 UL transmit timing test cases and do not apply it to the timing advance test.

R4-2521630	Huawei, HiSilicon

- Proposal 1: The TE-emulated channel model with varying Doppler and delay shifts is applied to UL transmit timing test cases only.

- Proposal 2: The TE-emulated channel model with varying Doppler and delay shifts is applied to R19 UE only.

R4-2522027	Samsung

- Proposal #1: Apply the new FR1-NTN channel model for R19 UL timing advance adjustment accuracy test cases

  - No impacts for the test procedure or test requirements; apply the following adaptation

    - Change the “Propagation condition” from “AWGN” to “AWGN with time varying Doppler and delay shifts”

    - During the tests, UE position is needed to be added and set by AT command at the beginning of T1 or other suitable method. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

Recommended WF:

- Please discuss the following options:

  - Option 1: The TE-emulated channel model with varying Doppler and delay shifts is applied to R19 UL transmit timing test cases only.

  - Option 2: The TE-emulated channel model with varying Doppler and delay shifts is applied to R19 UL timing advance adjustment accuracy test cases.

- The pre-R19 issues can be discussed in AI.6.6.3.1.2 under thread [328].

### Sub-topic 2-2: draftCR

A separate email thread will be initiated to collect comments for the following draftCR.

| R4-2520206 | Draft CR on IoT NTN timing tests for NGSO | Samsung |
| --- | --- | --- |

# Topic# 3: Testing for CB-Msg3-EDT (AI 6.20.3)

## Companies’ contributions summary

No discussion paper submitted under this AI.

## Open issues summary

### Sub-topic 3-1: draftCR

A separate email thread will be initiated to collect comments for the following draftCR.

| R4-2520037 | Introduction of test case for CB-Msg3 procedure for NB-IoT with Satellite Access | MediaTek inc. |
| --- | --- | --- |

# Topic# 4: IoT-NTN TDD mode (AI 6.21.2)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520607 | Iridium, Cambridge Consultants (CCL), THALES | - Update Annex B.3.25A to include IoT NTN TDD band 249 and with minimum NRSRP value of -133 dBm/15 kHz - RAN4 to consider updating NPRACH reference configurations for IoT NTN TDD mode based on Table A.3.18-1 as follows: Moderator: table to be added in the below proposal Proposal 3: The highlighted NPRACH procedure test configuration parameters from Table A.13.3.2.1.1-2 should be updated for IoT NTN TDD mode. Moderator: table to be added in the below proposal Proposal 4: For random access test in normal coverage, existing power level configurations from the test case A13.3.2.1.1 can be reused which results in UE selecting enhanced coverage level 0. Proposal 5: The highlighted timing advance accuracy test configuration parameters from Table A.13.4.2.1.1-2 should be updated for IoT NTN TDD mode. Moderator: table to be added in the below proposal Proposal 6: The Qin = 1 dB and Qout = -3.5 dB can be used as a reference for specifying SNR values in RLM test cases. Proposal 7: The highlighted radio link monitoring test for out-of-sync in DRX test configuration parameters from Table A.13.4.3.1.1-2 should be updated for IoT NTN TDD mode. Moderator: table to be added in the below proposal Proposal 8: The highlighted radio link monitoring test parameters for out-of-sync in DRX from Table A.13.4.3.1.1-3 should be updated for IoT NTN TDD mode. Moderator: table to be added in the below proposal Proposal 9: The highlighted radio link monitoring test parameters for in-sync in DRX from Table A.13.4.3.4.1-2 should be updated for IoT NTN TDD mode. Moderator: table to be added in the below proposal Proposal 10: The highlighted radio link monitoring test parameters for in-sync in DRX from Table A.13.4.3.4.1-3 should be updated for IoT NTN TDD mode. Moderator: table to be added in the below proposal Proposal 11: The highlighted radio link monitoring test parameters for in-sync without DRX from Table A.13.4.3.5.1-2 should be updated for IoT NTN TDD mode. Moderator: table to be added in the below proposal Proposal 12: The highlighted radio link monitoring test parameters for in-sync without DRX from Table A.13.4.3.5.1-3 should be updated for IoT NTN TDD mode. Moderator: table to be added in the below proposal Proposal 13: The highlighted radio link monitoring test parameters for out-of-sync without DRX from Table A.13.4.3.7.1-2 should be updated for IoT NTN TDD mode. Moderator: table to be added in the below proposal Proposal 14: The highlighted radio link monitoring test parameters for out-of-sync without DRX from Table A.13.4.3.7.1-3 should be updated for IoT NTN TDD mode. Moderator: table to be added in the below proposal |
| R4-2520701 | Nordic Semiconductor ASA | Proposal-1: NRSRP corresponding to at least target coverage of -1dB assuming noise floor and NF. Value could be somewhere around [-124 … -126] dBm. Proposal-2: Keep the current accuracy values in the Tables above. Proposal-3: Remove CE Level 2, NTN TDD target coverage is -1dB SNR, no CE Level 2 is needed. Adopt above modified table. Moderator: table to be added in the below proposal Proposal-4: Adopt proposed changes for random access test. Remove CE Level 2. Moderator: table to be added in the below proposal Proposal-5: Adopt proposed above changes for timing test. Moderator: table to be added in the below proposal |
| R4-2521653 | Huawei, HiSilicon | Proposal 1: UE supporting both FDD and TDD should pass the test cases with each of the FDD and TDD configurations as default. Proposal 2: RAN4 to discuss the applicable value for the following 2 parameters in TDD. - TSI_NB-IoT, the time required for receiving all the relevant system information (8320 ms for FDD). - TPRACH_NB-IoT, the additional delay caused by the random access procedure (80 ms for FDD). One is the time required for receiving all the relevant system information. In our understanding, it includes MIB, SIB1, SIB31 and SIB33. For FDD it is defined as 8320ms. It is not clear how this value was derived, and whether the assumptions behind this value apply for TDD. For example, if the transmission periodicity of SIB31 was assumed as 160ms, it may not apply for TDD. Another is the additional delay caused by the random access procedure. We understand it is the waiting time for PRACH, and it is unclear whether the existing value of 80ms can be re-used for TDD. |

## Open issues summary

### Sub-topic 4-1: RRM performance requirements

Open issues and candidate options before meeting: N/A

#### Issue 4-1-1: Measurement accuracy

Proposals from companies:

R4-2520607	Iridium, Cambridge Consultants (CCL), THALES

- Proposal 1: Update Annex B.3.25A to include IoT NTN TDD band 249 and with minimum NRSRP value of -133 dBm/15 kHz

Table B.3.25A-1: NB-IoT intra-frequency absolute NRSRP and NRSRQ Accuracy Requirements

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_SAB _G | -139.8 |
|  | NTDD_SAB_O | -133 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |

R4-2520701	Nordic Semiconductor ASA

- Proposal-1: NRSRP corresponding to at least target coverage of -1dB assuming noise floor and NF. Value could be somewhere around [-124 … -126] dBm.

- Proposal-2: Keep the current accuracy values in the Tables above.

Moderator’s Note: No need to discuss accuracy values further; the current values were confirmed in the WF after removing the brackets.

Recommended WF:

- Discuss option 1 and 2 for Minimum NRSRP on the following table.

Table B.3.25A-1: NB-IoT intra-frequency absolute NRSRP and NRSRQ Accuracy Requirements

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_SAB _G | -139.8 |
|  | NTDD_SAB_O | Option 1: -133 Option 2: [-124 … -126] |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |

### Sub-topic 4-2: Test case

#### Issue 4-2-1: Principle of testing TDD and FDD frame structure

Proposals from companies:

R4-2521653	Huawei, HiSilicon

- Proposal 1: UE supporting both FDD and TDD should pass the test cases with each of the FDD and TDD configurations as default.

Recommended WF:

- agree on Proposal 1.

#### Issue 4-2-2: Test parameters - TSI_NB-IoT  and TPRACH_NB-IoT

Proposals from companies:

R4-2521653	Huawei, HiSilicon

- Proposal 2: RAN4 to discuss the applicable value for the following 2 parameters in TDD.

- TSI_NB-IoT, the time required for receiving all the relevant system information (8320 ms for FDD).

- TPRACH_NB-IoT, the additional delay caused by the random access procedure (80 ms for FDD).

Moderator’s note: the following reasoning is provided in R4-2521653

One is the time required for receiving all the relevant system information. In our understanding, it includes MIB, SIB1, SIB31 and SIB33. For FDD it is defined as 8320ms. It is not clear how this value was derived, and whether the assumptions behind this value apply for TDD. For example, if the transmission periodicity of SIB31 was assumed as 160ms, it may not apply for TDD.

Another is the additional delay caused by the random access procedure. We understand it is the waiting time for PRACH, and it is unclear whether the existing value of 80ms can be re-used for TDD.

Recommended WF:

•	Discuss whether the current TSI_NB-IoT  and TPRACH_NB-IoT  are applicable for TDD.

#### Issue 4-2-3: Test configuration for random access test

Proposals from companies:

R4-2520607	Iridium, Cambridge Consultants (CCL), THALES

- Proposal 2: RAN4 to consider updating NPRACH reference configurations for IoT NTN TDD mode based on Table A.3.18-1 as follows:

Table X.3.18-1: NPRACH.R-X: IoT NTN TDD Reference NPRACH Configuration

| Field | Value |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH coverage level |  |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according to the requirement of individual test cases |  |
| nprach-CP-Length | us66dot7 |  |  | NPRACH format 0 |  |
| Parameters per NPRACH coverage Level |  |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 |  | Valid values as defined in TS 36.331 [2] |  |
| nprach-Periodicity | ms320 | ms640 | ms1280 |  | {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560} |  |
| nprach-StartTime | ms8 | ms8 | ms8 |  | {ms8, ms16, ms32, ms64, ms128, ms256, ms512, ms1024} |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  | {n0, n12, n24, n36, n2, n18, n34} |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  | {n12, n24, n36, n48} |  |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} | {one} |  | {zero, oneThird, twoThird, one} |  |
| maxNumPreambleAttemptCE | n3 | n5 | n7 |  | {n3, n4, n5, n6, n7, n8, n10} |  |
| numRepetitionsPerPreambleAttempt | n1 | n4 | n8 |  | {n1, n2, n4, n8, n16, n32, n64, n128} |  |
| npdcch-NumRepetitions-RA | r1 | r4 | r8 |  | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |  |
| npdcch-StartSF-CSS-RA | v64 | v32 | v4 |  | {v1dot5, v2, v4, v8, v16, v32, v48, v64} |  |
| npdcch-Offset-RA | zero | zero | zero |  | {zero, oneEighth, oneFourth, threeEighth} |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  | {n8, n10, n11, n12, n20, n22, n23, n24, n32, n34, n35, n36, n40, n44, n48} |  |
| Note 1: See Clause 6.7.3.2 in TS 36.331 [2] for further information on the parameters in this table. |  |  |  |  |  |

- Proposal 3: The highlighted NPRACH procedure test configuration parameters from Table A.13.3.2.1.1-2 should be updated for IoT NTN TDD mode.

Table X.13.3.2.1.1-3: NPRACH-Configuration parameters for IoT NTN TDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Field | Value |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH resource |  |  |  |  |  |  |
| RSRP-ThresholdsNPRACH-InfoList | {39, 24} |  |  |  | Corresponding to {-101, -116} dBm as defined in Table 9.1.4-1 |  |
| nprach-CP-Length | us66dot7 |  |  |  |  |  |
| nrs-Power | -5 dBm/15 kHz |  |  |  | As defined in clause 6.7.3 in TS 36.331. |  |
| Backoff Parameter Index | 1 |  |  |  | As defined in table 7.2-2 in TS 36.321 |  |
| Configured UE transmitted power () | 23 dBm for power class 3, 20 dBm for power class 5 |  |  |  | As defined in clause 6.2B.4 in TS 36.102 |  |
| powerRampingStep | dB2 |  |  |  |  |  |
| preambleInitialReceivedTargetPower | dBm-112 |  |  |  |  |  |
| preambleTransMax-CE | n6 |  |  |  |  |  |
| Parameters per NPRACH Resource |  |  |  |  |  |  |
| NPRACH Resource | Level 0 | Level 1 | Level 2 |  |  |
| nprach-Periodicity | ms640 | ms1280 | ms2560 |  |  |
| nprach-StartTime | ms8 | ms64 | ms512 |  |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart | zero | zero | zero |  |  |
| maxNumPreambleAttemptCE | n3 | n6 | n10 |  |  |
| numRepetitionsPerPreambleAttempt | n4 | n8 | n16 |  |  |
| npdcch-NumRepetitions-RA | r4 | r8 | r16 |  |  |
| npdcch-StartSF-CSS-RA | v16 | v48 | v64 |  |  |
| npdcch-Offset-RA | zero | zero | Zero |  |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  |  |
| ra-ResponseWindowSize (per NPRACH Resource) | pp2 | pp2 | pp2 |  |  |
| mac-ContentionResolutionTimer (per NPRACH Resource) | pp8 | pp8 | pp8 |  |  |
| Note 1: See Clause 6.7.3 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |  |  |

Proposal 4: For random access test in normal coverage, existing power level configurations from the test case A13.3.2.1.1 can be reused which results in UE selecting enhanced coverage level 0.

R4-2520701	Nordic Semiconductor ASA

  - Update the NPRACH reference configurations based on Table A.3.18-1 as follows:

Table X.3.18-1: NPRACH.R-1: HD-FDD Reference NPRACH Configuration

| Field | Value |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH coverage level |  |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according the requirement of individual test cases |  |
| nprach-CP-Length | us66dot7 |  |  | NPRACH format 0 |  |
| Parameters per NPRACH coverage Level |  |  |  |  |  |
| CE Level | Level 0 | Level 1 |  |  | Valid values as defined in TS 36.331 [2] |  |
| nprach-Periodicity | ms640ms40 | ms640ms40 |  |  | {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560} |  |
| nprach-StartTime | ms8 | ms8 |  |  | {ms8, ms16, ms32, ms64, ms128, ms256, ms512, ms1024} |  |
| nprach-SubcarrierOffset | n0 | n0 |  |  | {n0, n12, n24, n36, n2, n18, n34} |  |
| nprach-NumSubcarriers | n12 | n12 |  |  | {n12, n24, n36, n48} |  |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} |  |  | {zero, oneThird, twoThird, one} |  |
| maxNumPreambleAttemptCE | n3 | n5 |  |  | {n3, n4, n5, n6, n7, n8, n10} |  |
| numRepetitionsPerPreambleAttempt | n1 | n8 |  |  | {n1, n2, n4, n8, n16, n32, n64, n128} |  |
| npdcch-NumRepetitions-RA | r1 | r8 |  |  | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |  |
| npdcch-StartSF-CSS-RA | v64v8 | v32v2 |  |  | {v1dot5, v2, v4, v8, v16, v32, v48, v64} |  |
| npdcch-Offset-RA | zero | zero |  |  | {zero, oneEighth, oneFourth, threeEighth} |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 |  |  | {n8, n10, n11, n12, n20, n22, n23, n24, n32, n34, n35, n36, n40, n44, n48} |  |
| Note 1: See Clause 6.7.3.2 in TS 36.331 [2] for further information on the parameters in this table. |  |  |  |  |  |

- Proposal-3: Remove CE Level 2, NTN TDD target coverage is -1dB SNR, no CE Level 2 is needed.  Adopt above modified table.

Recommended WF:

- For IoT NTN TDD Reference NPRACH Configuration

  - Discuss whether to remove coverage level 2, based on the following table.

  - Discuss the applicability of the proposed values for coverage level 2.

- Table X.3.18-1: NPRACH.R-X: IoT NTN TDD Reference NPRACH Configuration

| Field | Value |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH coverage level |  |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according to the requirement of individual test cases |  |
| nprach-CP-Length | us66dot7 |  |  | NPRACH format 0 |  |
| Parameters per NPRACH coverage Level |  |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 |  | Valid values as defined in TS 36.331 [2] |  |
| nprach-Periodicity | ms320 | ms640 | ms1280 |  | {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560} |  |
| nprach-StartTime | ms8 | ms8 | ms8 |  | {ms8, ms16, ms32, ms64, ms128, ms256, ms512, ms1024} |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  | {n0, n12, n24, n36, n2, n18, n34} |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  | {n12, n24, n36, n48} |  |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} | {one} |  | {zero, oneThird, twoThird, one} |  |
| maxNumPreambleAttemptCE | n3 | n5 | n7 |  | {n3, n4, n5, n6, n7, n8, n10} |  |
| numRepetitionsPerPreambleAttempt | n1 | n4 | n8 |  | {n1, n2, n4, n8, n16, n32, n64, n128} |  |
| npdcch-NumRepetitions-RA | r1 | r4 | r8 |  | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |  |
| npdcch-StartSF-CSS-RA | v64 | v32 | v4 |  | {v1dot5, v2, v4, v8, v16, v32, v48, v64} |  |
| npdcch-Offset-RA | zero | zero | zero |  | {zero, oneEighth, oneFourth, threeEighth} |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  | {n8, n10, n11, n12, n20, n22, n23, n24, n32, n34, n35, n36, n40, n44, n48} |  |
| Note 1: See Clause 6.7.3.2 in TS 36.331 [2] for further information on the parameters in this table. |  |  |  |  |  |

- For NPRACH-Configuration parameters in random access test,

  - Discuss whether to remove coverage level 1&2, based on the following table.

  - Discuss the applicability of the proposed values for coverage level 0,1, 2.

Table X.13.3.2.1.1-3: NPRACH-Configuration parameters for IoT NTN TDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Field | Value |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH resource |  |  |  |  |  |  |
| RSRP-ThresholdsNPRACH-InfoList | {39, 24} |  |  |  | Corresponding to {-101, -116} dBm as defined in Table 9.1.4-1 |  |
| nprach-CP-Length | us66dot7 |  |  |  |  |  |
| nrs-Power | -5 dBm/15 kHz |  |  |  | As defined in clause 6.7.3 in TS 36.331. |  |
| Backoff Parameter Index | 1 |  |  |  | As defined in table 7.2-2 in TS 36.321 |  |
| Configured UE transmitted power () | 23 dBm for power class 3, 20 dBm for power class 5 |  |  |  | As defined in clause 6.2B.4 in TS 36.102 |  |
| powerRampingStep | dB2 |  |  |  |  |  |
| preambleInitialReceivedTargetPower | dBm-112 |  |  |  |  |  |
| preambleTransMax-CE | n6 |  |  |  |  |  |
| Parameters per NPRACH Resource |  |  |  |  |  |  |
| NPRACH Resource | Level 0 | Level 1 | Level 2 |  |  |
| nprach-Periodicity | ms640 | ms1280 | ms2560 |  |  |
| nprach-StartTime | ms8 | ms64 | ms512 |  |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart | zero | zero | zero |  |  |
| maxNumPreambleAttemptCE | n3 | n6 | n10 |  |  |
| numRepetitionsPerPreambleAttempt | n4 | n8 | n16 |  |  |
| npdcch-NumRepetitions-RA | r4 | r8 | r16 |  |  |
| npdcch-StartSF-CSS-RA | v16 | v48 | v64 |  |  |
| npdcch-Offset-RA | zero | zero | Zero |  |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  |  |
| ra-ResponseWindowSize (per NPRACH Resource) | pp2 | pp2 | pp2 |  |  |
| mac-ContentionResolutionTimer (per NPRACH Resource) | pp8 | pp8 | pp8 |  |  |
| Note 1: See Clause 6.7.3 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |  |  |

#### Issue 4-2-4: Test configuration for timing test

Proposals from companies:

R4-2520607	Iridium, Cambridge Consultants (CCL), THALES

Table X.13.4.2.1.1-2: General Test Parameters for E-UTRAN Timing Advance Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| Satellite information | Config 1 |  | SSC.1 |  |
|  | Config 2 |  | SSC.2 |  |
| Timing Advance Command (TA) value during T1 |  |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  |  | 39 | NTA = 128 |
| Number of repetitions | NPDCCH |  | 16 |  |
|  | NPDSCH |  | 16 |  |
|  | NPUSCH |  | 16 |  |
| DRX |  |  | OFF |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

R4-2520701	Nordic Semiconductor ASA

Table X.13.4.2.1.1-2: General Test Parameters for E-UTRAN Timing Advance Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| Satellite information | Config 1 |  | SSC.1 |  |
|  | Config 2 |  | SSC.2 |  |
| Timing Advance Command (TA) value during T1 |  |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  |  | 39 | NTA = 128 |
| Number of repetitons | NPDCCH |  | 128 4 | Smaller repetition values for NTN TDD |
|  | NPDSCH |  | 128 8 |  |
|  | NPUSCH |  | 32 8 |  |
| DRX |  |  | OFF |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Recommended WF:

- Discuss whether to choose Option 1 or Option 2 for the number of repetitions.

Table X.13.4.2.1.1-2: General Test Parameters for E-UTRAN Timing Advance Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| Satellite information | Config 1 |  | SSC.1 |  |
|  | Config 2 |  | SSC.2 |  |
| Timing Advance Command (TA) value during T1 |  |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  |  | 39 | NTA = 128 |
| Number of repetitions | NPDCCH |  | Option 1: 16 Option 2: 4 |  |
|  | NPDSCH |  | Option 1: 16 Option 2: 8 |  |
|  | NPUSCH |  | Option 1: 16 Option 2: 8 |  |
| DRX |  |  | OFF |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

#### Issue 4-2-5: Test configuration for RLM test

Proposals from companies:

R4-2520607	Iridium, Cambridge Consultants (CCL), THALES

- Proposal 6: The Qin = 1 dB and Qout = -3.5 dB can be used as a reference for specifying SNR values in RLM test cases.

- Proposal 7: The highlighted radio link monitoring test for out-of-sync in DRX test configuration parameters from Table A.13.4.3.1.1-2 should be updated for IoT NTN TDD mode.

- Table X.13.4.3.1.1-2: General test parameters for IoT NTN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | nCell 1 |  |
| CP length |  |  | Normal |  |
| Deployment Mode |  |  | Standalone |  |
| Satellite information | Config 1 |  | SSC.1 | GSO |
|  | Config 2 |  | SSC.2 | NGSO |
| NPDCCH transmission parameters Rmax |  |  | 2 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23B.2-1 G=48 to give NPDCCH period of 96ms |
| DRX cycle |  | ms | 256 | See Table A.13.4.3.1.1-4 |
| Layer 3 filtering Note 2,3 |  |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 |  | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 5.12 |  |
| dT |  | S | 0.8 |  |
| T2 |  | s | 10.24 |  |
| dT |  | S | 0.7 |  |
| T3 |  | s | 5.12 |  |
| dT |  | S | 1.4 |  |
| T4 |  | s | 5.12 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |  |

- Proposal 8: The highlighted radio link monitoring test parameters for out-of-sync in DRX from Table A.13.4.3.1.1-3 should be updated for IoT NTN TDD mode.

Table X.13.4.3.1.1-3: nCell specific test parameters for IoT NTN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| BWchannel | kHz | 200 |  |  |  |  |  |  |
| OCNG Pattern as defined in A.3.2.3.3 Note 1 |  | NOP.3 FDD |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.6.3 |  | R.30 NB-TDD |  |  |  |  |  |  |
| NPBCH_RA | dB | 0 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  |
| SNR Note 4, 5 | dB | 7 | Note 6 | -1 | Note 7 | -8 | Note 8 | 7 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNRs in time periods T1, T2, T3 and T4 are denoted as SNR1, SNR2, SNR3 and SNR1 respectively in figure A.13.4.3.1.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

- Proposal 9: The highlighted radio link monitoring test parameters for in-sync in DRX from Table A.13.4.3.4.1-2 should be updated for IoT NTN TDD mode.

Table X.13.4.3.4.1-2: General test parameters for IoT NTN TDD in-sync test with DRX for UE category NB1 Standalone mode in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| Active cell |  |  | nCell 1 |  |
| CP length |  |  | Normal |  |
| Satellite information | Config 1 |  | SSC.1 | GSO |
|  | Config 2 |  | SSC.2 | NGSO |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23A.2 and Table 7.23A.2-1 respectively, where NPDCCH Repetition level = Rmax/2, and Rmax is a configurable parameter defined in TS 36.331. G=48 to give NPDCCH period of 96ms |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 2 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212[21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23A.2 and Table 7.23A.2-1 respectively, where NPDCCH Repetition level = Rmax, and Rmax is a configurable parameter defined in TS 36.331. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 2 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| DRX cycle |  | ms | 256 | See Table A.13.4.3.4.1-4 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| - Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

- Proposal 10: The highlighted radio link monitoring test parameters for in-sync in DRX from Table A.13.4.3.4.1-3 should be updated for IoT NTN TDD mode.

- Table X.13.4.3.4.1-3: nCell 1 specific test parameters for IoT NTN TDD in-sync radio link monitoring test with DRX for UE category NB1 Standalone mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 200 |  |  |  |  |
| OCNG Pattern as defined in A.3.2.3.3 Note 1 |  | NOP.3 FDD |  |  |  |  |
| NPDCCH parameters defined in A.3.1.6.3 |  | R.30 HD-TDD |  |  |  |  |
| Ratio of NPDSCH to NRS EPRE | dB | 0 |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |
| NPBCH_RA | dB |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 5, Note 6 | dB | 7 | Note 7 | -8 | Note 8 | 7 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.13.4.3.4.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

- Proposal 11: The highlighted radio link monitoring test parameters for in-sync without DRX from Table A.13.4.3.5.1-2 should be updated for IoT NTN TDD mode.

Table X.13.4.3.5.1-2: General test parameters for IoT NTN TDD in-sync test without DRX for UE category NB1 Standalone mode in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| Active cell |  |  | nCell 1 |  |
| CP length |  |  | Normal |  |
| Satellite information | Config 1 |  | SSC.1 | GSO |
|  | Config 2 |  | SSC.2 | NGSO |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23A.2 and Table 7.23A.2-1 respectively, where NPDCCH Repetition level = Rmax/2, and Rmax is a configurable parameter defined in TS 36.331. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 2 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212[21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23A.2 and Table 7.23A.2-1 respectively, where NPDCCH Repetition level = Rmax, and Rmax is a configurable parameter defined in TS 36.331. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 2 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| - Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

- Proposal 12: The highlighted radio link monitoring test parameters for in-sync without DRX from Table A.13.4.3.5.1-3 should be updated for IoT NTN TDD mode.

- Table X.13.4.3.5.1-3: nCell 1 specific test parameters for IoT NTN TDD in-sync radio link monitoring test without DRX for UE category NB1 Standalone mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 200 |  |  |  |  |
| OCNG Pattern as defined in A.3.2.3.3 Note 1 |  | NOP.3 FDD |  |  |  |  |
| NPDCCH parameters defined in A.3.1.6.3 |  | R.30 HD-FDD |  |  |  |  |
| NPDSCH_RA | dB | 0 |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |
| NPBCH_RA | dB |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 5, Note 6 | dB | 7 | Note 7 | -8 | Note 8 | 7 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.13.4.3.5.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Proposal 13: The highlighted radio link monitoring test parameters for out-of-sync without DRX from Table A.13.4.3.7.1-2 should be updated for IoT NTN TDD mode.

Table X.13.4.3.7.1-2: General test parameters for IoT NTN TDD Radio Link Monitoring Test for out-of-sync tests without DRX for UE Category NB1 Standalone mode in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| Active cell |  |  | nCell 1 |  |
| CP length |  |  | Normal |  |
| Satellite information | Config 1 |  | SSC.1 | GSO |
|  | Config 2 |  | SSC.2 | NGSO |
| NPDCCH repetition level Rmax |  |  | 2 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23A.2-1 |
| DRX |  |  | OFF |  |
| Layer 3 filtering Note 2,3 |  |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 |  | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 |  | ms | 3000 | T311 is enabled |
| T1 |  | s | 2 |  |
| dT |  | s | 0.8 |  |
| T2 |  | s | 0.4 |  |
| dT |  | s | 0.7 |  |
| T3 |  | s | 0.5 |  |
| dT |  | s | 1.4 |  |
| T4 |  | s | 0.4 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |  |

Proposal 14: The highlighted radio link monitoring test parameters for out-of-sync without DRX from Table A.13.4.3.7.1-3 should be updated for IoT NTN TDD mode.

Table X.13.4.3.7.1-3: nCell1 specific test parameters for IoT NTN TDD Radio Link Monitoring Test for out-of-sync without DRX for UE Category NB1 Standalone mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| NB-IoT Channel Bandwidth (BWchannel) | kHz | 200 |  |  |  |  |  |  |
| OCNG Pattern as defined in A.3.2.3.3 Note 1 |  | NOP.3 FDD |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.6.3 |  | R.30 HD-TDD |  |  |  |  |  |  |
| NPBCH_RA | dB | 0 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RA Note 1 | dB |  |  |  |  |  |  |  |
| OCNG_RB Note 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 KHz | -98 |  |  |  |  |  |  |
| SNR Note 4,5 | - | 7 | Note 6 | -1 | Note 7 | -8 | Note 6 | 7 |
| Propagation Condition | - | AWGN |  |  |  |  |  |  |
| Antenna Configuration | - | 1x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNRs in time periods T1, T2, T3 and T4 are denoted as SNR1, SNR2, SNR3 and SNR1 respectively in figure A.13.4.3.7.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Recommended WF:

- Discuss whether the following proposal is acceptable.

  - The Qin = 1 dB and Qout = -3.5 dB can be used as a reference for specifying SNR values in RLM test cases.

- A separate email thread will be initiated to collect comments on the corresponding draft CR (R4-2520547), regarding whether the modifications to RLM in Proposals 7–14 of R4-2520607 are acceptable.

### Sub-topic 4-3: draftCR

A separate email thread will be initiated to collect comments for the following draftCR.

| R4-2520038 | Introduction of test case of RRC Re-establishment and Random Access for NB-IoT in TDD mode with Satellite Access | MediaTek inc. |
| --- | --- | --- |
| R4-2520261 | Draft CR for 36.133 RRM meas accuracy, reference NPRACH config for IoT_NTN_TDD | CAMBRIDGE CONSULTANTS LTD, Iridium Satellite LLC, THALES |
| R4-2520547 | Draft CR on 36.133 introduction of RLM test cases for IoT_NTN_TDD | CAMBRIDGE CONSULTANTS LTD, Iridium Satellite LLC, THALES |
| R4-2520608 | Draft CR on 36.133 introduction of DL CQI reporting accuracy test cases for IoT_NTN_TDD | CAMBRIDGE CONSULTANTS LTD, Iridium Satellite LLC, THALES |
| R4-2521558 | Test case on timing advance | ZTECorporation,Sanechips |
| R4-2521559 | Test case on transmit timing | ZTECorporation,Sanechips |
| R4-2521654 | draftCR on measurement test cases for IoT NTN TDD | Huawei, HiSilicon |
| R4-2521742 | CR Reselection test cases for IoT NTN TDD | Nokia |