# 3GPP TSG-RAN WG4 Meeting #117	R4-2522330

# Dallas, USA, Nov. 17-21, 2025

Agenda item:			8.1

Source:	Feature lead (CMCC)

Title:	Running summary of spectrum sharing

Document for:	Information

Introduction

In RAN #109 meeting, the revised SID of study on 6G Radio was agreed [RP-252912], and the spectrum sharing related scopes were duplicated as follows:

- (7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

In this running summary, the following information is captured for 6G RRM study:

- The key RAN4 agreements and tdocs from last RAN4 meeting, including topic summary, WF.

- Spectrum sharing related agreements summary from RAN1/RAN2

The information of feature lead for spectrum sharing is attached below.

| Feature Lead | Affiliation | Email |
| --- | --- | --- |
| Jingjing CHEN | CMCC | chenjingjing@chinamobile.com |

Status update

Following table is an abbreviated overview of the issues discussed under spectrum sharing in RAN4.

| Issue |  | Issue Status |
| --- | --- | --- |
| General aspects | Including migration scheme, scenarios for MRSS between 6GR and NR, etc | Ongoing |
| Aspects to facilitate MRSS | System parameters, e.g. numerology, channel raster, sync raster, etc | Ongoing |
|  | RF requirements | Ongoing |
|  | RRM requirements | Ongoing |
|  | Interference handling | Ongoing |

RAN4 - 6G spectrum sharing agreements

RAN4 #116bis meeting (Oct-2025)

Key tdocs

| R4-2514515 | Feature lead summary for [116bis][108] 6G spectrum sharing |
| --- | --- |
| R4-2514646 | WF for [116bis][108] 6G spectrum sharing |

Key agreements

  - Sub-topic 1-0: Agenda handling

- Agreements

MRSS should be considered in the ultimate RAN4 decision, including RAN4’s input to interim milestone.

Regarding some of the system parameters (e.g. SCS, sync raster, channel raster etc.), separated agenda will be designated for the scenarios of  with (in spectrum sharing)  and without (in system parameter) MRSS considered.

            - If feasible, RAN4 strives to define unified system parameters for the scenarios with and without MRSS.

            - Revisit this agenda arrangement after Feb. meeting in 2026 for potentially merging the discussion under the single agenda.

Before RAN4 receives concrete design from RAN1/2, RAN4 discussion will be based on the agreed hypothesis.

  - Sub-topic 1-1: General aspects

      - Issue 1-1-1: Migration scheme

- Agreements

- RAN4 focus on MRSS between 6GR and NR until further RANP guidance

- RAN4 focus on inter-RAT mobility between 6GR and NR

      - Issue 1-1-2: Scenarios for MRSS between 6GR and NR

- Agreements

- For operating frequency range or band for MRSS between 6GR and NR

- Start RAN4 6G-5G MRSS study in FR1

- Whether to consider 6G-5G MRSS in FR2-1 is further discussed

  - Sub-topic 1-2: Aspects to facilitate MRSS

      - Issue 1-2-0: General consideration

- Agreement

- To facilitate MRSS between 6GR and NR, take following aspects as starting point for RAN4 study

- System parameters

- RF requirements

- RRM requirements

- Interference handling

- Taking following table as starting point to discuss which system parameters are impacted by MRSS.

| System parameters | Whether impacted by MRSS |
| --- | --- |
| Numerology | Maybe impacted |
| Channel raster | Maybe impacted |
| Sync raster | Maybe impacted |
| Waveform | FFS |
| Modulation | FFS |
| Channel bandwidth | FFS |

RAN1 - spectrum sharing related agreements summary

MRSS agreements

RAN1 #122bis (Oct, 2025)

No further agreements.

RAN1 #122 (Aug, 2025)

Agreement

- Identify the high-level aspects which impact on the NR-6GR MRSS support

  - Including the lessons learned from LTE-NR DSS

System parameter agreements, which may have impact on MRSS discussion

Overview of 6GR air interface

RAN1 #122bis (Oct, 2025)

Agreement

- RAN1 provides methodology and corresponding initial analysis of potentially achievable coverage to RAN#110 to determine the coverage target(s)

Agreement

- High-level aspects to consider for the 6GR sync signal structure include, but not limited to

  - Sync raster design

  - Spectrum allocation

  - smallest maximum supported RF and BB UE BW without spectrum aggregation

  - mobile broadband service requirements as high priority

  - Energy efficiency for both BS and UE

  - Detection/tracking performance, latency, and complexity

  - Including initial cell search

  - Coverage target

  - Common design for diverse device types

  - Consideration of the supported deployment

  - Consideration on whether the single sync signal structure is sufficient

  - Note: Aspects impacting on the periodicity is to be discussed under AI11.5

Agreement

- The aspects to consider for supporting NTN include, but not limited to

  - Initial access, including cell search and SSB periodicity

  - Coverage

  - Duplexing

  - Capacity

  - Signalling overhead

  - GNSS-less/resilient/based operation

  - Large/varying doppler and propagation delay

  - Beamforming / beam management / beam hopping

Note:

- High-level aspects to consider to enable lower CAPEX/OPEX with respect to current networks include, but not limited to

  - UE/NW implementation complexity

  - UE/NW energy efficiency

  - MRSS

  - Spectrum efficiency

Agreement

For communication, 6GR considers NR frame structure used as a starting point for the study item,

  - Resource defined by one subcarrier and one symbol is called as resource element (RE).

  - Resource block (RB) is defined where the number of consecutive subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12

  - Radio Frame length is 10ms

  - Each radio frame is split into 10 subframes, each with a duration of 1 ms

  - For given SCS and for given symbol, the symbol duration, normal CP length and boundary is same as NR design.

  - A slot is defined as supporting 14 consecutive symbols for normal CP case and all subcarrier spacings.

Waveform

RAN1 #122bis (Oct, 2025)

Agreement

Final LS R1-2508069 is endorsed.

Agreement

- For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference

  - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

    - A realistic PA model should be used

    - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account.

    - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged.

    - FFS: Other evaluation metrics

    - Note: Companies to report how to calculate the Tx power gain, modulation and coding

Agreement

- Study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

RAN1 #122 (Aug, 2025)

Agreement (first agreement for 6G!!)

CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink

- Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions

- Other OFDM based waveforms are not precluded.

Agreement

CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink

- Enhancements/modifications on CP-OFDM will be studied as potential additions

- DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink

Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

Note:

Proponents are encouraged to provide more detailed information on their proposals for the next meeting, e.g.:

Proponents to characterize the main motivation for modification/additional waveform proposals:

- Targeted link direction, i.e. DL, UL or both

- Targeted use case (e.g. NTN, specific frequency range, etc.), if any

- Potential motivations metrics used, and quantified gains for a proposal, e.g.

  - Coverage

  - Network energy efficiency

  - UE energy efficiency

  - Spectral efficiency

  - High speed tolerance

  - Scheduling flexibility

  - Integration with ISAC

Proponents provide information on the following aspects, if applicable

- MRSS compatibility

- Target channels/signals, e.g. all channels, PxSCH only, etc.

- MIMO (SU and MU-MIMO) compatibility

- Target modulations, and impact to other modulations, if applicable

- Multi-user multiplexing/scheduling flexibility

- Multiplexing/coexistence with baseline waveforms

- Impact on synchronization and initial access

- Expected specification impact

- Transmitter/receiver complexity and impact to power consumption.

Modulation

RAN1 #122bis (Oct, 2025)

Agreement

For 6GR constellation shaping evaluation for CP-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. The evaluation and comparison should consider at least the following:

- BLER performance under AWGN channel (at least for performance calibration)

  - 1st transmission (baseline) and with HARQ re-transmission

- BLER performance under fading channel with fixed MCS

  - 1st transmission (baseline) and with HARQ re-transmission

- Throughput performance with link adaptation (adaptive MCS and rank) under fading channel

  - Needs to provide assumptions on rate adaptation (e.g., target BLER for 1st transmission, maximum # of retransmissions)

- Transmitter and receiver complexity (e.g., shaping/deshaping, demapper), latency, parallelism implementation, and storage requirements,

- Other KPI not excluded, such as PAPR, EVM, MPR/A-MPR

- Expected spec impact

- FFS detailed assumption of constellation shaping and improved MCS table

- System level evaluation can be done after link level evaluation.

Agreement

For 6GR constellation shaping study, proponent is encouraged to provide details for the PS/GS schemes considered for evaluation and comparison, including at least the following

- Probabilistic shaping for CP-OFDM and DFT-s-OFDM

  - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide constellation (including normalization), coding rate and target probabilistic distribution for each SE

    - If multiple coding rate and target probabilistic distribution pairs are provided for each SE, how to switch between them

  - Relationship between shaping and FEC, coded bits to modulation symbol mapping, and other modules (such as scrambling, interleaving), in transmit and receive chains. How to handle HARQ retransmission

  - PS algorithm details (for example, source coding based, channel coding based, etc) and parameters (such as block length, rate loss)

- Geometric shaping for CP-OFDM and DFT-s-OFDM

  - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide target constellation shape (including normalization) (1D-NUC, 2D-NUC, QAM-CS, etc) for each SE

    - If multiple constellation shapes are provided for each SE, how to switch between them

  - GS mapping details, such as bit to constellation point mapping and shape

  - Relationship with other blocks (such as scrambling, interleaving). How to handle HARQ retransmission

Agreement

For link level simulation for modulation evaluation, companies are encouraged to evaluate with the following assumptions and should report the exact scheme evaluated.

- channel configuration, including Channel profiles,Tx/RX antenna settings

- For MIMO scenario: SU-MIMO or MU-MIMO, follow agenda item 11.2 for MIMO when available.

- Precoder assumption

  - Close loop MIMO (reciprocal beamforming (e.g., SVD, SLR/RZF, etc.), codebook based)

    - Realistic CSI/SRS/AP-SRS periodicity and delay, and SRS chanEst assumptions,

    - or genie beamforming

  - Open loop MIMO

- Receiver assumption (for MIMO): LMMSE (baseline) for UL, rML or LMMSE for DL

- LLR demapper: Max-log (baseline) or Log-MAP

- Channel estimation: Realistic (baseline) or ideal

- Other assumptions: Channel coding NR-LDPC (baseline), PxSCH bandwidth, SCS, FD interleaver used or not, 5GNR BICM interleaver usage

- Note: For MIMO, SIMO, MISO and SISO are included when possible

Agreement

For 6GR constellation shaping evaluation for DFT-s-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. In addition to what has been agreed for CP-OFDM in earlier agreement, the evaluation and comparison should further consider at least the following:

- PAPR/CM of the resulting waveform

- EVM, MPR/A-MPR

Agreement

For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model).

- FFS: How to involve RAN4 early

- FFS: Shaping of higher order modulation

- System level evaluation can be done after link level evaluation.

RAN1 #122 (Aug, 2025)

Agreement

- For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

Max Channel bandwidth

RAN1 #122bis (Oct, 2025)

Agreement

RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz

- Study whether and how to enable UE to support 400MHz bandwidth

RAN1 #122 (Aug, 2025)

Agreement

Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

- Opt1: 3MHz

- Opt2: 5MHz

- Opt3: 10MHz

- Opt4: 20MHz

- FFS: the UL bandwidth may be different to the DL bandwidth.

- FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

- FFS: whether RF and BB UE BW are same or different.

- Note: RAN4 involvement is necessary.

Min Channel bandwidth

RAN1 #122bis (Oct, 2025)

No further agreements.

RAN1 #122 (Aug, 2025)

Agreement

For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

- Note: RAN4 involvement is necessary.

Numerology

RAN1 #122bis (Oct, 2025)

Agreement

6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band.

- FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1.

- Note: ISAC is separately discussed in ISAC session.

RAN1 #122 (Aug, 2025)

Agreement

- 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH.

  - For sub 6GHz

    - The following subcarrier spacing is at least supported

      - 15kHz SCS for FDD, 30kHz SCS for TDD

    - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz

    - FFS: 7.5kHz SCS for sub1GHz (FDD)

    - Whether to discuss the FFS will be subject to RANP decision.

  - For around 7GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz

  - FFS: For around 15GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz, 120kHz

    - Whether to discuss it will be subject to RANP decision

  - For between 24.25GHz - 52.6GHz

    - Subcarrier spacing 120kHz is supported

  - FFS whether to allow using additional subcarrier spacing for SSB

- FFS subcarrier spacing for PRACH and up to initial access discussion.

Conclusion

Numerologies for sensing is up to sensing agenda discussion.

Spectrum utilization

RAN1 #122bis (Oct, 2025)

No Further agreements.

RAN1 #122 (Aug, 2025)

Agreement

Study and identify the lessons learned from NR spectrum utilization and aggregation framework.

- DC is subject to RANP decision in June 2026.

- Note: MRSS aspects are separate discussion.

Sync raster

RAN1 #122bis (Oct, 2025)

No further agreements.

RAN1 #122 (Aug, 2025)

Agreement

Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity.