3GPP TSG-RAN WG4 Meeting #117	R4-2522382

Dallas, USA, November 17-21, 2025

Title: 	WF on n104 ACLR and MPR evaluation

Agenda Item:	10.2.2

Source: 	Huawei, HiSilicon

Document for:	Approval

# <Topic 1: Scope>

<Way forward>

- RAN4 introduces a band specific ACLR of 26 dB to n104 with a dedicated MPR for following cases.

-		PC3 for n104 single carrier

-		PC3 for CA_n104C, where

  - Only contiguous RBs allocations are considered

  - For non-contiguous RBs allocations, the existing MPR applies

-		1Tx and 2Tx are considered

## <Topic 2: Evaluation conditions>

<Way forward>: Following conditions will be used for MPR evaluation for n104 and for CA_n104C contiguous RB allocations with new ACLR of 26 dB

-	PA calibration point: PC3 -30dBc ACLR at MPR1 with the DFT-s-OFDM QPSK 20MHz Lcrb=100(RBstart=0) waveform

-	RF impairments: -28dBc I/Q, CIM3 : -60dBc, CIM5 : -70dBc

-	Definition of inner/outer/edge RB allocations is the same as is today

-	If required, modification of edge RB definition for n104 is not out of scope

## <Topic 3: How to introduce the ACLR>

<Way forward >

- A new n104 specific ACLR of 26 dB for PC3is introduced for single CC and intra-band contiguous CA without NS signaling.

## <Topic 4: MSD>

<Way forward >

- MSD value for CA_n78-n104 with respect to cross band isolation is the same as is today