3GPP TSG-RAN WG4 Meeting # 117	R4-2522148

# Dallas, US,  November 17 - 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 3146 | rev | 1 | Current version: | 18.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-1 for eRedCap channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 5.3I, there is a sentence on the RB number limitation for eRedCap UE with reduced BB BW capability: “When UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18 the requirements in this specification apply with maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH” The transmission bandwidth in above text can be interpreted as contiguous allocation and therefore, the statement is conflicting with RAN1 specification TS 38.213 where 25 RB for PDSCH allocation can be either contiguous or non-contiguous for eRedCap UE with reduced BB BW capability. The text in TS 38.213 is quoted below: 17.1A Second procedures for RedCap UE In this clause, the term 'UE' refers to a RedCap UE that indicates supportOfERedCap. A UE that has not indicated eRedCapNotReducedBB-BW does not expect to transmit a PUSCH over a bandwidth that is larger than 25 PRBs for 15 kHz SCS, or larger than 12 PRBs for 30 kHz SCS, per hop in a slot. A UE that has not indicated eRedCapNotReducedBB-BW does not expect to process a PDSCH reception that is scheduled by a DCI format with CRC scrambled by a C-RNTI, CS-RNTI, MCS-C-RNTI, G-RNTI for multicast, or G-CS-RNTI, or is associated with a SPS PDSCH configuration activated by a DCI format with CRC scrambled by CS-RNTI or G-CS-RNTI, over a number of PRBs that is larger than 25 PRBs for 15 kHz SCS, or larger than 12 PRBs for 30 kHz SCS, in a slot. […] The text above in TS 38.213 does not limit contiguous or non-contiguous allocation for PDSCH for eRedCap with reduced BB BW capability. Therefore, TS 38.101-1 should align the RB # limitation description for eRedCap on PDSCH with clause 17.1A in TS 38.213. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the words “transmission bandwidth” in clause 5.3I for eRedCap UE and add TS 38.213 as reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RB limitation description for PDSCH for eRedCap in TS 38.101-1 is not aligned with clause 17.1A in TS 38.213. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3I |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/ change start/

## 5.3I	Channel bandwidth for (e)RedCap

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 with maximum channel bandwidth of 20MHz. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths up to 20 MHz. When UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18 the requirements in this specification apply with maximum 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH as described in clause 17.1A of TS 38.213 [8]. The channel bandwidths are specified for both the TX and RX paths.

3MHz channel bandwidth is not applicable for (e)RedCap UE in the current release.

/ change end/