3GPP TSG RAN WG4 Meeting #117	R4-2522457

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6097 | rev | 3 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on RRM core requirements for support of AI/ML for NR Air Interface for beam management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-11-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19, the AI/ML for NR Air Interface feature was introduced. For the beam management use case, relevant RRM requirements need to be defined in RAN4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce correasponding requirements for Active TCI state switching delay for the AI/ML based BM scenario - Introduce correasponding requirements for L1-RSRP measurements for Prediction of beam management Reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS38.133 does not include the AI/ML for NR Air interface for BM feature in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10, 9.5X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2522359 |  |  |  |  |  |  |  |  |

### <Start of Change 1>

8.10.2X	Known conditions for TCI state with beam prediction

The TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource in resourcesForChannelMeasurement to the completion of active TCI state switch, where the target TCI state is either

-	RS resource in resourcesForChannelMeasurement  or QCLed to RS resource in resourcesForChannelMeasurement or QCLed to RS resource in resourcesForChannelMeasurement not configured for prediction

or

-	one of RS resource in resourcesForChannelPrediction-r19 or QCLed to one of RS resource in resourcesForChannelPrediction-r19 but not in the RS resource in resourcesForChannelMeasurement and UE supports [FG 59-1]

-	TCI state switch command is received within 1280 ms upon the last transmission of the RS resource in resourcesForChannelMeasurement.

-	The UE has sent at least 1 L1-RSRP report for the predicted RS quantities for the target TCI state before the TCI state switch command

-	The TCI state remains detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	SNR of the TCI state ≥ -3 dB

Otherwise, the TCI state is unknown.

### <End of Change 1>

### <Start of Change 2>

8.10.4	DCI based TCI state switch delay

If the target TCI state is known, when a UE is configured with the higher layer parameter tci-PresentInDCI which is set as 'enabled' for the CORESET scheduling PDSCH at slot n, UE shall be able to receive PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+timeDurationForQCL, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], the value of timeDurationForQCL is defined in TS 38.331 [2].

The known condition for TCI state defined in clause 8.10.2 or clause 8.10.2X is applied.

### <End of Change 2>

### <Start of Change 3>

9.5X	L1-RSRP measurements Reporting for Beam Prediction

9.5X.1	Introduction

For UE supporting UE-sided RS- prediction for reportQuantity-r19 and nroftimeinstance-r19 is not configured, when configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP for determine the reported P-CRI. For UE supporting UE-sided RS prediction with p-cri-RSRP-r19 for reportQuantity-r19 and nroftimeinstance-r19 is not configured, when configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP for determine the reported P-CRI, and P-L1-RSRP.

For UE supporting UE-sided RS- prediction for reportQuantity-r19 with 'p-cri-r19' or 'p-ssb-index-r19' and nroftimeinstance-r19 is configured, when configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP for determine the reported P-CRI, P-SSBRI. For UE supporting UE-sided RS prediction for reportQuantity-r19 with 'p-cri-RSRP-r19' or 'p-ssb-Index-RSRP-r19' and nroftimeinstance-r19 is configured, when configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP for determine the reported P-CRI, and P-L1-RSRP.

The measurements shall be performed for a serving cell, including PCell, on the resources configured for L1-RSRP measurements within the active BWP.

The UE shall be able to measure all CSI-RS resources and/or SSB resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet within the CSI-ResourceConfig settings configured for L1-RSRP for the active BWP, provided that the number of resources does not exceed the UE capability indicated by beamManagementSSB-CSI-RS.

The UE shall report the measurement quantity (reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19') and send periodic, semi-persistent or aperiodic reports, according to the reportConfigType according to the CSI reporting configuration(s) (CSI-ReportConfig) for the active BWP.

9.5X.2	Requirements applicability

The requirements in clause 9.5X apply, provided:

-	The CSI-RS or SSB or CSI-RS and SSB resources configured for L1-RSRP measurements are measurable.

An SSB resource configured for L1-RSRP shall be considered measurable when for each relevant SSB the following conditions are met:

-	L1-RSRP related side conditions given in clauses 10.x for FR2 for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.4.1 for a corresponding band.

A CSI-RS resource configured for L1-RSRP shall be considered measurable when for each relevant CSI-RS the following conditions are met:

-	L1-RSRP related side conditions given in clauses 10.x for FR2, respectively, for a corresponding band,

-	CSI-RS_RP and CSI-RS Ês/Iot according to Annex [B.2x] for a corresponding band.

A CSI-RS and SSB resource configured for L1-RSRP shall be considered measurable when the measurable resource conditions are met for both CSI-RS resource and SSB resource.

Requirements are defined for periodic, semi-persistent and aperiodic resources, with the exception for UE sided RS prediction when nroftimeinstance-r19 is configured, it is not applicable for aperiodic CSI-RS resources.

For UE sided RS prediction when nroftimeinstance-r19 is not configured, there is no requirement if UE is configured with reportQuantity-r19 set to 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19'.

For UE sided RS prediction when nroftimeinstance-r19 is configured, there is no requirement if higher layer parameter timeRestrictionForChannelMeasurement is not configured.

9.5X.3	Measurement Reporting Requirements

For a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-ssb-index-r19', the UE shall send P-CRI, P-SSBRI reports only for report configurations configured for the active BWP.

For a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', the UE shall send P-CRI, P-SSBRI, and P-L1-RSRP reports only for report configurations configured for the active BWP. The UE shall report the P-L1-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1dB step size according to clause 10.x for FR2 if nroftimeinstance-r19 is not configured or nroftimeinstance-r19 is configured with the value set to 1, and if nrofreportedpredictedrs-r19 is configured to be one. If nroftimeinstance-r19 is not configured or nroftimeinstance-r19 is configured with the value set to 1, and if nrofreportedpredictedrs-r19 is configured to be larger than 1, the UE shall use differential L1-RSRP based reporting as defined in clause 10.x for FR2. The largest predicted value of P-L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size and the differential P-L1-RSRP value is computed with 2 dB step size with a reference to the largest P-L1-RSRP value which is part of the same P-L1-RSRP reporting instance. The mapping between the reported P-L1-RSRP value and the predicted L1-RSRP quantity is described in 10.x.

9.5X.3.1	Periodic Reporting

Reported P-CRI, P-SSBRI, and/or P-L1-RSRP contained in periodic P-CRI, P-SSBRI, and/or P-L1-RSRP reports shall meet the requirements in clauses 10.x for P-CRI, P-SSBRI, and in clause 10.x for P-L1-RSRP, respectively.

The UE shall only send periodic P-CRI, P-SSBRI, and/or P-L1-RSRP reports for an active BWP.

The UE shall transmit the periodic P-CRI, P-SSBRI, and/or P-L1-RSRP reporting on PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

9.5X.3.2	Semi-Persistent Reporting

Reported P-CRI, P-SSBRI, and/or P-L1-RSRP contained in a Semi-Persistent P-CRI, P-SSBRI, and/or P-L1-RSRP report shall meet the requirements in clauses 10.1.20 for FR2. This requirement applies for semi-persistent P-CRI, P-SSBRI, and/or P-L1-RSRP P reports send on PUSCH or PUCCH.

The UE shall only send semi-persistent P-CRI, P-SSBRI, and/or P-L1-RSRP reports on PUSCH, if a DCI request has been received.

The UE shall only send semi-persistent P-CRI, P-SSBRI, and/or P-L1-RSRP reports on PUCCH, if an activation command [7] has been received.

The UE shall transmit the semi-persistent P-CRI, P-SSBRI, and/or P-L1-RSRP reporting on PUSCH or PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

9.5X.3.3	Aperiodic Reporting

Reported P-CRI, and/or P-L1-RSRP contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent reports shall meet the requirements in clauses 10.1.20 for P-CRI, and in clause 10.x for P-L1-RSRP, respectively.

The UE shall only send aperiodic P-CRI, and/or P-L1-RSRP reports, if a DCI trigger has been received.

After the UE receives CSI request in DCI, the UE shall transmit the aperiodic P-CRI, and/or P-L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

9.5X.4	L1-RSRP measurement and prediction requirements

9.5X.4.1	SSB based RS prediction reporting

The UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting P-CRI, P-SSBRI, and/or P-L1-RSRP based on the  L1-RSRP measurements over the prediction period of TL1-RSRP_Prediction_Period_SSB.

The value of TL1-RSRP_Prediction_Period_SSB is defined in Table 9.5X.4.1-2 for FR2, where

-	For UE sided RS prediction when nroftimeinstance-r19 is not configured, M= 1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise; And for UE sided RS prediction when nroftimeinstance-r19 is configured, M =1 if higher layer parameter timeRestrictionForChannelMeasurement is configured

-	N= 8

-	KBM=1 for UE sided RS prediction when nroftimeinstance-r19 is not configured. And for UE sided RS prediction when nroftimeinstance-r19 is configured, KBM is indicated by UE capability on number of latest consecutive transmission occasion.

For FR2,

-	P=11-TSSBTSMTCperiod, when SSB is not overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor, when SSB is not overlapped with measurement gap and SSB is fully overlapped with SMTC period (TSSB = TSMTCperiod).

-	P=11-TSSBMGRP-TSSBTSMTCperiod, when SSB is partially overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P is 11-TSSBMGRP* Psharing factor, when SSB is partially overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TSSB = 0.5*TSMTCperiod

-	P= 11-TSSBmin⁡(TSMTCperiod,MGRP), when SSB is partially overlapped with measurement gap (TSSB <MGRP) and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is  11-TSSBMRGP* Psharing factor, when SSB is partially overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	P is  11-TSSBMGRP* Psharing factor, when SSB is partially overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)Psharing factor = 1

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

Where:

TSSB = ssb-periodicityServingCell

TSMTCperiod = the configured SMTC period

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer prediction period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

Table 9.5X.4.1-2: Prediction period TL1-RSRP_Prediction_Period_SSB for FR2

| Configuration | TL1-RSRP_Prediction_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(KBM*M*P*N)*TSSB+Tinference) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*KBM*M*P*N)*max(TDRX,TSSB) +Tinference) |
| DRX cycle > 320ms | ceil(1.5*KBM*M*P*N)*TDRX+Tinference |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tinference is the time required for model inference, which is defined by d' as specified according to clause 5.4 in TS 38.214 [26]. |  |

9.5X.4.2	CSI-RS based RS prediction reporting

The UE shall be capable of performing L1-RSRP measurements based on the configured CSI-RS resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting P-CRI, and/or P-L1-RSRP based on the L1-RSRP measurements measured over the prediction period of TL1-RSRP_Prediction_Period_CSI-RS.

The value of TL1-RSRP_Prediction_Period_CSI-RS is defined in in Table 9.5X.4.2-2 for FR2, where

-	For periodic and semi-persistent CSI-RS resources, for UE sided RS prediction when nroftimeinstance-r19 is not configured, M= 1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise. And for UE sided RS prediction when nroftimeinstance-r19 is configured, M= 1 if higher layer parameter timeRestrictionForChannelMeasurement is configured.

-	For aperiodic CSI-RS resources M= 1

-	For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured for all resources in the resource set.

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured for with QCL-TypeD all resources in the resource set.

-	For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided with QCL-TypeD for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided qcl-info is configured for all resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.19.2 and 10.1.20.2 if number of resources in the resource set is smaller than maxNumberRxBeam. The requirements apply provided qcl-info is configured with QCL-TypeD for all resources in the resource set.

-	KBM=1 for UE sided RS prediction when nroftimeinstance-r19 is not configured. And for UE sided RS prediction when nroftimeinstance-r19 is configured, KBM is indicated by UE capability on number of latest consecutive transmission occasion.

For FR2,

-	P=1, when CSI-RS is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSMGRP, when CSI-RS is partially overlapped with measurement gap and CSI-RS is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=11-TCSI-RSTSMTCperiod, when CSI-RS is not overlapped with measurement gap and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P=Psharing factor, when CSI-RS is not overlapped with measurement gap and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=1, when aperiodic CSI-RS resource is not overlapped with measurement gap.

-	P= 11-TCSI-RSMGRP-TCSI-RSTSMTCperiod, when CSI-RS is partially overlapped with measurement gap and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5*TSMTCperiod

-	P=31-TCSI-RSMGRP, when CSI-RS is partially overlapped with measurement gap and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TCSI-RS = 0.5*TSMTCperiod

-	P= 11-TCSI-RSmin⁡(TSMTCperiod,MGRP), when CSI-RS is partially overlapped with measurement gap (TCSI-RS < MGRP) and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P=31-TCSI-RSMGRP, when CSI-RS is partially overlapped with measurement gap and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor = 1, if the CSI-RS configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured

-	Psharing factor = 3, otherwise.

Where:

TSMTCperiod = the configured SMTC period.

TCSI-RS = the periodicity of CSI-RS configured for L1-RSRP measurement

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Note: The overlap between CSI-RS for L1-RSRP measurement and SMTC means that CSI-RS for L1-RSRP measurement is within the SMTC window duration.

Longer prediction period would be expected if the combination of CSI-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

Table 9.5X.4.2-2: Prediction period TL1-RSRP_Prediction_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Prediction_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(KBM*M*P*N)*TCSI-RS+Tinference) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*KBM*M*P*N)*max(TDRX,TCSI-RS) +Tinference) |
| DRX cycle > 320ms | ceil(KBM*M*P*N)*TDRX+Tinference |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tinference is the time required for model inference, which is defined by d' as specified according to clause 5.4 in TS 38.214 [26]. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

### <End of Change 3>