3GPP TSG-RAN WG4 Meeting # 117				       			       			                  R4-2522644

Dallas, USA, Nov. 17-21, 2025

Agenda item:			6.15.1

Source:	Moderator (Huawei)

Title:	WF on Netw_Energy_NR_enh_RRM_Part2

Document for:	Information

Topic #1: Adaptation of common signal/channel transmission

Issue 1-1: Whether to verify DCI processing time in L1 based SSB adaptation test

Agreement:

  - DCI processing time is to be verified in L1 based SSB adaptation test

Issue 1-2: How many SSB index in L1 based SSB adaptation test

Agreement:

  - Only SSB#0 is configured and observed in L1 based SSB adaptation test

Issue 1-3: Details in test procedure for L1 based SSB adaptation test

Agreement:

  - For TC of L1-RSRP/SINR measurement of SSB adaptation, the details in test procedure are proposed as follows

    - Treport=20ms (Periodic L1-RSRP reporting) for FR1

    - timeRestrictionForChannelMeasurements is configured

    - DCI 2-9 indicates SSB adaptation:

L1-RSRP TC: SSB periodicity from 20ms to 80ms, and from 80ms to 40ms

L1-SINR TC: SSB periodicity from 80ms to 20ms

    - Condition of serving Cell is changed at the same time as DCI 2-9 indication

    - UE shall report L1-RSRP result with new value based on indicated SSB from the first SSB after DCI decoding plus OD-SSB post processing time

The below figure (from R4-2520928) which illustrates one-way SSB adaption (from 80ms to 20ms) is for information

Figure cited from R4-2520928

Topic #2: OD-SIB1

Issue 2-1: Ending point for cell reselection TC with longer TSI-NR for OD-SIB1Agreement:

Agreement:

  - The ending point of the TC is defined as legacy (i.e., starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2)

Issue 2-3: Test requirement

Agreement:

  - The cell reselection delay shall additionally consider the time for requesting and acquiring OD-SIB1 (i.e., Tevaluate, NR_Intra + TSI-NR + TOD-SIB1), where

    - The additional cell reselection delay due to OD-SIB1 is named TOD-SIB1. TOD-SIB1 comprises below three components:

      - UE transmits UL-WUS transmission

      - UE receives and decodes RAR

      - UE receives OD-SIB1 within OD-SIB1 window

    - TOD-SIB1 set to [1] s.

Issue 2-4: One direction or two directions for cell reselection TC with longer TSI-NR for OD-SIB1

Agreement:

- Only verify the case that UE reselect from Cell 1 to Cell2 (NES Cell), and no need to verify the case the UE reselect back from Cell2 (NES Cell) to Cell 1 as designed in legacy Cell reselection TC.

Issue 2-5: Verify known target cell case for cell reselection TC

Agreement:

Specify the new test case for known target cell only.