3GPP TSG-WG4 Meeting #117 	R4-2522670

# Dallas, USA, November 17th - November 21st, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR Applicability Rule for 6Rx 38.101-4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Perf |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Inclusion of Applicability Rule |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inclusion of Applicability Rule |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicability Rule not included |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

The number of RX antenna ports for different RF operating bands is up to UE declaration.

The UE shall support 2, 4, 6 or 8 RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in Clause 7.2 of TS 38.101-1 [6]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 5.1.1.2-1.

Table 5.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list |  |
| --- | --- | --- | --- |
| UE supports only 2RX | PDSCH | All tests in Clause 5.2.2 All tests in Clause 5.6.2 |  |
|  | PDCCH | All tests in Clause 5.3.2 |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 4RX or both 2RX and 4RX | PDSCH | All tests in Clause 5.2.3 (Note 2) All tests in Clause 5.6.3 |  |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2) |  |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 (Note 1) |  |
| UE supports 8Rx, 4Rx and 2Rx | PDSCH | All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4, Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6, Test 2-1 in Table 5.2.3.1.16 and Test 2-1 in Table 5.2.3.2.17. |
| or | PDCCH | All tests in Clause 5.3.3 (Note 2,3) |  |
| UE supports both 8Rx and 4Rx or UE supports 8Rx, 6Rx, 4Rx and 2Rx or UE supports 8Rx, 6Rx, and 4Rx | PBCH | All tests in Clause 5.4.3 (Note 1) |  |
| UE supports both 8Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4 |
| or | PDCCH | All tests in Clause 5.3.2 (Note 2,4) |  |
| UE supports 8Rx, 6Rx and 2Rx | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 8Rx or UE supports both 8Rx and 6Rx | PDSCH | All tests in Clause 5.2.4 (Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| UE supports 6Rx, 4Rx and 2Rx | PDSCH | All tests in Clause 5.2.3 (Note 2, 6) All tests in Clause 5.2.X (Note 2) | If UE passes tests in Clause 5.2.X, UE can skip Test 4-1 in Clause 5.2.3 Table 5.2.3.2.1-6 |
| or | PDCCH | All tests in Clause 5.3.3 (Note 2,6 |  |
| UE supports both 6Rx and 4Rx | PBCH | All tests in Clause 5.4.3 (Note 1) |  |
| UE supports both 6Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 (Note 2, 7) All tests in Clause 5.2.X (Note 2) All tests in Clause 5.6.2 |  |
|  | PDCCH | All tests in Clause 5.3.2 (Note 2,7) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 6Rx | PDSCH | All tests in Clause 5.2.X (Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. Note 5: Tests defined for 8Rx capable UEs according to whether the UE is a ‘simplified 8Rx receiver’ or ‘baseline 8Rx receiver’ are specified in Table 5.1.1.3-1. Note 6: 6Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 6 Rx with data source from system simulator, and the other 2 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 7: 6Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 6 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. |  |  |  |

<End of change 1>