3GPP TSG Meeting #117	R4-2522690

Dallas, USA, 17th Nov – 21st Nov

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Minimum TDDPMI requirements with Single PMI with 64TX TypeI-SinglePanel codebook |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TDD TypeI SingelPanel codebook PMI requirements with 64 ports, 4Rx are introduced in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test settings and PMI requirements are introduced in clause 6.3.3.2.Y1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No 64 4Rx TDD PMI requirements shall be present in spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.3.3.2.Y1 shall be added to 38.101-4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

##### 6.3.3.2.Y1	Single PMI with 64TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.1.Y1-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.Y1-2.

Table 6.3.3.1.Y1-1: Test parameters (dual-layer)

| Parameter |  | Unit |  | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz |  | 30 |  |  |
| Subcarrier spacing |  | kHz |  | 40 |  |  |
| Duplex Mode |  |  |  | TDD |  |  |
| TDD DL-UL configurations |  |  |  | FR1.30-1 as specified in Annex A |  |  |
| Propagation channel |  |  |  | TDLA30-5 |  |  |
| Antenna configuration |  |  |  | XP Medium 64 x 4 (N1,N2) = (8,4) |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |
|  | Density (ρ) |  |  | 1 |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(5) |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(7) |  |  |
|  | CSI-RS periodicity and offset | slot |  | 10/6 |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  |  | Resource #1 |  | Resource #2 |
|  | CSI-RS resource Type |  |  | Aperiodic |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  |  | 32 |  | 32 |
|  | CDM Type |  |  | CDM4 (FD2, TD2) |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  |  | 1 |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  |  | Row 17, (2, 4, 6, 8) |  | Row 17, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | Row 17, (2, 9) |  | Row 17, (5, 12) |
|  | CSI-RS interval and offset | slot |  | Not configured |  | Not configured |
|  | aperiodicTriggeringOffset |  |  | 0 |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic |  |  |
|  | CSI-IM RE pattern |  |  | Pattern 0 |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (5,7) |  |  |
|  | CSI-IM timeConfig interval and offset | slot |  | Not configured |  |  |
| ReportConfigType |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |
| Sub-band Size |  | RB |  | 8 |  |  |
| csi-ReportingBand |  |  |  | 11111111111111 |  |  |
| CSI-Report interval and offset |  | slot |  | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  | 12 |  |  |
| CSI request |  |  |  | 1 in slots i, where mod(i, 10) = 6, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration | Codebook Type |  |  | typeI-SinglePanel-r19 |  |  |
|  | Codebook Mode |  |  | modeA |  |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (8,4) |  |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,4) |  |  |
|  | (valueOfX1-typeI-CBSR-r19, valueOfX2-typeI-CBSR-r19) |  |  | Not configured |  |  |
|  | typeI-CBSR-r19 |  |  | Not configured |  |  |
|  | (valueOfX1-typeI-powerScaling-r19, valueOfX2-typeI-powerScaling-r19) |  |  | Not configured |  |  |
|  | typeI-softScalingRanks1-2-r19 |  |  | Not configured |  |  |
|  | RI Restriction |  |  | 00000010 |  |  |
|  | portMappingMethod |  |  | method1 |  |  |
| Physical channel for CSI report |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  | ms |  | 9 |  |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |  |
| Measurement channel |  |  |  | R.PDSCH 2-8.7 TDD |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-12), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |  |  |

Table 6.3.3.1.Y1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | TBD |

==============End of change==============