3GPP TSG-RAN4 Meeting # 117	R4-2522777

# Dallas, USA, Nov.17 – 21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on test case of Event-2, FR2, Inter-cell SSB based, no window time based, mode-A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the work split in WF R4-2514803, the test case of TC1 is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the test case for TC1 “Event-2, FR2, Inter-cell SSB based, no window time based” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases of Rel-19 MIMO are incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.6.X.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

A.7.6.X.1	Event triggered reporting for UE initiated beam management for UE configured with Inter-cell SSB based L1-RSRP measurement on FR2 when DRX is not used

A.7.6.X.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event based on L1-RSRP measurement. This test will partly verify the event triggered reporting for UE initiated beam management in clause 9.13.3.4 configured with event2-r19, modeA-r19 and without eventCountWindow-r19 configuration.

The AoA setup of FR2 cell for this test is Setup 3 as defined in clause A.3.15.

Table A.7.6.X.1.1-1: Applicable NR configurations for inter-cell SSB based Event triggered reporting for UE initiated beam management in FR2

| Config | Description |
| --- | --- |
| 1 | NR PCell 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR PCell 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | NR PCell 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 4 | NR PCell 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR SCell 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 5 | NR PCell 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR SCell 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 6 | NR PCell 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR SCell 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

A.7.6.X.1.2	Test parameters

There are two cells in the test, Cell 1 is the serving cell in CA, including a FR1 PCC and FR2 SCC. Cell 2 is a FR2 cell with different PCI from Cell 1. The test parameters for Cell 1 are given in table A.7.6.X.1.2-1. The test parameters for FR2 Cell (Cell 2) are given in table A.7.6.X.1.2-2 and table A.7.6.X.1.2-3.

SSB#0 is transmitted on Cell 1. SSB#1 and SSB#2 are transmitted on FR2 Cell 2. In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on SSB#1 and SSB#2, where SSB#1 is for current beam measurement and SSB#2 is for new beam measurement. The test consists of two successive time periods, with time duration of T1 and T2 respectively. At the beginning of T2, SSB#2 starts transmission and the UE is configured for L1-RSRP measurement on SSB#2 and the RSRP configuration of SSB#2 is better than SSB#1 to trigger the event-2 reporting. The test has higher layer parameter timeRestrictionForChannelMeasurements configured in CSI-ReportConfig and additionalPCIList configured in CSI-SSB-ResourceSet.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD on Cell 1 in FR1 and perform L1-RSRP measurements on the SSB#0in FR2.

Table A.7.6.X.1.2-1: Cell specific test parameters for FR1 PCell

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~6 |  | freq1 |
| Duplex mode | 1,4 |  | FDD |
|  | 2,5 |  | TDD |
|  | 3,6 |  | TDD |
| TDD Configuration | 1,4 |  | N/A |
|  | 2,5 |  | TDDConf.1.1 |
|  | 3,6 |  | TDDConf.2.1 |
| BWchannel | 1,4 | MHz | 10: NPRB,c = 52 |
|  | 2,5 |  | 10: NPRB,c = 52 |
|  | 3,6 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1,4 |  | SR.1.1 FDD |
|  | 2,5 |  | SR.1.1 TDD |
|  | 3,6 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1,4 |  | CR.1.1 FDD |
|  | 2,5 |  | CR.1.1 TDD |
|  | 3,6 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1,4 |  | CCR.1.1 FDD |
|  | 2,5 |  | CCR.1.1 TDD |
|  | 3,6 |  | CCR.2.1 TDD |
| SSB configuration | 1,4 |  | SSB.3 FR1 |
|  | 2,5 |  | SSB.3 FR1 |
|  | 3,6 |  | SSB.4 FR1 |
| OCNG Patterns | 1~6 |  | OP.1 |
| Initial BWP Configuration | 1~6 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~6 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~6 |  | SMTC.1 |
| CSI-RS for tracking | 1,4 |  | TRS.1.1 FDD |
|  | 2,5 |  | TRS.1.1 TDD |
|  | 3,6 |  | TRS.1.2 TDD |
| DRX configuration | 1~6 |  | Off |
| SSB index assigned as RLM RS | 1~6 |  | 0,1 |
| SSB Index assigned as BFD RS (q0) | 1~6 |  | 0 |
| SSB Index assigned as CBD RS (q1) | 1~6 |  | 1 |
| EPRE ratio of PSS to SSS | 1~6 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note2 | 1~6 | dBm/15 kHz | -104 |
|  | 1~6 | dB | 17 |
|  | 1~6 | dB | 17 |
| SS-RSRP Note3 | 1~6 | dBm/15 kHz | -87 |
| IoNote3 | 1,2,4,5 | dBm/ 9.36 MHz | -58.96 |
|  | 3,6 | dBm/ 38.16 MHz | -52.86 |
| Propagation condition | 1~6 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselvess. |  |  |  |

Table A.7.6.X.6.2-2: Cell specific test parameters for FR2 SCell

| Parameter |  | Config | Unit | Value |
| --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1~6 |  | Freq2 |
| Duplex mode |  | 1~6 |  | TDD |
| TDD Configuration |  | 1~2 |  | TDDConf.3.1 |
| BWchannel |  | 1~6 | MHz | 100: NPRB,c = 66 |
| Data PRBs allocated |  | 1~6 |  | 66 |
| PDSCH Reference measurement channel |  | 1,2,3 |  | SR.3.2 TDD |
|  |  | 4,5,6 |  | SR.3.3 TDD |
| RMSI CORESET Reference Channel |  | 1,2,3 |  | CR.3.1 TDD |
|  |  | 4,5,6 |  | CR.3.2 TDD |
| Dedicated CORESET Reference Channel |  | 1,2,3 |  | CCR.3.1 TDD |
|  |  | 4,5,6 |  | CCR.3.7 TDD |
| SSB configuration |  | 1,2,3 |  | SSB.1 FR2 |
|  |  | 4,5,6 |  | SSB.2 FR2 |
| OCNG Patterns |  | 1~6 |  | OP.1 |
| Initial BWP Configuration |  | 1~6 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~6 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration |  | 1~6 |  | SMTC.1 |
| TRS Configuration |  | 1~6 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration |  | 1~6 |  | TCI.State.2 |
| DRX configuration |  | 1~6 |  | Off |
| reportConfigType |  | 1~6 |  | periodic |
| reportQuantity |  | 1~6 |  | ssb-Index-RSRP |
| Number of reported RS |  | 1~6 |  | 2 |
| SSB index associated to a PCI different from serving cell |  | 1~6 |  | 0,1 |
| SSB index associated to indicated TCI |  | 1~6 |  | 0 |
| SSB index associated to newBeamResourceSet |  | 1~6 |  | 1 |
| eventTypeUE-IBR-r19 |  | 1~6 |  | event2-r19 |
| eventThreshold-r19 |  | 1~6 |  | 4 |
| reportTransmissionMode-r19 |  | 1~6 |  | modeA-r19 |
| currentBeamReport-r19 |  | 1~6 |  | disabled |
| PUCCH-Resource-r19 | periodicityAndOffset | 1~6 |  | sl5, 4 |
|  | pucch-Cell-r19 | 1~6 |  | Cell 2 |
| Timing offset between two SSBs associated to different PCI |  | 1~6 |  | <CP |
| T1 |  | 1~6 | s | 5 |
| T2 |  | 1~6 | s | 2 |
| EPRE ratio of PSS to SSS |  | 1~6 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition |  | 1~6 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table A.7.6.X.1.2-2: SSB specific test parameters for FR2 SCell

| Parameter | Config | Unit | SSB#1 |  | SSB#2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration | 1~6 |  | Setup 3 according to A.3.15.3 |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 4 | 1~6 |  | Rough |  | Rough |  |
| Note2 | 1~6 | dBm/15 kHz | -105 |  | -105 |  |
| Note2 | 1~3 | dBm/SSB SCS | -96 |  | -96 |  |
|  | 4~6 |  | -93 |  | -93 |  |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| SSB_RP Note3 | 1 | dBm/SSB SCS | -96 | -96 | -Infinity | -87 |
|  | 2 |  | -93 | -93 | -Infinity | -84 |
| Io Note3 | 1 | dBm/95.04 MHz | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 2 |  | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

A.7.6.X.1.3	Test Requirements

The UE shall send a first PUCCH message using the new UCI type, no later than 160 ms plus 5 slots from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

==============End of change==============