3GPP TSG-RAN WG4 Meeting #117														R4-2522976

Dallas, US, 17th to 21st November 2025

Agenda item:			7.2

Source:	Moderator (Ericsson)

Title:	Ad-hoc summary for [117][317] NR_BS_RF_Ph2

Document for:	Information

# Introduction

This summary collects the different proposals and main observations related to the AI 7.2 NR BS RF phase 2.

The WI have been divided into following topics:

-	General aspects and workplan

-	Enhancement of co-location requirements

-	SBFD BS to SBFD BS adjacent channel coexistence

The goal in the first meeting is to establish workplan, identify the common views and early agreements, refine the scope of the WI and list the different areas for further investigations, setting priorities when needed.

Note that in the following, proposals are not necessarily exclusive, they could be complementary. Some major observations have been captured to help the proposal’s understanding.

# Topic #1: General aspects and work plan

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2521048 | Ericsson | Proposal 1: Include updated work-plan in way-forward from this meeting. |

## Open issues summary

### Sub-topic 1-1

Sub-topic 1-1 is about the WI workplan for WI.

Issue 1-1: Workplan

- Proposals

  - Proposal 1:  Include updated work-plan in way-forward from this meeting.

- Recommended WF

  - Attach updated workplan in way-forward

Agreement: The way-forward is to attach an updated version of workplan in way-forward from this meeting.

# Topic #2: Enhancement of co-location requirements

This topic handles WID objective related to enhancement of co-location requirements.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520256 | CATT | Proposal 1: RAN4 to agree a single unified approach to define enhanced OTA co-location requirements to avoid mixed definitions and reduce test costs. Proposal 2: Enhanced OTA co-location core requirements for upper FR1 frequencies should be manufacturer-declared once specified. Proposal 3: For upper frequencies of FR1, Option 2 may offer better performance in terms of accuracy, reliability, cost, test efficiency and high frequencies. |
| R4-2521583 | Nokia | Observation 1: Using existing OTA metrics to define the requirements is challenging due to measuring low power levels. Observation 2: There might be a need to have different measurement method for different co-location requirement. Proposal 1: Consider to keep the 10cm separation and side-by-side deployment to guarantee co-location requirements are met in the field. |
| R4-2521697 | Ericsson | Observation 1: Compared with vertical separation, horizontal separation is seen as the worst-case co-location scenario. Observation 2: The evaluation of inter-band and intra-band coupling loss in Rel-20 should be focused on A-to-SA and A-to-A. Observation 3: Antenna mismatch for out-of-band response needs to be handled adequately. Observation 4: To be able to determine relevant coupling loss values for requirement derivations a simulation campaign including near-field modelling of victim BS and aggressor BS is necessary. Observation 5: Measurement of the SA-to-SA coupling loss presented so far did not capture the beam-forming aspects. If the beamforming aspects are to be added afterwards in simulation, the phase information will be needed to properly model the coupling loss in the near-field region. Observation 6: Simulation assumptions (ISD, Number of UEs, BS height, BS array configuration, band combinations, etc.) need to be defined to conduct the simulation campaign correctly and ensure RAN4 to agree on reasonable coupling loss values when concluding the Rel-20 co-location work. Observation 7: Rel-20 co-location work should focus on the Urban Macro (UMa) scenario, as the high power and line-of-sight conditions in UMa make it the most challenging/worst-case deployment scenario. Observation 8: The coupling loss assumptions should be captured for all band combinations picked from a division in sub-ranges. Observation 9: When considering AAS BS coupling loss characteristics for deriving requirement levels, choosing the minimum coupling loss represents an outlier case which would risk driving design complexity and cost. A more suitable percentile would be in the range 95 to 99%. Observation 10: The CLTA specification lacks a link to a physical OTA parameter. Observation 11: The current CLTA orientation leads to unnecessary low SNR and high MU. Observation 12: The current CLTA specification includes unnecessary requirements on beam width and polarization match. Observation 13: 10 cm separation colocation scenario may lead to inaccurate test results and disturbed installed performance. Proposal 1: Regarding Rel-20 coupling loss analysis, RAN4 focuses on the worst-case co-location scenario, i.e., horizontal separation, and A-to-SA (Array-to-Subarray) and A-to-A (Array-to-Array). Proposal 2: For tower sharing, transmitter spurious emission and receiver out-of-band blocking requirements are essential and should be prioritised, i.e., the inter-band coupling loss evaluation should be prioritised. Proposal 3: Establish simulation methodology required for side-by-side co-location scenario considering near-field effects. Proposal 4: Work on a calibration method that can be used to remove CLTA variations from the test result. Proposal 5: Allow for CLTA rotation and study the needed adjustments in required signal levels. Proposal 6: Remove, or make optional, beamwidths from CLTA specification. Proposal 7: Remove polarization matching from the test and add description to measure in two orthogonal polarizations and sum. Proposal 8: Use the maximum of 10 cm and 2λ (FFS) as smallest co-location scenario |
| R4-2521755 | ZTE Corporation, Sanechips | Proposal 1: In Rel-20 BS RF enhancement, propose to continue the investigation on the rest of co-located cases which are not covered in Rel-19. Proposal 2: For Rel-20 NR BS to BS co-located deployment, propose to consider more practical installation assumptions instead of reusing the legacy 10cm separation distance as default assumption which is used to mimic 30dBc coupling loss. Observation 1: For OTA transmitter OFF power improvement, don’t see too much implementation burden on the existing requirement except for the measurement for conformance testing; Proposal 3: For OTA transmitter OFF power in Rel-20, we propose to consider following aspects: 3) Step 1: to identify more appropriate MCL1 between two BSs according to the legacy deployment assumption from single operator and relevant BS’s antenna configuration to derive the core conductive requirement (no intention to change the conducted core requirements according to the WID objective); then check whether TRP based OFF power measurement in the OTA chamber is feasible or not; Step 2: If TRP based OFF power measurement in step 1 is not feasible, then to define more appropriate/relevant co-location reference antenna for the OTA requirement improvement and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement; Observation 2: For OTA transmitter intermodulation requirement, if NR BS is implemented with circulator for protection of external interference signal, IMD impacts due to transmitter intermodulation requirement should be quite limited. Proposal 4: For OTA transmitter intermodulation requirement in Rel-20, we propose to consider the following aspects: 3) Step 1: Discuss more appropriate co-location reference antenna for FR1 high bands as alternative of the existing testing antenna to emulate the more realistic deployment; 4) Step 2: Discuss whether the transmitter intermodulation requirement for band n79 and n104 except for band n77/n78 could be optionally removed from regions which don’t mandate the Tx intermodulation testing; Proposal 5: For OTA transmitter spurious co-location emission in Rel-20, we propose to consider following aspects: 3) Step 1: To identify more appropriate MCL1 between co-located BS from different bands, then use the following equation to derive the corresponding TRP based transmitter spurious co-location emission requirement and check its feasibility of TRP measurement in the OTA chamber firstly; = -174dBm/Hz + 5dB + 50dB－7dB + MCL1+9dB (scaling factor) 4) Step 2: If TRP based OTA transmitter spurious co-location emission in step 1 is not feasible, to discuss more appropriate co-location reference antenna and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement; Proposal 6: For OTA Receiver out-of-band blocking in Rel-20, we propose to consider following aspects: 3) Step 1: Discuss more appropriate co-location reference antenna for FR1 high bands as alternative of the existing testing antenna to emulate the more realistic deployment; a. NOTE: Larger inter-band MCL observed in TR 37.941 will cause less REFSENS degradation compared with the legacy 30dBc MCL assumption; 4) Step 2: Discuss whether we need to align the requirement for BS type 1-H and BS type 1-O; |

## Open issues summary

### Sub-topic 2-1

Issue 2-1-1: Co-location scenario

- Proposals

  - Proposal 1: Consider to keep the 10 cm separation and side-by-side deployment to guarantee co-location requirements are met in the field. (Nokia)

  - Proposal 2: Regarding Rel-20 coupling loss analysis, RAN4 focuses on the worst-case co-location scenario, i.e., horizontal separation, and A-to-SA (Array-to-Subarray) and A-to-A (Array-to-Array). (Ericsson)

  - Proposal 3: For Rel-20 NR BS to BS co-located deployment, propose to consider more practical installation assumptions instead of reusing the legacy 10 cm separation distance as default assumption which is used to mimic 30 dBc coupling loss. (ZTE)

- Recommended WF

  - For coming meeting consider following options

Option 1: Consider common scenario for non-SBFD BS and SBFD BS based on 10 cm separation side-by-side (defined in Rel-15 for BS type 1-O).

Option 2: Consider common scenario for non-SBFD BS and SBFD BS based on separation in the range 10 to 50 cm side-by-side.

Option 3: Consider SBFD BS unique separation side-by-side. Restriction in specification will be required.

Agreement: Consider side-by-side 10 cm separation according to WID

Comments:

Nokia: If we change case the work done in Rel-19 is not valid anymore. Meaning that more work is needed. We suggest to use Rel-19 work and make it complete. Leave SBFD outside the scope. Remove option 3. We suggest option 1

Huawei: Not mix non-SBFD and SBFD together. We could have different scenarios for SBFD. Focus on general case for regular BS.

Issue 2-1-2: Coupling loss evaluation

- Proposals

  - Proposal 1: In Rel-20 BS RF enhancement, propose to continue the investigation on the rest of co-located cases which are not covered in Rel-19. (ZTE)

  - Proposal 2: Establish simulation methodology required for side-by-side co-location scenario considering near-field effects. (Ericsson)

- Recommended WF

  - Continue the investigation on the rest of co-located cases which are not covered in Rel-19.

  - To capture beamforming effects properly establish simulation methodology.

  - Continue the coupling loss investigation for cases indicated in the table below.

| Band regions | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| 2.6 GHz | A-to-SA: FFS dB A-to-A: FFS dB | A-to-SA: FFS dB A-to-A: FFS dB | A-to-SA: FFS dB A-to-A: FFS dB | A-to-SA: FFS dB A-to-A: FFS dB |
| 3.5 GHz | -- | A-to-SA: M: 35 (R4-2507633) S: 55±15 (R4-2506307) A-to-A: FFS dB | A-to-SA: FFS dB A-to-A: FFS dB | A-to-SA: S: 57±10 (R4-2506307) A-to-A: S: 54±10 (R4-2506307) |
| 4.9 GHz | -- | -- | A-to-SA: M: 51 (R4-2507445) M: 69 (R4-2507511) A-to-A: FFS dB | A-to-SA: FFS dB A-to-A: FFS dB |
| 7 GHz | -- | -- | -- | A-to-SA: M: 55 (R4-2507633) S: 70±15 (R4-2506307) A-to-A: S: 77±15 (R4-2506307) |

Comments:

Nokia: Can we add company name also to all input

Nokia: Remove 2.6 GHz entry. Not touch the other frequencies

Huawei: What is the objective with coupling loss data?

Ericsson: We see that coupling is different for different bands from Rel-19. Now we need to make table complete to consider all 4 co-location requirements.

CATT: We think this table should be complete. Maybe the table needs to be updated to include study to make coupling loss evaluation.

Huawei: No clear objective to allow this work

CATT: Based on complete table we could pick one value to be used to derive new requirement for upper RF1 bands

Nokia: We cannot have just one value. The idea was to give the opportunity to redefine levels for some sub-ranges. What do we do with conducted requirements?

CATT: We need to follow the current WID. Later, we can also consider conducted if possible.

Nokia: The testability improvement is missed out at this meeting.

### Sub-topic 2-2

Issue 2-2-1: Requirement applicability

- Proposals

  - Proposal 1: RAN4 to agree a single unified approach to define enhanced OTA co-location requirements to avoid mixed definitions and reduce test costs.

- Recommended WF

  - Targeting a single option for specifying all co-location requirements.

Issue 2-2-2: Requirement priority

- Proposals

  - Proposal 1: For tower sharing, transmitter spurious emission and receiver out-of-band blocking requirements are essential and should be prioritised, i.e., the inter-band coupling loss evaluation should be prioritised. (Ericsson)

- Recommended WF

  - Prioritize transmitter spurious emission and receiver out-of-band blocking.

### Sub-topic 2-3:

Issue 2-3-1: OTA transmitter OFF power

- Proposals

  - Proposal 1: For OTA transmitter OFF power in Rel-20, we propose to consider following aspects: (ZTE)

    - Step 1: to identify more appropriate MCL1 between two BSs according to the legacy deployment assumption from single operator and relevant BS’s antenna configuration to derive the core conductive requirement (no intention to change the conducted core requirements according to the WID objective); then check whether TRP based OFF power measurement in the OTA chamber is feasible or not;

    - Step 2: If TRP based OFF power measurement in step 1 is not feasible, then to define more appropriate/relevant co-location reference antenna for the OTA requirement improvement and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement;

- Recommended WF

  - TBA

Issue 2-3-2: OTA transmitter intermodulation

- Proposals

  - Proposal 1: For OTA transmitter intermodulation requirement in Rel-20, we propose to consider the following aspects: (ZTE)

    - Step 1: Discuss more appropriate co-location reference antenna for FR1 high bands as alternative of the existing testing antenna to emulate the more realistic deployment;

    - Step 2: Discuss whether the transmitter intermodulation requirement for band n79 and n104 except for band n77/n78 could be optionally removed from regions which don’t mandate the Tx intermodulation testing;

- Recommended WF

  - TBA

Issue 2-3-3: OTA transmitter spurious co-location emission

- Proposals

  - Proposal 1:  For OTA transmitter spurious co-location emission in Rel-20, we propose to consider following aspects: (ZTE)

    - Step 1: To identify more appropriate MCL1 between co-located BS from different bands, then use the following equation to derive the corresponding TRP based transmitter spurious co-location emission requirement and check its feasibility of TRP measurement in the OTA chamber firstly; 
 = -174dBm/Hz + 5dB + 50dB－7dB + MCL1+9dB (scaling factor)

    - Step 2: If TRP based OTA transmitter spurious co-location emission in step 1 is not feasible, to discuss more appropriate co-location reference antenna and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement;

- Recommended WF

  - TBA

Issue 2-3-4: OTA receiver out-of-band blocking

- Proposals

  - Proposal 1: For OTA Receiver out-of-band blocking in Rel-20, we propose to consider following aspects: (ZTE)

    - Step 1: Discuss more appropriate co-location reference antenna for FR1 high bands as alternative of the existing testing antenna to emulate the more realistic deployment;

NOTE: Larger inter-band MCL observed in TR 37.941 will cause less REFSENS degradation compared with the legacy 30dBc MCL assumption;

    - Step 2: Discuss whether we need to align the requirement for BS type 1-H and BS type 1-O;

- Recommended WF

  - TBA

### Sub-topic 2-4

Issue 2-4-1: Improvement of CLRA/CLTA concept

- Proposals

  - Proposal 1: Work on a calibration method that can be used to remove CLTA variations from the test result. (Ericsson)

  - Proposal 2: Allow for CLTA rotation and study the needed adjustments in required signal levels. (Ericsson)

  - Proposal 3: Remove, or make optional, beamwidths from CLTA specification. (Ericsson)

  - Proposal 4: Remove polarization matching from the test and add description to measure in two orthogonal polarizations and sum. (Ericsson)

  - Proposal 5: Use the maximum of 10 cm and 2λ (FFS) as smallest co-location scenario. (Ericsson)

- Recommended WF

  - TBA

# Topic #3: SBFD BS to SBFD BS adjacent channel coexistence

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520132 | CATT | Proposal 1: When the SBFD BSs of Operator 1 and Operator 2 have the same DL and UL sub-band configuration, the mutual interference between them only needs to be analysed in one direction. Otherwise, mutual interference needs to be analysed separately. Proposal 2: Under the assumption of 1dB sensitivity degradation, the sum of ACLR, co-location coupling loss, and the suppression from the BPF after the PA in the UL sub-band of the victim BS must be greater than 144dB. Proposal 3: For the BPF used after PA, the required suppression levels on the UL sub-band of the victimized BS are shown in table 2. Proposal 4: Under the assumption of -43dBm in-band blocking, the sum of the co-location coupling loss and the rejection provided by the victim BS's uplink RF BPF against the interfering BS's downlink sub-band, shall exceed 92dB. Proposal 5: For the RF BPF used in UL link of victim BS, the required suppression levels on the DL sub-band of the aggressor BS are shown in table 4. Proposal 6: The optimal spectrum configuration for SBFD BSs with two operators is {UD} and {DU}. In the case of three operators, the optimal spectrum configuration becomes {UD}, {DUD}, and {DU}. |
| R4-2520318 | Tejas Network Limited | Observation 1: The coexistence issue comes from downlink signals interfering with uplink reception when they are placed next to each other. Observation 2: SBFD’s split-band design helps reduce interference by creating natural guard bands, especially when both operators follow similar patterns. Observation 3: DUD configurations introduce tighter guard spacing, requiring more stringent transmitter emission control and receiver blocking performance. Proposal 1: Begin with the UD/DU configuration as the baseline, then introduce DUD/DUD to assess performance when both operators adopt identical SBFD structures. Proposal 2: Consider the UD/DUD configuration to study how coexistence behaves when operators use different SBFD setups. Proposal 3: Define updated base station transmitter and receiver requirements, including emission masks and explicit in-band blocking thresholds. Proposal 4: Adopt a 10% GS as the baseline assumption for SBFD coexistence studies. |
| R4-2520756 | CMCC | Observation 1: On band n104, there is a situation where multiple adjacent operators jointly deploy SBFD. Proposal 1: It is suggested to study the scenarios of UD (Operator 1) and UD (Operator 2). Proposal 2: The priority of the co-located scenario could be higher. Proposal 3: It is suggested to analysis the coexistence of 100 MHz SBFD and 60 MHz SBFD for band n79. Observation 2: Band n104 is likely to be allocated to multiple operators. Proposal 4: Band n104 has a high priority and the scenarios that 100 MHz SBFD and 100 MHz SBFD are adjacent could be considered. |
| R4-2521424 | Samsung | Proposal 1: RAN4 prioritize the SBFD–SBFD adjacent coexistence study on bands n104, and band n41 and n79 can be further considered if allowed. Proposal 2: Only Scenario 1 should be adopted for the current SBFD-SBFD co-existence simulation. Observation 3: For the SBFD–SBFD coexistence problem, the size of guard band should be maximized in exchange for the better suppression of bandpass filter (BPF). Observation 4: DUD–DUD slot allocation has smallest guard band size, which would lead to the most stringent coexistence requirement. Proposal 3: RAN4 SBFD–SBFD coexistence study could start with UD–DU slot allocation for the SBFD–SBFD adjacent coexistence study. Proposal 4: RAN4 can discuss how to apply the co-existence study results to other cases. Proposal 5: RAN4 to consider 10% grid shift as a starting point for SBFD–SBFD coexistence simulation. |
| R4-2521454 | Nokia | Proposal 1: Companies are encouraged to discuss whether ACLR/ACS frequency-flat or frequency-dependent models should be used for the SBFD-SBFD coexistence study. Observation 3: The coexistence study should be based on the SBFD configuration arrangement that results in the lowest frequency separation between the DL subband of the aggressor gNB and the UL subband of the victim gNB. Observation 4: A bandwidth split of 80-20 and 40-20-40 is assumed for the DU and DUD SBFD configurations, respectively. Proposal 2: It is proposed to assume DUD configuration for Operator 1 and Operator 2 as first priority SBFD configuration Proposal 3: It is proposed to use TR 38.858 antenna assumptions as a baseline. Proposal 4: RAN4 shall also discuss whether it is feasible to achieve the identified BS TX and RX RF requirements considering practical BS implementation. |
| R4-2521698 | Ericsson | Observation 1: Both co-located and non-collocated scenarios are particularly important to support real-world deployment practices such as tower sharing and reuse of existing site grids (close to each other). Observation 2: Other bands (e.g., n41, n104) can be further discussed. Observation 3: 100 MHz is the largest carrier supported by NR FR1, and this is also aligned with assumptions made in previous studies in Rel-18/19. Observation 4: To suppress inter-operator interference, one operator is required to equip both transmitter and receiver analogue filters per polarization per subarray. The two analogue filters have different passbands and roll-offs. Observation 5: To suppress self-interference, yet another subband RX analogue filter, that may be same as or different from the RX filter for suppressing inter-operator interference, is required per polarization per subarray for each operator. Observation 6: For a single SBFD subband configuration, at least two analogue filters are required per polarization per subarray for each operator. Observation 7: DUD will be the only feasible solution when there are more than two operators, although the two outermost operators could have the possibility to do UD/DU, but their TX analogue filter design will be limited by the operator(s) in the middle deploying DUD. Observation 8: In the Rel-18 simulation assumptions, the adopted array configuration was not correct and does not accurately reflect the realistic case, which should be corrected in Rel-20 simulation work and use the IMT baseline instead. Observation 9: ACLR and ACS should be removed from the Rel-20 simulation assumptions. Proposal 1: Include both 0% grid shift (co-located) and 5% grid shift (non-collocated) in the simulation scenarios. Proposal 2: Use band n79 (4 GHz) as the reference band for initial SBFD-SBFD co-existence simulation. Proposal 3: Use 100 MHz bandwidth for SBFD in the simulation study. Proposal 4: Focus on SBFD subband configuration DUD-DUD case only for Rel-20 co-existence simulations. Proposal 5: Reuse Rel-18 SBFD simulation assumptions, down select the options, and make necessary updates, e.g., grid shift, subband configuration, antenna configuration, ACLR and ACS. |
| R4-2521701 | Huawei, HiSilicon | Proposal 1: RAN4 agree on the co-existence cases as shown in Table 2.2-1. Table 2.2-1: Adjacent channel co-existence cases Case; Aggressor; Victim; Simulation frequency; Slot allocation Aggressor Victim; Priority. 1; SBFD DL; SBFD UL; 7 GHz; D U D U; High. 2; SBFD DL; SBFD UL; 4 GHz; D U D U; High. Proposal 2: it is proposed to adopt the deployment parameters as in Table 2.3-1 for coexistence study Table 2.3-1: Deployment parameters Parameter; Urban Macro. Carrier frequency; 4 GHz, 7 GHz. Layout; Single layer with 19 hexagonal cells with wrap around.. Inter-BS distance; 500 m. Grid shift; [25%]. Path-loss model; BS-to-UE: UMa + penetration loss, see TR 38.803 BS-to-BS: UMa (h_UT = 25 m), see TR 38.828. LOS probability; BS-to-UE: UMa, see TR 38.803 BS-to-BS: UMa (h_UT = 25 m), see TR 38.803.. Site-interference level; Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens). BS height; 25 m. UE height; 1.5 m. UE distribution; Uniformly UE distribution 20% indoor and 80% outdoor. O-to-I penetration loss model; 80% low-loss model and 20% high-loss model. Minimum BS-UE (2D) distance; 35 m. DL power control; No. UL power control; Yes. Handover margin; 3 dB. BS mechanical down-tilt angle; 6 degrees. Proposal 3: for band n104, it is proposed to adopt the extended AAS model and parameters as defined in TR 38.922, subclause 5.4.1 Proposal 4: it is proposed to adopt the BS RF parameters as in Table 2.3-2 for coexistence study Table 2.3-2: FR1 BS RF parameters Parameter; Urban Macro (Wide Area); Urban Macro (Wide Area). Carrier frequency; 4 GHz; 7 GHz. Transmitter power (Total conducted power); 50 dBm/100 MHz; 50 dBm/100 MHz. Antenna configuration; (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ, (q3dB,j3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi; Extended AAS model and parameters in TR 38.922, subclause 5.4.1. ACIR; 60-80 dBc; 60-80 dBc. Observation 1: more than half of the base stations are more than 50 meters away from the interfering base stations in a real C band deployment. Observation 2: Interferer-victim separation of 70 metres is adopted to derive ECC BEM requirement for non-synchronised TDD in the 3400-3800 MHz band. Proposal 5: It is proposed to reuse ECC assumption for the interferer-victim separation for SBFD-SBFD co-existence, i.e. 25% grid shift. Proposal 6: it is proposed to run ACIR simulation to derive additional unwanted emission limit. Proposal 7: it is proposed to further discuss and decide on the simulation methed to derive the additional blocking limits. - Option 1: the power level CDF of blocking from the DL adjacent channel. - Option 2: adopt ACIR to derive the additional blocking limits Observation 3: the coupling loss is band/frequency dependent, the higher frequency larger coupling loss is observed. Observation 4: The 0.1 m edge-to-edge separation is the worst-case assumption and it is not suitable for SBFD-SBFD co-existence. Proposal 8: It is proposed to further discuss on the typical co-location scenario and deployments for SBFD-SBFD co-existence. |
| R4-2521756 | ZTE Corporation, Sanechips | Observation 1: For band n41 in the commercialized regions, there are no inter-operator adjacent channel coexistence scenarios identified so far from our understanding. Observation 2: For band n79 in the commercialized regions, there are no inter-operator adjacent channel coexistence scenarios identified so far from our understanding. Observation 3: For band n104 in the commercialized regions, it is possible to have SBFD BS coexisting with SBFD BS in the adjacent channel case. Proposal 1: Prioritize SBFD BS coexisting with SBFD BS in the adjacent channel in band n104 in Rel-20; for band n79 and n41, propose to consider the SBFD BS coexisting with SBFD BS with the certain frequency gap taken into account. Proposal 2: For the SBFD BS vs SBFD coexistence study in the co-located deployment, propose to consider one typical value for the coexistence study which could be based on values from TR 37.941. Proposal 3: For BS to BS co-located deployment, propose to consider more practical installation assumptions instead of reusing the legacy 10cm separation distance as default assumption. Proposal 4: For SBFD BS coexisting with SBFD BS, propose to use the simulation assumptions in Table E.2.1-1 and E.2.3-1 in this contribution and other relevant simulation assumptions in TR 38.858 to conduct the coexistence study. Observation 4: for aggressor BS and victim BS with 0% grid shift at 2.6GHz, 4GHz and 7GHz, the additional ACIR offset 30dB on top of baseline ACIR 42.6dB is needed which basically means that analog filter attenuation is needed. Observation 5: for aggressor BS and victim BS with 10% to 50% grid shift, the additional ACIR offset ranging from 30 to 40dB on top of baseline ACIR 42.6dB is needed. For different frequency, the additional ACIR offset varies. Proposal 5: Propose to consider the analog filter to provide the additional ACIR protection e.g. 30-40dBc. Proposal 6: For the potential tightening unwanted emission and blocking requirement for SBFD BS in Rel-20, it is proposed to consider the victim’s system DL sub-band as guard band to minimize the victim system’s performance degradation; Proposal 7: For RF requirements of SBFD BS in Rel-20, propose to consider the following factors: - Potential more tightening emission and blocking requirements from coexistence study - DL sub-band size of the victim system; - Coupling loss between the aggressor and victim system for co-site case - Analog filter performance of aggressor’s transmitter - Analog filter performance of victim’s receiver; Proposal 8: propose to consider the work procedure in RAN4 for Rel-20 SBFD vs SBFD coexistence study. Figure 2.5-1. The illustration of working procedure for SBFD study in Rel-20 |
| R4-2521881 | ISSDU, NYCU | Proposal 1: RAN4 to consider adding a configuration-dependent self-interference model (based on SBFD duplex arrangements defined in 5G-Advanced, i.e. DU, UD, DUD) to support more flexible SBFD deployments beyond 5G. Proposal 2: Include multi-operator SBFD coexistence as a key Release-20 uplink study scenario, specifically where adjacent uplink sub-bands are operated by different operators. Proposal 3: RAN4 to evaluate Release-20 SBFD coexistence using existing NR RF baselines but with separate assessments for each SBFD configuration. Proposal 4: RAN4 can discuss heterogeneous SBFD deployments in the Release-20 coexistence simulations by studying cases where different sectors within a site use mixed SBFD configurations. |
| R4-2521897 | Qualcomm Germany | Observation 1: Work in Rel-18 and Rel-19 considered TDD to SBFD adjacent channel coexistence, which is in general limited by legacy TDD BS requirements, especially regarding ACIR enhancements. Observation 2: RAN4 considered a flat leakage and selectivity model in TR 38.858, which implies that whether DUD or DU configuration is assumed, the leakage from the interfering gNB is the same. Proposal 1: RAN4 to discuss how to account for different ACLR/ACS steps from TR 36.942 in the Rel20 adjacent channel coexistence SBFD-SBFD work. Proposal 2: RAN4 to identify which Tx/Rx requirements will be studied via adjacent channel coexistence for the Rel-20 SBFD-to-SBFD coexistence. Observation 3: It is expected to start quantifying enhanced ACLR and ACS for SBFD-capable gNB as a first outcome of the Rel-20 SBFD-SBFD coexistence work. Proposal 3: RAN4 to adopt the same performance metric in TR 38.858 Section 11 to determine the level of ACIR enhancement needed for the SBFD-capable gNB. |

## Open issues summary

Issue 3-1: Sub-band configuration

- Proposals

  - Option 1: UD (Operator 1) and DU (Operator 2) (CATT, Tejas, Huawei, Samsung)

  - Option 2: UD (Operator 1) and UD (Operator 2) (CMCC)

  - Option 3: DUD (Operator 1) and DUD (Operator 2) (Tejas, Ericsson, Nokia, ISSDU)

  - Option 4: UD (Operator 1) and DUD (Operator 2) (Tejas)

  - Option 5: UD (Operator 1) and DUD (Operator 2) and DU (Operator 3) (CATT)

- Recommended WF

  - For co-existence evaluation adopt sub-band configuration DUD (Operator 1) and DUD (Operator 2)

Agreement: For co-existence evaluation consider Option 1 and Option 3. Encourage companies to indicate ACLR model assumption.

Comments:

Qualcomm: Do we keep the assumption of flat ACLR?

Samsung: Keep flat ACLR assumption during the co-existence evaluation and consider stepped ACLR when requirement is derived.

Ericsson: In Rel-20 we need to do two types of simulations. For in-band blocking we don’t need ACLR model. For emission model we are looking for ACLR, so no model is used.

Qualcomm: If we use Rel-18 assumption option 1 and option 3 will give the same results. Maybe capture in WF how ACLR is modelled.

CATT: The model o ACLR is not the problem. We derive with fixed performance metric to derive ACLR

Nokia: If we go with the stepped ACLR model, how do we agree on how the requirement is defined. It’s better to use the flat model now for the coexistence evaluation.

Ericsson: The step-wised model is more for UE. But for BS a flat model is more relevant considering DPD effects.

Huawei: Let’s keep both options.

Nokia: Keep both options.

Issue 3-2: Carrier bandwidth

- Proposals

  - Option 1: 100 MHz for band n104 and 60 MHz SBFD for band n79 (CMCC)

  - Option 2: 100 MHz (Ericsson, Huawei, Nokia)

- Recommended WF

  - For co-existence evaluation adopt 100 MHz carrier bandwidth.

Agreement: For co-existence evaluation adopt 100 MHz carrier bandwidth.

Issue 3-3: Grid shift

Issue 3-3a: co-existence

- Proposals

  - Option 1: 10% grid shift (Tejas, Samsung, ZTE)

  - Option 2: 25% grid shift (Huawei)

  - Option 3:  5% grid shift (Ericsson)

  - Option 4: 20% grid shift (ZTE)

  - Option 5: 50% grid shift (ZTE)

Comments:

Nokia: Better coexistence is expected, which would require small grid-shift number.

ZTE: Option 1, Option 4 and 5

Ericsson: Whole purpose is to remove requirement restriction of 288m. If we pick too large grid-shift we need a new restriction. We need to collect operator option on this issue.

Nokia: We support option 1 with 10 % grid-shift. If high value is assumed we need notes with restrictions in the specification.

Huawei: Option 1 and Option 2

Issue 3-3a: co-location scenarios

- Proposals

  - Proposal 1: The co-located scenario should be considered (CMCC, Ericsson, ZTE).

    - Option 1a: 0% grid shift (Ericsson)

  - Proposal 2: It is proposed to further discuss on the typical co-location scenario (Huawei)

- Recommended WF

  - TBA

Issue 3-4: RF assumptions

- Proposals

  - Option 1: Reuse Rel-18 SBFD simulation assumptions, down select the options, and make necessary updates, e.g., grid shift, subband configuration, antenna configuration, ACLR and ACS. (Ericsson)

  - Option 2: Introduce a configuration-dependent self-interference model (define minimum ASBIR for DU/UD/DUD) to refine SBFD coexistence RF assumptions. (ISSDU)

- Recommended WF

  - TBA

Issue 3-5: n79 simulation assumptions

- Proposals

  - Option 1: For band n79, it is proposed to adopt AAS BS parameters defined in TR 38.858 (8x8 single element structure) (Huawei)

  - Option 2: For band n79, it is proposed to adopt AAS BS parameters defined in TR 38.922 (sub-array structure) (Ericsson)

- Recommended WF

  - TBA

Issue 3-6: n104 simulation assumptions

- Proposals

  - Option 1: For band n104, it is proposed to adopt the extended AAS model and parameters as defined in TR 38.922, subclause 5.4.1 (Huawei)

  - Option 2:  Use TR 38.858 antenna assumptions as a baseline for 6GHz. (Nokia)

- Recommended WF

  - TBA

Issue 3-7: Simulation scenario

- Proposals

  - Option 1: Only Scenario 1 should be adopted for the current SBFD-SBFD co-existence simulation. (Samsung, Huawei)

  - Option 2: Consider a heterogeneous SBFD deployment scenario where different sectors (or operators) at a site use different SBFD configurations. (ISSDU)

- Recommended WF

  - For the co-existence evaluation consider Scenario 1.

Issue 3-8: Modelling aspects & simulation metric

- Proposals

  - Proposal 1: It is proposed to run ACIR simulation to derive additional unwanted emission limit. (Huawei)

  - Proposal 2: It is proposed to further discuss and decide on the simulation method to derive the additional blocking limits. (Huawei)

    - Option 1: the power level CDF of blocking from the DL adjacent channel.

    - Option 2: adopt ACIR to derive the additional blocking limits

  - Proposal 3: RAN4 to discuss how to account for different ACLR/ACS steps from TR 36.942 in the Rel-20 adjacent channel coexistence SBFD-SBFD work. (Qualcomm)

  - Proposal 4: Companies are encouraged to discuss whether ACLR/ACS frequency-flat or frequency-dependent models should be used for the SBFD-SBFD coexistence study. (Nokia)

- Recommended WF

  - TBA