3GPP TSG-RAN WG4 Meeting # 117	 R4-2523024

# Dallas,  US ,  November 17 - 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 19.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for 38.101-1: Adding 3MHz for e_RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Sony, Nokia, Anterix |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph6-Core |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 3MHz CBW support is added for (e) RedCap in WID RP-252955 |  |  |  |  |  |  |  |  |
| Summary of change: |  | adding the 3MHz CBW for (e) RedCap UE according to WF in RAN4#116bis |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3MHz CBW not supported for (e) RedCap UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3I, 7.3I.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/* start change */

## 5.3I	Channel bandwidth for (e)RedCap

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 with maximum channel bandwidth of 20MHz. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths up to 20 MHz. When UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18 the requirements in this specification apply with maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths.

/* next change */

### 7.3I.2	Reference sensitivity power level for RedCap

For a RedCap UE equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a and Table 7.3.2-1b for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 2 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For a power class 2 RedCap UE equipped with 2 Rx antenna ports operating in FD-FDD mode, certain degradation of the reference sensitivity in Table 7.3.2-1a is allowed. The maximum amount of degradation is specified in Table 7.3.2-1c.

For a RedCap UE equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2-1 for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 1 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For a power class 2 RedCap UE equipped with 2 Rx or 1 Rx antenna port(s) operating in HD-FDD mode or with TDD band, the reference sensitivity is same as the power class 3 RedCap UE.

For a power class 2 RedCap UE equipped with 1 Rx antenna ports operating in FD-FDD mode, the reference sensitivity in Table 7.3I.2-1b shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3.

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2.5 |
| FDD band | 3,5 | 2.5 |
| FDD band | 10, 15, 20 | 3.0 |

Table 7.3I.2-1b: The reference sensitivity for PC2 RedCap UE with 1 Rx antenna port operating in FD-FDD mode

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -97.5 | -93.8 | -92 | -90.8 |
|  | 30 |  | -94.1 | -92.1 | -91 |
|  | 60 |  | -94.5 | -92.4 | -91.2 |
| n3 | 15 | -93.6 | -90.3 | -88.5 | -87.3 |
|  | 30 |  | -90.7 | -88.7 | -87.6 |
|  | 60 |  | -91.1 | -89.0 | -87.8 |

For a RedCap UE equipped with 2 Rx antenna ports operating in HD-FDD mode, reference sensitivity for 2Rx antenna ports in Table 7.3I.2-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-4.

Table 7.3I.2-2: HD-FDD RedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band |  | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 |  | 15 |  | -100.0 | -96.8 | -95.0 | -93.7 |
|  |  | 30 |  |  | -97.2 | -95.2 | -93.9 |
|  |  | 60 |  |  | -97.5 | -95.4 | -94.2 |
| n2 |  | 15 |  | -98.8 | -95.6 | -93.8 | -92.5 |
|  |  | 30 |  |  | -96.0 | -94.0 | -92.7 |
|  |  | 60 |  |  | -96.3 | -94.2 | -93.0 |
| n3 |  | 15 |  | -97.8 | -94.6 | -92.8 | -91.5 |
|  |  | 30 |  |  | -95.0 | -93.0 | -91.7 |
|  |  | 60 |  |  | -95.3 | -93.2 | -92.0 |
| n5 |  | 15 |  | -98.8 | -95.6 | -93.8 | -92.5 |
|  |  | 30 |  |  | -96.0 | -94.0 | -92.7 |
| n7 |  | 15 |  | -98.8 | -95.6 | -93.8 | -92.5 |
|  |  | 30 |  |  | -96.0 | -94.0 | -92.7 |
|  |  | 60 |  |  | -96.3 | -94.2 | -93.0 |
| n8 |  | 15 |  | -97.8 | -94.6 | -92.8 | -91.5 |
|  |  | 30 |  |  | -95.0 | -93.0 | -91.7 |
| n12 |  | 15 |  | -97.8 | -94.6 | -92.8 |  |
|  |  | 30 |  |  | -95.0 | -93.0 |  |
| n13 |  | 15 |  | -97.8 | -94.6 |  |  |
|  |  | 30 |  |  | -95.0 |  |  |
| n14 |  | 15 |  | -97.8 | -94.6 |  |  |
|  |  | 30 |  |  | -95.0 |  |  |
| n18 |  | 15 |  | -100.0 | -96.8 | -95.0 |  |
|  |  | 30 |  |  | -97.2 | -95.2 |  |
| n20 |  | 15 |  | -97.8 | -94.6 | -92.8 | -91.5 |
|  |  | 30 |  |  | -95.0 | -93.0 | -91.7 |
| n24 |  | 15 |  | -100.0 | -96.8 |  |  |
|  |  | 30 |  |  | -97.2 |  |  |
|  |  | 60 |  |  | -97.5 |  |  |
| n25 |  | 15 |  | -97.3 | -94.1 | -92.3 | -91.0 |
|  |  | 30 |  |  | -94.5 | -92.5 | -91.2 |
|  |  | 60 |  |  | -94.8 | -92.7 | -91.5 |
| n26 |  | 15 | -100.5 | -98.3 | -95.1 | -93.3 | -92.0 |
|  |  | 30 |  |  | -95.5 | -93.5 | -92.2 |
| n28 |  | 15 | -101.5 | -99.3 | -96.1 | -94.3 | -93.0 |
|  |  | 30 |  |  | -96.5 | -94.5 | -93.2 |
| n30 |  | 15 |  | -99.5 | -96.3 |  |  |
|  |  | 30 |  |  | -96.7 |  |  |
| n65 |  | 15 |  | -100.0 | -96.8 | -95.0 | -93.7 |
|  |  | 30 |  |  | -97.2 | -95.2 | -93.9 |
|  |  | 60 |  |  | -97.5 | -95.4 | -94.2 |
| n66 |  | 15 |  | -100.0 | -96.8 | -95.0 | -93.7 |
|  |  | 30 |  |  | -97.2 | -95.2 | -93.9 |
|  |  | 60 |  |  | -97.5 | -95.4 | -94.2 |
| n70 |  | 15 |  | -100.0 | -96.8 | -95.0 | -93.7 |
|  |  | 30 |  |  | -97.2 | -95.2 | -93.9 |
|  |  | 60 |  |  | -97.5 | -95.4 | -94.2 |
| n71 |  | 15 |  | -98.0 | -94.8 | -93.0 | -91.7 |
|  |  | 30 |  |  | -95.2 | -93.2 | -91.9 |
| n74 |  | 15 |  | -100.0 | -96.8 | -95.0 | -93.7 |
|  |  | 30 |  |  | -97.2 | -95.2 | -93.9 |
|  |  | 60 |  |  | -97.5 | -95.4 | -94.2 |
| n85 |  | 15 | -100.0 | -97.8 | -94.6 | -92.8 |  |
|  |  | 30 |  |  | -95.0 | -93.0 |  |
| n91 |  | 15 |  | -100.0 |  |  |  |
| n92 |  | 15 |  | -100.0 | -96.8 | -95.0 | -93.7 |
|  |  | 30 |  |  | -97.2 | -95.2 | -93.9 |
| n93 |  | 15 |  | -100.0 |  |  |  |
| n94 |  | 15 |  | -100.0 | -96.8 | -95.0 | -93.7 |
|  |  | 30 |  |  | -97.2 | -95.2 | -93.9 |
| n100 |  | 15 | -102.2 |  |  |  |  |
| n105 |  | 15 |  | -981 | -94.8 | -93 | -91.7 |
|  |  | 30 |  |  | -95.2 | -93.2 | -91.9 |
| n106 |  | 15 | -100.0 |  |  |  |  |
|  | NOTE 1: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |  |  |  |

For a RedCap UE equipped with 1 Rx antenna ports and operating in HD-FDD mode, reference sensitivity for 1Rx antenna ports in Table 7.3I.2-3 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-4.

Table 7.3I.2-3: HD-FDD RedCap UE with 1 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band |  | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 |  | 15 |  | -97.5 | -94.3 | -92.5 | -91.2 |
|  |  | 30 |  |  | -94.7 | -92.7 | -91.4 |
|  |  | 60 |  |  | -95.0 | -92.9 | -91.7 |
| n2 |  | 15 |  | -96.3 | -93.1 | -91.3 | -90.0 |
|  |  | 30 |  |  | -93.5 | -91.5 | -90.2 |
|  |  | 60 |  |  | -93.8 | -91.7 | -90.5 |
| n3 |  | 15 |  | -95.3 | -92.1 | -90.3 | -89.0 |
|  |  | 30 |  |  | -92.5 | -90.5 | -89.2 |
|  |  | 60 |  |  | -92.8 | -90.7 | -89.5 |
| n5 |  | 15 |  | -96.3 | -93.1 | -91.3 | -90.0 |
|  |  | 30 |  |  | -93.5 | -91.5 | -90.2 |
| n7 |  | 15 |  | -96.3 | -93.1 | -91.3 | -90.0 |
|  |  | 30 |  |  | -93.5 | -91.5 | -90.2 |
|  |  | 60 |  |  | -93.8 | -91.7 | -90.5 |
| n8 |  | 15 |  | -95.3 | -92.1 | -90.3 | -89.0 |
|  |  | 30 |  |  | -92.5 | -90.5 | -89.2 |
| n12 |  | 15 |  | -95.3 | -92.1 | -90.3 |  |
|  |  | 30 |  |  | -92.5 | -90.5 |  |
| n13 |  | 15 |  | -95.3 | -92.1 |  |  |
|  |  | 30 |  |  | -92.5 |  |  |
| n14 |  | 15 |  | -95.3 | -92.1 |  |  |
|  |  | 30 |  |  | -92.5 |  |  |
| n18 |  | 15 |  | -97.5 | -94.3 | -92.5 |  |
|  |  | 30 |  |  | -94.7 | -92.7 |  |
| n20 |  | 15 |  | -95.3 | -92.1 | -90.3 | -89.0 |
|  |  | 30 |  |  | -92.5 | -90.5 | -89.2 |
| n24 |  | 15 |  | -97.5 | -94.3 |  |  |
|  |  | 30 |  |  | -94.7 |  |  |
|  |  | 60 |  |  | -95.0 |  |  |
| n25 |  | 15 |  | -94.8 | -91.6 | -89.8 | -88.5 |
|  |  | 30 |  |  | -92.0 | -90.0 | -88.7 |
|  |  | 60 |  |  | -92.3 | -90.2 | -89.0 |
| n26 |  | 15 | -98.0 | -95.8 | -92.6 | -90.8 | -89.5 |
|  |  | 30 |  |  | -93.0 | -91.0 | -89.7 |
| n28 |  | 15 | -99.0 | -96.8 | -93.6 | -91.8 | -90.5 |
|  |  | 30 |  |  | -94.0 | -92.0 | -90.7 |
| n30 |  | 15 |  | -97.0 | -93.8 |  |  |
|  |  | 30 |  |  | -94.2 |  |  |
| n65 |  | 15 |  | -97.5 | -94.3 | -92.5 | -91.2 |
|  |  | 30 |  |  | -94.7 | -92.7 | -91.4 |
|  |  | 60 |  |  | -95.0 | -92.9 | -91.7 |
| n66 |  | 15 |  | -97.5 | -94.3 | -92.5 | -91.2 |
|  |  | 30 |  |  | -94.7 | -92.7 | -91.4 |
|  |  | 60 |  |  | -95.0 | -92.9 | -91.7 |
| n70 |  | 15 |  | -97.5 | -94.3 | -92.5 | -91.2 |
|  |  | 30 |  |  | -94.7 | -92.7 | -91.4 |
|  |  | 60 |  |  | -95.0 | -92.9 | -91.7 |
| n71 |  | 15 |  | -95.5 | -92.3 | -90.5 | -89.2 |
|  |  | 30 |  |  | -92.7 | -90.7 | -89.4 |
| n74 |  | 15 |  | -97.5 | -94.3 | -92.5 | -91.2 |
|  |  | 30 |  |  | -94.7 | -92.7 | -91.4 |
|  |  | 60 |  |  | -95.0 | -92.9 | -91.7 |
| n85 |  | 15 | -97.5 | -95.3 | -92.1 | -90.3 |  |
|  |  | 30 |  |  | -92.5 | -90.5 |  |
| n91 |  | 15 |  | -97.5 |  |  |  |
| n92 |  | 15 |  | -97.5 | -94.3 | -92.5 | -91.2 |
|  |  | 30 |  |  | -94.7 | -92.7 | -91.4 |
| n93 |  | 15 |  | -97.5 |  |  |  |
| n94 |  | 15 |  | -97.5 | -94.3 | -92.5 | -91.2 |
|  |  | 30 |  |  | -94.7 | -92.7 | -91.4 |
| n100 |  |  |  | 15 |  |  | -99.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| n105 |  | 15 |  | -95.51 | -92.3 | -90.5 | -89.2 |
|  |  | 30 |  |  | -92.7 | -90.7 | -89.4 |
| n106 |  |  | 15 |  |  | -97.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |  |  |  |

Table 7.3I.2-4: Uplink configuration for HD-FDD reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band |  | SCS kHz | 3MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
|  |  | 60 |  |  | 10 | 18 | 24 |
| n2 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
|  |  | 60 |  |  | 10 | 18 | 24 |
| n3 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
|  |  | 60 |  |  | 10 | 18 | 24 |
| n5 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
| n7 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
|  |  | 60 |  |  | 10 | 18 | 24 |
| n8 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
| n12 |  | 15 |  | 25 | 50 | 75 |  |
|  |  | 30 |  |  | 24 | 36 |  |
| n13 |  | 15 |  | 25 | 50 |  |  |
|  |  | 30 |  |  | 24 |  |  |
| n14 |  | 15 |  | 25 | 50 |  |  |
|  |  | 30 |  |  | 24 |  |  |
| n18 |  | 15 |  | 25 | 50 | 75 |  |
|  |  | 30 |  |  | 24 | 36 |  |
| n20 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
| n24 |  | 15 |  | 25 | 50 |  |  |
|  |  | 30 |  |  | 24 |  |  |
|  |  | 60 |  |  | 10 |  |  |
| n25 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
|  |  | 60 |  |  | 10 | 18 | 24 |
| n26 |  | 15 | 15 | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
| n28 |  | 15 | 15 | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
| n30 |  | 15 |  | 25 | 50 |  |  |
|  |  | 30 |  |  | 24 |  |  |
| n65 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
|  |  | 60 |  |  | 10 | 18 | 24 |
| n66 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
|  |  | 60 |  |  | 10 | 18 | 24 |
| n70 |  | 15 |  | 25 | 50 | 75 | NOTE 1 |
|  |  | 30 |  |  | 24 | 36 | NOTE 1 |
|  |  | 60 |  |  | 10 | 18 | NOTE 1 |
| n71 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
| n74 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
|  |  | 60 |  |  | 10 | 18 | 24 |
| n85 |  | 15 | 15 | 25 | 50 | 75 |  |
|  |  | 30 |  |  | 24 | 36 |  |
| n91 |  | 15 |  | 25 |  |  |  |
| n92 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
| n93 |  | 15 |  | 25 |  |  |  |
| n94 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
| n100 |  |  |  | 15 |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
| n105 |  | 15 |  | 25 | 50 | 75 | 100 |
|  |  | 30 |  |  | 24 | 36 | 50 |
| n106 |  |  | 15 |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. |  |  |  |  |  |  |

/* next change */

### 7.3I.3	Reference sensitivity power level for eRedCap

For UE supporting IE supportOfERedCap-r18 and IE eRedCapNotReducedBB-BW-r18, the REFSENS requirements for RedCap UE in clause 7.3I.2 apply.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, the reference sensitivity requirements for 5 MHz channel bandwidth with 15 kHz SCS defined in clause 7.3I.2 apply for 5, 10, 15 and 20 MHz channel bandwidths with 15 kHz SCS and the reference sensitivity requirements for 3 MHz channel bandwidth with 15 kHz SCS defined in clause 7.3I.2 apply for 3MHz channel banwidth. In case the reference sensitivity requirements for 5 MHz channel bandwidth are not defined, the reference sensitivity requirements for 10 MHz channel bandwidth apply with the reference sensitivity level reduced by 3.2 dB, and the UL configuration shall be less than or equal to minimum between RB number specified in Table 7.3I.2-4  and 25 RBs for HD-FDD operation, and minimum between RB number specified in Table 7.3.2-3 and 25RBs otherwise. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths. Both Tx RBs in UL configuration and Rx RBs in FRC for 10, 15 and 20MHz channel bandwidth, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 25 RBs for 15 kHz SCS.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, for 30 kHz SCS, the reference sensitivity requirements defined for 10 MHz channel bandwidth in clause 7.3I.2 apply with reference sensitivity level reduced by 3.0 dB and the UL configuration shall be less than or equal to minimum between RB number specified in Table 7.3I.2-4 and 12 RBs for HD-FDD operation, and minimum between RB number specified in Table 7.3.2-3 and 12RBs otherwise. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths with 30 kHz SCS. Both Tx RBs in UL configuration and Rx RBs in FRC , when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 12 RBs for 30 kHz SCS.

NOTE:	It is not necessary to repeat verification when same requirement applies for multiple channel bandwidths.

/* End change */