3GPP TSG-RAN4 Meeting #117	R4-2523057

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7404 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (IoT_NTN_TDD-Perf) CR on removing support for segmented precompensation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_TDD-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 has received communication from RAN1 (R1-2508064) indicating updated agreements related to segmented pre-compensation that requires RAN4 to update its requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update requiremetns to be in line with R1-2508064 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will be misaligned with TS 36.213 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.20B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2521735 |  |  |  |  |  |  |  |  |

==============First change==============

## 7.20B	UE transmit timing for NB-IoT for frame structure type 1 for NTN-TDD

### 7.20B.1	Introduction

The Category NB1 UE shall have capability to follow the frame timing change of the connected eNode B. The uplink frame transmission takes place NTA+NTA,offset+NTA,adjcommon+NTA,adjUE×Ts before the reception of the first detected path (in time) of the corresponding downlink frame from the reference NB-IoT cell.

UE shall use the serving NB-IoT cell as the reference cell for deriving the UE transmit timing. UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are defined in the following requirements.

### 7.20B.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.20B.2-1. This requirement applies when it is the first transmission in each set of consecutive uplink subframes; or the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA+NTA,offset+NTA,adjcommon+NTA,adjUE×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA for NPRACH is defined as 0. NTA+NTA,offset+NTA,adjcommon+NTA,adjUE (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.22B was applied. NTA for other channels is not changed until next timing advance is received.

Table 7.20B.2-1: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97*TS |
| Note 1: TS is the basic timing unit defined in TS 36.211 |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for NPUSCH the UE shall, when no repetitions are configured on the uplink or the repetition period is R=1, be capable of changing the transmission timing according to the received downlink frame of the serving NB-IoT cell except when the timing advance in clause 7.22B is applied such that the UE transmission timing error shall be less than or equal to ±Te, where the timing error limit value Te is specified in Table 7.20B.2-1.

When no repetition period is configured, or the configured repetition period is R=1, all adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, in one adjustment shall be 58.33*TS seconds.

2)	The minimum aggregate adjustment rate, apart from a change ofNTA,adjUE due to satellite position update and NTA,adjcommonbetween the previous transmission and the current transmission, shall be 7*TS per 1second.

3)	The maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, shall be 58.33*TS per 200ms.

when a repetition is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the first transmission in each set of consecutive uplink subframe and pre-compensation gaps are not allowed..

==============End of change==============